3GPP TSG-SA WG1 Meeting #100	S1-223248

Toulouse, France, 14 – 18 Nov. 2022

Source:	Lenovo

pCR Title:	modes of 5GS sensing service

Draft Spec:	3GPP TR 22.837

Agenda item:	7.1

Document for:	Approval

Contact:	Omid Taghizadeh (smotlagh@lenovo.com)

Abstract: This contribution proposes to add possible modes of 5GS sensing service to the informative Annex.

# 1	Proposal

It is proposed to agree the following changes to 3GPP TR 22.837.

FIRST CHANGE

###### Annex <Z> (informative):
Modes of 5GS sensing service

Network-based sensing: this includes a mode of 5GS sensing service, where the 5GS sensing service includes exposing the obtained sensing results by the 5G network to the requesting entity.

UE-initiated sensing: this includes a mode of 5GS sensing service, where the 5GS sensing service is triggered/initiated by a request from a UE device.

UE-based sensing: this includes a mode of 5GS sensing service, where the sensing results is computed by the UE requesting/triggering the 5GS sensing service. The 5GS sensing service facilitates the requesting UE to obtain the needed sensing measurements data based on which the requesting UE calculates the sensing results.

UE-assisted sensing: this includes a mode of 5GS sensing service, where the sensing measurement data and/or sensing results are generated, at least in part, based on the assistance of a UE device.

Network-assisted sensing: this includes a mode of a UE-based 5GS sensing service, where the sensing measurement data and/or sensing results are generated, at least in part, based on the assistance of the 5G network.

Network-initiated sensing: this includes a mode of 5GS sensing service, where the 5GS sensing service is initiated/triggered by the 5G network.

The Table A-1 shows the scenarios which are envisioned based on the abovementioned different modes of 5GS sensing service.

Table A-1: Classification of sensing scenarios

| Scenario | Sensing service initiator | Calc. of sensing results | Sensing Meas. Process | 5GS delivered service | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Network | Network | Sensing Results | UE-initiated, Network-based sensing |
| 2 | UE | Network | Network and UE | Sensing Results | UE-initiated, UE-assisted, Network-based sensing |
| 3 | UE | UE | UE | Sensing Meas. data | UE-based sensing, e.g., UE as a monostatic radar |
| 4 | UE | UE | Network and UE | Sensing Meas. data | UE-based sensing, Network-assisted |
| 5 | Network | Network | Network | Sensing Results | Network-initiated, Network-based sensing |
| 6 | Network | Network | Network and UE | Sensing Results | Network- initiated, UE-assisted, Network-based sensing |

END OF CHANGE