3GPP TSG SA WG 1 Meeting #100		 S1-223291

Toulouse, France, 14 – 18 November 2022

Title:	Reply LS on SENSE for home PLMN and disaster roaming PLMN

Response to:	LS C1-226124 on SENSE for home PLMN and disaster roaming PLMN from CT1

Release:	Rel-18

Work Item:	SENSE

Source:	SA1

To:	CT1

Cc:

Contact person:	Kurt Bischinger

kurt.bischinger@magenta.at

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CRs #0345 and #0342 to TS22.011

# 1	Overall description

SA1 thanks CT1 for their LS C1-226124 on SENSE for home PLMN and disaster roaming PLMN. SA1 would like to provide the following answers:

Question 1: if a UE supports SENSE, the UE can register to a VPLMN (non-HPLMN/EHPLMN) depending on signal quality of this VPLMN compared to HPLMN/EHPLMN at the time of PLMN selection. Up until now when both HPLMN/EHPLMN and VPLMN are available, the UE selects the former not the latter. Can SA1 confirm the UE behavior and usage of the threshold in this case?

Answer 1: SA1 has agreed the attached CR#0345 to 22.011 for clarification.

Question 2: CT1 has identified conflicting requirements for the combination of SENSE and MINT in the Stage-1 specifications. If a UE for which SENSE and MINT are both supported and enabled, detects a disaster condition and during the subsequent PLMN selection(s) finds an allowable PLMN with a signal quality below the SENSE threshold and a forbidden PLMN offering disaster roaming for which the conditions for disaster roaming according to 23.122 are fulfilled with a signal level above the SENSE threshold, shall the UE prioritise and select the forbidden PLMN, even if an allowable PLMN is available? Current definitions in TS 22.011 seems to imply that the forbidden PLMN is chosen in the given example, while TS 22.261 requirement "The 3GPP system shall be able to provide means to enable a UE to access PLMNs in a forbidden PLMN list if a Disaster condition applies and no other PLMN is available except for PLMNs in the forbidden PLMN list." seems to imply that the allowable PLMN is chosen.

Answer 2: SA1 has agreed the attached CR#0342 to 22.011 for clarification.

# 2	Actions

To CT1:

ACTION: 	SA1 kindly asks CT1 to take the above answers and the attached CR into account.

# 3	Dates of next TSG SA WG 1 meetings

SA1#101	13-17 Feb 2023			Wroclav, Poland (fallback: e-meeting)

SA1#102	15-19 May 2023			TBD