3GPP TSG-SA1 Meeting #100	S1-223322

Toulouse, France, 14 – 18 November 2022

Title:	Pseudo-CR on Ambient power and energy storage for Ambient IoT

Agenda Item:	7.3

Source:	OPPO

Contact:	Weijie Xu xuweijie@oppo.com;

Zhisong Zuo zuozhisong@oppo.com

Yang Xu xuyang@oppo.com

Abstract: In the combination contribution S1-223162, ambient power and energy storage is discussed. This document provides a Text Proposal on aspects such as typical ambient power sources suitable for ambient IoT, brief introduction of the technical principle on how energy is harvested from the ambient power, the amount of power can be available and the potential applicable use cases for each kind of ambient power source.

# Proposal

It is proposed to add the following reference and use case description in TR 22.840.

************* Start of Changes ***************

References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	International Energy Agency (IEA), "World Energy Outlook 2020". Available online: https://www.iea.org/reports/world-energy-outlook-2020

[3] 	Motlagh, N.H.; Mohammadrezaei, M.; Hunt, J.; Zakeri, B. "Internet of Things (IoT) and the Energy Sector." Energies 2020, 13, 494.

[4] 	Pereira F., Correia R., Carvalho N.B. "Passive Sensors for Long Duration Internet of Things Networks." Sensors 2017;17:2268. doi: 10.3390/s17102268.

[5] 	https://www.gs1.org/standards/rfid.

[6]	3GPP TS 22.368: "Service requirements for Machine-Type Communications (MTC)".

[7]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.011: “Service accessibility”.

[10]	5G for smart manufacturing: https://www.gsma.com/iot/wp-content/uploads/2020/04/2020-04_GSMA_SmartManufacturing_Insights_On_How_5G_IoT_Can_Transform_Industry.pdf

[11] 	https://forcetechnology.com/en/articles/batteryless-electronics-energy-harvesting

[12] 	R. Ding and B. Xing, "Comparative Research on the Way of Energy Harvesting of the Wireless Sensor Network Nodes," 2013 6th International Conference on Intelligent Networks and Intelligent Systems (ICINIS), 2013

[13] 	P. D. Hung et al., "A Self-Powered Wireless Gas Sensor Node Based on Photovoltaic Energy Harvesting," 2021 Symposium on VLSI Circuits, 2021.

[14] 	GS1: "EPC Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard (Release 2.1) "

[15]           EPC Tag Data standards version 1.1 Rev.1.24

[X1           ]Noghabaei S M, Radin R L, Savaria Y, et al. A high-efficiency ultra-low-power CMOS rectifier for RF energy harvesting applications[C]//2018 IEEE International Symposium on Circuits and Systems (ISCAS). IEEE, 2018: 1-4

[X2]          Wu Z, Zhao Y, Sun Y, et al. A Self-Bias Rectifier with 27.6% PCE at-30dBm for RF Energy Harvesting[C]//2021 IEEE International Symposium on Circuits and Systems (ISCAS). IEEE, 2021: 1-5.

[X3]          Valenta C R, Durgin G D. Harvesting wireless power: Survey of energy-harvester conversion efficiency in far-field, wireless power transfer systems[J]. IEEE Mi crowave Magazine, 2014, 15(4): 108-120.

[X4]          Kim S, Vyas R, Bito J, et al. Ambient RF energy-harvesting technologies for self-sustainable standalone wireless sensor platforms[J]. Proceedings of the IEEE, 2014, 102(11): 1649-1666.

[X5]          Green M, Dunlop E, Hohl‐Ebinger J, et al. Solar cell efficiency tables (version 57)[J]. Progress in photovoltaics: research and applications, 2021, 29(1): 3-15.

[X6]          Mishu M K, Rokonuzzaman M, Pasupuleti J, et al. An adaptive TE-PV hybrid energy harvesting system for self-powered iot sensor applications[J]. Sensors, 2021, 21(8): 2604.

[X7]          Prauzek M, Konecny J, Borova M, et al. Energy harvesting sources, storage devices and system topologies for environmental wireless sensor networks: A review[J]. Sensors, 2018, 18(8): 2446.

[X8]          Kim S, Vyas R, Bito J, et al. Ambient RF energy-harvesting technologies for self-sustainable standalone wireless sensor platforms[J]. Proceedings of the IEEE, 2014, 102(11): 1649-1666.

[X9]          H. S. Kim, J. -H. Kim, and J. Kim, ‘‘A review of piezoelectric energy harvesting based on vibration,’’ Int. J. Precision Eng. Manuf., vol. 12, no. 6, pp. 1129–1141, Dec. 2011.

[X10]         Hoang T, Ferin G, Bantignies C, et al. Aging assessment of piezoelectric energy harvester using electrical loads[C]//Journal of Physics: Conference Series. IOP Publishing, 2019, 1407(1): 012078.

************* Start of next Change ***************

Annex X (Informative):Ambient power and energy storage
X.1 Typical ambient power

For ambient IoT devices, energy can be harvested from different types of ambient power sources. Some examples of which could be ambient power includes radio waves, solar energy/light, thermal energy and mechanical vibration etc.

RF Energy

RF-based energy can be harvested from radio waves ranging from 3 kHz to 300 GHz using a single-stage or multistage converter (i.e., rectifier circuit, as shown in Figure 1). The amount of power that can be harvested depends on the source power, antenna gain, and the distance from the RF source. Ambient RF energy has a relatively low energy density, e.g., from several microwatts to tens of microwatts.

Figure 1: rectifier circuit For RF energy harvesting

Based on the current state of art, the minimum RF power can be harvested is around -30dB [X1] [X2] . The conversion efficiency for RF Energy is listed in the table below.

Table 4.1-1 Conversion efficiency for RF Energy [X3]

| Efficiency（%） | Input power（dBm） | Center frequency（MHz） | Reflector unit |
| --- | --- | --- | --- |
| 1.2 | -14 | 950 | 0.3-μm CMOS convertor |
| 5.1 | -14.1 | 920 | 0.18-μm CMOS convertor |
| 10 | -22.6 | 906 | 0.25-μm CMOS convertor |
| 11 | -14 | 915 | 90-μm CMOS convertor |
| 12.8 | -19.5 | 900 | 0.18-μm CMOS , CoSi2 - Si Schottky |
| 13 | -14.7 | 900 | 0.35-μm CMOS convertor |
| 16.4 | -9 | 963 | 0.35-μm COMS convertor |
| 18 | -19 | 869 | 0.5-μm CMOS convertor |
| 26.5 | -11.1 | 900 | 0.18-μm CMOS convertor |
| 36.6 | -6 | 963 | 0.35-μm CMOS convertor |
| 47 | -8 | 915 | 0.18-μm CMOS convertor |
| 49 | -1 | 900 | Skyworks SMS7630 Si Schottky |

The main advantage of RF-based energy harvesting is its availability in deployed environments and the fact that RF power is controllable (e.g., power can be sent by a transmitter on demand or periodically). Potential applications include logistics/warehouse, manufacturing, smart homes, health monitoring, and environmental monitoring etc.

Solar Energy/Light

Solar power/light can be transformed into electrical power using photovoltaic cells and it uses photovoltaic effect for energy harvesting with conversion efficiency of 10-40% [X4] . For the outdoor case, solar energy is one of the most common ambient power, it can supply inexhaustible clean energy and has high power density of up to 100 mW/cm2 [X5] .

Figure 2: The equivalent electrical circuit of a single diode solar PV cell [X6]

Solar power is unstable, inconsistent, and intermittent. It is highly dependent on the atmospheric condition, surrounding obstructions, etc. It is available during daytime but inefficient on a cloudy day or during the night. Solar energy harvesting can be mainly used for outdoor environmental monitoring, agriculture, husbandry, transportation, etc.

For the indoor cases, light from the lighting equipment can be used. Although the power density is lower than solar, e.g., 100uw/cm2, it is much stable and controllable. Energy harvested from light can be used for manufacturing, indoor environmental monitoring etc.

Thermal Energy

Thermal energy is another ambient power source that are available for lots of use cases. Electrical power is directly generated by exploiting the temperature difference in thermoelectric devices taking advantage of thermoelectric effects, such as the Seebeck effect or the Thomson effect. Thermoelectric generators have low efficiency (only about 5–6%) [X7] . The power density is 25~1000uw/cm2 depending the environment condition.

Figure 3: Seebeck effect

Although with low conversion efficiency, thermal energy can be used in many outdoor applications or indoor cases as long as temperature difference or temperature fluctuation can be expected in the environment. For example, outdoor environmental monitoring, smart grid, agriculture, husbandry etc.

Mechanical Vibration

The piezoelectric effect generates electrical voltages or currents from mechanical strains, such as vibration or deformation. Typical piezoelectric-based energy harvesters keep creating power when there is a continuous mechanical motion, such as acoustic noises and wind, or they sporadically generate power for intermittent strains, such as human motion (walking, clicking a button, etc.). The volume of the piezoelectric power generators is relatively small and typical output power density values of usual piezoelectric materials are around 250 μW/cm3 but they can create more power when a motion or deformation is intense [X8] [X9] .

Figure 5: Piezoelectric energy harvesting generator [X10]

X.2 Energy storage

From the discussion above, it can be seen that kinds of ambient power have the following characteristics:

- For typical ambient power, it can be observed the power harvested is very limited, e.g. from 1uW to 100mW (per cm2/cm3).

- For some ambient power from artificial power source (e.g., light, RF waves), the power can be stable and constant. But for some other kind of ambient power such as solar, heat or vibration, the ambient power will be unstable (intermittent, not constant). It is impossible to use the ambient power as a direct power source for electronic devices.

Therefore, Energy storage element is needed, at least for some ambient IoT devices due to the following reasons:

- The energy storage element is able to stabilize and control the power output, smooth the fluctuation.

- It is able to collect the weak harvested power (e.g., in the level of micro Ampere or even nano Ampere) and provide the required higher peak discharge current (e.g., in the level of tens of micro ampere to hundreds of micro ampere) for the ambient IoT devices.

- Therefore, it makes it possible to use more kinds of ambient power sources for ambient IoT by using the energy storage element.

Note: It is still necessary to have no power storage for some types of ambient IoT devices (e.g. using energy from radio waves).

Capacitor can be considered as the basic energy storage elements for ambient IoT devices. Capacitor have limited power perseverance time and storage capacity, which can restrict the ambient IoT application.

For example, with a fully charged capacitor of 24uF, it can drive the ambient IoT devices for 3.6k bits communication (1.5V, 10uA and 1kbit/s are assumed).

        - 24uF*1.5V= 36uC = 36 uAs

The communication is depending on the power consumption. Capacitor could be used in case power source are stable and constant.

Solid-state battery can be considered as an additional power storage with similar durability and higher capacity. With a solid-state battery of 1uAh@1.5V as example, it can drive the ambient IoT devices for even 360k bits communication (1.5V and 10uA).

************* End of Changes ***************