3GPP TSG-SA WG1 Meeting #100 	S1-223326

Toulouse, France, 14 – 18 November 2022,	(revision of S1-223129)

Title:	New use case: Ambient IoT in Smart Agriculture

Agenda Item:	7.2

Source:	vivo

Contact:		Yanchao Kang <kangyanchao@vivo.com>

Abstract: This document provides a Text Proposal for the use case about using Ambient IoT services in smart agriculture.

# Discussion

In the use case described in this contribution, authorized UEs (e.g. CPEs, smartphones) and/or RAN (e.g. pico cells) can be deployed in a farm (e.g. in a greenhouse) to monitor the environment and control the facilities such as irrigation system and temperature control system. In the existing use cases of the TR, it is not clear whether authorized UEs can be used to communicate with such Ambient IoT devices and transfer sensor data and control signalling between the Ambient IoT devices and the 3rd part application server. Therefore, it is proposed to add the description of this use case, along with the service requirements and KPI table.

Regarding the KPI table, based on online reports of some agricultural production bases in China, the background for the suggested values are listed below:

1) Max. allowed end-to-end latency: There is no strict requirement for end-to-end latency. Therefore >1 s is suggested.

2) Communication Service Availability: 99.9% is suggested.

3) User-experienced data rate: It is configured based on the value of latency and message size.

4) Message Size: To monitor environmental parameters, (e.g. air temperature and humidity, carbon dioxide concentration, light, soil temperature, humidity and PH), it would not be larger than 1000bits per message. Also control signaling would not exceed this value.

5) Device density: Typically, 5 sensors (Ambient IoT devices) are deployed in one greenhouse. About 5 Ambient IoT devices are deployed for control of the window and irrigation system. It is calculated based on the area of a greenhouse.

6) Communication Range: For direct communication with a UE, 10 m is required; For communication with a RAN node, 50 m is required.

7) Service area dimension: Typically, a greenhouse is about 500 m2, while a farm may be 2 km2.

8) Device speed: The Ambient IoT devices are statically deployed.

9) Traffic interval: Typically, 1 hour report is performed in current intelligent greenhouse control system.

Since the Ambient IoT devices are static, there is no requirement for positioning of the Ambient IoT devices for this use case.

# Text Proposal to TR 22.840

**************** Start of Change ******************

## 5.x	Use case on Smart Agriculture

### 5.x.1	Description

Ambient power-enabled IoT devices can be used in smart agriculture to monitor the environment and control the facilities such as irrigation system and temperature control system.

Farm A built a smart greenhouse for tomato planting. Some sensing Ambient power-enabled IoT devices are placed in the smart greenhouse to monitor the air temperature and humidity, carbon dioxide concentration, light, soil temperature, humidity and PH. Some operation Ambient power-enabled IoT devices are placed in the smart greenhouse to control the window and irrigation system. A pico cell or a reader UE (with subscription of Operator O) is placed in the smart greenhouse to communicate with the Ambient IoT devices.

Considering the greenhouse environmental control, crop growth characteristics and economic benefits, the optimal scale of greenhouse construction is 8 ~10 meters span, 80~100 meters length.  The size of the greenhouse could be very huge, it is reported recently that a single greenhouse area reaches nearly 70,000 square meters, equivalent to ten standard football field size.

(http://www.haofengfood.com.cn/index.php?m=content&c=index&a=show&catid=46&id=68)

In charge of the technology is a team of eight people whose main task is to ensure that all the crops in the greenhouse grow well. To provide all-round technical support for a greenhouse of nearly 70,000 square meters, a "super brain" is needed. This computer monitors tens of thousands of sensors in the greenhouse, people can sit in front of the computer and know everything that's going on in the greenhouse. For example, the temperature and humidity in each area of the 60,000 square meters greenhouse, the temperature of the underground heating tube, the concentration of carbon dioxide are how much, whether the fan is opened, and whether the nutrition is enough for each tomato.

### 5.x.2	Pre-conditions

The pico cell(s) placed in the smart greenhouse is configured by Operator O to support the 5GS Ambient IoT service. The UE(s) is capable to and authorized by Operator O to directly communicate with the 5GS Ambient IoT device.

### 5.x.3	Service Flows

- Operator O’s 5GS authenticates Ambient IoT devices in farm A. The communication between the Ambient IoT device and 5GS are transferred through the authorized UE or the pico cell. For example, the reader UE or the pico cell obtains Ambient IoT device ID and sends the ID to the 5GC for authentication.

- Periodically, or upon request from the application server of farm A, the authorized UE or the pico cell obtains the environmental monitoring information from the sensing Ambient IoT devices and sends the information to the application server of farm A.

- Based on the received information and preconfigured logic, the application server requests 5GS to send control signaling to the operation Ambient IoT devices to control their operations, e.g. open or close the window of the greenhouse, start or close the irrigation system.

- The 5GS transfers the control signalling from the application server to Ambient IoT devices through the authorized UE or the pico cell.

### 5.x.4	Post-conditions

Thanks to the Ambient IoT service provided by the 5G system, operation Ambient power-enabled IoT devices automatically take care of the plants.

### 5.x.5	Existing features partly or fully covering the use case functionality

None.

### 5.x.6	Potential New Requirements needed to support the use case

### 5.x.6.1 Service requirements for use of Ambient IoT devices in Smart Agriculture

[PR.5.x.6.1-001] The 5G system shall support communication with Ambient IoT device with 3rd party application server via an authorized UE.

[PR.5.x.6.1-002] The 5G system shall be able to support direct communication between authorized UE(s) and Ambient IoT devices.

[PR.5.x.6.1-003] The 5G system shall be able to authenticate an Ambient IoT device.

### 5.x.6.2 KPIs for the use of Ambient IoT devices in Smart Agriculture

[PR.5.x.6.2-001] The 5G system shall be able to provide Ambient IoT service with following KPIs:

Table 5.x.6.2-1 Ambient_IoT KPI for Smart Agriculture

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Ambient IoT Device-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Traffic interval |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Smart Agriculture | > 1 s | 99.9% | < 1 kbit/s | <1000 bits | 1 per 1 m2 | 10 m with UE 50 m with RAN | 500 ~70000 m2 per greenhouse | static | 1 hour |

NOTE: There is no requirement for positioning of the Ambient IoT devices for this use case.

************* End of Change ***************