3GPP TSG SA WG 1 Meeting #100 	S1-223355

Toulouse, France, 14 – 18 November 2022	(revision of S1-223250)

Source:	KPN, Interdigital, Huawei, Vodafone

pCR Title:	Pseudo-CR on pressure powered switch

Draft Spec:	3GPP TR 22.840

Agenda item:	7.2

Document for:	Approval

Contact:	Toon Norp (<firstname>.<secondname>@tno.nl)

Abstract: <provide a short description of the content>

1. Introduction

Ambient IoT devices (e.g. a pressure powered switch) can harvest kinetic energy. Very limited power is available and only for a very small amount of time.

2. Reason for Change

This use case introduces requirements to optimise Ambient IoT signalling and communication to take into account that power is available only for a very limited time.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840 v 0.2.0.

* * * First Change * * * *

## 5.x	Use case on pressure powered switch

### 5.x.1	Description

A pressure powered switch can harvest energy from the kinetic energy of the pushing on the switch. Existing non-3GPP products can e.g. connect wirelessly to a controller over approximately 25 m indoor and 150 m outdoor. These products do not adhere to 3GPP protocols for signalling and user communication.

Figure 5.x.1-1: A pressure powered switch

The registration procedure for 5G according to TS 23.502 [16] shows 25 different steps which 8 involve the UE. Several of these steps involve multiple interactions. This registration procedure does not even include the actual data communication. Compared to the minimal information transfer requirement for a button switch (only its ID), it is clear that the registration involves far more information than the actual data transport. Furthermore, a registration procedure will likely take longer than the time the switch has power available. This runs the risk that the switch can start a registration (or other) signaling procedure, but cannot successfully complete it.

The purpose of the use case is to propose a requirement to optimize Ambient IoT signaling to reduce the amount of signaling interactions and thus save power and time. Note that the pressure powered switch is an example for other Ambient IoT devices that only have a very limited amount of energy for a very short amount of time (e.g. door/window sensors, vibration sensors).

### 5.x.2	Pre-conditions

None

### 5.x.3	Service Flows

1. The switch is being pushed.

2. The switch harvests the energy from the push, wakes up and communicates to the network.

3. Through optimized protocols only a minimal number of signalling interactions is needed. This allows the switch to complete the signalling procedure. As part of the signalling, the switch also transmits its identity.

4. The network transfers the identity information from the switch to an application server, which e.g. determines that a light needs to be turned on.

### 5.x.4	Post-conditions

The light is successfully switched on.

### 5.x.5	Existing features partly or fully covering the use case functionality

Existing signaling procedures and data transfer procedures for 5G are specified in TS 23.502 [16]. It is clear that these procedures are not optimized for a minimal number of signaling interactions. This runs the risk that an Ambient IoT device cannot complete these procedures before running out of power.

### 5.x.6	Potential New Requirements needed to support the use case

[P.R.5.1.6-001] The 5G system shall support optimized procedures for Ambient IoT control and user data transmission; to use a minimum number of interactions with the Ambient IoT device.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	International Energy Agency (IEA), "World Energy Outlook 2020". Available online: https://www.iea.org/reports/world-energy-outlook-2020

[3] 	Motlagh, N.H.; Mohammadrezaei, M.; Hunt, J.; Zakeri, B. "Internet of Things (IoT) and the Energy Sector." Energies 2020, 13, 494.

[4] 	Pereira F., Correia R., Carvalho N.B. "Passive Sensors for Long Duration Internet of Things Networks." Sensors 2017;17:2268. doi: 10.3390/s17102268.

[5] 	https://www.gs1.org/standards/rfid.

[6]	3GPP TS 22.368: "Service requirements for Machine-Type Communications (MTC)".

[7]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.011: “Service accessibility”.

[10]	5G for smart manufacturing: https://www.gsma.com/iot/wp-content/uploads/2020/04/2020-04_GSMA_SmartManufacturing_Insights_On_How_5G_IoT_Can_Transform_Industry.pdf

[11] 	 https://forcetechnology.com/en/articles/batteryless-electronics-energy-harvesting

[12] 	R. Ding and B. Xing, "Comparative Research on the Way of Energy Harvesting of the Wireless Sensor Network Nodes," 2013 6th International Conference on Intelligent Networks and Intelligent Systems (ICINIS), 2013

[13] 	P. D. Hung et al., "A Self-Powered Wireless Gas Sensor Node Based on Photovoltaic Energy Harvesting," 2021 Symposium on VLSI Circuits, 2021.

[14] 	GS1: "EPC Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard (Release 2.1) "

[15]                      EPC Tag Data standards version 1.1 Rev.1.24

[16]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

* * * Next Change * * * *

<Proposed change in revision marks>