3GPP TSG-SA WG1 Meeting #100 	S1-223360

Toulouse, France, 14 – 18 November 2022	(revision of S1-223128)

Title:	KPI update for Ambient IoT in personal belongings finding

Agenda Item:	7.2

Source:	vivo

Contact:		Yanchao Kang <kangyanchao@vivo.com>

Abstract: This document provides a Text Proposal for updating KPI table for personal belongings finding.

# Discussion

Based on the rapporteur’s suggestion, this contribution proposes to update the KPI table in clause 5.12.6.2.

Regarding the suggested values in the KPI table, the background is listed below:

1) Max. allowed end-to-end latency: The original value is 2s for Ambient IoT communication service latency.  It is about twice of the time of end-to-end latency. Therefore 1 s is suggested for end-to-end latency.

2) Communication Service Availability: 99.9% is suggested.

3) User-experienced data rate: It is configured based on the value of latency and message size.

4) Message Size: In this use case, the message mainly includes identifier of the Ambient IoT device and location information. Based on [14], the maximum length of EPC identifier is 464 bits, and maximum length of TID is 464 bits. Beside the TID, the message could always contain location information timestamp and etc. Therefore, the suggested value is less than 1k bits.

5) Device density: Typically, one person may use several Ambient IoT devices. Considering the typical area of an apartment, the suggested values are < 5 per 100 m2 for indoor. For outdoor case, there may be several people loss their belongings in the same place. Therefore, the suggested value is < 100 per 100 m2 for outdoor.

6) Communication Range: For indoor direct communication with a UE, 10 m is required; For outdoor communication with a RAN node, 100 m is required.

7) Service area dimension: Typically, an apartment is less than 200 m2. For outdoor case, the service area depends on the area where the operator wants to provide such service. It could be at some business centres, can also be up to the whole PLMN.

8) Device speed: The Ambient IoT device may be carried to difference places. Once it is lost, it would be static in normal case.

9) Traffic interval: Since this is not a communication scenario that frequently happens, the suggested value is 1 per hour.

10) Positioning Accuracy: Based on the actual human experience, the suggested value is 1-3m for indoor positioning and 5-10m for outdoor positioning.

# Text Proposal to TR 22.840

**************** Start of Change ******************

### 5.12.6	Potential New Requirements needed to support the use case

[PR.5.12.6.1-001] The 5G system shall support to authorize a UE to obtain device information of an Ambient IoT device.

[P.R.5.12.6.1-002] The 5G system shall be able to collect information from a specific Ambient IoT device.

[P.R.5.12.6.1-003] The 5G system shall be able to provide information of a specific Ambient IoT device to the trusted 3rd party.

NOTE: The request from 3rd party can include the requested Ambient IoT device identity, the requested service area to find the Ambient IoT device, the requested information of an Ambient IoT device includes position information.

[PR 5.12.6.1-004] The 5G system shall be able to support indoor and outdoor positioning for Ambient IoT devices.

[PR. 5.12.6.1-005] The 5G system shall be able to support an Ambient IoT device to validate a UE which communicates with the device.

[PR. 5.12.6.1-006] The 5G system shall support to validate an Ambient IoT device.

[PR.5.12.6.1-007] The 5G system shall be able to provide Ambient IoT service with following KPIs:

Table 5.12.6.2-1 Ambient IoT service KPI for personal belongings finding

| Scenario | Max. allowed end-to-end latency | Communication service availability |  | Ambient IoT Device--experienced data rate | Message Size (UL) (NOTE) | Device density | Communication Range | Service area dimension | Device speed | Traffic interval | positioning accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Personal belongings finding (indoor) | 1 s | 99.9% |  | < 1 kbit/s | < 1 kbits | < 5 per 100 m2 | 10 m | < 200 m2 | Static | 1 per hour | [1-3] m, 90% availability |
| Personal belongings finding (outdoor) | 1 s | 99.9% |  | < 1 kbit/s | < 1 kbits | < 10 per 100 m2 | 100 m | Up to the whole PLMN | Static | 1 per hour | FFS |
| NOTE: The payload includes Ambient IoT device information, e.g., Ambient IoT device ID [14] [15]. |  |  |  |  |  |  |  |  |  |  |  |

************* End of Change ***************