3GPP TSG SA WG 1 Meeting #100	S1-223366

Toulouse, 14 November – 18 November 2022 	(revision of S1-223208)

Source:	KPN, Huawei

pCR Title:	Update of KPI values in traffic scenario 6.1

Draft Spec:	3GPP TR 22.840

Agenda item:	7.2

Document for:	Approval

Contact:	Toon Norp (toon .norp@tn o.nl)

Abstract: This pCR provides an update to traffic scenario 6.1

1. Introduction

A traffic scenario on flower auction and the corresponding KPIs were accepted in SA1 #98-e, power scenario was added in SA1 #99e. During SA1 #99e, it was observed that some KPI values need to be updated for accuracy or correctness, therefore this pCR is proposed. Also some other changes were done to the introduction text.

2. Reason for Change

The message size of 500 bits is longer than is needed. This also resulted in a higher than needed data rate. This is now corrected. Furthermore, a KPI for latency was added.

Furthermore, this pCR proposes a resolution for a Editor’s Note. Terminology related to “activation” in the description was conflicting with “activation” terminology in clause 5.17. New terminology is now proposed.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840 v0.2.0.

There was a discussion in the meeting on the KPI for devices per base station. In light of this a new tdoc for revision was provided. However, it was later agreed that changes to the KPI tables should be done in the Oppo general contribution. They have therefore not been included in this revision. Therefore this revision does not have any changes compared to S1-223208.

* * * First Change * * * *

## 6.1	Traffic scenario Flower Auction

### Description

In the Netherlands, there is extensive logistics industry for flowers and vegetables. A specific case are the auctions where flowers from all over the world are brought in by the growers, then auctioned and subsequently distributed to buyers all over the world.

Flowers are transported on four-wheel containers that can be rented and are that used throughout the logistic chain. These containers are now equipped with a RFID tag. It would be beneficial if Ambient IoT tags could be used. RFID tags are scanned when containers with flowers arrive or leave the auction; tracking and tracing with Ambient IoT could get regular reports from all containers anywhere at the auction.

Figure 6.1-1: Logistics at a flower auction

Shipments of flowers can be tracked and traced based on the containers they are on. This is of interest for growers, buyers and the auction. There is also logistics of empty containers, where also the company that owns the containers can benefit from Ambient IoT. Finally, the auction has an interest in managing its space. Companies that own or rent containers are charged for leaving containers on the auction grounds overnight.

Communication service availability is important. Tracking and tracing is mainly done to find out where things have gone wrong in the logistics, e.g., missing containers. If the communication from tags is not significantly more reliable than the logistics itself, then tracking and tracing does not provide a benefit.

Some numbers:

-	There are multiple flower auctions in The Netherlands.

-	The size of the flower auction location in Aalsmeer is 1 732 769 m2.

-	44 million flowers are auctioned per day

### Assumptions

We assume every four wheeled container is equipped with a tag in the form of an Ambient IoT device.

Figure 6.1-2: Container with flowers (Photo: Container Centralen)

Density of containers can be estimated based on the dimensions of the containers. A container is (lxwxh) 1350 mm x 565 mm x 1900m. Packing these containers closely together gives a density of 740 x 1770 = 1,3 million containers per km2.

Figure 6.1-3: Density of flower containers (Photo: Royal FloraHolland)

Ceiling in the flower auction is at 9 meters. We assume that base stations are attached to the ceiling. Number of base stations that is needed to cover the flower auction is dependent on the communication range and on the number of devices per base station. Here we assume a base station spacing of one base station for every 50 m x 50 m of ceiling. This gives a maximum range of approx. 35 meters from ceiling to container. The number of containers in that 50 m x 50 m area is approximately 3000.

The assumption is that the Ambient IoT devices are woken up and triggered for communication on demand by the 5G network, where e.g., the flower auction or a flower grower can decide when to wake up and trigger the Ambient IoT devices for communication. When woken up and triggered the Ambient IoT devices respond by transmitting information, e.g., their identity numbers, to the network.

The flower auction can decide in which parts of the flower auction the Ambient IoT devices are woken up. This can be done by e.g. only providing the wake up via some of the base stations. The flower auction can also decide to trigger only part of the Ambient IoT devices.

Assumption is that an identity can be provided within 96 bits (is EPC length used for identification).

Assumption is that probability of errors in logistics handling (e.g., a container is left behind) is <1%. A communication service availability of 99,99% would imply that the chance that communication for tracking a container is approximately 2 orders of magnitude better than the logistics handling reliability.

There is no strict latency requirement when large amounts of containers are triggered. A latency in the order of 10 seconds is acceptable.

### Potential Functional Requirements

None identified.

### Potential Key Performance Requirements

[PR 6.1-001] The 5G system shall be able to provide Ambient IoT service with the following KPIs:

Table 6.1-1: KPIs for Flower Auction scenario

| Scenario | Communication Service Availability | Message Size | Data rate per UE | Communication range | Device density | Devices per base station | Latency | Service area dimension |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Container logistics in a flower auction | [99,99%] (note 1) | [96] bits (note 2) | < 1 kbit/s (note 3) | [35] m Indoors | [< 1,3] Million/km2 (note 4) | <[3000] | [<10 s] | [1 700 000] m2 (note 5) |
| NOTE 1: Chance of communication service unavailability needs to be significantly lower than chance of errors in logistics handling NOTE 2: Only an identifier for the tag is sent (EPC lengths used for identification is 96 bits). NOTE 3: This value is calculated as the instant data rate for transmitting 96 bits within 100 ms time period. The need for data transmission is infrequent. NOTE 4: Based on closely packing containers. NOTE 5: Size of the flower auction location in Aalsmeer. |  |  |  |  |  |  |  |  |