3GPP TSG-SA WG1 Meeting #100 	S1-223388

Toulouse, France, 14 – 18 November 2022	(revision of S1-223377-3046)

Source:	Ericsson, Deutsche Telekom

pCR Title:	Pseudo-CR on RVAS Consolidation

Draft Spec:	3GPP TR 22.877 v0.1.0

Agenda item:	7.9

Document for:	Approval

Contact:	Peter Bleckert (peter.bleckert at Ericsson.com)

Abstract: <provide a short description of the content>

1. Introduction

All the requirements found in subchapter 5.1.6, 5.2.6, and 5.3.6 is consolidated and moved to chapter 6.

No change to the requirments are suggested.

2. Proposal

It is proposed to add the following text to 3GPP TR 22.877, and align the text based on any changes in 5.3.6.

# 6	Consolidated Potential Requirements

Table 6-1 –Consolidated Potential Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6-001 | The 5G system shall be able to support mechanisms for the HPLMN to provide a notification, including equipment and subscription identifiers, to a trusted application server when a UE successfully registers in a VPLMN. In response to the notification, the trusted application server can indicate specific actions to the HPLMN (e.g., send an SMS to the UE). NOTE: The trusted application server can be hosted by the home operator or a trusted 3rd party and is out of 3GPP scope. | PR 5.1.6-001 |  |
| CPR 6-002 | The 5G system shall be able to support mechanisms enabling the HPLMN to: - provide a notification, including subscription and equipment identifiers, to a trusted application server when a UE tries to register in a VPLMN - receive a notification reply from the trusted application server indicating specific actions to the HPLMN e.g., reject UE registration (with a specific cause), trigger a SoR command. NOTE: The trusted application server can be hosted by the home operator or a trusted 3rd party and is out of 3GPP scope. | PR 5.2.6-001 |  |
| CPR 6-003 | The 5G system shall be able to support a mechanism for forwarding signalling traffic pertaining to UEs of specific subscribers from their HPLMN to a target PLMN, e.g., to enable further handling of those UEs by the target PLMN. The forwarding mechanism shall minimize signalling traffic in the HPLMN, e.g., by using efficient means to forward traffic from selected UEs. NOTE1: The above requirement assumes that the HPLMN has an agreement with the target PLMN, and routing policies are in place. NOTE2: In case of UEs connected via a VPLMN, it is assumed that signalling traffic is forwarded to the target PLMN by the HPLMN. | PR 5.3.6-001 |  |