3GPP TSG SA WG 1 Meeting #100 	S1-223396

Toulouse, France, 14 – 18 November 2022	(revision of S1-223117)

Source:	CATT

pCR Title:	Use case on vehicle fleet management in the desert

Draft Spec:	3GPP TR 22.865 v0.1.0

Agenda item:	7.7

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: The contribution proposes a use case about the fleet management of the vehicles through 5G network with satellites and describes potential requirements for TR22.865.

1. Introduction

Introduce a use case about the vehicle fleet management in the desert based on the communication between UEs using satellites and UEs using indirect connection to 5G network.

2. Reason for Change

Service requirements about UE communications under the same satellite network shall be addressed to Section 5 “Use Cases” of TR 22.865.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree on the following changes to 3GPP TR22.865 v0.1.0

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[2]					3GPP TS 22.261: "Service requirements for the 5G system".

[x1]	Noman Shaikh, Significance of fleet management in logistics industry, January 02, 2020, https://www.peerbits.com/blog/significance-fleet-management-solution-for-logistics-industry.html

* * * Next Change * * * *

5 Use cases

5. X	Use case on vehicle fleet management in the desert

5.X.1	Description

With the help of the construction of the transportation infrastructure, communication and information infrastructure, the modern logistics can deliver the goods to almost any corner of the world in a fast and reliable manner. Fleet management is a critical part of the logistics industry[x1], which is being changed by IoT technologies in live vehicle monitoring, cargo management, driver behaviours’ monitoring and etc. The real-time data exchange is important for the staff of fleet management in route scheduling, decision making and safety assurance.

The convoy sometimes need to go across the area with sparse population or in extreme condition, where the network status fluctuates. Thus, besides remote management, local management by the team leader will also play a role in ensuring the speed and reliability of the transportation.

5.X.2	Pre-conditions

The logistic company ExpressX provides transportation services all over the world and well known for long distance transport. NetX, a mobile network operator has signed the contract with ExpressX to offer the communication service for all the vehicles and the staff, and promise the full coverage along all their transport routes including satellite service. NetX has deployed NGSO satellites to realize the radio coverage in rare population area and the deserts.

All the vehicles for long distance transport are equipped with Telematics Box supporting all 5G RATs (e.g. NR, LEO) as well as on-vehicle IoT devices only capable of 5G NR for data service. Also, all on-vehicle devices support 3GPP Device-to-Device communication. The man-held UEs of fleet team leaders support all 5G RATs.

Figure 5.X.2-1: Fleet management in the desert via satellite access

5.X.3	Service Flows

1. UE1, held by the fleet team leader is registered to 5G network via terrestrial access network at departure.

2. The T-box Device#2 and other on-vehicle devices such as Device#3 are registered to 5G network indicating UE capability when the transport starts, and report the vehicle status and the driver’s behaviours to remote management platform and UE1 via terrestrial access network and ground core network .

3. When the convey approaches the desert highway, the remote management platform requests fleet team leader to manage the fleet locally considering the delay and instability of the network. Device#2, Device#3 and UE1 are authorized by 5G network to use satellite services regarding the operator’s policy (e.g. access network selection, route selection).

4. When the convey moves into the desert, Device#2 and UE1are authorized by remote core network to exchange data between each other via satellites without going to the remote ground network. Also, Device#3 is authorized to communicate with UE1 via Device#2 and satellites without going to the remote ground network.

5. As the movement, there is coverage of terrestrial access network. Device#2 and UE1 can continue the communication with minimum interruption via terrestrial access network and remote ground network. Device#3 can be steered to connect to 5G network via terrestrial access network directly.

5.X.4	Post-conditions

UE1 can obtain the vehicle status and driver’s information in real-time, and issue the action commands and distribute the route adjustment information in time.

5.X.5	Existing features partly or fully covering use case functionality

SA1 has performed several studies on satellite access and backhaul, and the associated service requirements are introduced to TS 22.261[2].

Clause 6.5.2 defines the requirements of efficient user plane about satellite access as below.

For a 5G system with satellite access, the following requirements apply:

A 5G system with satellite access shall be able to select the communication link providing the UE with the connectivity that most closely fulfils the agreed QoS

Clause 6.9.2.5 defines the requirements of connectivity models about satellite access as below.

A 5G system with satellite access shall be able to support relay UE's with satellite access.

NOTE: The connection between a relay UE and a remote UE is the same regardless of whether the relay UE is using satellite access or not.

A 5G system with satellite access shall support mobility management of relay UEs and the remote UEs connected to the relay UE between a 5G satellite access network and a5G terrestrial network, and between 5G satellite access networks.

There is no explicit discussion about the efficient user data path when all the UEs are connecting to the 5G network via the same satellite but in different network connection modes.

5.X.6	Potential New Requirements needed to support the use case

[PR 5.X.6-001] The 5G system shall support mechanisms to authorize the communication between UEs using satellite access without going through the ground network in direct network connection mode or indirection network connection mode.

NOTE: The connection mode refers to direct network connection (UE connects to the network directly) and indirect network connection (UE connects to the network using another UE as a relay UE).

[PR 5.X.6-002]

Subject to operator’s policy, the 5G system shall support the traffic data exchange between UEs via satellite access without going through the ground network in the same or different network connection modes

NOTE: The connection mode refers to direct network connection (UE connects to the network directly) or indirect network connection (UE connects to the network using another UE as a relay UE).

Editor’s note: it is FFS whether there is new requirements about mobility management.

* * * End of Changes * * * *