3GPP TSG SA WG 1 Meeting #100 	S1-223153

Toulouse, France, 14 – 18 November 2022	(revision of S1-22xxxx)

Source:	vivo

pCR Title:	Pseudo-CR on update the power consumption for immersive AR Interactive Experience

Draft Spec:	3GPP TR 22.856-020

Agenda item:	7.3

Document for:	Approval

Contact:	Xiaowen Sun

sunxiaowen@vivo.com

Abstract: <provide a short description of the content>

1. Introduction

The power consumption for UE communication in immersive AR interactive experience has been analysed and updated in the existed use case.

2. Reason for Change

In order to fulfill user's immersive experience e.g. AR movie, wireless wearable devices not only need to support low-latency and high data rata transmission,  but also need to have a battery life of at least two hours, i.e. duration of a movie, power consumption requirements of wearable devices needs to be identified.

It is proposed to focus on UE’s communication power consumption, that is, energy efficiency of delivery, how the UE can support ‘mobile metaverse services’ with energy efficiency;  or in the other words,  in order to delivery the same data transmission bit rate, communication power consumption needs to be more efficient, the smaller, the better.

3. Conclusions

For the use case AR Enabled Immersive Experience, the communication power consumption is fully investagated and calculated, the power consumption value KPI can be updated.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.856-020.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.228: "Service requirements for the Internet Protocol (IP) Multimedia core network Subsystem (IMS)".

[3]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services".

[4]	3GPP TS 22.101: "Service principles".

[5]	3GPP TS 22.261: "Service requirements for the 5G system".

[6] 	Talaba D, Horvath I, Lee K H, "Special issue of Computer-Aided Design on virtual and augmented reality technologies in product design," Computer-Aided Design 42:361 - 363.

[7] 	Wang X, Tsai J J-H, "Collaborative Design in Virtual Environments," In: ISCA 48:1-2.

[8] 	Benford S, Greenhalgh C, Rodden T, "Collaborative virtual environments." Communications of the ACM 44: 79–85.

[9]	M. Eid, J. Cha, and A. El Saddik, "Admux: An adaptive multiplexer for haptic-audio-visual data communication", IEEE Tran. Instrument. and Measurement, vol. 60, pp. 21–31, Jan 2011.

[10]	K. Iwata, Y. Ishibashi, N. Fukushima, and S. Sugawara, "QoE assessment in haptic media, sound, and video transmission: Effect of playout buffering control", Comput. Entertain., vol. 8, pp. 12:1–12:14, Dec 2010.

[11]	N. Suzuki and S. Katsura, "Evaluation of QoS in haptic communication based on bilateral control", in IEEE Int. Conf. on Mechatronics (ICM), Feb 2013, pp. 886–891.

[12]	E. Isomura, S. Tasaka, and T. Nunome, "A multidimensional QoE monitoring system for audiovisual and haptic interactive IP communications", in IEEE Consumer Communications and Networking Conference (CCNC), Jan 2013, pp. 196–202.

[13]	A. Hamam and A. El Saddik, "Toward a mathematical model for quality of experience evaluation of haptic applications", IEEE Tran. Instrument. and Measurement, vol. 62, pp. 3315–3322, Dec 2013.

[14]	O. Holland et al., "The IEEE 1918.1 "Tactile Internet" Standards Working Group and its Standards," Proceedings of the IEEE, vol. 107, no. 2, Feb. 2019.

[15]	Lee, Donghwan et. al., “Large-scale Localization Datasets in Crowded Indoor Spaces,” NAVER LABS, NAVER LABS Europe, CVPR 2021.

[16]	3GPP TR 22.837, " Study on Integrated Sensing and Communication."

[17]	Alriksson, F., Kang, D.H., Phillips, C., Pradas, J.L. and Zaidi, A., 2021. XR and 5G: Extended reality at scale with time-critical communication. Ericsson Technology Review.

[18]	3GPP TR 26.928, "Extended Reality (XR) in 5G."

[19] 	Ge, Y., Wen, F., Kim, H., Zhu, M., Jiang, F., Kim, S., Svensson, L. and Wymeersch, H., 2020. 5G SLAM using the clustering and assignment approach with diffuse multipath. Sensors, 20(16), p.4656.

[20] 	Kim, H., Granström, K., Gao, L., Battistelli, G., Kim, S. and Wymeersch, H., 2020. 5G mmWave cooperative positioning and mapping using multi-model PHD filter and map fusion. IEEE Transactions on Wireless Communications, 19(6), pp.3782-3795.

[21] 	Liu, A., Huang, Z., Li, M., Wan, Y., Li, W., Han, T.X., Liu, C., Du, R., Tan, D.K.P., Lu, J. and Shen, Y., 2022. A survey on fundamental limits of integrated sensing and communication. IEEE Communications Surveys & Tutorials, 24(2), pp.994-1034.

[22] 	Dwivedi, S., Shreevastav, R., Munier, F., Nygren, J., Siomina, I., Lyazidi, Y., Shrestha, D., Lindmark, G., Ernström, P., Stare, E. and Razavi, S.M., 2021. Positioning in 5G networks. IEEE Communications Magazine, 59(11), pp.38-44.

[23]	Suhr, C. "S.L.A.M. and Optical Tracking for XR", https://medium.com/desn325-emergentdesign/s-l-a-m-and-optical-tracking-for-xr-cfabb7dd536f <accessed 1.9.22>

[24]	Kim, H., Granstrom, K., Svensson, L., Kim, S. and Wymeersch, H., 2022. PMBM-based SLAM Filters in 5G mmWave Vehicular Networks. IEEE Transactions on Vehicular Technology.

[25]	5GAA " C-V2X Use Cases Volume II: Examples and Service Level Requirements", 5G Automobile Assocaition White Paper,  https://5gaa.org/wp-content/uploads/2020/10/5GAA_White-Paper_C-V2X-Use-Cases-Volume-II.pdf <accessed 02.09.22>

[26]	A. Ebrahimzadeh, M. Maier and R. H. Glitho, "Trace-Driven Haptic Traffic Characterization for Tactile Internet Performance Evaluation," 2021 International Conference on Engineering and Emerging Technologies (ICEET), 2021, pp. 1-6.

[27]	Lee, L.-H., Braud, T., Zhou, P., Wang, L., Xu, D., Lin,  Z., Kumar, A., Bermejo, C., and Hui, P. All One Needs to Know about Metaverse: A Complete Survey on Technological Singularity, Virtual Ecosystem, and Research Agenda, 2021.

[28]	Halbhuber, David & Henze, Niels & Schwind, Valentin. (2021). Increasing Player Performance and Game Experience in High Latency Systems. Proceedings of the ACM on Human-Computer Interaction. 5. 1-20. 10.1145/3474710.

[29] 	Holland, O., Steinbach, E., Prasad, R.V., Liu, Q., Dawy, Z., Aijaz, A., Pappas, N., Chandra, K., Rao, V.S., Oteafy, S. and Eid, M., 2019. The IEEE 1918.1 “tactile internet” standards working group and its standards. Proceedings of the IEEE, 107(2), pp.256-279.

[30] 	Skalidis, I., Muller, O. and Fournier, S., 2022. CardioVerse: The Cardiovascular Medicine in the Era of Metaverse. Trends in Cardiovascular Medicine.

[31] 	Koo, H., 2021. Training in lung cancer surgery through the metaverse, including extended reality, in the smart operating room of Seoul National University Bundang Hospital, Korea. Journal of educational evaluation for health professions, 18.

[32] 	Mozumder, M.A.I., Sheeraz, M.M., Athar, A., Aich, S. and Kim, H.C., 2022, February. Overview: technology roadmap of the future trend of metaverse based on IoT, blockchain, AI technique, and medical domain metaverse activity. In 2022 24th International Conference on Advanced Communication Technology (ICACT) (pp. 256-261). IEEE.

[33] 	Ning, H., Wang, H., Lin, Y., Wang, W., Dhelim, S., Farha, F., Ding, J. and Daneshmand, M., 2021. A Survey on Metaverse: the State-of-the-art, Technologies, Applications, and Challenges. arXiv preprint arXiv:2111.09673.

[34] 	https://www.xrtoday.com/virtual-reality/ukrainian-doctors-perform-first-vr-surgery/

[35] 	https://www.cryptotimes.io/surgeon-performed-surgery-remotely-through-metaverse/

[36]	 Senk, S., Ulbricht, M., Tsokalo, I., Rischke, J., Li, S.C., Speidel, S., Nguyen, G.T., Seeling, P. and Fitzek, F.H., 2022. Healing Hands: The Tactile Internet in Future Tele-Healthcare. Sensors, 22(4), p.1404.

[37] 	Chen, D. and Zhang, R., 2022. Exploring Research Trends of Emerging Technologies in Health Metaverse: A Bibliometric Analysis. Available at SSRN 3998068.

[38] 	https://www.yankodesign.com/2022/05/15/the-metaverse-has-the-power-to-improve-healthcare-and-it-has-already-begun/

[39] 	3GPP TR 23.501 release-17 - https://www.3gpp.org/ftp/Specs/archive/23_series/23.501/23501-h50.zip

[40] 	3GPP TR 38.300 release-17 - https://www.3gpp.org/ftp/Specs/archive/38_series/38.300/38300-h10.zip

[41] 	3GPP TR 22.826 release-17 - https://www.3gpp.org/ftp/Specs/archive/22_series/22.826/22826-h20.zip

[42] 	Tataria, H., Shafi, M., Molisch, A.F., Dohler, M., Sjöland, H. and Tufvesson, F., 2021. 6G wireless systems: Vision, requirements, challenges, insights, and opportunities. Proceedings of the IEEE, 109(7), pp.1166-1199.

[x]                       VESA Compression Codecs - vesa.org/vesa-display-compression-codecs

* * * Second Change * * * *

5.7	AR Enabled Immersive Experience

5.7.1	Description

In addition to watching movies at the cinema, people will also choose to watch a movie on their mobile phones, laptops or TVs when they don’t have time to go to the cinema, e.g. when travelling or at home. By watching through these terminals, because the users always keep their head down, they will feel more or less uncomfortable in neck or cervical spine. Moreover, the screen is relatively small, if users want to see more realistic screen details, or want to watch an immersive 3D movie, using these terminals is not feasible.

Figure 5.7.1-1: AR Enabled Immersive Experience

In this use case, users can get an immersive experience of watching movies in certain circumstances, such as at home or on the train. Even, they can also invite some of their friend to watch a movie at different place at the same time by wearing a wearable device, such as AR glasses. A large screen like a movie theatre will be presented in the field of vision of the wearable device, which not only provides an immersive watching experience like private cinema but also has very low demanding on the environment and space of the user's location, 3D cinematic effects can also be easily rendered in the device.

In order to achieve immersive experience through AR glasses, 5G system is required to fulfil the high data rate and high reliability transmission of uplink and downlink data. In mobility scenarios e.g. when user is travelling on the train, the continuity of data transmission also needs to be guaranteed. Moreover, when AR glasses is wireless, power supply relies on the battery integrated inside the AR glasses. If the 5G system (through direct device connection or NG-RAN) is used to support UE to establish data connection with the Metaverse service server, the 5G system is required to collaborate with AR glasses to achieve low power consumption and high data rate transmission.

5.7.2	Pre-conditions

Bob wants to watch an AR movie to relax when at home or travelling. He wears a wearable equipment such as AR glasses which can access into the 5G network (through direct device connection or NG-RAN) to receive movie data information.

The AR glasses has subscribed the immersive movie services in Metaverse. And in Metaverse, movie resources are stored in the cloud, existed movie image can be pushed to the AR glasses, the battery capacity of the AR glasses is enough to watch a two-hour movie.

Considering the wearing comfort of users, mainstream AR glasses should not be too heavy (normally less than 150 grams). The maximum capacity of the battery (50 grams) is about 1000mAh or 3850mWh. Usually, 25% of the battery capacity of AR glasses, that is, 962.5mWh, is allocated to the mobile communication module. Not to mention that some of this power is used to transmit control signalling, leaving less power for media data transmission.

When a user watches a 4K movie, some video compression techniques are usually used to reduce the amount of data transmitted while maintaining the image quality. However, high video compression rates require responsive decoders for AR glasses which will also increase the weight of the AR glasses. Meanwhile, large compression ratio will cause delay increase. Considering the overall factors of delay and energy consumption, the compression ratio of the media data can be 3:1 [x], and the corresponding communication power consumption of the AR glasses is required to be 500mW.

5.7.3	Service Flows

1. Data connections are set up between AR glasses and Metaverse server which can provide immersive movie service with 5G module registered into the 5G network via direct device connection or NG-RAN. The QoS of the data connections require low power consumption and high data rate transmission.

2. The Metaverse server continuously pushes the movie image to the AR glasses through the downlink data stream.

5.7.4	Post-conditions

Bob is able to watch an immersive movie at home or when travelling, obtaining a good user experience. The battery capacity of the AR glasses is enough to watch a two-hour movie.

The 5G system is able to support communication for immersive AR services, with characteristics of high data rate and low power consumption transmission.

5.7.5	Existing features partly or fully covering the use case functionality

The functional and performance requirements for high data rate AR services has been captured in TS 22.261 clause 7.6. The functional and performance requirements for UE to network relaying in 5G system has been captured in TS 22.261 clause 7.7. And existing requirements has not consider the power consumption of the 5G UE or AR terminals.

5.7.6	Potential New Requirements needed to support the use case

[PR 5.7.6-1]  Subject to operator policy, the 5G system shall support a means to provide high data rate communication with a UE such that power consumption can be reduced sufficiently.

NOTE:     High data rate communication of media over an extended period of time requires significant power consumption by the UE. In some cases, a device with no external power supply cannot sustain downloading and rendering of media over a long interval, e.g. for the duration of an entire feature film or athletic event, etc.

Table 5.x.6-1 – Potential key performance requirements for Immersive AR interactive experience

|  | Service bit rate: user-experienced data rate | Reliability | Power Consumption for Communication |
| --- | --- | --- | --- |
| Immersive AR Experience (note 1) | [200-2000] Mbit/s | [> 99.9%] | [2.3*10^-10 – 2.3*10^-9]J/bit (note 2) |

[PR 5.7.6-2]  Subject to operator policy, the 5G system shall support a mechenism that enable flexible adjustment of communcation services, such that the services can be operated with reduced energy utilization.

* * * End of Change * * * *