3GPP TSG SA WG 1 Meeting #100 	S1-223464

Toulouse, France, 14 - 18 November 2022	(revision of S1-223055)

Source:	Samsung

pCR Title:	22.856 pCR: New Use Case on IMS-based 3D Avatar Communication

Draft Spec:	3GPP TR 22.856-020

Agenda item:	7.3

Document for:	Approval

Contact:	Erik Guttman

erik.guttman@samsung.com

Abstract: This new use case introduces a new teleservice to 3GPP as a use case, the ability to perform 3D Avatar communication as an enhancement to the IMS beyond voice calls, video calls, etc.

1. Introduction

In SA1 98e and 99e, aspects of the use of Avatars in different contexts were discussed as part of the FS_Metaverse use case. This new use case introduces a new teleservice to 3GPP as a use case, the ability to perform 3D Avatar communication as an enhancement to the IMS beyond voice calls, video calls, etc..

The images used in this contribution in Figure 5.A.3-1 are under creative commons licensing:

Adonis_Mazarin_Louvre_MR239.jpg (1800×3600) (wikimedia.org)

File:Bomba de Calor.jpg - Wikimedia Commons

https://commons.wikimedia.org/wiki/File:Aphrodite8.jpg

File:The Derveni krater, late 4th century B.C., an exhausted Menead seating on the shoulder of the vase, Archaeological Museum, Thessaloniki, Greece (7457828286).jpg - Wikimedia Commons

2. Reason for Change

It is proposed that this use case presents a new and attractive possible service that the 5G system could offer.

revision in 464

- remove avatar from the requirement [Francesco]

- requirement on the IMS not 5G system [Peter]

- the second requirement was unclear - remove it and return at the next meeting [Alice]

- remove the requirement to create a new codec [Peter]

- add EN to the second requirement [Francesco stated this should be a note... it was not clearly stated]

- add that facial data is captured [Alice]

- add that the intent is to create an animated user digital representation [Orlett requested more clarity], please see the new figure 5.x.3-1 and related text

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.856-020.

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.228: "Service requirements for the Internet Protocol (IP) Multimedia core network Subsystem (IMS)".

[3]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services".

[4]	3GPP TS 22.101: "Service principles".

[5]	3GPP TS 22.261: "Service requirements for the 5G system".

[6] 	Talaba D, Horvath I, Lee K H, "Special issue of Computer-Aided Design on virtual and augmented reality technologies in product design," Computer-Aided Design 42:361 - 363.

[7] 	Wang X, Tsai J J-H, "Collaborative Design in Virtual Environments," In: ISCA 48:1-2.

[8] 	Benford S, Greenhalgh C, Rodden T, "Collaborative virtual environments." Communications of the ACM 44: 79–85.

[9]	M. Eid, J. Cha, and A. El Saddik, "Admux: An adaptive multiplexer for haptic-audio-visual data communication", IEEE Tran. Instrument. and Measurement, vol. 60, pp. 21–31, Jan 2011.

[10]	K. Iwata, Y. Ishibashi, N. Fukushima, and S. Sugawara, "QoE assessment in haptic media, sound, and video transmission: Effect of playout buffering control", Comput. Entertain., vol. 8, pp. 12:1–12:14, Dec 2010.

[11]	N. Suzuki and S. Katsura, "Evaluation of QoS in haptic communication based on bilateral control", in IEEE Int. Conf. on Mechatronics (ICM), Feb 2013, pp. 886–891.

[12]	E. Isomura, S. Tasaka, and T. Nunome, "A multidimensional QoE monitoring system for audiovisual and haptic interactive IP communications", in IEEE Consumer Communications and Networking Conference (CCNC), Jan 2013, pp. 196–202.

[13]	A. Hamam and A. El Saddik, "Toward a mathematical model for quality of experience evaluation of haptic applications", IEEE Tran. Instrument. and Measurement, vol. 62, pp. 3315–3322, Dec 2013.

[14]	O. Holland et al., "The IEEE 1918.1 "Tactile Internet" Standards Working Group and its Standards," Proceedings of the IEEE, vol. 107, no. 2, Feb. 2019.

[15]	Lee, Donghwan et. al., “Large-scale Localization Datasets in Crowded Indoor Spaces,” NAVER LABS, NAVER LABS Europe, CVPR 2021.

[16]	3GPP TR 22.837, " Study on Integrated Sensing and Communication."

[17]	Alriksson, F., Kang, D.H., Phillips, C., Pradas, J.L. and Zaidi, A., 2021. XR and 5G: Extended reality at scale with time-critical communication. Ericsson Technology Review.

[18]	3GPP TR 26.928, "Extended Reality (XR) in 5G."

[19] 	Ge, Y., Wen, F., Kim, H., Zhu, M., Jiang, F., Kim, S., Svensson, L. and Wymeersch, H., 2020. 5G SLAM using the clustering and assignment approach with diffuse multipath. Sensors, 20(16), p.4656.

[20] 	Kim, H., Granström, K., Gao, L., Battistelli, G., Kim, S. and Wymeersch, H., 2020. 5G mmWave cooperative positioning and mapping using multi-model PHD filter and map fusion. IEEE Transactions on Wireless Communications, 19(6), pp.3782-3795.

[21] 	Liu, A., Huang, Z., Li, M., Wan, Y., Li, W., Han, T.X., Liu, C., Du, R., Tan, D.K.P., Lu, J. and Shen, Y., 2022. A survey on fundamental limits of integrated sensing and communication. IEEE Communications Surveys & Tutorials, 24(2), pp.994-1034.

[22] 	Dwivedi, S., Shreevastav, R., Munier, F., Nygren, J., Siomina, I., Lyazidi, Y., Shrestha, D., Lindmark, G., Ernström, P., Stare, E. and Razavi, S.M., 2021. Positioning in 5G networks. IEEE Communications Magazine, 59(11), pp.38-44.

[23]	Suhr, C. "S.L.A.M. and Optical Tracking for XR", https://medium.com/desn325-emergentdesign/s-l-a-m-and-optical-tracking-for-xr-cfabb7dd536f <accessed 1.9.22>

[24]	Kim, H., Granstrom, K., Svensson, L., Kim, S. and Wymeersch, H., 2022. PMBM-based SLAM Filters in 5G mmWave Vehicular Networks. IEEE Transactions on Vehicular Technology.

[25]	5GAA " C-V2X Use Cases Volume II: Examples and Service Level Requirements", 5G Automobile Assocaition White Paper,  https://5gaa.org/wp-content/uploads/2020/10/5GAA_White-Paper_C-V2X-Use-Cases-Volume-II.pdf <accessed 02.09.22>

[26]	A. Ebrahimzadeh, M. Maier and R. H. Glitho, "Trace-Driven Haptic Traffic Characterization for Tactile Internet Performance Evaluation," 2021 International Conference on Engineering and Emerging Technologies (ICEET), 2021, pp. 1-6.

[27]	Lee, L.-H., Braud, T., Zhou, P., Wang, L., Xu, D., Lin,  Z., Kumar, A., Bermejo, C., and Hui, P. All One Needs to Know about Metaverse: A Complete Survey on Technological Singularity, Virtual Ecosystem, and Research Agenda, 2021.

[28]	Halbhuber, David & Henze, Niels & Schwind, Valentin. (2021). Increasing Player Performance and Game Experience in High Latency Systems. Proceedings of the ACM on Human-Computer Interaction. 5. 1-20. 10.1145/3474710.

[29] 	Holland, O., Steinbach, E., Prasad, R.V., Liu, Q., Dawy, Z., Aijaz, A., Pappas, N., Chandra, K., Rao, V.S., Oteafy, S. and Eid, M., 2019. The IEEE 1918.1 “tactile internet” standards working group and its standards. Proceedings of the IEEE, 107(2), pp.256-279.

[30] 	Skalidis, I., Muller, O. and Fournier, S., 2022. CardioVerse: The Cardiovascular Medicine in the Era of Metaverse. Trends in Cardiovascular Medicine.

[31] 	Koo, H., 2021. Training in lung cancer surgery through the metaverse, including extended reality, in the smart operating room of Seoul National University Bundang Hospital, Korea. Journal of educational evaluation for health professions, 18.

[32] 	Mozumder, M.A.I., Sheeraz, M.M., Athar, A., Aich, S. and Kim, H.C., 2022, February. Overview: technology roadmap of the future trend of metaverse based on IoT, blockchain, AI technique, and medical domain metaverse activity. In 2022 24th International Conference on Advanced Communication Technology (ICACT) (pp. 256-261). IEEE.

[33] 	Ning, H., Wang, H., Lin, Y., Wang, W., Dhelim, S., Farha, F., Ding, J. and Daneshmand, M., 2021. A Survey on Metaverse: the State-of-the-art, Technologies, Applications, and Challenges. arXiv preprint arXiv:2111.09673.

[34] 	https://www.xrtoday.com/virtual-reality/ukrainian-doctors-perform-first-vr-surgery/

[35] 	https://www.cryptotimes.io/surgeon-performed-surgery-remotely-through-metaverse/

[36]	 Senk, S., Ulbricht, M., Tsokalo, I., Rischke, J., Li, S.C., Speidel, S., Nguyen, G.T., Seeling, P. and Fitzek, F.H., 2022. Healing Hands: The Tactile Internet in Future Tele-Healthcare. Sensors, 22(4), p.1404.

[37] 	Chen, D. and Zhang, R., 2022. Exploring Research Trends of Emerging Technologies in Health Metaverse: A Bibliometric Analysis. Available at SSRN 3998068.

[38] 	https://www.yankodesign.com/2022/05/15/the-metaverse-has-the-power-to-improve-healthcare-and-it-has-already-begun/

[39] 	3GPP TR 23.501 release-17 - https://www.3gpp.org/ftp/Specs/archive/23_series/23.501/23501-h50.zip

[40] 	3GPP TR 38.300 release-17 - https://www.3gpp.org/ftp/Specs/archive/38_series/38.300/38300-h10.zip

[41] 	3GPP TR 22.826 release-17 - https://www.3gpp.org/ftp/Specs/archive/22_series/22.826/22826-h20.zip

[42] 	Tataria, H., Shafi, M., Molisch, A.F., Dohler, M., Sjöland, H. and Tufvesson, F., 2021. 6G wireless systems: Vision, requirements, challenges, insights, and opportunities. Proceedings of the IEEE, 109(7), pp.1166-1199.

[x]	"MPEG-4 Face and Body Animation", https://visagetechnologies.com/mpeg-4-face-and-body-animation/ (accessed 2.11.22).

[y]	3GPP TS 22.105, "Services and service capabilities".

Second Change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

FACS	Facial Action Coding System

Third Change

## 5.A	Use case of IMS-based 3D Avatar Communication

### 5.A.1	Description

This use case identifies two fundamental scenarios and one sub-scenario for 3D Avatar Communication by means of the IMS. The intention of the proposal is to fully specify this system in 3GPP, to provide a standard for a new form of media to be used in telecommunication by mobile users.

An Avatar Call is similar to a Video Call in that both are visual, interactive, provide live feedback to participants regarding their emotions, attentiveness and other social information. They differ in that an Avatar call can be more private - neither revealing the environment where the caller is, nor their actual appearance. An avatar may be preferable to display to one's own face in a call for a number of reasons - a user may not feel presentable, may want to make a specific impression, or may have to communicate when only limited data communication is possible. The key difference between an Avatar Call and a Video Call is that the Avatar Call requires only a very constrained data rate, e.g. 5 kbps, to support.

This use case is timely because the key enabling technologies have reached a sufficient maturity. The key avatar technologies are the means to (1) capture facial and calculate values according to a model, (2) efficiently send both media and model components through a communication chanel, both initially and over time, (3) produce media for presentation to a user for the duration of the communication. We anticipate services will be increasingly available in the coming months and years. The current approaches under development are effectively proprietary and they are not integrated with the IMS.

The scenarios considered in this use case are:

1(a). IMS users initiate an avatar call.

1(b). An IMS users initiate a video call, but one (or both) users decide instead to provide Avatar Call representation instead of video representation.

For both 1(a) and 1(b) the goal is to capture sensing data of the communicating users (especially facial data) to create an animated user digital representation (avatar). This media is provided to communicating users as a new teleservice user experience enabled by the IMS.

2. A user interacts with a computer generated system. Avatar communication is used to generate an appearance for a simulated entity with whom the user communicates.

### 5.A.2	Pre-conditions

Users Adonis and Aphrodite are 3GPP subscribers. Both have terminal equipment sufficient to capture their facial expression and movements adequately for computing avatar modeling information. The terminal equipment also includes a display, e.g. a screen, to display visual media. The terminal equipment is capable of initiating and terminating the IMS multimedia application 'avatar call.' The terminal equipment is also capable of capturing the facial appearance and movements sufficiently to produce data required by a Facial Action Coding System (FACS).

A network accessible service is capable of initiating and terminating an IMS session and the IMS multimedia application 'avatar call.'

### 5.A.3	Service Flows

1(a).	Avatar Call

i.	Adonis is on a business trip and filthy after a day servicing industrial equipment. He calls Aphrodite, who is several time zones away and reading in bed after an exhausting day.

ii.	Adonis doesn't want to initiate a video call since he hasn't had a chance to clean up and is still at work, surrounded by ugly machines.  He initiates an 'avatar call' explicitly with his terminal equipment interface.

iii.	Aphrodite, several time zones away, reading in bed after an exhausting day, is alerted of an incoming call. She sees it is from Adonis and that it is an avatar call. She accepts the call, pleased that she will be presented on the call as an avatar.

Figure 5.A.3-1: Avatar media prepared for an avatar call

1(b).	Video call falls back to an Avatar call

i.	Adonis is striking as can be, and standing at an awe-inspiring vista on mount Olympus. He initiates a video call with Aphrodite.

ii.	Unfortunately, Adonis has forgotten to consider the time zone difference. For Aphrodite, it is the middle of the night. What's more, Aphrodite has been up for several hours in the middle of the night to clean up a mess made by her sick cat. While she wants to take the call from Adonis, she prefers to be presented by an avatar, and not to take the call as a video call from her side. She explicitly requests an 'avatar presentation' instead of a 'video presentation' and picks up Adonis' call.

iii.	The call between Adonis and Aphrodite is established. Adonis sees Aphrodite's avatar representation. Aphrodite sees Adonis in the video media received as part of the call.

iv.	Adonis walks further along the mountain trail while still speaking to Aphrodite. The coverage gets worse and worse until it is no longer possible to transmit video uplink adequately. Rather than switching to a voice-only call, Adonis activates 'avatar call' representation. This requires very little data throughput.

v.	Adonis and Aphrodite enjoy the rest of their avatar call.

2.	Aphrodite calls automated customer service

i.	Aphrodite calls a customer service of company "Inhabitabilis" to initite a video call.

ii.	Inhabitabilis customer service employs a 'receptionist' named Nemo, who is actually not a peson at all. He is a software construct. There is an artificial intelligence algorithm that generates his utterances. At the same time, an appearance is generated as a set of code points using a FACS system, corresponding to the dialog and interaction between Aphrodite and Nemo.

iii.	Aphrodite is able to get answers to her questions and thanks Nemo.

In all the above scenarios, the following applies.

UE1

IMS

UE2

(a)

(b)

(c)

Figure 5.A.3-1: Avatar functional flow (capture, encode, transmit, decode, display)

Editor's Note: Figure 5.A.3-1 shall be replaced with a superior figure in future.

Once the avatar call is established the communicating parties provide information uplink. The terminal (a) captures facial information of the call participants and (b) locally determines an encoding that captures the facial information (e.g. consisting of data points, coloring and other metadata.) This information is transmitted as a form of media uplink, and provided by the IMS to the other participant(s) in the avatar call. When (c) the media is received by a participant, the media is rendered as a two (or three) dimensional digital representation, shown above as the 'comic figure' on the right.

### 5.A.4	Post-conditions

In each of the scenarios above, avatar media provides an acceptable interactive choice for a video call experience. The advantages are privacy, efficiency and ease of integration with computer software to animate a simulated conversational partner.

### 5.A.5	Existing feature partly or fully covering use case functionality

TS 22.228 define the service requirements for IMS. IMS supports different IMS multimedia applications. IMS supports a wide range of services, notably voice and video calls. There is extensive support for services, tightly integrated with the 3GPP system, with extensive support for roaming and integration with both PSTN and ISDN telephony, emergency services and more. The requirements for a 3D avatar application are largely covered by existing requirements in the 5G standard for IMS.

TS 22.173 defines the media handling capabilities of the IMS Multimedia Telephony service

The specific gaps that are addressed in 5.A.6 include: extended feature negotiation, enabling the user to decide whether to present video or avatar communciation, the ability to support Avatar communication and content efficiently, the ability to support standardized Avatar media in the 5G system.

The following KPIs are easily supported by the 5G system. They are included in order to contrast the requirements of an avatar call with a video call.

| Use Case | Characteristic parameter (KPI) |  |
| --- | --- | --- |
|  | End-to-end latency | Service bit rate: user-experienced data rate |
| Avatar call | [TBD] | <5 kbps [x] |
| Video call | < 150 msec preferred <400 msec limit Lip-synch: < 100 msec [y] | 32-384 kb/s [y] |
| NOTE: The video call KPIs are from TS 22.105 and have not changed since Rel-3. Actual transactional video call parameters may be higher now. |  |  |

### 5.A.6	Potential New Requirements needed to support the use case

[P.R. 5.A.6-1]	The IMS shall allow multimedia conversational communications between two or more users providing real time conversational transfer of animated user digital representation and speech data.

End of Changes