3GPP TSG SA WG 1 Meeting #100 	S1-223501

Toulouse, France, 14 – 18 November 2022	 (revision of S1-223157)

Source:	vivo

pCR Title:	Update use case on sleep monitoring

Draft Spec:	3GPP TR22.837

Agenda item:	7.2

Document for:	Approval

Contact:	Amy Zhang, amy.zhang@vivo.com

Abstract: <provide a short description of the content>

1. Introduction

To add missing KPIs, and delete ENs.

2. Reason for Change

EN “The requirement is FFS” is deleted.

- The reason of FFS is that people have a concern about which description is better, “perform sensing” or “perform sensing measurement”.

- To be consistent with other potential requirements (e.g. clause 5.2, 5.7, 5.8, 5.14), it is proposed to delete the EN, and utilize “perform sensing”

EN “it is FFS of whether there are or what additional requirements are needed to support this use case” is deleted, as 2 corresponding requirements and KPIs are added.

- Requirement is on how the 5G system provides the sensing results, including event trigger use case.

- Requirement is on how the 5G system performs the sensing measurement considering different sensing requirements as received from 3rd party.

- KPIs are considered to have the following:

  - Sensing service area: Indoor bedroom

  - Confidence Level: [95%]

  - Motion (i.e. Respiration) rate accuracy: [<=2 times/min], [ 0.033Hz]

  - Refreshing rate: [1min], [0.0167Hz]

  - Latency: [10s]

  - Max sensing service latency: [60s]

  - Missed detection: [5%]

  - False alarm: [5%]

The sleep monitoring application is executed in an indoor environment, especially in a bedroom where people sleep. To achieve accuracy in detecting the respiration rate, we need the sensing system, i.e. 5G system to have 95% confidence. The accuracy is on respiration rate, which is derived from processing the micro doppler shift, the vibration caused by the chest movement when people breathe. Missed detection is setting performance requirements on the event that is requested by the application server, whether or not the sensing system can detect this event. A false alarm is also setting performance requirements for the event.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR22.837 v0.2.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	W. Favoreel, "Pedestrian sensing for increased traffic safety and efficiency at signalized intersections," 2011 8th IEEE International Conference on Advanced Video and Signal Based Surveillance (AVSS), 2011, pp. 539-542, doi: 10.1109/AVSS.2011.6027406.

[3]	Advances in Wildlife Crossing Technologies: https://highways.dot.gov/public-roads/septoct-2009/advances-wildlife-crossing-technologies.

[4]	Protection Detection: Making Roads Safe for Drivers and Wildlife: https://onlinepubs.trb.org/onlinepubs/webinars/201118.pdf.

[5]	F. Liu et al., "Integrated Sensing and Communications: Towards Dual-functional Wireless Networks for 6G and Beyond," in IEEE Journal on Selected Areas in Communications, doi: 10.1109/JSAC.2022.3156632.

[6]	T. S. Rappaport, G. R. MacCartney, M. K. Samimi and S. Sun, "Wideband Millimeter-Wave Propagation Measurements and Channel Models for Future Wireless Communication System Design," in IEEE Transactions on Communications, vol. 63, no. 9, pp. 3029-3056, Sept. 2015, doi: 10.1109/TCOMM.2015.2434384.

[7]	C. Han, Y. Bi, S. Duan and G. Lu, "Rain Rate Retrieval Test From 25-GHz, 28-GHz, and 38-GHz Millimeter-Wave Link Measurement in Beijing," in IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing, vol. 12, no. 8, pp. 2835-2847, Aug. 2019, doi: 10.1109/JSTARS.2019.2918507.

[8]	IEEE 802.11-18/0611r16: “Wireless LANs, WiFi Sensing Uses Cases”

[9]	TEM STANDARDS TEM STANDARDS AND RECOMMENDED PRACTICE: https://unece.org/fileadmin/DAM/trans/main/tem/temdocs/TEM-Std-Ed3.pdf

[10]	ETSI TS 103 300-2: "Intelligent Transport Systems (ITS); Vulnerable Road Users (VRU) awareness; Part 2: Functional Architecture and Requirements definition; Release 2".

[11]	3GPP TR 22.856, "Localized Mobile Metaverse Services".

[12]	3GPP TR 29.238, "Uplink Streaming".

[13]	“Velodyne™ LiDAR VPL-16 User Manual,” 63-9243 Rev. E, Velodyne™ LiDAR, https://velodynelidar.com/wp-content/uploads/2019/12/63-9243-Rev-E-VLP-16-User-Manual.pdf  <accessed 9.8.22>

[14] 		Liu, A., Huang, Z., Li, M., Wan, Y., Li, W., Han, T.X., Liu, C., Du, R., Tan, D.K.P., Lu, J. and Shen, Y., 2022. A survey on fundamental limits of integrated sensing and communication. IEEE Communications Surveys & Tutorials, 24(2), pp.994-1034.

[15] 		https://medium.com/desn325-emergentdesign/s-l-a-m-and-optical-tracking-for-xr-cfabb7dd536f

[16] 		Dwivedi, S., Shreevastav, R., Munier, F., Nygren, J., Siomina, I., Lyazidi, Y., Shrestha, D., Lindmark, G., Ernström, P., Stare, E. and Razavi, S.M., 2021. Positioning in 5G networks. IEEE Communications Magazine, 59(11), pp.38-44.

[17]	T. Murakami et al, “Wildlife Detection System Using Wireless LAN Signal,” in NTT Technical Review vol.17, No.6, pp. 45-48, June 20019, https://www.ntt-review.jp/archive/ntttechnical.php?contents=ntr201906fa13.pdf&mode=show_pdf

[18]	Eradication of elephant mortality and injury due to railway accidents through automatic tracking and alert system in IEEE Conference Publication, IEEE Xplore

[19]	Impact of wild animals (deer and bears) on train operations 210616_KO_Animal2.pdf (jrhokkaido.co.jp) (in Japanese) [z]	Rail Industry Safety Induction Handbook: https://railsafe.org.au/__data/assets/pdf_file/0009/32022/Rail-Industry-Safety-Induction-RISI-Handbook-V5.1.pdf

[20]	Rail Industry Safety Induction Handbook: https://railsafe.org.au/__data/assets/pdf_file/0009/32022/Rail-Industry-Safety-Induction-RISI-Handbook-V5.1.pdf

[21]	Society of Automotive Engineers (SAE), “Taxonomy and definition for terms related to Driving automation systems for on-Road Motor Vehicles”, https://www.sae.org/standards/content/j3016_202104/

[22]	Census of Fatal Occupational Injuries Summary, 2020   https://www.bls.gov/news.release/cfoi.nr0.htm

[23]	Javed MA, Muram FU, Hansson H, Punnekkat S, Thane H. Towards dynamic safety assurance for Industry 4.0. Journal of Systems Architecture. 2021 Mar 1; 114:101914.

[24]	American National Standards Institute/Industrial Truck Safety Development Foundation, Safety standard for driverless, automatic guided industrial vehicles and automated functions of manned industrial vehicles, December 2019, 2019, [Online] http://www.itsdf.org.

[25]	Moore, Erik George, "Radar Detection, Tracking and Identification for UAV Sense and Avoid Applications" (2019). Electronic Theses and Dissertations. 1544.

[26] 	I. Sobron, I. Landa, I. Eizmendi and M. Velez, "Adaptive TOA Estimation with Imperfect LOS and NLOS Knowledge in UWB Positioning Systems," 2020 IEEE SENSORS, 2020, pp. 1-4. available at: https://sci-hub.wf/10.1109/sensors47125.2020.9278589

[27] 	L. Wang, Z. Zhang, X. Di and J. Tian, "A Roadside Camera-Radar Sensing Fusion System for Intelligent Transportation," 2020 17th European Radar Conference (EuRAD), 2021, pp. 282-285.

[28]	5GAA_White_Paper_C-V2X Use Cases Volume II: Examples and Service Level Requirements

[29]	Moore, Erik George, "Radar Detection, Tracking and Identification for U AV Sense and Avoid Applications" (2019). Electronic Theses and Dissertations. 1544.

[30]	Moore, Erik George, "Radar Detection, Tracking and Identification for U AV Sense and Avoid Applications" (2019). Electronic Theses and Dissertations. 1544

[31]	X. Liu, J. Cao, S. Tang and J. Wen, "Wi-Sleep: Contactless Sleep Monitoring via WiFi Signals," 2014 IEEE Real-Time Systems Symposium, 2014, pp. 346-355, doi: 10.1109/RTSS.2014.30.

[32]	Chen V C. The micro-Doppler effect in radar. Artech house, 2019.

[33]	3GPP TS 22.261: “Service requirements for the 5G system”.

[34]	A. Chebrolu, "FallWatch: A Novel Approach for Through-Wall Fall Detection in Real-Time for the Elderly Using Artificial Intelligence", 2021 Third International Conference on Transdisciplinary AI (TransAI), 2021, pp. 57-63, doi: 10.1109/TransAI51903.2021.00018, https://ieeexplore.ieee.org/document/9565618

[35]	B. A. Alsaify et al., “A CSI-Based Multi-Environment Human Activity Recognition Framework” Applied Sciences 12, no. 2: 930, 2022. https://doi.org/10.3390/app12020930

[36]	U. Saeed U et al., "Discrete Human Activity Recognition and Fall Detection by Combining FMCW RADAR Data of Heterogeneous Environments for Independent Assistive Living", Electronics 10(18):2237, 2021. https://doi.org/10.3390/electronics10182237

[37]	C. Dou, H. Huan, "Full Respiration Rate Monitoring Exploiting Doppler Information with Commodity Wi-Fi Devices". Sensors 21, 3505, 2021. https://www.ncbi.nlm.nih.gov/pmc/articles/PMC8157398/

[38]	J. Pu, H. Zhang, "RF-Heartbeat: Robust and Contactless Heartbeat Monitoring Based on FMCW Radar", 2021. TechRxiv Preprint. https://doi.org/10.36227/techrxiv.15021645.v2

[xa]	https://www.rwjbh.org/trinitas-regional-medical-center/treatment-care/sleep-disorders/sleep-apnea/

[xb]	https://my.clevelandclinic.org/health/articles/10881-vital-signs

* * * Next Change * * * *

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

sensing measurement data: data collected about radio/wireless signals impacted (e.g. reflected, refracted, diffracted) by an object or environment of interest for sensing purposes.

sensing measurement process: process of collecting sensing measurement data.

sensing result: the information derived from processing sensing measurement data.

NOTE: Examples of sensing result are characteristics of an object or environment, etc.

Sensing service area location:  an area location whether with or without obstacle, the 5G system can provide sensing service with certain quality.

transparent sensing: sensing measurements are communicated such that they can be discerned and interpreted by the 5GS, e.g. the data is communicated using a standard protocol to an interface defined by the 5GS.

Motion rate accuracy: describes the closeness of the measured magnitude of the target object’s vibration frequency caused by part(s) of the target object to the true magnitude of the target object’s vibration frequency.

Editor’s Note: terminology of motion rate accuracy needs to be refined.

* * * Next Change * * * *

## 5.15	Use case on contactless sleep monitoring service

### 5.15.1	Description

Compared with wearable devices, contactless sensing technologies have more advantages in health status detection. With more room has reserved for un-intrusion, contactless sensing-based health monitoring gives people, especially who view object contact as uncomfortable, more options. 3GPP system are designed for catering people’s communication purpose, whose wireless signals are very rich and can be accessible ubiquitously. With additional processing, 3GPP system will breed new opportunities with contactless sensing technologies applied, such as smart health, smart home, smart city and even smart space.

Sleep Monitoring application describes the case that a human’s sleep situation is monitored without any wearable device [31]. Instead of utilizing capacitors as propagation medium, Sleep Monitoring application effectively reuses the current ubiquitously accessible medium, that is wireless signals to realize the sensing purpose. People’s presence, movement and even respiration will affect the wireless signal propagation, which on the receiving side will be presented as the fluctuation of waveform’s intensity, phase shift and etc.

Figure 5.15.1-1 describes how the wireless signals that propagated via the established direct network connection (i.e. between the radio access network and 5G UE) will be affected and distorted by the target sensing object. Generally, when people is sleeping, regular chest rise and fall will cause additional vibration of the target object when detecting the doppler [37], this is defined as the micro doppler effect in radar [32]. By observing the micro doppler effect, people’s respiration rate per minute can be counted.

Figure 5.15.1-1: People’s respiration affected 3GPP wireless signal propagation in an indoor environment

NOTE 1: The transmitter as shown in Figure 5.15.1-1 is an indoor small base station as described in TS22.261 [33].

NOTE 2: The transmitter as shown in Figure 5.15.1-1 can also be a CPE that is used for this service.

This sleep monitoring application can help to diagnose early symptoms of some diseases, e.g. milder symptoms of sleep apnea before it develops worse [xa]. Through monitoring people’s breathing, i.e. respiration rate, and the breathing stoppage duration, the application server can give instructions to the user on whether or not the user is experiencing sleep apnea, and the user in return can adjust lifestyles such as losing weight or quitting smoking to avoid worse cases.

-	A person's respiratory rate is the number of breaths you take per minute. The normal respiration rate for an adult at rest is 12 to 20 breaths per minute. A respiration rate under 12 or over 25 breaths per minute while resting is considered abnormal [xb].

-	The breathing stoppage duration is the amount of time that a sleep apnea patient stops breathing, which can be from 10 seconds to two minutes or more [xa]. We take breathing stoppage duration = 10 seconds for example as the trigger of the event reporting to the application server. When the user triggers this sensing service, the sensing system will monitor this special event and report it to the application server.

### 5.15.2	Pre-conditions

The device installing this sleep monitoring application is 5G UE.

There is a service agreement between MNO and sleep monitoring operator. The MNO can also be the sleep monitoring application provider.

### 5.15.3	Service Flows

1. The application user Bob triggers the sleep monitoring application on the 5G UE. When the application server receives the request, the application server contacts the 5G system to trigger the sensing service to monitor Bob’s respiration rate.

2. 5G system discovers a base station (or CPE) to start the sleep monitoring sensing service.

3. The base station (or CPE) coordinates with Bob’s phone (5G UE) to perform the sensing measurement. The base station and the 5G UE can be transmitter and receiver or vice versa. The receiver measures the 5G wireless signals (e.g., number of detected transmission paths, micro doppler shift, etc.) and collects them as the sensing measurement data.

4. Sensing measurement data is processed to derive the sensing results (e.g. respiration rate) locally or is provided to the 5G network: 5G network processes the sensing measurement data to derive the sensing results and exposes the sensing results to the sleep monitoring application server.

5. The 5G UE receives the sleep monitoring feedback from the application server and shows it to the application user Bob. Bob may have sleep apnea and needs the application to further monitor his breathing stoppage duration. An event with “breathing stoppages duration = 10 seconds” is triggered by Bob and received by the application server, which then contacts the 5G system to trigger this event.

6. 5G system adjusts the sensing measurement process and executes Steps 3-5. When the event report criteria are satisfied, i.e. Bob is detected to have a 10-second breathing stoppages duration, the application server will receive the notification sent by 5G system.

### 5.15.4	Post-conditions

The user experiences the sleep monitoring application enabled by the 5G network. Bob changes his lifestyle, he does more exercise, and tries to lose weight to avoid the sleep apnea problem.

### 5.15.5	Existing feature partly or fully covering use case functionality

None

### 5.15.6	Potential New Requirements needed to support the use case

[PR.5.15.6-1] The 5G system shall support mechanisms to discover and configure a UE and a base station to perform sensing measurements in a certain sensing service area.

[PR.5.15.6-2] The 5G system shall support mechanisms to derive and expose sensing results to a trusted 3rd party.

[PR.5.15.6-4] The 5G system shall be able to provide 5G wireless sensing service with the following KPIs:

| Scenario |  |  | Sensing service area |  | Confidence level [%] |  |  | Accuracy of positioning estimate by sensing |  |  |  | Acuracy of Velocity estimate by sensing |  |  |  |  |  | Motion rate accuracy |  |  |  | Sensing resolution |  | Max sensing service latency [ms] |  | Refreshing rate [s] |  | Missed detection [%] |  | False alarm [%] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Horizontal [m] |  | Vertical [m] |  | Horizontal [m/s] |  |  | Vertical [m/s] |  |  |  |  |  |  | Range resolution (horizontal/ vertical) [m x m] |  | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |  |  |  |
| Sleep Monitoring | Indoor (Bedroom) | 95% | - | - | - | - | 2 times/min [0.033Hz] (NOTE 1) | - | - | 60s | 1 min | 5% (NOTE 2) | 5% (NOTE 2) |
| NOTE 1: we take respiration rate = 18 times/min as reference, any detected value in [16,20] satisfies accuracy requirements NOTE 2: we take detect event = “breathing stoppages duration >= 10 seconds” as reference |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *

<Proposed change in revision marks>