| 3GPP TR 22.882 V0.2.0 (2022-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group TSG SA; Study on Energy Efficiency as service criteria (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Abbreviations	7

4	Overview	7

5	Use cases	8

5.1	Energy Utilization as a Performance Criteria for Best Effort Communication	8

5.1.1	Description	8

5.1.2	Pre-conditions	9

5.1.3	Service Flows	9

5.1.4	Post-conditions	9

5.1.5	Existing feature partly or fully covering use case functionality	9

5.1.6	Potential New Requirements needed to support the use case	9

5.2	Use case on supporting different energy efficiency modes in industrial campus	10

5.2.1	Description	10

5.2.2	Pre-conditions	10

5.2.3	Service Flows	10

5.2.4	Post-conditions	10

5.2.5	Existing features partly or fully covering the use case functionality	10

5.2.6	Potential New Requirements needed to support the use case	11

5.3	Energy usage exposure considering possible deployment scenarios	11

5.3.1	Description	11

5.3.2	Pre-conditions	11

5.3.3	Service Flows	11

5.3.4	Post-conditions	11

5.3.5	Existing features partly or fully covering the use case functionality	11

5.3.6	Potential New Requirements needed to support the use case	12

5.4	Energy usage information exposure under NPN RAN sharing	12

5.4.1	Description	12

5.4.2	Pre-conditions	12

5.4.3	Service Flows	12

5.4.4	Post-conditions	12

5.4.5	Existing features partly or fully covering the use case functionality	12

5.4.6	Potential New Requirements needed to support the use case	12

5.5	Use Case on Service Energy Monitoring by an Application Server	12

5.5.1	Description	12

5.5.2	Pre-conditions	13

5.5.3	Service Flows	13

5.5.4	Post-conditions	14

5.5.5	Existing feature partly or fully covering use case functionality	14

5.5.6	Potential New Requirements needed to support the use case	14

6	Consolidated potential requirements	15

7	Conclusion and recommendations	15

Annex A (informative): Existing Energy Efficiency Standardisation	16

Annex B (informative): Change history	19

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document provides stage 1 use cases and potential 5G requirements on the following aspects regarding enhancements to Energy Efficiency of 5G network.

- Defining and supporting energy efficiency criteria as part of communication service to user and application services,

- Supporting information exposure of systematic energy consumption or level of energy efficiency to vertical customers.

- Gap analysis between the identified potential requirements and existing 5GS requirements or functionalities.

- Potential requirements on security, charging and privacy aspects.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ETSI ES 201 554: "Environmental Engineering (EE); Measurement method for Energy efficiency of Mobile Core network and Radio Access Control equipment"

[3] 	ETSI ES 203 228: "Environmental Engineering (EE); Assessment of mobile network energy efficiency"

[4] 	 GSMA Intelligence: "Going green: benchmarking the energy efficiency of mobile", June 2021.

[5] 	3GPP TR 21.866: "Study on Energy Efficiency Aspects of 3GPP Standards"

[6] 	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G"

[7] 	3GPP TR 28.813: "Management and orchestration; Study on new aspects of Energy Efficiency (EE) for 5G"

[8] 	3GPP RP-213554: "Study on network energy savings for NR "

[9]	ETSI ES 202 336-1: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 1: Generic Interface".

[10]	ETSI ES 202 336-12: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[11]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[12]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[13]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[14]	"LS on Energy Efficiency as guiding principle for new solutions", Sent TSG SA #94e 12.21 (SP-211621), Received SA1 #97e 02.22 S1-220063.

[15]	3GPP TS 22.261, " Service requirements for the 5G system".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Energy efficiency modes: refer to the power saving mode, switching off status which are defined in TS 28.310.

Editor’s Note: the definition of energy efficiency modes is for FFS.

energy charging rate: a means of determining the energy usage consequence (use of energy credit) associated with charging events.

NOTE:	The rate listed above is a charging rate, completely distinct and unrelated to the 'Maximum Energy Utilization Rate' as discussed in 5.1.

energy credit: a quantity of credit associated with the subscriber that can be used for credit control by the 5G system.

maximum energy credit limit: a policy establishing an upper bound on the quantity of energy used by the 5G system to provide services provided to a specific subscriber.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Overview

Climate change and globally energy shortage are issues that requires international cooperation and coordinated solutions at all levels, many regions and countries have published related policies and requirements to control carbon release and promote energy efficiency. These policies have made energy efficiency a strategic priority for many telecoms operators around the world. Energy efficiency has been considered in many standard groups and specifications.

The existing studies concentrate more on how to satisfy user experience and try to achieve energy efficiency at the same time and achieve energy efficiency within the network, so the requirements, use cases and solutions are basically within the network itself. Verticals and customers have no approach for energy efficiency related information from network.

Introducing energy efficiency as a service will allow users to have the choice to select proper energy efficiency criteria as well as other network performance parameters when they need it, which may include:

- Define and support energy efficiency criteria as part of communication service to user and application services.

- Provide information exposure on systematic energy consumption or level of energy efficiency to vertical customers.

Such as in satellite and terrestrial convenience scenario, for some regions where both satellite and terrestrial coverage exist, energy saving could be taken as a dimension while providing the communication service, users or operators could have the choice to find out a best way in satisfying both user experience and energy efficiency. From another perspective, the network could also react to different energy consumption modes of application or adjust network resource.

Both the two aspects above need more interaction between application and network on energy consumption status. It is worth considering how to deliver services with energy efficiency as service criteria, associated with verticals’ preferences, and how to support the policy of handling energy as part of a subscription.

5	Use cases

5.1	Energy Utilization as a Performance Criteria for Best Effort Communication

5.1.1	Description

Currently energy utilization and efficiency can be monitored and considered through OAM and network operation, but not as a service performance criterion, as for example bit rate, latency or availability. The guidance from SA to all working groups in [14] states

"The EE-specific efforts so far undertaken e.g., in SA5 have aimed mostly at improving the energy efficiency by impacting the operations of the system. As we now are starting to specify the 5G-Advanced features, TSG SA kindly requests the recipient WGs and TSGs to consider EE even more as a guiding principle when developing new solutions and evolving the 3GPP systems specification, in addition to the other established principles of 3GPP system design.

TSG SA clarifies that in addition to EE, other system level criteria shall continue to be met (i.e.  the energy efficiency aspects of a solution defined in 3GPP is not to be interpreted to take priority or to be alternative to security, privacy, complexity etc. and to meeting the requirements and performance targets of the specific feature(s) the solution addresses)."

There is an important type of traffic where energy efficiency policy, for example a maximum amount of energy to be utilized could be applied without conflict with this guidance. Best effort traffic is a type of traffic that is provided as a service to customers everything else being equal. Of course security, privacy and complexity principles will not be sacrificed, but there is no conflict between a service policy that constrains performance (e.g. latency, throughput, even availability) on the basis of energy utilization and a best effort service, since there are no guarantees in the case of best effort traffic. We can say that best effort traffic is not associated with QoS policy service performance level criteria.

Today the 5GS works to support services efficiently, though does not take into account energy utilization at the service level. The use case explores a particular opportunity to identify this information and use it to make more efficient use of all network resources without sacrificing service quality. In particular, information gathered through OAM, and in the future possibly from the network (see 6.F.5 which identifies a gap and opportunity), can be leveraged to make it possible to employ energy utilization information as part of service delivery.

In the following use case, the possibility of using energy utilization as a new service criterion for this less constrained type of mobile telecommunication service is explored.

A large scale logistics company L has deployed a large number of communicating components. These are integrated into vehicles, palettes, facilities, etc.. Essentially, IoT terminals enable remote tracking and monitoring functions. The information gathered is relevant, but not constrained with respect to latency. In fact, eventual delivery (e.g. after hours or even a full day) of communication is entirely acceptable for L. The MNO M offers a 'green service' which limits the rate of energy utilized for communication over a particular time interval (e.g. per day) and this service is appropriate for L, whose overall corporate goals are also served by 'green service,' as they strive to operate with energy efficiency.

5.1.2	Pre-conditions

L deploys many UEs with associated 'green service' subscriptions from M. These subscriptions policies include the following criteria:

-	Best Effort Service (service that is not associated with QoS policy service performance level criteria)

-	Energy Constraints applied to service delivery

5.1.3	Service Flows

1.	The fleet of trucks belonging to L leave the logistic center located in the middle of the uninhabited region 100s of kilometers northeast of the major city Erehwon. There are many devices located in this fleet. The trucks and their contents comprise a physically dense group of UEs, all communicating periodically with the network. This 'massive IoT' group leaves the coverage of the logistics center. The network coverage over the road through the uninhabited region is very sparse.

2.	As the trucks proceed into extreme low coverage, the energy utilized to communicate with the IoT devices increases. This energy utilization increase is monitored by the 5G System and can be aggregated, e.g. at the slice level.

3.	The 'green service' policy for the service provided to L includes a maximum energy utilization rate. At a certain point the IoT communication of the fleet exceeds this maximum energy utilization rate.

4.	The policy indicates that latency can be traded off with energy utilization for service to L; the communication service is delay tolerant in this condition. As the energy utilization rate has exceeded the maximum, the latency is increased to enforce this policy. In effect, L's fleet receives very limited service, with high latency, even for a limited period of time, no service at all.

NOTE:	This use case does not describe how latency is increased, but does assume that this increase will result in a reduction of energy utilization. It is possible to reduce energy utilization by offering less service.

5.1.4	Post-conditions

The IoT devices in the fleet belonging to L is able to communicate with varying latency, depending on the energy utilization required to serve the devices. When the UEs are in poor coverage, they communicate seldom, when under good coverage, they can communicate more frequently.

The total energy utilization of the M's network has reduced while still providing adequate service to the customer L.

It is important to emphasize that there has been no trade off between 'energy efficiency' and 'service quality.' The customer L received what was necessary while using less energy precisely because the energy utilization was taken into account in the service delivery.

5.1.5	Existing feature partly or fully covering use case functionality

The 5G System can monitor energy utilization. The existing energy utilization monitoring is done at an OAM level, per network node, per cell and per slice. The number of UEs per network node, cell and slice are also known. Please see Annex A for an overview to existing energy efficiency standardization, which includes the determining energy consumption for use in calculating energy efficiency.

The 5G System can enforce performance criteria, as described in TS 22.261, 6.7 [15]. Most of the enforcement requirements refer to prioritization, but policies that result in other enforcement are possible too, e.g. gating, charging, credit control, restrictions with respect to maximum allowed resources, etc.

Gap: there is currently no means to determine the per UE or per QoS Flow (in stage 1 terms) service flow energy utilization. This information is not included in network data analytic services.

5.1.6	Potential New Requirements needed to support the use case

[PR 5.1.6-1]	Subject to operator’s policy, the 5G system shall support subscription policies that define a maximum energy utilization rate for services without QoS criteria (also termed "best effort" services.)

[PR 5.1.6-2]	Subject to operator’s policy, the 5G system shall support enforcement of subscription policies that define a maximum energy utilization rate for services without associated QoS criteria (also termed "best effort" services.)

Editor's Note: This note applies to both potential requirements above. It is FFS what are the granularities of the subscription (e.g., per service?  per user? ) associated with the energy utilization rate.

Editor's Note: This note applies to both potential requirements above. It is FFS what are the granularities of the energy utilization rate and how the energy utilization rate can be applied to a particular service and what the implications are for the potential new requirements.

5.2	Use case on supporting different energy efficiency modes in industrial campus

5.2.1	Description

Industrial campus are very typical scenarios of edge computing and local traffic offload. Dedicated network facilities are usually deployed near the campus for lower latency and local data protection. This brings a problem that these network facilities are used only for the campus, so while the manufacturing load is light or during vacation, these network facilities will be in very light load or even no load. Under this scenario, the energy consumption of these network facilities will be unnecessary.

5.2.2	Pre-conditions

Factory F, a smart manufacturing factory locates in a remote area outside city. Factory F requires low latency in AGV transporting services and local data processing using computing vision to support image comparison for fault detection in circuit boards. Factory F has an agreement with Operator T on the communication service with certain SLA. As the manufacturing activity is not consistent, Operator T provides a replaceable SLA which can be used during off-peak time. This replaceable SLA can save energy consumption by changing energy efficiency mode of network functions used locally, and the action can be activated either by pre-configured policy or by notification from Factory F.

5.2.3	Service Flows

1.  Operator T provided a dedicate set of UPF and MEC platform for factory F. Factory F is an environmental friendly enterprise are care about energy efficiency along its whole industrial chain.

2. While the manufacturing load of Factory F reaches a certain threshold, which is evaluated by Factory F, a notification will be sent to Operator T.

3.  Operator T will change the energy efficiency mode of network functions based on the pre-agreed policy with Factory F.

4. After one year of this kind of usage, the charging information of communication service will consider the actual usage time of different energy efficiency mode.

5.2.4	Post-conditions

Manufacturing of Factory F will be not affected, while energy consumption of communication could be saved by dynamically changing energy efficiency modes of network functions, and the expenses of communication service will be lower to encourage this kind of environmental-friendly action.

5.2.5	Existing features partly or fully covering the use case functionality

In TS 28.310, there’s existing requirements on switch off edge UPFs during off-peak hours

REQ-SOUPF-FUN-1: The management service producer responsible for energy saving should have the capability allowing its authorized consumer to collect the traffic load performance measurements of its edge UPFs.

REQ-SOUPF-FUN-2: The management service producer responsible for energy saving should have the capability allowing its authorized consumer to administratively prohibit selected edge UPFs from performing services for its users, either with immediate effect or only when no more users are using these UPFs.

5.2.6	Potential New Requirements needed to support the use case

[PR 5.2.6-1] 5G system shall support different energy efficiency modes of network functions based on pre-configured policy with authorised 3rd party.

[PR 5.2.6-2] 5G system shall support dynamic changes of energy efficiency modes of network functions based on pre-configured policy with authorised 3rd party.

NOTE 1: Pre-configured policy may include the time of changing energy efficiency modes, which energy efficiency modes map to which level of network function load, etc.

[PR 5.2.6-3] 5G system shall support different charging mechanisms considering different energy efficiency modes of network functions during the usage within one service.

5.3	Energy usage exposure considering possible deployment scenarios

5.3.1	Description

When considering Energy as a service or network performance criteria, it is necessary to consider different 5G network deployment scenarios, e.g. for RAN network with dual connectivity, RAN network with CU-DU deployment, RAN sharing, etc.  That means whatever the deployment scenario, the energy usage of the 5G network which relates to the industry customer is expected to be acquired and exposed to the authorized third parties.

5.3.2	Pre-conditions

The network operator A deploys 5G network “N” to serve industry customers C and D.

In the 5G network “N”, some of the gNBs can support dual-connectivity. In order to achieve more flexible deployment and reduce the cost, the operator A also deploys a large number of DUs in some hotspot area, each DU is for covering a certain area. For customer C, dual-connectivity is utilized, while for customer D, multiple DUs have been configured.

Industry customers C and D have also subscribed the “Green Energy Moni” value-added service from network operator A, thus they can access energy usage information corresponding to their respective network functions from web application provided by Operator A.

5.3.3	Service Flows

1. The 5G network “N” of operator A acquires the energy usage information of related 5G network functions serving the customers C and D.

2. Customer C asks the “Green Energy Moni” of Operator A to provide the network energy usage information associated the 5G network functions serving it via dual-connectivity deployment.

3. Operator A provides the network energy usage information to customer C.

4. Customer D asks the “Green Energy Moni” of Operator A to provide the network energy usage information associated the 5G network functions serving it via DU deployment.

5. Operator A provides the network energy usage information to customer D.

5.3.4	Post-conditions

Customers C and D can get the energy usage information of the network functions serving them, independently from NG-RAN deployment scenarios.

5.3.5	Existing features partly or fully covering the use case functionality

None.

5.3.6	Potential New Requirements needed to support the use case

[PR.5.3.6-1] Subject to operator’s policy and consent by the vertical customer, the 5G system shall be able to acquire energy usage information of the network functions serving the customer, independently from NG-RAN deployment scenarios, and expose this information to the customer and authorized third parties.

5.4	Energy usage information exposure under NPN RAN sharing

5.4.1	Description

In the practice of 5G NPN networks deployment, in order to save time and cost, RAN sharing (i.e. NG-RAN is shared by any combination of PLMNs and NPNs) is a common deployment scenario for vertical industries. The customers will concern about the energy efficiency of their communication service especially in RAN sharing cases. Thus, it is reasonable for 5G system to acquire and expose the energy usage information of the customer including it is served by RAN sharing network.

5.4.2	Pre-conditions

The 5G network operator A deploys local NPN “N1” network in factories for customer C which is sharing resource of operator A’s PLMN “R”.

Customer C has subscribed the “Green energy Moni” value-added service for its NPN “N1” from network operator A, thus it can access energy usage information corresponding to the “N1” network from web application provided by Operator A.

5.4.3	Service Flows

1. The 5G network of operator A acquires the energy usage information of the NPN ”N1” and PLMN “R”.

2. Customer C asks the “Green Energy Moni” of Operator A to provide the energy usage information of its network “N1”.

3. The operator A acquires and provides energy usage information of the network “N1” for customer C.

5.4.4	Post-conditions

Customer C can get the energy usage information of its network “N1”.

5.4.5	Existing features partly or fully covering the use case functionality

None.

5.4.6	Potential New Requirements needed to support the use case

[PR.5.4.6-1] Subject to operator’s policy and consent by the customer of NPN, the 5G system shall be able to acquire energy usage information of the NPN, including the shared network function(s) which is (are) serving the NPN, and expose this information to the NPN customer and authorized third parties.

5.5	Use Case on Service Energy Monitoring by an Application Server

5.5.1	Description

In this scenario, a service provider monitors events resulting from energy utilization policy triggers in the 5G system. These triggers correspond to monitoring policy in the 5G system as well as energy enforcement policies.

Figure 5.5.1: Monitoring of Energy Events by the 5G network for an AS

In Figure 5.5.1, the AS obtains information corresponding to the energy consequences of a UE 'A' served by the 5G network.

This use case will provide a description of a scenario in which the service provider (who operates an application server) cares about energy utilization in the 5G system as a result of the service to UE A. This could be for 3 reasons.

1 - the service provider needs to show they are saving energy.

2 - the service has an associated energy cost, and the service provider wants to reduce it. This is analogous to the user of industrial or consumer electronics when energy rates are lower, and also as an incentive to operate more efficiently.

3 - the service provider recognizes that there are policies that limit energy use (such as aggregate energy use of a slice) and controls the overall use of the service to operate within those constraints.

The use case introduces five new concepts related to new energy events and energy event monitoring.

i)	the ability for the network operator to create a 'maximum energy credit' policy, after which services are gated.

ii)	the ability for the network operator to inform an AS of the 'maximum energy credit expired' event.

iii)	the ability for the 5G system to calculate 'energy credit' use

iv)	the ability to monitor and provide to the AS the use of 'energy credits' (or other energy 'quantum');

v)	the support a new policy that establishes the energy consequence for charging control - either charging for use of energy or establishing an 'energy credit limit' for enforcement by the 5G system.

5.5.2	Pre-conditions

The UE "A" has a subscription that enables it to make use of 'best effort communication subject to energy constraints' policy for communication. This class of communication was introduced in TR 22.882, 5.1.

The application service provider "AS" is capable monitoring service aspects of the 3GPP system, e.g. through network exposure of information as described in 22.261 for QoS monitoring or 22.115 related to credit limit policy and control.

5.5.3	Service Flows

1.	AS is aware that there is an energy policy related to the service for the subscription related to UE "A". As a result, AS requests to monitor 'Energy Use', which is a kind of usage monitoring supported by the 5G system. The monitoring policy has an established 'threshold' for the 5G system to notify the AS.

In addition, the AS requests to monitor 'Out of Energy Credit' events.

2.	The 5G system provides service to UE A according to a 'best effort communication subject to energy constraints' policy, where the policy charges for energy use and also imposes an 'energy credit' limit, after which the UE A subsription is 'gated' (receives no further services from the 5G system until more 'energy credit is available.'

3.	UE A proceeds to use services of the 5G system, especially data communication. As it does so, the charging system is triggered and generates records. The 3GPP charging system uses a means to identify how much credit is used and whether a credit limit is exceeded. The 3GPP charging system in this use case also uses a means of calculating energy credits on the basis of charging events. That is, there is a 'rating policy' used to multiply a 'charging event' by an 'energy utilization' unit.

NOTE 1:	The actual amount of energy corresponding to an 'energy unit' used in energy credit control is out of scope of this use case. A mobile network operator can develop a model by which they analyze the total energy needed to provide services and assign fractions of these to each event triggered in the charging system.

4.	When the total 'energy units' exceed the reporting threshold according to the energy monitoring policy, the 5G system exposes this energy usage information to AS.

NOTE 2:	Monitoring of energy usage could be done by other means than 'energy units' corresponding to the same units as the credit limit. This could be useful for the third party. However, only by exposing units that result in charging or gating enforcement by the network operator can the third party determine the consequences of their use of services and potentially change their use of those services, e.g. to communicate sparingly, to communicate more efficiently (e.g. at times in less energy use is reported per 'byte', etc. of communication, as calculated by the third party based on their own measurements & the monitoring reports of the 5G system.)

5.	When the total 'energy units' exceed the energy credit limit, this results in the 5G system exposing this event to the AS. The AS could take some action to restore energy credit, but this is out of scope of the use case.

5.5.4	Post-conditions

The UE A's use of energy can be monitored by AS. The AS can alter their activity (e.g. communicate less intensely or less frequently) to remain within their expectation - be it to keep the charging per energy utilization to their expectation, or to avoid exhausting A's energy credit limit.

The MNO is able to create and enforce policies that attach consequences to use of energy. This can lead to energy efficient behavior on the part of service providers which is both in their interest and the interest of the MNO.

5.5.5	Existing feature partly or fully covering use case functionality

The 5G system provides support for credit limits [22.115, 8.2] and for performance monitoring [22.261]. There are a number of other events that are exposed by the 5G system to third parties by the Policy and charging control framework by the 5G System [23.503]. These events and their triggers, which are not detailed in stage 1, allow for usage monitoring to be exposed to a third party in specific circumstances, e.g. sponsored connectivity. The scenario in this use case is similar to sponsored connectivity, as the AS is a directly concerned party that seeks to operate successfully in an efficient manner, as there are charging and even gating consequences as the UE communicates with AS.

Note that the existing usage monitoring and reporting for sponsored connectivity is not sufficient to support this use case because these do not in any way take into account the energy consequence of service. Only traffic volume and time based monitoring are supported today. Other chargeable (and therefore significant events from an energy perspective) are not captured by usage monitoring as supported in the 5G system.

5.5.6	Potential New Requirements needed to support the use case

[P.R 5.5.6-1]	Subject to operator’s policy, the 5G system shall support subscription policies that define a maximum energy credit limit for services.

[P.R 5.5.6-2]	Subject to operator’s policy, the 5G system shall support subscription policies that define an energy charging rate for services.

[P.R 5.5.6-3]	Subject to operator’s policy, the 5G system shall support a means to expose energy utilization to authorized third parties for services, such that the energy utilization information clearly identifies the 'approaching' enforcement of an energy credit limit.

[P.R 5.5.6-4]	Subject to operator’s policy, the 5G system shall support a mechanism to perform energy utilization credit limit control for services. The result of the credit control is not specified by this requirement. Examples include gating, increased charging rates, etc.

Editor's Note: It is for further study how the proposed energy credit control and existing credit control are related and whether and if so how energy credit control and existing credit control are compatible. This editor's note applies to all four of the above potential requirements.

6	Consolidated potential requirements

7	Conclusion and recommendations

Annex A (informative): Existing Energy Efficiency Standardisation

## A.1 Overview of existing energy efficiency standardisation

In ETSI, GSMA and 3GPP, there were many reports, studies, specifications related to energy efficiency. And now there are also ongoing R18 studies on energy efficiency in both SA5 and RAN.

In ETSI, existing specifications cover several aspects of energy efficiency, which includes energy efficiency metrics and measurement methods for mobile core equipment, metrics and methods to measure energy performance of Mobile Radio Access Networks, measurement and monitoring of power, energy and environmental parameters for ICT equipment in telecommunications. [2] [3]

GSMA has done lots work in assessing energy consumption in different field within communication system. In "Going green: benchmarking the energy efficiency of mobile", GSMA states that 73% of the energy of the participating operators is consumed in the radio access network (RAN). The network core (13%), owned data centres (9%) and other operations (5%) account for the rest. [4] The statistics shows that energy efficiency is an end-to-end issue.

In 3GPP, energy efficiency has been studied in SA, SA5 and RAN. SA have studied system requirement and principle, provided Energy Efficiency Control Framework. [5] SA5 have specified concepts, use cases, requirements and solutions for the energy efficiency assessment and optimization for energy saving, as well as Energy Efficiency (EE) KPIs. [6] [7] RAN EE study will concentrate on definition of network energy consumption model, evaluation methodology and KPIs, also study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception. [8]

## A.2 Energy efficiency KPIs

3GPP Energy Efficiency KPI definitions are under SA5 (Telecom Management) responsibility. They are based on measurements collected on RAN or CN network elements / network functions via OA&M. The KPI calculation is a generalisation of the work in ETSI TC EE. Figure A.1-1 below shows the KPI derivation with notes to the source specifications.

Performance Measurements

(TS28.552, 3GPP SA5)

Energy Consumption Measurements a.k.a. PEE parameters (Power, Energy and Environmental)

(TS28.552 28.554, 3GPP SA5, based on input from ETSI ES 203 336-12)

3GPP generic EE KPI = Performance Energy Consumption

is a generalisation of

OA&M Collection Method

(TS28.550, TS 28.532, 3GPP SA5)

Defined in ETSI TC EE 203 228

Figure A.2-1: KPI derivation and sources

## A.3 Summary of existing energy efficiency standards

Table A.2-1 below shows the standards relevant to the FS_EnergyServ study with a synopsis taken from the Scope clause of the standard.

| SDO | Group | Standard | Summary |
| --- | --- | --- | --- |
| 3GPP |  |  |  |
|  | SA | TR 21.866: Study on Energy Efficiency Aspects of 3GPP Standards [5] | Identifies and studies the key issues and the potential solutions in defining Energy Efficiency Key Performance Indicators and the Energy Efficiency optimization operations in existing and future 3GPP networks. |
|  | SA5 | TS28.310: Management and orchestration; Energy efficiency of 5G [6] | Specifies concepts, use cases, requirements and solutions for the energy efficiency assessment and optimization for energy saving of 5G networks. |
|  |  | TS28.552: Management and orchestration; 5G performance measurements [11] | Specifies the performance measurements for 5G networks including network slicing. Performance measurements for NG-RAN are defined in this document, and some L2 measurement definitions are inherited from TS 38.314. The performance measurements for 5GC are all defined in this document. Related KPIs are defined to those measurements are defined in TS 28.554. |
|  |  | TS28.554: Management and orchestration; 5G end to end Key Performance Indicators (KPI) [12] | Specifies end-to-end Key Performance Indicators (KPIs) for the 5G network and network slicing |
|  |  | TS28.622: Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) [13] | Specifies the Generic network resource information that can be communicated for telecommunication network management purposes, including management data about energy efficiency |
|  |  | TR28.813: Management and orchestration; Study on new aspects of Energy Efficiency (EE) for 5G.[7] | Investigates the opportunities for defining new Energy Efficiency (EE) KPIs and new Energy Saving (ES) solutions. |
| ETSI |  |  |  |
|  | TC EE | ETSI ES 203 228: "Environmental Engineering (EE); Assessment of mobile network energy efficiency" [3] | Defines the topology and level of analysis to assess the energy efficiency of mobile networks (excluding terminal) |
|  |  | ETSI ES 202 336-1: "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 1: Generic Interface" [9] | Defines monitoring and control of Infrastructure Environment i.e. power, cooling and building environment systems for telecommunication centres and access network locations. |
|  |  | ETSI ES 202 336-12: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model" [10] | Defines measurement and monitoring of power, energy and environmental parameters for ICT equipment in telecommunications or datacenter or customer premises |

Table A.3-2: List of EE specifications

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | SA1#99-e | S1-222412 |  |  |  | TR skeleton | 0.0.0 |
| 2022-08 | SA1#99-e | S1-222413 S1-222414 S1-222415 S1-222416 |  |  |  | Output of approved pCRs from SA1 #99e | 0.1.0 |
| 2022-11 | SA1#100 | S1-223431 S1-223654 S1-223656 S1-223657 S1-223658 |  |  |  | Output of approved pCRs from SA1 #100 | 0.2.0 |