3GPP TSG SA WG 1 Meeting #100 	S1-223568

Toulouse, France, 14 – 18 November 2022	(revision of S1-2233354)

Source:	KPN, InterDigital, Wiliot

pCR Title:	pCR on New use case on end-to-end logistics

Draft Spec:	3GPP TR 22.840v0.2.0

Agenda item:	7.3

Document for:	Approval

Contact:	Toon Norp (Toon .norp @ tno. nl)

Abstract: <provide a short description of the content>

1. Introduction

Several Ambient IoT use cases related to logistics have been included. But these have not specifically addressed a supporting a complete logistics chain from factory to delivery at the door of an end-customer.

2. Reason for Change

Whithout a use case for a complete logistics chain, we miss the aspects of roaming support for Ambient IoT across different types of networks. Furthermore, we miss the aspect of a network that only wants to wake up / trigger some of the Ambient IoT devices that are present locally.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840v0.2.0 (all new text).

## * * * First Change * * * *

## 5.x	Use case on end-to-end logistics

### 5.x.1	Description

In a logistics scenario, Ambient IoT devices can be used to track specific goods (e.g. TVs) from the factory where they are made until delivery at the final end-customer. E.g. an Ambient IoT device could be attached to the packaging of a TV, which can then be used for tracking in the factory, in one or more warehouses on the way from factory to the customer and finally to the track the delivery at the end customer.

In this end-to-end logistics trajectory, an Ambient IoT device will encounter multiple different networks (NPNs, PLMNs) in different regions and countries. Each of these networks may have to follow different frequency regulations and may have different frequency licenses. The Ambient IoT device needs to be able to deal with these differences.

Limitations in range for Ambient IoT may can imply that some additional infrastructure needs to be deployed. However, it is not assumed that a network needs to be deployed specifically for Ambient IoT support. It should be possible to integrate Ambient IoT communication in networks that are also used for other 5G communication.

The assumption in this use case is that the Ambient IoT devices need to be woken up and triggered to send their identification. When the Ambient IoT devices are not woken up triggered they remain in a suspended state and cannot send or receive information.

The owner of a network (e.g. in a factory or warehouse) can decide which part of the network (e.g. which base stations) to use to wake up trigger Ambient IoT devices. This way, e.g. Ambient IoT devices in only part of the factory are woken up triggered. It should also be possible to trigger only a selected group of Ambient IoT devices (e.g. through broadcasting a group ID), or to trigger the Ambient IoT devices for specific actions (e.g. receive information instead of broadcasting an ID).

Figure 5.x.1-1: Tracking a cardboard box a TV is transported in by attaching an Ambient IoT Device to the box

Where the network itself does may not have the support to wake up and trigger the Ambient IoT device (e.g. in the PLMN where the TV is delivered to the door of the customer), a handheld device can may be used. This handheld device can, provide the power locally to wake up the Ambient IoT device and when needed, may relay the communication from the Ambient IoT device towards the network.

### 5.x.2	Pre-Conditions

Factory A has deploys a standalone NPN.

TV_company has an agreement with the NPN in Factory A and with PLMN_X to use a third party interface to wake up and trigger Ambient IoT devices.

The Ambient IoT device has credentials for the NPN in factory A and has credentials for a PLMN_X in Country_X. PLMN_X has roaming agreements with operators in other countries.

Warehouse B deploys a NPN implemented as a slice on PLMN_Y.

### 5.x.3	Service Flows

- TV_company uses Factory A to manufacture its TVs. After manufacturing, the TVs are packaged in a cardboard box and the box is then equipped with an Ambient IoT device (in the form of a sticker) for tracking/tracing of the TV.

- Within the Factory A, the TVs are stored for a while in internal storage. Tracking of the TVs in storage is possible through Ambient IoT. The TV_company instructs the NPN that is deployed in the Factory A to wake up trigger the TVs in the internal storage only. TVs that are still in the production area do not need to be triggered woken up. The Ambient IoT devices are triggered to send their ID. This way the TV_company can create an inventory of all TVs that are still in store at the factory. The Ambient IoT devices can also be triggered to receive information (e.g. instructions).

- A shipment of TV is sent to a warehouse in a Country_X. The warehouse uses a NPN implemented as a slice on PLMN_Y. Because of roaming agreements between PLMN_Y and PLMN_X, the Ambient IoT devices can access the NPN network in the warehouse. Note for the NPN in Country_X different regulations apply than for the NPN in Factory A.

- TV_company wants to create an inventory of the TVs that are stored in the warehouse. However, the warehouse also stores products from other companies. Therefore the TV_company instructs the NPN to trigger only the TVs from TV_company through the use of a group ID that identifies these TVs. Ambient IoT devices on other products may be woken up, but will not be triggered to respond.

- Finally the TV is delivered to the end-customer. The delivery driver uses a handheld device to wake up trigger the Ambient IoT device on the box and to trigger it to send its ID to the network. If the network is within range, the Ambient IoT device can communicate directly to the network. Otherwise the handheld device can may relay the communication.

### 5.x.4	Post-Conditions

TV_Company has full traceability of its products from the factory in one country, to the delivery at the end-customer in another country.

### 5.x.5	Existing features partly or fully covering the use case functionality

3GPP TS22.101 Clause 29.2 contains the following requirements:

Support of 3rd party requested broadcast

-	The 3GPP Core Network shall enable a 3rd party service provider to request sending a broadcast message in a specified geographic area (as specified in TS 22.368 [52]) expecting to reach a group of devices that are served by the 3rd party service provider.

A 3rd party requested broadcast can be used to trigger a group of IoT devices in a specific geographic area. But this requirement does not include the feature of waking up Ambient IoT devices.

### 5.x.6	Potential New Requirements needed to support the use case

[PR.5.x-001] The 5G system shall enable support of Ambient IoT communication on an infrastructure (e.g. base stations) also used for other 3GPP communication.

[PR.5.x-002] The 5G system shall be able to support an Ambient IoT device to function in multiple different regions and countries in accordance with local regulations (e.g. frequency regulations which may be different in different regions).

[PR.5.x-003] The 5G system shall support functionality to wake up and trigger Ambient IoT devices to communicate.

Editor’s Note: terminology of triggering to be checked with existing MTC triggering terminology. Definitions still to be provided in the definition section

[PR.5.x-004] The 5G network shall be able to wake up Ambient IoT devices. The 5G system shall enable an authorized 3rd party to instruct the 5G network in which area (e.g. through which base stations) Ambient IoT devices need to be woken up.

[PR.5.x-005] The 5G network shall be able to trigger Ambient IoT devices. The 5G system shall enable an authorized 3rd party to instruct the 5G network in which area, which group of Ambient IoT devices needs to be triggered and which action the Ambient IoT devices need to perform when triggered (e.g. send ID, receive further information, send measurement value).

[PR.5.x-006] The 5G system shall support relaying communication from an Ambient IoT device to the network.