3GPP TSG SA WG 1 Meeting #100 	S1-223586

14 – 18 November 2022, Toulouse, FR	(revision of S1-223506)

Source:	Qualcomm Incorporated, Charter Communications, AT&T

pCR Title:	Use case on Seamless XR Streaming

Draft Spec:	3GPP TR 22.837

Agenda item:	7.1

Document for:	Approval

Contact:	Lola Awoniyi-Oteri <oawoniyi@qti.qualcomm.com>

Abstract: This document proposes a use case using on Seamless XR Streaming.

1. Introduction

Introduce an Seamless XR Streaming use case into the sensing study

2. Reason for Change

Adding a use case with some functional requirements to the study

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 v0.2.0.

* * * First Change * * * *

## 5.x. Use case of Seamless XR streaming

### 5.x.1	Description

Extended Reality (XR) is an important 5G use case. Split-rendering architectures, where the heavy XR video rendering computation is done at the application server based on control information received from the UE, poses strict Quality-of-Service (QoS) requirements in terms of round-trip latency and throughput for delivering the video and control info.

It is therefore crucial to always maintain a high-quality wireless link for XR. Thus, it is critical to predict and adapt fast  to  wireless channel changes. This is especially true in Millimeter wave bands in which the channel and propagation characteristics are very sensitive to user and environment changes such as blockages, user motion or rotation.

To adapt fast to the wireless channel changes, an understanding of the wireless channel dynamics is required. The channel dynamics depend on understanding the surrounding environment such as the transmitter and receiver locations, geometry of the buildings, moving scatterers, location and material of blockers, etc.

Interestingly, most of the XR streaming devices (e.g., 5G phones, AR/VR headsets) and 3rd party entities that support 5G are also provided with sensors, such as RF sensors, Inertial Measurement Units (IMU) sensors, RGB cameras, position sensors, and others.

In light of the availability of sensors and the need of environment understanding, it is therefore natural to utilize the overall sensing information to acquire an understanding of the surrounding environment.

To this end, a “Sensing Assistance Service” can be envisioned that enables the collection of sensing information from multitude of sensors, process  and provide information that information to 5GS  entities and/or  other 3rd party applications. The service is described in the Figure 5.x.1-1.

- The input to this service, “Sensing  Assistance information” could be measurements or processed sensing data from multiple sensors, e.g., RF sensing measurement data and sensing results, communication reference signal measurements or reports, XR user/RAN entity  position, XR user pose, camera images, depth maps, hand tracking, motion type, etc. Such input could be produced by a 5GS entity ( e.g., XR UE, RAN entity)  sensors or by a 3rd party (e.g., surveillance camera). It is essential to note that the collection of this sensing assistance information should be done with appropriate user consent and adherence to regional and national regulations.

- The output of this service is some understanding of the environment and/or impact to communication performance of a service consumer, e.g., RF environment mapping, etc.

- The consumer of this service may can be a 5GS entity (e.g., XR UE or RAN entity), or a 3rd party application.

It is important to note that while this service is provided by 5GS, business model for the monetization of  this service would need to consider factors such as the entities and the value of sensing assistance information produced by these entities as well as the value to the consumer of the service.

5GS entity

(VR Headset)

5GS entity

(RAN entity)

Sensing Assistance Service

3rd party producer

(Camera Surveillance company)

3rd party consumer

(Application client/server)

5GS

Sensing Assistance info.

Sensing Assistance info.

Sensing Assistance info.

5.x.1-1 Seamless XR streaming

Note: Arrows indicate input and output to the service

### 5.1.2	Pre-conditions

Peter is playing a game inside a gaming arena using a VR headset that is connected to a RAN entity.  The VR headset, RAN entity and 3rd party surveillance system are configured to provide “Sensing Assistance Information” to the “Sensing Assistance Service.”

The VR headset is equipped with NR RF sensors, IMU sensors and cameras, and it can provide Sensing Assistance Information e.g., NR RF sensing measurement data and sensing results, headset pose and location, velocity, images of the environment and processed images (such as motion pattern and maps). Also, the VR headset can provide communication reference signal measurements or reports to the Sensing Assistance Service.

RAN entity has NR RF capabilities and can provide NR RF sensing measurement data and sensing results.

The gaming arena also has cameras deployed by a trusted 3rd party surveillance camera company and can provide images of the environment and processed images (such as motion pattern and maps).

### 5.1.3	Service Flows

Step 1: Peter is playing a game using a VR headset in an arena with some obstacles and other gamers in the environment. Peter moves through the arena and approaches a communication blocker which could potentially impact the performance of the wireless communication between VR headset and the RAN entity.

Step 2: Peter’s VR headset, RAN entity and the 3rd party surveillance system provide Sensing Assistance Information to the  Sensing Assistance Service. The Sensing Assistance Service fuses together the Sensing Assistance Information to produce a comprehensive RF map of the environment surrounding the headset (information such as the location of RAN entities, reflectors, static blockers, etc) and an indication of wireless link blockage event, e.g., people walking by blocking the 5G link.

Editor’s Note: Further clarification of RF Map is FFS.

Step 3: The Sensing Assistance Service sends the RF map information to the 5GS entity (e.g., UE in the VR headset)  so that the UE may can request communication parameters and resources changes (e.g., beam switch and handover candidate) to ensure the application’s  performance is not degraded in the presence of a blocker.

Step 4: Alternatively, headset uses the RF map to predict that Peter’s communication link is about to be blocked if he comes close to the blocker and such prediction is sent to the application layer of the game (in the VR headset).Also, the Sensing Assistance Service may can send the RF map information directly to the application layer  in the server without going through the UE. The application layer (in the VR headset or server) adjusts the content of rendered video frames accordingly (e.g., lowers the frame rate, adds a virtual obstacle in the rendered video to prevent Peter from coming close to the blocker.)

Editor’s Note: Further clarification of the relation between this use case and “Use case on Transparent Sensing” in Clause 5.4 of the TR 22.837 is FFS.

### 5.1.4	Post-conditions

Peter enjoys seamless XR gaming application without video frame drops, i.e., no video glitches. This is because Sensing Assistance Information was leveraged to assist both the communication service as well as the application.

### 5.1.5	Existing features partly or fully covering the use case functionality

None

### 5.1.6	Potential New Requirements needed to support the use case

[PR.5.x.6 - 001] Subject to user consent and national or regional regulation, based on operator policy, the 5G system shall be able to support secure means for RAN entities and authorized UEs to provide sensing NR RF sensing measurement data and/or sensing results to a 5GS entity hosting the sensing service.

[PR.5.x.6 - 002] Subject to user consent and national or regional regulation, based on operator policy, the 5G system shall be able to support mechanisms to collectively process sensing measurement data and/or sensing results from RAN entities and authorized UEs to obtain and expose sensing information (e.g., environmental, and spatial awareness information) to a 5GS entity or a trusted 3rd party.

* * * End of Changes * * * *