3GPP TSG SA WG 1 Meeting #100 	S1-223591

Toulouse, France, 14 – 18 November 2022

(revision of S1-223585,S1-223483,S1-223149)

Source:	NTT DOCOMO, NTT

pCR Title:	Pseudo-CR on Full tracking for immersive experience in Metaverse

Draft Spec:	3GPP TR 22.837 version 0.2.0

Agenda item:	7.1

Document for:	Approval

Contact:	Atsushi Minokuchi, minokuchi@nttdocomo.com

Yutaka Musaka, yutaka.musaka.nw@nttdocomo.com

Yoshifumi Morihiro, morihiro@nttdocomo.com

Shinya Otsuki, shinya.otsuki.ma@hco.ntt.co.jp

Tomoki Murakami, tomoki.murakami.nm@hco.ntt.co.jp

Abstract: This contribution proposes a new use case for FS_Sensing.

1. Introduction

<Introduction part >

2. Reason for Change

This contribution proposes a new use case.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 version 0.2.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	W. Favoreel, "Pedestrian sensing for increased traffic safety and efficiency at signalized intersections," 2011 8th IEEE International Conference on Advanced Video and Signal Based Surveillance (AVSS), 2011, pp. 539-542, doi: 10.1109/AVSS.2011.6027406.

[3]	Advances in Wildlife Crossing Technologies: https://highways.dot.gov/public-roads/septoct-2009/advances-wildlife-crossing-technologies.

[4]	Protection Detection: Making Roads Safe for Drivers and Wildlife: https://onlinepubs.trb.org/onlinepubs/webinars/201118.pdf.

[5]	F. Liu et al., "Integrated Sensing and Communications: Towards Dual-functional Wireless Networks for 6G and Beyond," in IEEE Journal on Selected Areas in Communications, doi: 10.1109/JSAC.2022.3156632.

[6]	T. S. Rappaport, G. R. MacCartney, M. K. Samimi and S. Sun, "Wideband Millimeter-Wave Propagation Measurements and Channel Models for Future Wireless Communication System Design," in IEEE Transactions on Communications, vol. 63, no. 9, pp. 3029-3056, Sept. 2015, doi: 10.1109/TCOMM.2015.2434384.

[7]	C. Han, Y. Bi, S. Duan and G. Lu, "Rain Rate Retrieval Test From 25-GHz, 28-GHz, and 38-GHz Millimeter-Wave Link Measurement in Beijing," in IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing, vol. 12, no. 8, pp. 2835-2847, Aug. 2019, doi: 10.1109/JSTARS.2019.2918507.

[8]	IEEE 802.11-18/0611r16: “Wireless LANs, WiFi Sensing Uses Cases”

[9]	TEM STANDARDS TEM STANDARDS AND RECOMMENDED PRACTICE: https://unece.org/fileadmin/DAM/trans/main/tem/temdocs/TEM-Std-Ed3.pdf

[10]	ETSI TS 103 300-2: "Intelligent Transport Systems (ITS); Vulnerable Road Users (VRU) awareness; Part 2: Functional Architecture and Requirements definition; Release 2".

[11]	3GPP TR 22.856, "Localized Mobile Metaverse Services".

[12]	3GPP TR 29.238, "Uplink Streaming".

[13]	“Velodyne™ LiDAR VPL-16 User Manual,” 63-9243 Rev. E, Velodyne™ LiDAR, https://velodynelidar.com/wp-content/uploads/2019/12/63-9243-Rev-E-VLP-16-User-Manual.pdf  <accessed 9.8.22>

[14] 		Liu, A., Huang, Z., Li, M., Wan, Y., Li, W., Han, T.X., Liu, C., Du, R., Tan, D.K.P., Lu, J. and Shen, Y., 2022. A survey on fundamental limits of integrated sensing and communication. IEEE Communications Surveys & Tutorials, 24(2), pp.994-1034.

[15] 		https://medium.com/desn325-emergentdesign/s-l-a-m-and-optical-tracking-for-xr-cfabb7dd536f

[16] 		Dwivedi, S., Shreevastav, R., Munier, F., Nygren, J., Siomina, I., Lyazidi, Y., Shrestha, D., Lindmark, G., Ernström, P., Stare, E. and Razavi, S.M., 2021. Positioning in 5G networks. IEEE Communications Magazine, 59(11), pp.38-44.

[17]	T. Murakami et al, “Wildlife Detection System Using Wireless LAN Signal,” in NTT Technical Review vol.17, No.6, pp. 45-48, June 20019, https://www.ntt-review.jp/archive/ntttechnical.php?contents=ntr201906fa13.pdf&mode=show_pdf

[18]	Eradication of elephant mortality and injury due to railway accidents through automatic tracking and alert system in IEEE Conference Publication, IEEE Xplore

[19]	Impact of wild animals (deer and bears) on train operations 210616_KO_Animal2.pdf (jrhokkaido.co.jp) (in Japanese) [z]	Rail Industry Safety Induction Handbook: https://railsafe.org.au/__data/assets/pdf_file/0009/32022/Rail-Industry-Safety-Induction-RISI-Handbook-V5.1.pdf

[20]	Rail Industry Safety Induction Handbook: https://railsafe.org.au/__data/assets/pdf_file/0009/32022/Rail-Industry-Safety-Induction-RISI-Handbook-V5.1.pdf

[21]	Society of Automotive Engineers (SAE), “Taxonomy and definition for terms related to Driving automation systems for on-Road Motor Vehicles”, https://www.sae.org/standards/content/j3016_202104/

[22]	Census of Fatal Occupational Injuries Summary, 2020   https://www.bls.gov/news.release/cfoi.nr0.htm

[23]	Javed MA, Muram FU, Hansson H, Punnekkat S, Thane H. Towards dynamic safety assurance for Industry 4.0. Journal of Systems Architecture. 2021 Mar 1; 114:101914.

[24]	American National Standards Institute/Industrial Truck Safety Development Foundation, Safety standard for driverless, automatic guided industrial vehicles and automated functions of manned industrial vehicles, December 2019, 2019, [Online] http://www.itsdf.org.

[25]	Moore, Erik George, "Radar Detection, Tracking and Identification for UAV Sense and Avoid Applications" (2019). Electronic Theses and Dissertations. 1544.

[26] 	I. Sobron, I. Landa, I. Eizmendi and M. Velez, "Adaptive TOA Estimation with Imperfect LOS and NLOS Knowledge in UWB Positioning Systems," 2020 IEEE SENSORS, 2020, pp. 1-4. available at: https://sci-hub.wf/10.1109/sensors47125.2020.9278589

[27] 	L. Wang, Z. Zhang, X. Di and J. Tian, "A Roadside Camera-Radar Sensing Fusion System for Intelligent Transportation," 2020 17th European Radar Conference (EuRAD), 2021, pp. 282-285.

[28]	5GAA_White_Paper_C-V2X Use Cases Volume II: Examples and Service Level Requirements

[29]	Moore, Erik George, "Radar Detection, Tracking and Identification for U AV Sense and Avoid Applications" (2019). Electronic Theses and Dissertations. 1544.

[30]	Moore, Erik George, "Radar Detection, Tracking and Identification for U AV Sense and Avoid Applications" (2019). Electronic Theses and Dissertations. 1544

[31]	X. Liu, J. Cao, S. Tang and J. Wen, "Wi-Sleep: Contactless Sleep Monitoring via WiFi Signals," 2014 IEEE Real-Time Systems Symposium, 2014, pp. 346-355, doi: 10.1109/RTSS.2014.30.

[32]	Chen V C. The micro-Doppler effect in radar. Artech house, 2019.

[33]	3GPP TS 22.261: “Service requirements for the 5G system”.

[34]	A. Chebrolu, "FallWatch: A Novel Approach for Through-Wall Fall Detection in Real-Time for the Elderly Using Artificial Intelligence", 2021 Third International Conference on Transdisciplinary AI (TransAI), 2021, pp. 57-63, doi: 10.1109/TransAI51903.2021.00018, https://ieeexplore.ieee.org/document/9565618

[35]	B. A. Alsaify et al., “A CSI-Based Multi-Environment Human Activity Recognition Framework” Applied Sciences 12, no. 2: 930, 2022. https://doi.org/10.3390/app12020930

[36]	U. Saeed U et al., "Discrete Human Activity Recognition and Fall Detection by Combining FMCW RADAR Data of Heterogeneous Environments for Independent Assistive Living", Electronics 10(18):2237, 2021. https://doi.org/10.3390/electronics10182237

[37]	C. Dou, H. Huan, "Full Respiration Rate Monitoring Exploiting Doppler Information with Commodity Wi-Fi Devices". Sensors 21, 3505, 2021. https://doi.org/10.3390/s21103505

[38]	J. Pu, H. Zhang, "RF-Heartbeat: Robust and Contactless Heartbeat Monitoring Based on FMCW Radar", 2021. TechRxiv Preprint. https://doi.org/10.36227/techrxiv.15021645.v2

[x]	W. Jiang et al., "Towards 3D Human Pose Construction Using WiFi", MobiCom '20, September 21–25, 2020, London, United Kingdom.

* * * Next Change * * * *

5.x	Use case on full tracking for immersive experience in Metaverse

5.x.1	Description

NOTE:	A term Metaverse used in this use case is not directly linked to FS_Metaverse.

Participants to Metaverse recently reflect their body movements to those of their avatar's in Metaverse by using a certain tracking equipment. The tracking equipment can be either (i) equipment that comprises several parts, each of which is separately attached to each part of the participant's body, and that tracks the participant's body movement without assistance of any external device, or (ii) equipment similar to (i) but with assistance of external devices sometimes fixed to the wall. Either way, such attachments are cumbersome. Tracking can be done also by using a camera. But privacy becomes concern to participants. And a camera cannot be used if the surrounding is dark. By using wireless sensing for tracking, those participants are freed from such cumbersome attachments, do not need to worry about privacy, and can act even in poorly lit places.

5.x.2	Pre-conditions

MNO#A's indoor small NR base station supports the sensing function. By using two or more such NR base stations and utilizing the NR-based sensing technology, tracking of human body movement can be performed. MNO#A offers a tracking service for customers who buy two or more such NR base stations. A customer can switch on this tracking service, test some body movements, calibrate a certain setting, and then reflect their body movements onto their avatars in Metaverse.

5.x.3	Service flows

1.	Alice is already an inhabitant in Metaverse. She has a tracking equipment and uses it every day.

2.	Alice wants to improve the communication environment of her home and buys three indoor small NR base stations from MNO#A. Noticing these NR base stations provide a tracking service, Alice just tries.

3.	She switches on the tracking service.

4.	The system (e.g., which uses a mechanism similar to that described in [x] or another mechanism) starts working. The system utilizes the NR-based sensing technology and combines multiple sensing measurement data related to multiple target points belonging to a body into one to construct a sensing result, which is a body tracking result.

5.	She checks by using her smart phone if the tracking is well performed. She finds there is slight deviation between the body movement in the physical world and that in the VR world. She quickly calibrates by using an application feature and thinks it's fine.

6.	She configures a setting so that the 5G system provides a tracking data to Metaverse that she usually visits.

7.	She enjoys Metaverse without getting distracted by body attachments that she uses so far. In the mid-night, she goes to a room in Metaverse and sleeps (called VR-sleep). She feels like sleeping at night on a school excursion as a junior high school student. Next morning, she feels she had a good sleep.

5.x.4	Post-conditions

Alice got a comfortable immersive experience to Metaverse, while also getting a good indoor communication environment.

5.x.5	Existing features partly or fully covering the use case functionality

None.

5.x.6	Potential New Requirements needed to support the use case

[PR. 5.x.6 - 001] The 5G system shall be able to utilize the NR-based sensing technology and obtain sensing result by processing multiple sensing measurement data related to multiple target points that belong to the same object.

Editor's Note:	KPI is FFS.

* * * End of Change * * * *