3GPP SA WG1 Meeting #100e 	S1-223072

Toulouse, France, 14 -18 November 2022	(revision of xx-yyxxxx)

Source:	vivo

Title:	New WID on Personal IoT Networks phase 2

Document for:	Approval

Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	New WID on Personal IoT Networks phase 2

Acronym:	PIN_ph2

Unique identifier:

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No |  |  | X |  |  |
| Don't know | X |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_Resident | SA1 | 880040 | Study of Enhancements for Residential 5G |
| FS_PIN | SA1 | 880041 | Study on Personal IoT Networks |
| PIRates | SA1 | 930029 | Personal IoT and Residential networks Service Requirements |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

3	Justification

SA1 rel18 WID PIRates specify requirements for using the 5G system for Customer Premises and Personal IoT Networks as derived from TR 22.858  (FS_RESIDENT [SP-200576]) and TR 22.859( FS_PIN [SP-200592]),  mainly including:

-	Devices within these local “Networks” can communicate with other devices, services and applications within the same local “Network”. Furthermore, these local “Networks” connect to the 5G Network via a gateway to allow devices within the local “Network” to communicate with UEs, services and applications available on the 5G network.

However, rel18 PIRates WID specifies no service requirements for collaboration of different PINs. In practical life, a family would have different PINs (e.g. one PIN for the house and different PINs for different family members that live in the house). All these separate PINs may still want to share specific PIN elements or applications.

It seems to be able to fit the momentary needs by continuously expanding and shrinking one PIN. But from a user point of view, it is easier if different users can manage their own PINs (i.e. PIN A and PIN B), where specific PIN elements from PIN A can make use of applications/services/5G access from PIN B, subject to agreement between the PIN owners.

Consider the following use case:

Figure 3.1

1.  Lucy bought a watch, and joins the PIN of Lucy.

2.  The watch(non-3GPP device) is provisioned with credentials linked the subscription of gateway UE(Lucy’s UE), and get services according to Lucy’s subscription.

3.  Lucy’s UE run out of power;

4.  Lucy and Bob have some agreement to provide PIN access for each other’s PIN device(SLA); while Lucy and John have no such agreement.

5. 	According to the service agreement between Lucy and Bob, Lucy’s watch can use Bob’s UE as gateway to access 3GPP network and get services according to Lucy’s subscription.

6. 	Lucy’s watch can not use John’s UE as gateway because there is no SLA between them.

Therefore it is proposed to agree the PIN phase 2 WID to capture service requirements for collaboration of different PINs, subject to agreement between the PIN owners.

4	Objective

This work item proposes service requirements to support 5GS to support collaboration of different PINs (e.g. PIN A and PIN B), subject to agreement between the PIN owners, including:

-	Authorization /deauthorization for a specific PIN element of PIN A:

-	which specific PIN elements in PIN B it can communicate with;

-	which applications/service or services in PIN B it can access;

-	Authorize/deauthorize for a specific non-3GPP device of PIN A:

-	whether it can use a PIN Element with gateway capability in the PIN B;

-	which external applications/services in the 5GS it can use via the PIN element with gateway capability in the PIN B;

-	charging aspect of non-3GPP device of PIN A in PIN B.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 22.261 | Requirements for PIN phase 2 | SA#98 | New requirements in 22.261 to capture requirements related to PIN phase 2 |

6	Work item Rapporteur(s)

Yanchao Kang, vivo, kangyanchao@vivo.com

7	Work item leadership

SA1

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

vivo

OPPO

Toyota

Xiaomi

China Telecom

CATT

China Unicom

KPN

Philips

Interdigital