3GPP TSG-SA WG1 Meeting #100 	S1-223623

Toulouse, France, 14 Nov – 18 Nov 2022	(revision of S1-223596 of S1-223410 of S1-223025)

Source:	China Unicom, vivo, Charter Communications

pCR Title:	Updated use case on network access control and mobility requirements

Draft Spec:	3GPP TR 22.851v0.2.0

Agenda item:	7.4

Document for:	Approval

Contact:	Tianqi Xing <xingtq@chinaunicom.cn>, Yanchao Kang <kangyanchao@vivo.com>, Richard Wu<richard.wu@charter.com>

Abstract: introduce the network access control and mobility scenarios and potential requirements for the indirect network sharing in TR22.851.

1. Introduction

The purpose of this use case is to introduce access related requirements to TR22.851.

2. Reason for Change

Update the “Use Cases” section 5 of the TR 22.851V0.2.0.

These access related requirements were provided in the last meeting, but not yet captured in the TR.

It is proposed to include new service flows and define new requirements. The access related scenarios and potential requirements need to be discussed and agreed for the indirect network sharing.

3. Conclusions

none

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.851.

* * * First Change * * * *

5	Use cases

5.3	use case on network access control and mobility between sharing parties

5.3.1	Description

It is worth mentioning that 5G networks have been designed to be able to provide shared facilities from the beginning. This means that in the case that the 4G network has both non-shared and shared E-UTRAN at the same time, there could be a number of different types of coverage in the same region:

- Non-shared E-UTRA coverage,

- Shared E-UTRA coverage,

- Non-shared 5G NR coverage,

- Shared 5G NR coverage.

This study introduces a new sharing method arises as without direct connection between shared NG-RAN and the core network of participating operators. Therefore, network access control and mobility management need to be considered when introducing the potential requirements of this new sharing method with existing network.

5.3.2	Pre-conditions

1. It is assumed that OP1, OP2 and OP3 has deployed 4G and 5G networks.

NOTE: The home 4G and 5G network of OP1, OP2 and OP3 may be deployed with non-shared and shared wireless access technology.

- Both OP1 and OP3 are Hosting RAN Operators, which shared NG-RAN with Participating OP2.

- UEs subscribe to OP2’s home PLMN.

- UEs may register successfully to OP1 and OP3’s shared 5G network.

2. Both operators (i.e., OP1 and OP3) agreed to share their networks via indirect connection between the shared radio access network and the OP2’s core network.

3. Potential scenario1: The coverage of OP1 and OP3’s shared 5G network may overlap with OP2’s 4G network; may also overlap with OP2’s 5G network (i.e., at OP1 and OP2’s border, at OP3 and OP2’s border).

4. Potential scenario2: The coverage of OP1’s shared 5G network may overlap with OP3’s shared 5G network (i.e., at OP1’s and OP3’s border). Some parts of shared areas do not overlap the OP2’s network. There are none-shared areas in OP1 or OP3 networks as well.

5.3.3	Service Flows

Mobility and access control scenarios are shown in the following pictures of shared network, including:

Figure 5.3.3-1a:  Scenario 1, a UE with a subscription from OP2 makes a handover between OP2’s own 4G access networks and either OP1’s or OP3’s shared 5G networks.

Figure 5.3.3-1b: Scenario 1, a UE with a subscription from OP2 moves between OP2’s own 5G access networks and either OP1’s or OP3’s shared 5G networks.

NOTE 1: OP1_5G/OP3_5G are OP1/OP3’s shared 5G network via indirect connection between the shared radio access network and the OP2’s core network in the figure 5.3.3-1a and figure 5.3.3-1b.

NOTE 2: OP2_5G/OP2_4G is OP2’s network, may be MOCN networks or non-shared network.

1. UE connects to the participating OP2_4G network then accesses to the hosting OP1_5G or OP3_5G network when the UE crosses the border between the shared network managed by Hosting operator and the OP2’s own 4G access networks in the scenario 1 (As shown as ① in figure 5.3.3-1a).

2. UE connects to the participating OP2_5G network then accesses to the hosting OP1_5G or OP3_5G network when the UE crosses the border between the shared network managed by Hosting operator and the OP2’s own 5G access networks in the scenario 1 (As shown as ② in figure 5.3.3-1b).

Figure 5.3.3-2:  Scenario 2, a UE with a subscription from OP2 moves between coverage of OP1’s and OP3’s shared 5G access networks

3. UE connects to the hosting OP1_5G network then accesses to the hosting OP3_5G network when the UE crosses the border between the two shared networks managed by different Hosting operators in the scenario 2 (As shown as ③ in figure 5.3.3-2).

- The UE accesses the shared network via indirect sharing network method in the specific geographical area as OP1_5G (shared) and/or OP3_5G (shared) shown in figure 5.3.3-2, based on the agreements between the hosting and the participating operators.

- The UE connects an appropriate access network, when the user moves to an area where more than one operator's access networks provide connectivities, e.g., existing OP3’s 5G network and the OP2’s 4G network, or existing OP1’s 5G network and OP3’s 5G network, based on the agreement between the hosting and participating operators.

5.3.4	Post-conditions

All forms of mobility (i.e., between participating operators RAN and shared RAN for both CONNECTED mode and IDLE mode UE, see clause 3.1 of TS 23.122 [5]) are successfully processed in a sharing scenario without direct connections between the shared access and the core networks of the participating operator.

5.3.5	Existing feature partly or fully covering use case functionality

SA1 has performed various studies on mobility and network sharing in previous releases, where related normative stage 1 requirements are introduced in 3GPP TS 22.101 [2] and 22.261 [3].

3GPP TS 22.261 [3] introduces requirements of Diverse mobility management, stated as follows:

The 5G system shall support inter- and/or intra- access technology mobility procedures within 5GS with minimum impact to the user experience (e.g. QoS, QoE).

3GPP TS 22.261 [3] describes various access related requirements, stated as follows:

Based on operator policy, the 5G system shall support steering a UE to select certain 3GPP access network(s).

3GPP TS 22.101 [2] introduces requirements of mobility of network sharing, stated as follows:

It shall be possible to support different mobility management rules, service capabilities and access rights as a function of the home PLMN of the subscribers.

The above requirements are based on MOCN.

5.3.6	Potential New Requirements needed to support the use case

[PR 5.3.6-001] In case of indirect network sharing, the 3GPP system shall support mechanisms to minimize service interruptions for UEs that are moving between different Shared RANs or between a Shared RAN and a non-shared RAN (managed by Hosting and Participating Operators).

[PR 5.3.6-002] In case of Indirect Network Sharing and subject to Hosting and Participating Operators’ policies, the 3GPP system shall support access control for a UE accessing a shared network.

[PR 5.3.6-003] In case of Indirect Network Sharing and subject to Hosting and Participating Operators’ policies, the 3GPP system shall be able to select an appropriate radio access network (i.e. 4G, 5G) for a UE.

[PR 5.3.6-004] In case of Indirect Network Sharing and subject to Hosting and Participating Operators’ policies, the 3GPP system shall support a mechanism to enable a UE with a subscription to a Participating Operator to access an authorized shared network.