3GPP TSG SA WG 1 Meeting #100 															S1-223716

Toulouse, France, 14 – 18 November 2022                                                                  (revision of S1-223095, S1-223491, S1-223576)

Source:	China Telecom, vivo, ZTE

pCR Title:	Update of use case on Rainfall Monitoring

Draft Spec:	3GPP TR 22.837v0.2.0

Agenda item:	7.1

Document for:	Approval

Contact:	Yuying Zhang, zhangyy45@chinatelecom.cn

Amy Zhang, amy.zhang@vivo.com

Abstract: The contribution proposes to align with the common KPI table format.

1. Introduction

This contribution is to align with the common KPI table and clarify rainfall information in the use case.

2. Reason for Change

The refreshing rate is set for the following reason that the refreshing rate of weather monitoring in the observatory is typical from 10-15 min, the same time interval is assumed to be achieved for the rainfall monitoring by sensing. The refreshing rate in the KPI table is set to 10 min and could be configured by application.

Rainfall estimation accuracy is added with the following definition for this specific use case: Rainfall estimation accuracy: describes the closeness of the measured sensing result (i.e. rainfall estimation) to its true rainfall value.

3. Conclusions

KPI table is added for rainfall monitoring use case.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837v0.2.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	W. Favoreel, "Pedestrian sensing for increased traffic safety and efficiency at signalized intersections," 2011 8th IEEE International Conference on Advanced Video and Signal Based Surveillance (AVSS), 2011, pp. 539-542, doi: 10.1109/AVSS.2011.6027406.

[3]	Advances in Wildlife Crossing Technologies: https://highways.dot.gov/public-roads/septoct-2009/advances-wildlife-crossing-technologies.

[4]	Protection Detection: Making Roads Safe for Drivers and Wildlife: https://onlinepubs.trb.org/onlinepubs/webinars/201118.pdf.

[5]	F. Liu et al., "Integrated Sensing and Communications: Towards Dual-functional Wireless Networks for 6G and Beyond," in IEEE Journal on Selected Areas in Communications, doi: 10.1109/JSAC.2022.3156632.

[6]	T. S. Rappaport, G. R. MacCartney, M. K. Samimi and S. Sun, "Wideband Millimeter-Wave Propagation Measurements and Channel Models for Future Wireless Communication System Design," in IEEE Transactions on Communications, vol. 63, no. 9, pp. 3029-3056, Sept. 2015, doi: 10.1109/TCOMM.2015.2434384.

[7]	C. Han, Y. Bi, S. Duan and G. Lu, "Rain Rate Retrieval Test From 25-GHz, 28-GHz, and 38-GHz Millimeter-Wave Link Measurement in Beijing," in IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing, vol. 12, no. 8, pp. 2835-2847, Aug. 2019, doi: 10.1109/JSTARS.2019.2918507.

[8]	IEEE 802.11-18/0611r16: “Wireless LANs, WiFi Sensing Uses Cases”

[9]	TEM STANDARDS TEM STANDARDS AND RECOMMENDED PRACTICE: https://unece.org/fileadmin/DAM/trans/main/tem/temdocs/TEM-Std-Ed3.pdf

[10]	ETSI TS 103 300-2: "Intelligent Transport Systems (ITS); Vulnerable Road Users (VRU) awareness; Part 2: Functional Architecture and Requirements definition; Release 2".

[11]	3GPP TR 22.856, "Localized Mobile Metaverse Services".

[12]	3GPP TR 29.238, "Uplink Streaming".

[13]	“Velodyne™ LiDAR VPL-16 User Manual,” 63-9243 Rev. E, Velodyne™ LiDAR, https://velodynelidar.com/wp-content/uploads/2019/12/63-9243-Rev-E-VLP-16-User-Manual.pdf  <accessed 9.8.22>

[14] 		Liu, A., Huang, Z., Li, M., Wan, Y., Li, W., Han, T.X., Liu, C., Du, R., Tan, D.K.P., Lu, J. and Shen, Y., 2022. A survey on fundamental limits of integrated sensing and communication. IEEE Communications Surveys & Tutorials, 24(2), pp.994-1034.

[15] 		https://medium.com/desn325-emergentdesign/s-l-a-m-and-optical-tracking-for-xr-cfabb7dd536f

[16] 		Dwivedi, S., Shreevastav, R., Munier, F., Nygren, J., Siomina, I., Lyazidi, Y., Shrestha, D., Lindmark, G., Ernström, P., Stare, E. and Razavi, S.M., 2021. Positioning in 5G networks. IEEE Communications Magazine, 59(11), pp.38-44.

[17]	T. Murakami et al, “Wildlife Detection System Using Wireless LAN Signal,” in NTT Technical Review vol.17, No.6, pp. 45-48, June 20019, https://www.ntt-review.jp/archive/ntttechnical.php?contents=ntr201906fa13.pdf&mode=show_pdf

[18]	Eradication of elephant mortality and injury due to railway accidents through automatic tracking and alert system in IEEE Conference Publication, IEEE Xplore

[19]	Impact of wild animals (deer and bears) on train operations 210616_KO_Animal2.pdf (jrhokkaido.co.jp) (in Japanese) [z]	Rail Industry Safety Induction Handbook: https://railsafe.org.au/__data/assets/pdf_file/0009/32022/Rail-Industry-Safety-Induction-RISI-Handbook-V5.1.pdf

[20]	Rail Industry Safety Induction Handbook: https://railsafe.org.au/__data/assets/pdf_file/0009/32022/Rail-Industry-Safety-Induction-RISI-Handbook-V5.1.pdf

[21]	Society of Automotive Engineers (SAE), “Taxonomy and definition for terms related to Driving automation systems for on-Road Motor Vehicles”, https://www.sae.org/standards/content/j3016_202104/

[22]	Census of Fatal Occupational Injuries Summary, 2020   https://www.bls.gov/news.release/cfoi.nr0.htm

[23]	Javed MA, Muram FU, Hansson H, Punnekkat S, Thane H. Towards dynamic safety assurance for Industry 4.0. Journal of Systems Architecture. 2021 Mar 1; 114:101914.

[24]	American National Standards Institute/Industrial Truck Safety Development Foundation, Safety standard for driverless, automatic guided industrial vehicles and automated functions of manned industrial vehicles, December 2019, 2019, [Online] http://www.itsdf.org.

[25]	Moore, Erik George, "Radar Detection, Tracking and Identification for UAV Sense and Avoid Applications" (2019). Electronic Theses and Dissertations. 1544.

[26] 	I. Sobron, I. Landa, I. Eizmendi and M. Velez, "Adaptive TOA Estimation with Imperfect LOS and NLOS Knowledge in UWB Positioning Systems," 2020 IEEE SENSORS, 2020, pp. 1-4. available at: https://sci-hub.wf/10.1109/sensors47125.2020.9278589

[27] 	L. Wang, Z. Zhang, X. Di and J. Tian, "A Roadside Camera-Radar Sensing Fusion System for Intelligent Transportation," 2020 17th European Radar Conference (EuRAD), 2021, pp. 282-285.

[28]	5GAA_White_Paper_C-V2X Use Cases Volume II: Examples and Service Level Requirements

[29]	Moore, Erik George, "Radar Detection, Tracking and Identification for U AV Sense and Avoid Applications" (2019). Electronic Theses and Dissertations. 1544.

[30]	Moore, Erik George, "Radar Detection, Tracking and Identification for U AV Sense and Avoid Applications" (2019). Electronic Theses and Dissertations. 1544

[31]	X. Liu, J. Cao, S. Tang and J. Wen, "Wi-Sleep: Contactless Sleep Monitoring via WiFi Signals," 2014 IEEE Real-Time Systems Symposium, 2014, pp. 346-355, doi: 10.1109/RTSS.2014.30.

[32]	Chen V C. The micro-Doppler effect in radar. Artech house, 2019.

[33]	3GPP TS 22.261: “Service requirements for the 5G system”.

[34]	A. Chebrolu, "FallWatch: A Novel Approach for Through-Wall Fall Detection in Real-Time for the Elderly Using Artificial Intelligence", 2021 Third International Conference on Transdisciplinary AI (TransAI), 2021, pp. 57-63, doi: 10.1109/TransAI51903.2021.00018, https://ieeexplore.ieee.org/document/9565618

[35]	B. A. Alsaify et al., “A CSI-Based Multi-Environment Human Activity Recognition Framework” Applied Sciences 12, no. 2: 930, 2022. https://doi.org/10.3390/app12020930

[36]	U. Saeed U et al., "Discrete Human Activity Recognition and Fall Detection by Combining FMCW RADAR Data of Heterogeneous Environments for Independent Assistive Living", Electronics 10(18):2237, 2021. https://doi.org/10.3390/electronics10182237

[37]	C. Dou, H. Huan, "Full Respiration Rate Monitoring Exploiting Doppler Information with Commodity Wi-Fi Devices". Sensors 21, 3505, 2021. https://doi.org/10.3390/s21103505

[38]	J. Pu, H. Zhang, "RF-Heartbeat: Robust and Contactless Heartbeat Monitoring Based on FMCW Radar", 2021. TechRxiv Preprint. https://doi.org/10.36227/techrxiv.15021645.v2

[xa] 	H. V. Habi and H. Messer, “Recurrent Neural Network for Rain Estimation Using Commercial Microwave Links,” in IEEE Transactions on Geoscience and Remote Sensing, vol. 59, no. 5, pp. 3672-3681, May 2021, doi: 10.1109/TGRS.2020.3010305.

[xb] 	Polz, Julius & Chwala, Christian & Graf, Maximilian & Kunstmann, Harald. (2020). Rain event detection in commercial microwave link attenuation data using convolutional neural networks. Atmospheric Measurement Techniques. 13. 3835-3853. 10.5194/amt-13-3835-2020.

* * * Next Change * * * *

## 5.3	Use case on rainfall monitoring

### 5.3.1	Description

Rainfall monitoring is a topic of great importance for several application contexts: hydraulic structure design, agriculture, weather forecasting, climate modelling, etc. At present, the most widely used measurement method is rain gauge.

Traditional rainfall monitoring use rain gauges, which are located at a particular location. Wide-area rainfall monitoring using traditional rain gauges would be costly. The base stations are deployed by the operators with radio cell planning that could cover a wider area. With base stations monitoring the rainfall, for example rain rate (mm/h), it could obtain a horizontally wider-area measurement.

Radio signals, as they propagate through the atmosphere, are reduced in intensity by constituents of the atmosphere. Oxygen and water vapor are the two major components which are responsible for the signal absorption. If it is a rainy day, an additional attenuation caused by rain further increases the propagation path loss. [7] The rain attenuation depends on the size and distribution of the water droplets, hence, by quantifying and modelling the base station signal measurements, we are able to know the rain rate.

The mmWave bands, such as 28GHz and 38GHz have been used to assess coverage, large-scale path loss, and fading and multipath effects [6]. Since the 28 GHz and 38 GHz bands are also licensed for wireless backhaul communications, these frequencies can used for rainfall monitoring [7].

The granularity of the rainfall monitoring could be smaller than the traditional measurements.

### 5.3.2	Pre-conditions

Peter is a farmer who takes care of a big farm that grows different crops. Peter needs to monitor the rainfall of his farm to manage reasonable irrigation, drainage and fertilizer. When there is less rainfall, Peter can select reasonable irrigation plans to improve the farmland water content condition. When there is high rainfall, Peter should improve the drainage system and fertilize the crops to avoid crop losses.

### 5.3.3	Service Flows

- Peter has a subscription for the premium service of rainfall monitoring for a more granular location.

- Peter is at daily working routine and wants to check the timely rainfall information from the weather application on his phone.

- The base station obtains the NR based sensing measurement data every hour and the 5G system processes the sensing measurement data to obtain sensing results and exposes the NR based sensing results to the weather application via the core network.

- Based on the sensing results above, the application server obtains the rainfall information (i.e. rainfall and whether it is raining) associated with location information.

- Peter obtains timely rainfall information from weather application on his phone.

### 5.3.4	Post-conditions

Peter could check the rainfall information at any time on his phone. Based on the timely rainfall information, Peter could plan the irrigation, drainage and fertilizer for the crops in his farm.

### 5.3.5	Existing feature partly or fully covering use case functionality

None

### 5.3.6	Potential New Requirements needed to support the use case

[PR. 5.3.6 - 001] The 5G system shall support  collection of the NR based sensing measurement data from the base station.

[PR. 5.3.6 - 002] Based on operator’s policy, the 5G system shall support mechanisms to process the sensing measurement data to derive the sensing results.

[PR. 5.3.6 - 003] Based on operator’s policy, the 5G system shall provide mechanisms to expose NR based sensing results with assisted information, e.g. location, to a trusted 3rd party application via the core network.

[PR. 5.3.6 – 004] The 5G system shall support sensing services with KPIs as given Table 5.3.6-1.

Table 5.3-X	Performance requirements of sensing results for integrated sensing and communication scenarios

| Scenario | Sensing service area | Confidence level [%] | Rainfall estimation accuracy (for a target confidence level) | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Rainfall monitoring | outdoor | 95% | [1mm/h] NOTE 1 | N/A | N/A | N/A | N/A | N/A | N/A | 1min | 10min, application configurable | 5% | 5% |
| NOTE: The terms in Table 7.2-X are found in Section 3.1. NOTE 1: For rainfall rain rate >1 mm/h. [xa] |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *