| 3GPP TR 22.837 V0.3.0 (2022-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group TSG SA; Feasibility Study on Integrated Sensing and Communication (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

Introduction	8

1	Scope	9

2	References	9

3	Definitions of terms, symbols and abbreviations	11

3.1	Terms	11

3.2	Symbols	13

3.3	Abbreviations	13

4	Overview	13

5	Use cases	13

5.1	Use case of intruder detection in smart home	13

5.1.1	Description	13

5.1.2	Pre-conditions	13

5.1.3	Service Flows	14

5.1.4	Post-conditions	14

5.1.5	Existing features partly or fully covering the use case functionality	14

5.1.6	Potential New Requirements needed to support the use case	14

5.2	Use case on pedestrian/animal intrusion detection on a highway	15

5.2.1	Description	15

5.2.2	Pre-conditions	16

5.2.3	Service Flows	16

5.2.4	Post-conditions	17

5.2.5	Existing features partly or fully covering the use case functionality	17

5.2.6	Potential New Requirements needed to support the use case	17

5.3	Use case on rainfall monitoring	18

5.3.1	Description	18

5.3.2	Pre-conditions	18

5.3.3	Service Flows	18

5.3.4	Post-conditions	18

5.3.5	Existing feature partly or fully covering use case functionality	18

5.3.6	Potential New Requirements needed to support the use case	19

5.4	Use Case on Transparent Sensing Use Case	19

5.4.1	Description	19

5.4.2	Pre-conditions	20

5.4.3	Service Flows	20

5.4.4	Post-conditions	20

5.4.5	Existing feature partly or fully covering use case functionality	20

5.4.6	Potential New Requirements needed to support the use case	21

5.5	Use case on sensing for flooding in smart cities	21

5.5.1	Description	21

5.5.2	Pre-conditions	21

5.5.3	Service Flows	22

5.5.4	Post-conditions	22

5.5.5	Existing features partly or fully covering the use case functionality	22

5.5.6	Potential New Requirements needed to support the use case	22

5.6	Use case on intruder detection in surroundings of smart home	23

5.6.1	Description	23

5.6.2	Pre-conditions	23

5.6.3	Service Flows	23

5.6.4	Post-conditions	23

5.6.5	Existing features partly or fully covering the use case functionality	23

5.6.6	Potential New Requirements needed to support the use case	23

5.7	Use case on sensing for railway intrusion detection	24

5.7.1	Description	24

5.7.2	Pre-conditions	25

5.7.3	Service Flows	25

5.7.4	Post-conditions	26

5.7.5	Existing feature partly or fully covering use case functionality	26

5.7.6	Potential New Requirements needed to support the use case	26

5.8	Use Case on Sensing Assisted Automotive Maneuvering and Navigation	26

5.8.1	Description	26

5.8.2	Pre-conditions	27

5.8.3	Service Flows	27

5.8.4	Post-conditions	29

5.8.5	Existing features partly or fully covering the use case functionality	29

5.8.6	Potential New Requirements needed to support the use case	29

5.9	Use case on AGV detection and tracking in factories	29

5.9.1	Description	29

5.9.2	Pre-conditions	30

5.9.3	Service Flows	30

5.9.4	Post-conditions	31

5.9.5	Existing features partly or fully covering the use case functionality	31

5.9.6	Potential New Requirements needed to support the use case	31

5.10	Use case on UAV flight trajectory tracing	31

5.10.1	Description	31

5.10.2	Pre-conditions	32

5.10.3	Service Flows	32

5.10.4	Post-conditions	33

5.10.5	Existing features partly or fully covering the use case functionality	33

5.10.6	Potential New Requirements needed to support the use case	33

5.11	Use case on sensing at crossroads with/without obstacle	34

5.11.1	Description	34

5.11.2	Pre-conditions	34

5.11.3	Service Flows	35

5.11.4	Post Conditions	36

5.11.5	Existing features partly or fully covering the use case functionality	36

5.11.6	Potential New Requirements needed to support the use case	36

5.12	Network assisted sensing to avoid UAV collision	37

5.12.1	Description	37

5.12.2	Pre-Conditions	38

5.12.3	Service Flows	38

5.12.4	Post-Conditions	39

5.12.5	Existing features partly or fully covering the use case functionality	39

5.12.6	Potential New Requirements needed to support the use case	39

5.13.	Use case on sensing for UAV intrusion detection	40

5.13.1	Description	40

5.13.2	Pre-conditions	41

5.13.3	Service Flows	41

5.13.4	Post-conditions	42

5.13.5	Existing features partly or fully covering the use case functionality	42

5.13.6	Potential New Requirements needed to support the use case	42

5.14.	Use case on sensing for tourist spot traffic management	43

5.14.1	Description	43

5.14.2	Pre-conditions	43

5.14.3	Service Flows	44

5.14.4	Post-conditions	44

5.14.5	Existing features partly or fully covering the use case functionality	44

5.14.6	Potential New Requirements needed to support the use case	44

5.15	Use case on contactless sleep monitoring service	45

5.15.1	Description	45

5.15.2	Pre-conditions	46

5.15.3	Service Flows	46

5.15.4	Post-conditions	46

5.15.5	Existing feature partly or fully covering use case functionality	46

5.15.6	Potential New Requirements needed to support the use case	46

5.16	Use case on Protection of Sensing Information	47

5.16.1	Description	47

5.16.2	Pre-conditions	47

5.16.3	Service Flows	47

5.16.4	Post-conditions	47

5.16.5	Existing features partly or fully covering the use case functionality	47

5.16.6	Potential New Requirements needed to support the use case	47

5.17	Use case on health monitoring at home	48

5.17.1 	Description	48

5.17.2	Pre-conditions	48

5.17.3	Service Flows	49

5.17.4	Post-conditions	49

5.17.5	Existing features partly or fully covering the use case functionality	49

5.17.6	Potential New Requirements needed to support the use case	49

5.18	Use case on service continuity of unobtrusive health monitoring.	49

5.18.1 	Description	49

5.18.2	Pre-conditions	50

5.18.3	Service Flows	50

5.18.4	Post-conditions	51

5.18.5	Existing features partly or fully covering the use case functionality	51

5.18.6	Potential New Requirements needed to support the use case	51

5.19	Use case on Sensor Groups	51

5.19.1	Description	51

5.19.2	Pre-conditions	52

5.19.3	Service Flows	53

5.19.4	Post-conditions	54

5.19.5	Existing feature partly or fully covering use case functionality	54

5.19.6	Potential New Requirements needed to support the use case	55

5.20	Use case of Sensing for Parking Space Determination	55

5.20.1	Description	55

5.20.2	Pre-conditions	56

5.20.3	Service Flows	57

5.20.4	Post-conditions	57

5.20.5	Existing features partly or fully covering the use case functionality	57

5.20.6	Potential New Requirements needed to support the use case	57

5.21.	Use case of Seamless XR streaming	58

5.21.1	Description	58

5.21.2	Pre-conditions	59

5.21.3	Service Flows	59

5.21.4	Post-conditions	59

5.21.5	Existing features partly or fully covering the use case functionality	59

5.21.6	Potential New Requirements needed to support the use case	59

5.22	Use case of UAVs/vehicles/pedestrians detection near Smart Grid equipment	60

5.22.1	Description	60

5.22.2	Pre-conditions	60

5.22.3	Service Flows	60

5.22.4	Post-conditions	60

5.22.5	Existing features partly or fully covering the use case functionality	61

5.22.6	Potential New Requirements needed to support the use case	61

5.23	AMR collision avoidance in smart factories	61

5.23.1	Description	61

5.23.2	Pre-Conditions	62

5.23.3	Service Flows	62

5.23.4	Post-Conditions	63

5.23.5	Existing features partly or fully covering the use case functionality	63

5.23.6	Potential New Requirements needed to support the use case	63

5.24	Use case on roaming for sensing service of sports monitoring	64

5.24.1	Description	64

5.24.2	Pre-conditions	64

5.24.3	Service Flows	64

5.24.4	Post-conditions	64

5.24.5	Existing features partly or fully covering the use case functionality	64

5.24.6	Potential New Requirements needed to support the use case	65

5.25	Use Case on immersive experience based on sensing	65

5.25.1	Description	65

5.25.2	Pre-conditions	66

5.25.3	Service Flows	67

5.25.4	Post-conditions	67

5.25.5	Existing features partly or fully covering the use case functionality	67

5.25.6	Potential New Requirements needed to support the use case	67

5.26	Use case on accurate sensing for automotive maneuvering and navigation service	68

5.26.1	Description	68

5.26.2 	Pre-conditions	68

5.26.3	 Service Flows	68

5.26.4	Post-conditions	69

5.26.5	 Existing features partly or fully covering the use case functionality	69

5.26.6	 Potential New Requirements needed to support the use case	69

6	Considerations	69

6.1	Considerations on confidentiality and integrity,	69

6.1.1	General	69

6.1.2	Potential New Requirements	69

6.2 Considerations on Privacy	70

6.3 Considerations on Mission Critical and other priority services	70

7	Consolidated potential requirements and KPIs	70

7.1	Consolidated potential requirements	70

7.2	Consolidated potential KPIs of sensing results	70

8	Conclusion and recommendations	71

Annex <Z> (informative): Change history	72

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

Wireless sensing technologies aim at acquiring information about a remote object or environment and its characteristics without physically contacting it. The perception data of the object and its surrounding can be utilized for analysis, so that meaningful information about the object or environment and its characteristics can be obtained.

# 1	Scope

The present document describes use cases and potential requirements for enhancement of the 5G system to provide sensing services addressing different target verticals/applications, e.g. autonomous/assisted driving, V2X, UAVs, 3D map reconstruction, smart city, smart home, factories, healthcare, maritime sector.

Use cases focus on NR-based sensing, while some use cases might make use of information already available in EPC and E-UTRA (e.g. cell/UE measurements, location updates). This study will not lead to impacts on EPC and E-UTRA. Some use cases could also include non-3GPP type sensors (e.g. Radar, camera).

The aspects addressed in the present document include collecting and reporting of sensing information, sensing related KPIs. Security, privacy, regulation and charging are additional topics of concern.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	W. Favoreel, "Pedestrian sensing for increased traffic safety and efficiency at signalized intersections," 2011 8th IEEE International Conference on Advanced Video and Signal Based Surveillance (AVSS), 2011, pp. 539-542, doi: 10.1109/AVSS.2011.6027406.

[3]	Advances in Wildlife Crossing Technologies: https://highways.dot.gov/public-roads/septoct-2009/advances-wildlife-crossing-technologies.

[4]	Protection Detection: Making Roads Safe for Drivers and Wildlife: https://onlinepubs.trb.org/onlinepubs/webinars/201118.pdf.

[5]	F. Liu et al., "Integrated Sensing and Communications: Towards Dual-functional Wireless Networks for 6G and Beyond," in IEEE Journal on Selected Areas in Communications, doi: 10.1109/JSAC.2022.3156632.

[6]	T. S. Rappaport, G. R. MacCartney, M. K. Samimi and S. Sun, "Wideband Millimeter-Wave Propagation Measurements and Channel Models for Future Wireless Communication System Design," in IEEE Transactions on Communications, vol. 63, no. 9, pp. 3029-3056, Sept. 2015, doi: 10.1109/TCOMM.2015.2434384.

[7]	C. Han, Y. Bi, S. Duan and G. Lu, "Rain Rate Retrieval Test From 25-GHz, 28-GHz, and 38-GHz Millimeter-Wave Link Measurement in Beijing," in IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing, vol. 12, no. 8, pp. 2835-2847, Aug. 2019, doi: 10.1109/JSTARS.2019.2918507.

[8]	IEEE 802.11-18/0611r16: “Wireless LANs, WiFi Sensing Uses Cases”

[9]	TEM STANDARDS TEM STANDARDS AND RECOMMENDED PRACTICE: https://unece.org/fileadmin/DAM/trans/main/tem/temdocs/TEM-Std-Ed3.pdf

[11]	3GPP TR 22.856, "Localized Mobile Metaverse Services".

[13]	“Velodyne™ LiDAR VPL-16 User Manual,” 63-9243 Rev. E, Velodyne™ LiDAR, https://velodynelidar.com/wp-content/uploads/2019/12/63-9243-Rev-E-VLP-16-User-Manual.pdf

[14] 		Liu, A., Huang, Z., Li, M., Wan, Y., Li, W., Han, T.X., Liu, C., Du, R., Tan, D.K.P., Lu, J. and Shen, Y., 2022. A survey on fundamental limits of integrated sensing and communication. IEEE Communications Surveys & Tutorials, 24(2), pp.994-1034.

[15] 		https://medium.com/desn325-emergentdesign/s-l-a-m-and-optical-tracking-for-xr-cfabb7dd536f

[16] 		Dwivedi, S., Shreevastav, R., Munier, F., Nygren, J., Siomina, I., Lyazidi, Y., Shrestha, D., Lindmark, G., Ernström, P., Stare, E. and Razavi, S.M., 2021. Positioning in 5G networks. IEEE Communications Magazine, 59(11), pp.38-44.

[17]	T. Murakami et al, “Wildlife Detection System Using Wireless LAN Signal,” in NTT Technical Review vol.17, No.6, pp. 45-48, June 20019, https://www.ntt-review.jp/archive/ntttechnical.php?contents=ntr201906fa13.pdf&mode=show_pdf

[18]	Eradication of elephant mortality and injury due to railway accidents through automatic tracking and alert system in IEEE Conference Publication, IEEE Xplore

[19]	Impact of wild animals (deer and bears) on train operations 210616_KO_Animal2.pdf (jrhokkaido.co.jp) (in Japanese) [z]	Rail Industry Safety Induction Handbook: https://railsafe.org.au/__data/assets/pdf_file/0009/32022/Rail-Industry-Safety-Induction-RISI-Handbook-V5.1.pdf

[21]	Society of Automotive Engineers (SAE), “Taxonomy and definition for terms related to Driving automation systems for on-Road Motor Vehicles”, https://www.sae.org/standards/content/j3016_202104/

[22]	Census of Fatal Occupational Injuries Summary, 2020, https://www.bls.gov/news.release/cfoi.nr0.htm

[23]	Javed MA, Muram FU, Hansson H, Punnekkat S, Thane H. Towards dynamic safety assurance for Industry 4.0. Journal of Systems Architecture. 2021 Mar 1; 114:101914.

[24]	American National Standards Institute/Industrial Truck Safety Development Foundation, Safety standard for driverless, automatic guided industrial vehicles and automated functions of manned industrial vehicles, December 2019, 2019, [Online] http://www.itsdf.org.

[25]	Moore, Erik George, "Radar Detection, Tracking and Identification for UAV Sense and Avoid Applications" (2019). Electronic Theses and Dissertations. 1544.

[26] 	I. Sobron, I. Landa, I. Eizmendi and M. Velez, "Adaptive TOA Estimation with Imperfect LOS and NLOS Knowledge in UWB Positioning Systems," 2020 IEEE SENSORS, 2020, pp. 1-4. available at: https://sci-hub.wf/10.1109/sensors47125.2020.9278589

[27] 	L. Wang, Z. Zhang, X. Di and J. Tian, "A Roadside Camera-Radar Sensing Fusion System for Intelligent Transportation," 2020 17th European Radar Conference (EuRAD), 2021, pp. 282-285.

[28]	5GAA_White_Paper_C-V2X Use Cases Volume II: Examples and Service Level Requirements

[29] 	https://www.ieee802.org/11/Reports/tgbf_update.htm

[30]	https://mentor.ieee.org/802.11/dcn/20/11-20-1712-02-00bf-wifi-sensing-use-cases.xlsx

[31]	X. Liu, J. Cao, S. Tang and J. Wen, "Wi-Sleep: Contactless Sleep Monitoring via WiFi Signals," 2014 IEEE Real-Time Systems Symposium, 2014, pp. 346-355, doi: 10.1109/RTSS.2014.30.

[32]	Chen V C. The micro-Doppler effect in radar. Artech house, 2019.

[33]	3GPP TS 22.261: “Service requirements for the 5G system”.

[34]	A. Chebrolu, "FallWatch: A Novel Approach for Through-Wall Fall Detection in Real-Time for the Elderly Using Artificial Intelligence", 2021 Third International Conference on Transdisciplinary AI (TransAI), 2021, pp. 57-63, doi: 10.1109/TransAI51903.2021.00018, https://ieeexplore.ieee.org/document/9565618

[35]	B. A. Alsaify et al., “A CSI-Based Multi-Environment Human Activity Recognition Framework” Applied Sciences 12, no. 2: 930, 2022. https://doi.org/10.3390/app12020930

[36]	U. Saeed U et al., "Discrete Human Activity Recognition and Fall Detection by Combining FMCW RADAR Data of Heterogeneous Environments for Independent Assistive Living", Electronics 10(18):2237, 2021. https://doi.org/10.3390/electronics10182237

[37]	C. Dou, H. Huan, "Full Respiration Rate Monitoring Exploiting Doppler Information with Commodity Wi-Fi Devices". Sensors 21, 3505, 2021. https://doi.org/10.3390/s21103505

[38]	J. Pu, H. Zhang, "RF-Heartbeat: Robust and Contactless Heartbeat Monitoring Based on FMCW Radar", 2021. TechRxiv Preprint. https://doi.org/10.36227/techrxiv.15021645.v2

[39] 	H. V. Habi and H. Messer, “Recurrent Neural Network for Rain Estimation Using Commercial Microwave Links,” in IEEE Transactions on Geoscience and Remote Sensing, vol. 59, no. 5, pp. 3672-3681, May 2021, doi: 10.1109/TGRS.2020.3010305.[40]                    Roberto Opromolla, etc., “Perspectives and Sensing Concepts for Small UAS Sense and Avoid”, 2018 IEEE/AIAA 37th Digital Avionics Systems Conference (DASC)

[41]	https://www.rwjbh.org/trinitas-regional-medical-center/treatment-care/sleep-disorders/sleep-apnea/

[42]	https://my.clevelandclinic.org/health/articles/10881-vital-signs

[43]	3GPP TR 22.855, "Study on Ranging-based Services".

[44]	Guoxuan Chi, et. al., "Wi-Drone: Wi-Fi-based 6-DoF Tracking for Indoor Drone Flight Control", MobiSys 22, Association for Computing Machinery, 2022.

[45]	Report on Automated Valet Parking: technology assessment and use case implementation description – 5G Automotive Association (5gaa.org). https://5gaa.org/news/report-on-automated-valet-parking-technology-assessment-and-use-case-implementation-description/

[46]	Nie Y B , Zhang L . Main amendments to Working Safety Regulation of State Grid Company(Dynamical Part for Hydrodynamic Power Plant)[J]. East China Electric Power, 2008.

[47] 	Giuseppe Fragapane, René de Koster, Fabio Sgarbossa, Jan Ola Strandhagen, Planning and control of autonomous mobile robots for intralogistics: Literature review and research agenda, European Journal of Operational Research, Volume 294, Issue 2,2021, Pages 405-426

[48]	Li S, Li X, Lv Q, et al. WiFit: Ubiquitous bodyweight exercise monitoring with commodity wi-fi devices, 2018 IEEE SmartWorld, Ubiquitous Intelligence & Computing, Advanced & Trusted Computing, Scalable Computing & Communications, Cloud & Big Data Computing, Internet of People and Smart City Innovation, IEEE, 2018: 530-537.

[49]	https://www.synopsys.com/automotive/autonomous-driving-levels.html

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G Wireless sensing: 5GS feature providing capabilities to get information about characteristics of the environment and/or objects within the environment (e.g. shape, size, speed, location, distances or relative motion between objects, etc) using NR RF signals and, in some cases, previously defined information available in EPC and/or E-UTRA.

Motion rate accuracy describes the closeness of the measured magnitude of the target object’s vibration frequency caused by part(s) of the target object to the true magnitude of the target object’s vibration frequency.

Editor’s Note: terminology of motion rate accuracy needs to be refined.

sensing group: a set of UEs that support sensing operations all of whose location is known and whose sensing measurement data can be collected synchronously.

sensing measurement data: data collected about radio/wireless signals impacted (e.g. reflected, refracted, diffracted) by an object or environment of interest for sensing purposes.

sensing measurement process: process of collecting sensing measurement data.

Sensing receiver: A Sensing receiver is an entity that receives the sensing signal which the sensing service will use in its operation. A sensing receiver is an NR RAN node or a UE. A Sensing receiver can be located in the same or different entity as the Sensing transmitter.

sensing result: the information derived from processing sensing measurement data.

NOTE: Examples of sensing result are characteristics of an object or environment, etc.

Sensing service area location:  an area location whether with or without obstacle, the 5G system can provide sensing service with certain quality.

Sensing transmitter: A Sensing transmitter is the entity that sends out the sensing signal which the sensing service will use in its operation. A Sensing transmitter is an NR RAN node or a UE. A Sensing transmitter can be located in the same or different entity as the Sensing receiver.

transparent sensing: sensing measurements are communicated such that they can be discerned and interpreted by the 5G system, e.g. the data is communicated using a standard protocol to an interface defined by the 5G system.

The following KPIs apply to the definition of the use cases on sensing qualitative requirements:

-	Sensing service area: a service area where sensing services would solely rely on infrastructures and sensing technologies that can be assumed to be present anywhere where 5G is present. This includes both indoor and outdoor environments.

The following KPIs apply to the definition of the use cases on sensing quantitative requirements:

-	Accuracy of positioning estimate describes the closeness of the measured sensing result (i.e. position) of the target object to its true position value. It can be further derived into a horizontal sensing accuracy – referring to the sensing result error in a 2D reference or horizontal plane, and into a vertical sensing accuracy – referring to the sensing result error on the vertical axis or altitude.

-	Accuracy of velocity estimate describes the closeness of the measured sensing result (i.e. velocity) of the target object’s velocity to its true velocity.

-	Confidence level describes the percentage of all the possible measured sensing results that can be expected to include the true sensing result considering the accuracy.

-	Sensing Resolution describes the difference of the measured magnitude of two target objects, which is the magnitude needed to distinguish the two target objects.

-	Range resolution denotes the minimum difference in distance between target objects to have a measurably different range.

-	Velocity resolution denotes the minimum difference in velocity between target objects to have a measurably different velocity.

-	Missed detection describes the probability of missing to acquire a sensing result when the 5G system attempts to acquire a sensing result. It applies only to binary sensing results.

-	False alarm describes the probability of detecting a false sensing result that does not represent the characteristics of a target object or environment when the 5G system attempts to acquire a sensing result. It applies only to binary sensing results.

-	Max sensing service latency: time elapsed between the event triggering the determination of the sensing result and the availability of the sensing result at the sensing system interface.

-	Refreshing rate: rate at which the sensing result is generated by the sensing system. It is the inverse of the time elapsed between two successive sensing results reporting to the application server.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Overview

# 5	Use cases

## 5.1	Use case of intruder detection in smart home

### 5.1.1	Description

Sensing in smart home is a kind of the typical scenarios of indoor/local-area sensing [8]. Considering people spends most of lifetime indoor, how to improve the user experience for indoor scenario is important. Nowadays, various 5G UEs, e.g. wearable device, sensor, smart phone and customer premise equipment (CPE), are deployed at home. In order to enjoy more comfortable and convenient indoor life, various devices are connected via wireless signals to build a smart home platform.

In addition to communication purposes, wireless signals can also be used for sensing, e.g., monitoring the home environment continuously.

For intruder detection in smart home scenario, due to the activities of indoor object or human, the 3GPP signal measured by UE or network would be influenced. By analysing and collecting the sensing information such as Doppler frequency shift, amplitude change and phase change, the behaviour of indoor object or human could be detected as shown in following figure 5.1.1-1 which takes sensing entity that transceives (transmits and receives) the signal case as example.

.

Fig. 5.1.1-1 An example of sensing operation of UE

### 5.1.2	Pre-conditions

Mary and her husband Tom live in a house with little daughter Alice.

On every working day, Mary and Tom have to leave home to work, and Alice needs to go to school. Since the community where the house is located is not stable, Mary and Tom have concern on the safety of their property.

In order to address their concerns, considering protecting the personal privacy and save family cost, Mary sets up some 5G CPEs (i.e. UE) in each room at home, which support sensing functionalities.

### 5.1.3	Service Flows

Mary and her all family members travel to Hawaii in a holiday. At this time, her house is empty. Since she worries about the safety of property, she enables the sensing service on intruder detection of the 5G CPEs (i.e. UE) at home.

Mary’s CPE (i.e. UE) in the living room is activated to perform the sensing operation. While the 5G CPE transmit 5G signals to provide communication services at home, the reflected signals are also be received and measured at the CPE as sensing information. The CPE reports the sensing information to 5G network or further process locally. Via the analysing the differences between the 5G signals and the received reflected signals provided by sensing service performed by 5G system, any potential intruder will not be missed.

Also, Mary’s CPE in the living room can work with other 5G UEs in other rooms. The CPE discovers that the living room has another 5G device (i.e. UE) which could assist the sensing service as secondary device via direct device connection.  The connectivity used in this case is direct device connection, and CPE and this 5G device play as the role of transmitter and receiver, respectively. The receiver measures the 5G signal (e.g., number of detected transmission paths), then provides sensing information to 5G network or further process locally. Via the analysing the differences between the 5G signals and the received reflected signals provided by sensing service performed by 5G system, any potential intruder will not be missed.

An intruder breaks into Mary’s house someday. The sensing service provided by 5G network system assists detecting that the presence of an intruder based on analysing the change of collected signals is aligned with the known feature of the activities of indoor human, and the alarm of intruder is sent to Mary’s smart phone. Mary calls the police for help, and the property is protected.

### 5.1.4	Post-conditions

Thanks to the sensing service provided by 5G UE and network, an intruder is found when Mary is out of home.

### 5.1.5	Existing features partly or fully covering the use case functionality

None.

### 5.1.6	Potential New Requirements needed to support the use case

[P.R.5.1.6-1] The 5G system shall provide a mechanism for an operator to authorize a UE for sensing, e.g., based on location.

Editor's Note:	Terminology of UE/base station needs to be adapted

[P.R.5.1.6-2] The 5G system shall support a UE to perform sensing measurement process based on the trusted 3rd party’s request.

[P.R.5.1.6-3] The 5G system shall provide mechanisms for an operator to only collect or expose the sensing information requested by a trusted 3rd party according to agreement.

[P.R.5.1.6-4] The 5G system shall support UE to perform sensing measurement process using signals received from other UE(s).

[P.R.5.1.6-5] The 5G system shall support UE to perform sensing measurement process in licensed or unlicensed band.

[P.R.5.1.6-6] The 5G system shall be able to provide the sensing service with following KPIs:

Table 5.1.6-1	Performance requirements of sensing results for intruder detection in smart home

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Intruder detection in smart home | Indoor | 95 | ≤10 | ≤10 | N/A | N/A | N/A | N/A | <1000 | < 1 | < 5 | < 2 |
| NOTE: The terms in Table 5.1.6-1 are found in Section 3.1. |  |  |  |  |  |  |  |  |  |  |  |  |

Editor's Note:	it is FFS whether other potential requirements will be identified.

## 5.2	Use case on pedestrian/animal intrusion detection on a highway

### 5.2.1	Description

Transportation as a basic and essential industry plays one of the important roles in a human’s life. Making transportation smarter can make life more convenient and benefit economic development. Highways are an important part of smart transportation. Due to the strong road safety demand on smart transportation, it is necessary to monitor the road situation so as to make appropriate management of road traffic, give guidance or assistance information to vehicles and/or highway traffic safety administration [2].

For example, major accidents caused by pedestrians or animals crossing highways occur frequently [3] [4]. Currently, the highway supervision systems are mainly based on traditional sensors (e.g. radars, cameras) equipped in the roadside infrastructure, but there are still many problems in road supervision system, e.g. it only has partial coverage along the roadside, and the radar may be dedicated for a single usage which requires deploying different types of transportation radars in the same place to satisfy the respective sense use cases and requirements in the area of interest.

Base stations on the roadside are already used to provide 5G coverage for communication, and the radio signals can also be used to sense the environment for object detection.

The assumption of this use case is the following:

-	There is at least 10km long and 33m wide dual three-lane carriageway, which has a central reservation to separate the carriageways, six 3.75m wide lanes and two 3.00m wide shoulders to permit emergency stops [9].

-	The size and typical velocity of traffic participant is described in the Table 5.2.2-1.

Table 5.2.1-1

|  | Size (Length x Width x Height) | Typical velocity |
| --- | --- | --- |
| Pedestrian (Adult) | 0.5m x 0.5m x 1.75m | 5km/h [9] |
| Animal (Sheep/deer) | 1.5m x 0.5m x 1 m | 5km/h [9] |
| Vehicle | 4m x 1.75m x 1.5m | 60km/h - 120km/h |

As described in the figure 5.2.1-1, when the pedestrian/animal standing at the outermost side of the shoulder starts walking on the traffic lane, it means the highway intrusion happens. The distance that pedestrian/animal move perpendicular to the traffic lane (i.e. y direction in the figure 5.2.1-1) is more sensitive for road safety, compared to the distance parallel to the traffic lane (i.e. x direction in the figure 5.2.1-1).

Figure 5.2.1-1: Intrusion detection on a dual three-lane carriageway

### 5.2.2	Pre-conditions

Good partnership and cooperation are established between the road supervision department and Mobile Operator#A in City#B. Requested by the supervision departments for the sensing service, the suitable base stations around/along a highway are selected, which enable Mobile Operator#A to constantly sense the road situation including moving objects (e.g. vehicles and pedestrians). The sensing signal emitted from the base station arrives at vehicles/pedestrians/objects on the road and is bounced (reflected) back to the transmitting base station.

### 5.2.3	Service Flows

Figure 5.2.3-1: Pedestrian/animal intrusion detection

- Fei is a tourist, who is taking a taxi to enjoy the view around the highway in City#B. The base stations around/along the highway constantly sense the road situation. While the taxi is driving on the highway, Fei rolls down the window to take some pictures. Suddenly his mobile phone falls out the window.

- Fei tells the driver to stop and cautiously gets out of the taxi. He crosses the highway and wants to find his mobile phone. Meanwhile, some animals (e.g. sheep and deer) from a farm near the highway approach the road. More and more surrounding vehicles are passing at very high speed. The pedestrian and animals are detected and closely tracked with sufficient accuracy in the sensing area of a base station, and then the sensing measurement is transferred to the core network from the base station and further processed into the sensing results in the core network.

- The sensing results are exposed by the Mobile Operator#A to the road supervision departments and map provider. The map provider adds the position of the vulnerable pedestrian and animals into the HD dynamic maps and transmits warning messages to the vehicles approaching them. Alternatively, RSUs are connected to the traffic control centre for management and control purposes. RSUs transmit warning messages to the vehicles approaching them. The staff working for supervision departments immediately responds to the emergency, launching temporary traffic management, and rushes to the emergency site to fetch the mobile phone for Fei and drive the animals away from the highway.

- Finally, Fei and animals leaves the highway safely. Potential road accident(s) caused by the pedestrian/animal intrusion are avoided.

### 5.2.4	Post-conditions

Thanks to the area-coverage, long-distance sensing capability of the base station (which provides a bird’s-eye-view for monitoring the highway environment) the precision and efficiency of highway management and safety supervision is improved. The network-based sensing can provide timely, continuous, accurate, and comprehensive sensing results, which is a reliable basis for highway safety services.

### 5.2.5	Existing features partly or fully covering the use case functionality

None.

### 5.2.6	Potential New Requirements needed to support the use case

[PR 5.2.6-1] The 5G system shall be able to support a base station to perform sensing.

[PR 5.2.6-2] The 5G system shall be able to support means to select suitable base station(s) to perform sensing, e.g. based on the base station’s location, sensing capability, and the sensing service information requested by trusted third party application.

[PR 5.2.6-3] The 5G system shall be able to support means to configure the sensing operation of a base station(e.g. authorization, sensing activation and/or deactivation, sensing duration, sensing accuracy, target sensing location area).

[PR 5.2.6-4] The 5G system shall be able to support means to enable a base station to transfer sensing measurement data to the core network.

[PR 5.2.6-5] The 5G system shall be able to support means to enable the core network to process sensing measurement data for obtaining sensing results.

[PR 5.2.6-6] Based on operator’s policy, the 5G system shall expose a suitable API to a trusted third party to provide the information regarding sensing results.

[PR 5.2.6-7] The 5G system shall be able to support charging data collection for the sensing services (e.g. considering service type, sensing accuracy, target area, duration) requested by a trusted third party application.

[PR 5.2.6-8] The 5G system shall be able to support the following KPIs:

Table 5.2.6-1	Performance requirements of sensing results for pedestrian/animal intrusion detection

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Pedestrian/animal intrusion detection on a highway | Outdoor (Highway) | 95 | ≤1 | N/A | N/A | N/A | N/A | N/A | ≤5000 | ≤ 0.1 | ≤5 | ≤5 |
| NOTE: The terms in Table 5.2.6-1 are found in Section 3.1. |  |  |  |  |  |  |  |  |  |  |  |  |

## 5.3	Use case on rainfall monitoring

### 5.3.1	Description

Rainfall monitoring is a topic of great importance for several application contexts: hydraulic structure design, agriculture, weather forecasting, climate modelling, etc. At present, the most widely used measurement method is rain gauge.

Traditional rainfall monitoring use rain gauges, which are located at a particular location. Wide-area rainfall monitoring using traditional rain gauges would be costly. The base stations are deployed by the operators with radio cell planning that could cover a wider area. With base stations monitoring the rainfall, for example rain rate (mm/h), it could obtain a horizontally wider-area measurement.

Radio signals, as they propagate through the atmosphere, are reduced in intensity by constituents of the atmosphere. Oxygen and water vapor are the two major components which are responsible for the signal absorption. If it is a rainy day, an additional attenuation caused by rain further increases the propagation path loss. [7] The rain attenuation depends on the size and distribution of the water droplets, hence, by quantifying and modelling the base station signal measurements, we are able to know the rain rate.

The mmWave bands, such as 28GHz and 38GHz have been used to assess coverage, large-scale path loss, and fading and multipath effects [6]. Since the 28 GHz and 38 GHz bands are also licensed for wireless backhaul communications, these frequencies can used for rainfall monitoring [7].

The granularity of the rainfall monitoring could be smaller than the traditional measurements.

### 5.3.2	Pre-conditions

Peter is a farmer who takes care of a big farm that grows different crops. Peter needs to monitor the rainfall of his farm to manage reasonable irrigation, drainage and fertilizer. When there is less rainfall, Peter can select reasonable irrigation plans to improve the farmland water content condition. When there is high rainfall, Peter should improve the drainage system and fertilize the crops to avoid crop losses.

### 5.3.3	Service Flows

- Peter has a subscription for the premium service of rainfall monitoring for a more granular location.

- Peter is at daily working routine and wants to check the timely rainfall information from the weather application on his phone.

- The base station obtains the NR based sensing measurement data every hour and the 5G system processes the sensing measurement data to obtain sensing results and exposes the NR based sensing results to the weather application via the core network.

- Based on the sensing results above, the application server obtains the rainfall information (i.e. rainfall and whether it is raining) associated with location information.

- Peter obtains timely rainfall information from weather application on his phone.

### 5.3.4	Post-conditions

Peter could check the rainfall information at any time on his phone. Based on the timely rainfall information, Peter could plan the irrigation, drainage and fertilizer for the crops in his farm.

### 5.3.5	Existing feature partly or fully covering use case functionality

None

### 5.3.6	Potential New Requirements needed to support the use case

[PR. 5.3.6 - 001] The 5G system shall support  collection of the NR based sensing measurement data from the base station.

[PR. 5.3.6 - 002] Based on operator’s policy, the 5G system shall support mechanisms to process the sensing measurement data to derive the sensing results.

[PR. 5.3.6 - 003] Based on operator’s policy, the 5G system shall provide mechanisms to expose NR based sensing results with assisted information, e.g. location, to a trusted 3rd party application via the core network.

[PR. 5.3.6 – 004] The 5G system shall support sensing services with KPIs as given Table 5.3.6-1.

Table 5.3.6-1	Performance requirements of sensing results for rainfall monitoring

| Scenario | Sensing service area | Confidence level [%] | Rainfall estimation accuracy (for a target confidence level) | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Rainfall monitoring | outdoor | 95 | [1mm/h] NOTE 2 | N/A | N/A | N/A | N/A | N/A | N/A | 1 min | 10min, application configurable | 5 | 5 |
| NOTE 1: The terms in Table 5.3.6-1 are found in Section 3.1. NOTE 2: For rainfall rain rate >1 mm/h. [39] |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 5.4	Use Case on Transparent Sensing Use Case

### 5.4.1	Description

In general, a UE senses using either or combination of the sensors such as camera, NR-based sensing and Lidars, Radars. In NR-based sensing, the UE and BS senses for stationary and moving Objects around UE – using time-difference-of-arrival (TDoA), angle-of-arrival (AoA), angle-of-departure (AoD) measurements, RSSI etc. as shown in Figure 5.4.1-1 [14] Transparent sensing is a use case in which sensing data is captured by the UE and communicated so that the 5GS is aware of the sensing information. From this information, service enablers can be defined. One example of such information is location data, whose corresponding service enabler is Location Based Services.

Figure 5.4.1-1: BS and UE sensing Objects

In this use case, sensing data is made available to the 5GS, and the requirements for this exposure are considered. The data so obtained can be used for diverse purposes. One such purpose is Localization (identifying both a three- dimensional position and orientation.) Transparent Sensing data used for Localization is described in TR 22.856 [11]

Figure 5.Q.1-2: Opaque and Transparent Sensing Data

The distinguishing characteristic of this use case is that the sensing data is provided to the 5GS itself, not merely by the 5GS to an AF out of scope of 3GPP standardization.

### 5.4.2	Pre-conditions

A UE has access to one or more sensors. In this use case. the UE has access to four sensors: NR-based sensing, 3D LiDAR, an RGB Camera and a Smart Phone Camera. The sensors' physical configuration is known (e.g. the cameras are 10 cm apart). The NR-based sensing capabilities of the UE and its connected BS are used to capture information about the nearby environment by the UE.

A mobile network MN supports the acquisition of sensing data. We term this support by the network a 'sensing data consuming service.'

### 5.4.3	Service Flows

The user U activates a mechanism to enable NR-based sensing at UE and MN and provide sensing measurement data to the 5GS. This process is analogous to activating or enabling a location tracking service.

MN acquires sensing measurement data provided by U's UE, for a period of time.

The user U deactivates the mechanism to provide sensing measurement to the 5GS.

Editor's Note: Additional details on the use of LiDAR, cameras and/or spatial localization need to be added.

### 5.4.4	Post-conditions

The sensing data acquired by the 5GS is processed in order to enable other services. The processed information can for example provide 'Spatial Localization' information that can be exposed to authorized third parties, as discussed in 22.856 [11]. "Spatial Localization Use Case".

### 5.4.5	Existing feature partly or fully covering use case functionality

Positioning in 5G Networks been proposed in 3GPP release-16, it specifies positioning signals and measurements for the 5G NR. In release-16, 5G Positioning architecture extends 4G positioning architecture by adding Location Management Function (LMF) and Transmission reception points (TRP).  5GS provides new positioning methods based on multi-cell round-trip time measurements, multiple antenna beam measurements, multiple to enable downlink angle of departure (DL-AoD) and uplink angle of arrival (UL-AoA) [15][16]. The Rel-17 5G system supports positioning of the device-based but not device-free – objects that do not radiate EM signals. [14][15][16]

The 5GS already supports transport of sensor data. The table below provides indicative performance requirements for media used for sensor information communication.

| Sensor Type | Uplink KPI | Remarks |
| --- | --- | --- |
| 3D Lidar | 30 Mbps | An example 3D LiDAR: 16 channel, 0.3M data points, dual return mode 2 bytes distance, 1byte [13] |
| Industrial RGB Camera | 16 ~ 800 Mbps | 2,592 x 2,048 x 10bits x 2.5 Hz x 6 EA, compression ratio 2% |
| Smart Phone Camera | 4 ~ 200 Mbps | 2,160 x 2,880 x 8bits x 1 Hz x 4 EA, compression ratio 2% |

Figure 5.4.5-1: Performance Requirements (already possible to fulfill with the 5GS)

### 5.4.6	Potential New Requirements needed to support the use case

[PR.5.4.6 - 001] Subject to user consent and national or regional regulation, based on operator policy, the 5GS shall support a mechanism to receive uplink sensing measurements from a UE that is authorized to provide uplink sensing measurements to the network.

NOTE: 	This requirement assumes there is some functionality in the 5GS to discern and interpret the acquired sensing measurements.

[PR.5.4.6 - 002] Subject to user consent and national or regional regulation, based on operator policy, the 5GS shall support a mechanism to expose sensing result to authorized third parties.

[PR.5.4.6 - 003] Subject to user consent, network operator policy and national or regional regulation, the 5GS shall support a mechanism to enable RAN entities and UEs to acquire NR-based sensing measurement data to capture information about the nearby environment.

Editor's Note: It is FFS if explicit requirements for LiDAR, cameras and other sensors need to be added.Editor's Note:	Using existing E-UTRA measurements is FFS.

## 5.5	Use case on sensing for flooding in smart cities

### 5.5.1	Description

Due to the climate change in recent years, a larger amount of rain sometimes falls within a short duration of time inside a small area. This result, in particular in urban areas, in inundation and flooding even in areas where these did not happen in the past. When flooding is about to happen on roads, people might enter areas getting in danger without knowing it. Once flooding really happens there, this might result in loss of human life. At places where flooding is expected to occur, monitoring of flooding is performed using cameras and other sensors. However, due to the recent climate change, it may be difficult to recognize places where flooding is expected to occur. Using radio waves, it is possible to recognize places where flooding occurs in an efficient way.

### 5.5.2	Pre-conditions

Good partnership and cooperation are established between Mobile Operator #A and administrators of roads such as a local government in City #B. Mobile Operator #A constantly senses the surface of the road and informs results of sensing to the administrator of the road.

### 5.5.3	Service Flows

Figure 5.5.3-1: Sensing for flooding in smart cities

- Base stations owned by Mobile Operator #A are deployed around the road. Mobile Operator #A carries out sensing of the surface of the road in City #B. This sensing is performed using radio wave. Results of sensing information, incl. whether flooding occurs on the road, are informed to the administrator of the road in City #B.

- The administrator of the road usually monitors the state of flooding on the road using information from sensors including information from Mobile Operator #A. In addition, in the case of heavy rain, the administrator may request Mobile Operator #A to increase frequency of monitoring of situation of roads and Mobile Operator #A monitors the situation more frequently responding to this request.

- If there is information received that flooding occurs, the administrator advises people in the areas concerned to evacuate the areas. The administrator advises via mobile networks.

- People who received the advice evacuate the areas or do not enter such areas.

- Now City #B trusts Mobile Operator #A and allows it to advise people about evacuation without City #B's intervention in case of flooding. Next time a similar flooding occurs, Mobile Operator #A sends advice for evacuation directly to people.

### 5.5.4	Post-conditions

Damage of the flooding has been kept at minimum.

### 5.5.5	Existing features partly or fully covering the use case functionality

TBD.

### 5.5.6	Potential New Requirements needed to support the use case

[PR. 5.5.6 - 001] Subject to operator policy, the 5G system shall be able to provide sensing result indicating disasters or other emergencies (e.g., flooding) in a given geographic area to authorized third parties in a timely manner.

[PR. 5.5.6 - 002] Subject to regional or national regulatory requirements and operator policy, the 5G system shall be able to provide its public warning system with a warning notification based on sensing result indicating disasters or other emergencies (e.g., flooding) in a given geographic area in a timely manner.

## 5.6	Use case on intruder detection in surroundings of smart home

### 5.6.1	Description

Detection of an intruder including a person or a harmful animal into a private property is an important piece to ensure residents at home in the private property feel comfortable and secure. For the surroundings monitoring, various technologies, such as cameras, infrared cameras, and microwave radars are being used. However, these technologies require line-of-sight, and therefore locations which can be monitored may be limited.

Wireless signals make it possible to monitor locations without line-of-sight and to monitor wider areas [17]. Sensing by wireless signals can complement the afore-mentioned technologies and can improve accuracy of the detection. Sensing by wireless signals gives residents time to prepare against intruders or to drive them away.

### 5.6.2	Pre-conditions

UEs such as smart phones and consumer premise equipment are installed inside a house, in particular, near a wall or a window. Residents have a contract with a mobile operator for the UEs.

### 5.6.3	Service Flows

Figure 5.6.3-1: Intruder detection in surroundings of smart home

- The UEs such as smart phones and CPE communicate with base stations in the outdoor or in the indoor and monitor 3GPP signals which are influenced by outdoor objects such as humans and animals. In addition, the UEs communicate with base stations of the mobile operator and monitor the radio wave state between the UEs and the base stations.

- When an intruder enters the site, the radio signals are changed. The core network processes the data and yields sensing result indicating detection of the intruder.

NOTE:	Cases that such an intruder or an animal is already indoor are addressed in the use case in clause 5.1.

- The residents are informed of detection of the intruder.

### 5.6.4	Post-conditions

The residents report to the police or the security service and request them to take an appropriate action.

### 5.6.5	Existing features partly or fully covering the use case functionality

TBD.

### 5.6.6	Potential New Requirements needed to support the use case

[PR. 5.6.6 - 001] Subject to operator policy, the 5G system shall be able to collect sensing measurement data and yield sensing result from the data for detection of outdoor objects.

## 5.7	Use case on sensing for railway intrusion detection

### 5.7.1	Description

Extensive railway deployment and the changing wildlife habitat area due to the changing global environment has led to increase of crash of wildlife to trains. Once a crush happens, its recovery costs, takes time, and impairs convenience [18, 19]. Such a crush should be avoided, but it appears difficult to proactively predict wildlife's intrusion onto railway track. It's different from e.g., weather forecast. Passively detecting wildlife's intrusion onto railway track appears an option to take. Monitoring with cameras serves the same purpose. However, this requires LOS (i.e., line of sight) and a dense deployment of cameras, which is not necessarily efficient. Another traditional mechanism using fibre optic sensing techniques is costly and requires manual intervention, making it very difficult to meet the increasing demand for railway monitoring. Thanks to the 5G NR based sensing, the base station as transmitter and receiver along the railway can constantly sense the railway situation such as railway intrusion.

The assumption of this use case is the following:

-	There is at least 300km train line as depicted in the figure 5.7.1-1[19] owned by railway operator. The safe place is a place where a person and their equipment cannot be struck by rail traffic, which is used for minimizing damage caused by possible railway accident or crash and for ensuring safe operation of railway. The danger zone is anywhere within 3m horizontally from the nearest track.

-	The typical size and velocity of intruder and train in this use case are described in the Table 5.7.1-1.

Table 5.7.1-1

|  | Size (Length x Width x Height) | Velocity |
| --- | --- | --- |
| Intruder | Pedestrian(Adult): 0.5m x 0.5m x 1.75m | 5km/h |
|  | Animal(Sheep/deer): 1.5m x 0.5m x 1 m | 5km/h |
| Trains | 24m x 3.5m x 3 m | 100km/h - 350km/h |

When the intruder standing at the outermost side of safe place starts walking on the danger zone, it means the intrusion happens. The distance that intruder move perpendicular to the railway track is more sensitive for road safety, compared to the distance parallel to the railway track.

Figure 5.7.1-1

### 5.7.2	Pre-conditions

Base stations are deployed near and along a railway track which enable the mobile operator to constantly sense the railway including intruder (e.g., pedestrians and animal). For sensing, signaling transmitted by a base station is influenced or bounced by objects around the railway and then monitored by the base station and other base stations. Sensing result is being notified to a railway operator by the mobile operator. The railway operator knows locations of trains.

### 5.7.3	Service Flows

Figure 5.7.3-1 Railway intrusion detection

- Base stations are deployed near and along a railway track. In order to acquire the sensing information of railway, railway operator requests sensing service from mobile operator. The mobile operator configures the base stations along the train line to perform sensing. Suddenly, an intruder (e.g. pedestrian or animal) is walking on the danger zone.

- The sensing measurement data is reported from base stations and further processed into the sensing results by the core network. The mobile operator exposes the sensing results to the railway operator. Based on the sensing results, the location of the intruder can be estimated.

3.	Trains running on the railway track measure their own location and velocity. These trains inform that information to a controller of the railway operator.

4.	The controller identifies a train that is affected by an intruder based on the sensing results from mobile operator and train's location and velocity.

5.	The controller orders the train to slow down or stop. In addition, the staff working for railway operator immediately responds to the emergency. The intruder leaves the danger zone safely.

### 5.7.4	Post-conditions

The controller judges the intruder is gone and safety can be ensured. The controller permits the train to start again or speed up.

### 5.7.5	Existing feature partly or fully covering use case functionality

TBD.

### 5.7.6	Potential New Requirements needed to support the use case

[PR. 5.7.6 - 001] Subject to operator policy, the 5G system shall enable the core network to collect and aggregate sensing measurement data from base stations.

[PR. 5.7.6 - 002] Subject to operator policy, the 5G system shall enable the core network to expose a suitable API to provide the information regarding sensing results to authorized third parties.

[PR. 5.7.6 - 002] The 5G system shall be able to support the following KPIs:

Table 5.7.6-1	Performance requirements of sensing results for railway intrusion detection

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Intrusion detection on a railway | Outdoor (Along railway) | 95 | ≤1.5 | N/A | N/A | N/A | N/A | N/A | ˂1500 | ≤ 0.1 | ≤0.1 | ≤0.1 |
| NOTE: The terms in Table 5.7.6-1 are found in Section 3.1. |  |  |  |  |  |  |  |  |  |  |  |  |

Editor's Note: the KPI values for Missed Deteciton and False Alarm are FFS.

## 5.8	Use Case on Sensing Assisted Automotive Maneuvering and Navigation

### 5.8.1	Description

To support smart transportation and autonomous driving, more vehicle and devices are equipped with sensing technologies. For example, cameras, Radar, and Lidar systems are the most used sensors by the automotive industry to maintain the perception for autonomous vehicles at various levels of autonomy. Accurate sensing results are crucial to enable the safe and reliable control of the vehicles.

Due to the mounting position of the sensors (e.g., NR-based sensors) information collected from a single vehicle's sensors may not be sufficient or accurate enough to satisfy the advanced automotive use cases, e.g., autonomous driving, coordinated manoeuvre, etc. Therefore, the 5G system could coordinate sensing to enable the vehicles derive NR sensing inputs (i.e., sensing measurement data or sensing results) which could be consumed at the vehicle and used for the vehicular control and driver assistance, e.g., feed into the Automated Driving System (ADS) in the car [21]. Alternatively, the NR sensing measurement data or sensing results collected by the UE can be sent alongside relevant sensing information to other sensing entities (including other vehicles, roadside units, and network) for further processing (if required) before sharing with a 3rd party application as shown in Figure 5.8.1-1.

The network facilitated NR based sensing described above could significantly improve the sensing reliability and quality, enabling new and advanced automotive use cases.

Figure 5.8.1-1: 5G System Assisted Automotive maneuvering and navigation

### 5.8.2	Pre-conditions

In this use case, Joe and Bob’s vehicles are equipped with NR-based sensing technology. Non-3GPP-based sensors like radar, camera and Lidar sensors could also be available in the vehicles. Additionally, the vehicles are capable of 5G communications, including direct communication with other vehicles, communication with 5G system via RAN entities.

### 5.8.3	Service Flows

5G system assisted coordination of sensing service

Step 1 (Network provides configurations and policies): When Bob’s car registers for NR sensing service, the network provides policies and configurations to enable UEs take appropriate actions during sensing e.g., obtaining sensing inputs from another UEs/RAN entities.  For example, the policies provided by network could provide guidance for the discovery UEs/RAN entities with appropriate NR RF sensing capabilities, when to trigger requests, when to stop sending requests, messaging formats, the communication configurations (such as which 5G communication mode to use and under which conditions), etc. These polices and configurations could be updated frequently as determined by the network.

Step 2 (Bob determines his sensors are blocked): Bob's sensor(s) is(are) blocked by Joe's vehicle, and cannot adequately detect its surroundings (e.g., detect if there is another vehicle in front). This could result in the vehicle miscalculating the needed distance to stop before a traffic light. In other cases, Joe's vehicle could also reduce the valid sensing region and result in misdetection of incoming vehicles size or shape, especially near intersections. The sensing results cannot fully satisfy the autonomous driving needs and requirement.

Step 3 (Bob recognizes need for sensing inputs): Due to unsatisfactory autonomous driving needs and requirements, the UE in Bob's vehicle is notified that its sensors are blocked and needs 5G System assistance for coordination of the sensing service.

Step 4 (Bob’s vehicle discovers Joe’s vehicle): With the policies and configurations provided by the 5G system, Bob’s vehicle can search for neighbouring UEs/RAN entities or ask the network to provide recommendations for UEs/RAN entities and their NR RF sensing capabilities (e.g., if UE/RAN entity supports sensing service). This information would be used to discover other vehicles and RAN entities with NR RF sensors that can support sensing in the area. In this example, Bob's vehicle discovered Joe's vehicle could be useful in providing sensing inputs.

Step 5 (Bob’s vehicle connects to Joe’s vehicle): Bob's vehicle then establishes 5G communication connection with Joe's vehicle and/or RAN entities as shown in Figure 5.y.3-1. The most suitable 5G communication mode (e.g., broadcast, unicast, etc) is determined by the Bob’s vehicle based on 5G system configuration and policies.

Step 6 (Bob's vehicle requests sensing info from Joe’s vehicle). The request could indicate the information needed to perform sensing, e.g., the additional region to be covered, additional sensing target, synchronization info, etc.

Step 7 (Joe sends sensing results/sensing measurement data to Bob’s vehicle) Based on the information provided by Bob’s vehicle; Joe sends Bob sensing results identifying objects in its surroundings. Joe’s car also supports the capability to forward sensor measurement data so it forwards that also to Bob’s car.

Step 8a (Bob processes sensing data locally) Based on the fact that Bob’s has non-NR sensors (e.g., camera, Lidar), Bob’s car combines the sensing measurement data from Joe’s vehicle with other sensors and displays the sensing results on the application in its autonomous driving application in his car.

Step 8b (Bob sends NR and non-NR sensing results to 3rd party application) Alternatively, Bob sends NR sensing results and results from the camera and Lidar to a 3rd party application server (through the network) for fusion of the sensing results from NR and non-NR sensors.

Figure 5.8.3-1:  5G system assisted automotive maneuvering and navigation

With the sensing information provided by Joe's vehicle and/or the network, Bob's vehicle obtains a full map of the region. The autonomous driving algorithm can make corresponding decisions reliably.

### 5.8.4	Post-conditions

Using 5G system assistance, Bob's vehicle would be able to achieve highly reliable and accurate sensing results, by coordinating the operation with other vehicles to collaborate with other sensing devices to improve quality. With this high-quality sensing results, advanced smart transportation use cases and autonomous driving could be achieved.

### 5.8.5	Existing features partly or fully covering the use case functionality

V2X communication supports the information exchange among the vehicles, between vehicle and infrastructure or network.

Editor’s note: Further gap analysis is FFS.

### 5.8.6	Potential New Requirements needed to support the use case

[PR 5.8.6-1] The 5G system shall be able to support mechanisms to control UEs and RAN entities for a sensing service.

NOTE1:	In the requirement above, control may include configuration such as sensing specific policies and settings (e.g., conditions for triggering sensing requests, location,  etc.) coordinated amongst UE and RAN entities.

[PR 5.8.6-2] For a sensing service, the 5G system shall be able to support mechanisms for the UEs and RAN entities to provide sensing measurement data.

NOTE2:	This requirement can cover scenarios making use of information already available in the EPC and E-UTRA (assuming no new functionalities are required in the EPC and E-UTRA).

[PR 5.8.6-3] The 5G system shall be able to support an authorized UE in the discovery of UEs and RAN entities with the required NR RF sensing capabilities for the sensing service.

[PR 5.8.6-4] The 5G system shall be able to provide means to authorize and configure a UE for sensing operation (e.g., based on location, time, etc) and for establishing the communication connection needed to assist the sensing service.

NOTE3: 	The above requirement assumes that the communication connection used for sensing service can include direct network communication, direct device connection and indirect network connection.

[PR 5.8.6-5] The 5G system shall be able to support exposure of UEs' sensing results, capabilities, and configurations to a trusted 3rd-party application, e.g., to facilitate 3rd-party control and coordination of sensing inputs from one or more sensing UEs.

## 5.9	Use case on AGV detection and tracking in factories

### 5.9.1	Description

Improving safety and work conditions in factories and industrial environments is a critical component for industry 4.0. Replacing communication cables with wireless connections has already positively changed the factory environment, by providing reliable ethernet-like communications, and enabling time-sensitive networking over the air. Nevertheless, despite automation and improvement, accidents in factories still occur, leaving room for improvement. Indeed, 5250 fatal work injuries were recorded in the US only in 2018, according to the Bureau of Labour Statistics [22], a 2% increase from 2017.

Automated Guided Vehicles (AGVs) are key components of the new smart factories, used for a variety of tasks such as heavy or hazardous materials transportation and distribution. Simultaneous presence of AGVs and human workers at the industrial side creates safety challenges and calls for stringent safety requirements [23]. For example, the driverless, automated guided industrial vehicles ANSI/ITSDF B56.5 [24] safety standard requires that “the AGV shall detect and avoid both static and dynamic obstacles appearing in the path of travel direction”. Reliable detection of AGV/human presence or proximity is therefore an important safety criterion.

5G system can be deployed in a factory which uses RAN entities and/or UEs to measure sensing measurement data, that are made available to sensing management entities in order to derive sensing results such as the detection of the presence or proximity of AGVs and humans. This use case assumes support of NR-based RF sensing.

### 5.9.2	Pre-conditions

Company #A operates multiple AGV in its factory. Each AGV is programmed to perform certain tasks, such as transporting large containers from point #B to point #C following a programmed route. AGVs can be of various sizes and operate at different speeds and locations. In a factory, workers are dispersed throughout the area, performing different tasks. Workers-AGVs interactions are a source of potential injuries, and extra care needs to be taken to avoid any harm.

The factory deploys 5G based integrated communication and sensing system with RAN entities and UEs throughout the factory floor. The RAN entities deployment is done to optimize communication, positioning, and sensing.

The RAN entities and/or UEs perform sensing operations over certain target areas throughout the factory. The deployed RAN entities (or a subset of the RAN entities) transmit sensing reference signals, which are received by a subset of RAN entities and/or selected UEs. Some UEs are authorized and configured to monitor the sensing reference signals and report sensing measurement data to a sensing entity in the 5G system. The sensing entity can be deployed either locally in the factory or in the cloud/edge.

In this use case it is important to note that AGVs do not actively participate in the sensing signals transmission or reception, and hence it is more applicable to AGVs which are not equipped with UEs, e.g., legacy AGVs. For those AGVs with UEs, the UEs can be helpful in sensing and tracking humans on the factory floor.

### Service Flows

Figure 5.9.3-1: AGV presence and proximity detection

- Alex is working in his section of the factory (shown in the lower left area in Figure 1), performing regular maintenance work around a conveyor belt.

- An AGV, AGV#1, is approaching the area where Alex is working, carrying a heavy load to be placed at a designated location next to the conveyor belt.

- Using the sensing measurement data from the RAN entities and the UE carried by Alex, the sensing entity processes the data to obtain sensing results and detects the proximity of the AGV1 to Alex. The sensing results is shared with a safety monitoring application of the factory, and a notification is sent to Alex to warn him of the approaching AGV.

- Another AGV, AGV#2, enters an area (lower right area in Figure 1) with increased risk for workers due to higher workers presence and higher equipment and machines density. Based on the sensing measurement data from RAN entities, the sensing entity processes the data to obtain sensing results and detects the presence of AGV#2 and exposes the detection event to the factory safety monitoring application. The safety monitoring application triggers a warning sound to warn the workers (e.g., John and Emma) in that area of the approaching AGV.
Note that, in this scenario, none of the UEs was involved in the sensing session. However, the sensing entity can use sensing measurements from UEs in the area (e.g., UE carried by Emma) in its sensing processing if available.

- In another scenario, John (lower right area in Figure 1), working on his section, not having a UE, is being tracked using radio sensing measurements from RAN entities and/or UEs. When John comes in proximity with an AGV, which has or does not have a UE, a warning message is sounded to alert John.

### 5.9.4	Post-conditions

Thanks to the warning messages, workers are safe and potential accidents caused by workers-AGVs interactions are avoided. By leveraging the sensing capability of the 5G based integrated communication and sensing system, the factory safety supervision is upgraded, and workers safety is enhanced.

### 5.9.5	Existing features partly or fully covering the use case functionality

Editor’s note: Gap analysis for this use case is FFS.

### 5.9.6	Potential New Requirements needed to support the use case

NOTE1: The following requirements apply to networks managed by PLMN or NPN.

[PR 5.9.6-1] The 5G system shall be able to provide means to support NR-based sensing in a certain area or location.

[PR 5.9.6-2] Based on operator policy and location area, the 5G system shall be able to provide means to support per-UE authorization for NR-based sensing.

[PR 5.9.6-3] The 5G system shall be able to support means to enable RAN entities and UEs to transfer sensing measurement data to sensing processing entities in the 5G system responsible for processing and aggregation of the sensing measurement data.

NOTE2:	The “Sensing processing entities” in the above requirement refer to one or more entities in the 5G system responsible for aggregating and processing of sensing measurement data (e.g., core network).

[PR 5.9.6-4] Based on operator’s policy, the 5G system shall be able to support means to expose sensing results to a trusted 3rd-party application.

Editor’s note: The KPIs for this use case are FFS.

## 5.10	Use case on UAV flight trajectory tracing

### 5.10.1	Description

With the development of UAV technologies and the increase of demands on rapid logistics, aerial photographing, environmental monitoring and public security, a variety of commercial UAV services gradually become reality.

Normally the commercial UAVs fly based on predetermined flight routes, following regulated positions, heights, speeds, and directions. E.g., a package-delivery UAV flies from the package sender to the package recipient; a task-execution (such as environmental monitoring) UAV flies from the UAV airport to the target area.

On-route flying is important for these commercial UAVs. Their flight routes are optimized and permitted by UAV service operators, UAV management department, or USS (Uncrewed Aerial System Service Supplier)/UTM (Uncrewed Aerial System Traffic Management). Usually, they have the shortest flight distance, avoid no-fly zone, and keep safe distance from obstacles (e.g., building, trees, hills) and other commercial UAVs.

Although a UAV is equipped with sensors to keep itself along the flight route, the external UAV flight trajectory tracing function is still necessary because these sensors sometimes are restricted. E.g., the camera is impacted by light situation; the UAV-borne radar is impacted by rainfall or snowfall, etc. If these events occur, UAV cannot correctly decide its own position, height or speed, and thus cannot follow the traced route.

Although there exist dedicated UAV surveillance equipment and radar, their large-scale deployment has great challenges due to lack of available sites and high installation and maintenance cost.

In comparison, using the 5G system can provide a cost-effective way to trace these UAVs, e.g., 5G network infrastructures with ubiquitous coverage can better trace the flight trajectory of each UAV.

Specifically, 5G RAN entities can relies on radio sensing to obtain the information on UAV position and motion (e.g., distance, angle) and send sensing measurement data to a sensing processing entity located in the 5G system.

As shown in Figure 5.10.3-1, the UEs that are connected to the 5G RAN entities can be configured to assist in the sensing operations, which can increase the sensing coverage, provide more positioning reference points, and improve sensing result accuracy and robustness. This improvement is a result of  higher density of UEs compared to the base stations,  which increases the probability that some UEs are located in positions that have shorter distance away from UAV than 5G RAN entities (e.g., UAV located in the middle of two 5G RAN entities while UE locates under UAV), or some UEs are located in the reflection directions that have larger radar cross section (RCS) than 5G RAN entities considering the UAV RCS variation in different reflection directions.

The 5Gsensing processing entity can collect the sensing data from one or multiple network infrastructures.

The 5G network operator can provide the UAV flight trajectory tracing service to a trusted 3rd party application (e.g., UAV service operator, UAV management department, USS/UTM) as requested.

Figure 5.10.1-1:  UAV flight trajectory tracing by 5G system

### 5.10.2	Pre-conditions

A UAV operator/UTM provides package delivery service in an area which is covered by 5G network. The UAV operator/UTM subscribes to the UAV flight trajectory tracing service from the 5G network operator.

The UAV operator/UTM provides the 5G network operator the characteristics of the UAV to be sensed, time and space (covering the regulated UAV flight routes and possible off-route locations) of the UAV flight trajectory tracing service.

### 5.10.3	Service Flows

When the appointed time starts, 5G network operator activates the UAV flight trajectory tracing function at the appointed space until the appointed time ends.

The UAV operator controls UAV#1 to take off from package delivery source and fly toward package delivery destination along a regulated flight route.

By radio sensing, a set of 5G base stations and UEs detect UAV#1, and then estimate the position and motion related metrics (e.g., distance, angle) as well as the target object is in coverage, resulting in sensing measurement data. The 5G base stations and UEs then send the sensing measurement data to the 5Gsensing processing entity.

In certain cases, during the flying course, based on sensing and location information, if it is detected that UAV#1 has left the coverage of an old base station and entered the coverage of a new base station, the old base station could stop radio sensing and operate in a power saving mode. The new base station starts and keeps on sensing UAV#1 until it is out of coverage. Note that the determination that the UAV#1 has left the coverage of a base station or not could be determined based on the UAV positions and velocities estimated at the 5G sensing processing entity. Therefore, the network could then decide to activate and deactivate sensing in certain base stations based on this information. In other cases, the network could configure a start and stop of sensing operations for a base station based on a specified time period.

The 5G sensing processing entity collects the UAV sensing measurement data from one or multiple base stations and UEs, and estimates the positions and velocities, and sends in real time the sensing results (e.g., UAV positions, velocities) to the UAV operator and/or UTM.

Based on the received sensing results, the UAV operator and/or UTM traces the flight trajectory of UAV#1. Once the UAV operator and/or UTM detects an off-route event, it further steers UAV#1.

### 5.10.4	Post-conditions

UAV#1 delivers package to the destination along the traced flight route or its off-route behavior is sensed.

### 5.10.5	Existing features partly or fully covering the use case functionality

None.

### 5.10.6	Potential New Requirements needed to support the use case

[P.R 5.10.6-1] Based on operator policy, request from UTM and sensing configuration (e.g. sensing area), the 5G system shall be able to support RAN entities and UEs in sensing the characteristics of an airborne object of interest (e.g., UAV), including generating sensing measurement data related to the object’s location and motion metrics (see examples in Table 5.10.6-1).

[P.R 5.10.6-2] The 5G system shall be able to support means to authorize RAN entities and UEs in certain location area generating and reporting sensing measurement data (e.g., related to a UAV position, velocity) to a 5G sensing processing entity.

NOTE: 	The requirement above assumes that the sensing measurement data is post-processed in 5G sensing processing entity which is located within the 5G system.

[P.R 5.10.6-3] The 5G system shall be able to support means to process the sensing measurement data and expose in real time the sensing results (e.g., related to a UAV position, velocity) from a 5G sensing processing entity to a trusted 3rd party application.

[P.R 5.10.6-4] The 5G system shall support energy efficient sensing operations.

[P.R 5.10.6-5] The 5G system shall support sensing services with KPIs as given in Table 5.10.6-1.

Table 5.10.6-1	Performance requirements of sensing results for UAV flight trajectory tracing

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution (horizontal/vertical) [mxm] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| UAV flight trajectory tracing | Outdoor | N/A | N/A | N/A | N/A | N/A | 1m x 1m ~10m x 10m (NOTE 2) | 1m/s x 1m/s ~ 10m/s x 10m/s (NOTE 3) | 5~50 (NOTE 4) | 0.3~5Hz (NOTE 5) | 5 | 5 |
| NOTE 1: The terms in Table 5.10.6-1 are found in Section 3.1. NOTE 2: To detect the UAV existence (e.g., for invation detection), the sensing resolution of distance is 10m [25]. To track the UAV flying (e.g., for collision detection and warning), the sensing resolution of distance is 1m [25] NOTE 3: To detect the UAV existence, the sensing resolution of velocity is 10m/s [25]. To track the UAV flying, the sensing resolution of velocity is 1m/s [25] NOTE 4: To detect the UAV existence, the sensing latency is approximate 50ms [25]. To track the UAV flying, the sensing latency is approximate 5ms [25] NOTE 5: To detect the UAV existence, the sensing refreshing rate is 0.3~5Hz. To track the UAV flying, the sensing latency is 0.3~2Hz. Echodyne MESA-DAATM has approximate 1Hz scan rate [40]. |  |  |  |  |  |  |  |  |  |  |  |  |

## 5.11	Use case on sensing at crossroads with/without obstacle

### 5.11.1	Description

The various ways of transportation (e.g. vehicles, walking people, motor vehicle, non-motor vehicle) and the dense buildings make the traffic condition complicated. Typically, traffic accidents often happen at the crossroads for example the pedestrians suddenly rush to the road from the invisible place (e.g., behind the high buildings, behind the tall trees), which cause an urgent need to monitor the real-time road status for all days, thus with the collaboration of trusted 3rd party e.g. map service provider or ITS management platform, driving warning or assistant driving information can be provide timely to vehicles.

The road status includes vehicle moving information, VRU(Vulnerable road user) information (e.g. VRU location, VRU moving direction, VRU moving speed, etc.), abnormal vehicle behaviour, road obstacles  and road condition.

The road status information can be sensed by the cameras and radars on RSU (Road Side Unit). But considering the crossroad condition is very complicated, there are always some blind points. 5G based sensing can provide sensing information to fill these gaps.

For example, it is expected that the base station can sense the surrounding environment e.g. the road, and send the sensing measurement data to the core network. The core network can carry out systematic calculation and analysis of the sensing measurement data for outputting the sensing result. Such sensing result can be sent to a trusted 3rd party e.g. map service provider for combination with navigation map data, so as to make the driver aware of the congestion and traffic accidents in advance, and effectively increase the comfort and safety of driving. The base station sensing operations could improve the real-time map service with high reliability and quality.

But in some cases of above, the obstacles (e.g., high buildings or trees) block the transmission of radio signals. The availability and accuracy of the sensing service for the target objects which are located in the area will be greatly impacted.

To guarantee sensing service in this area, multiple 5G system sensing entities can work together.

### 5.11.2	Pre-conditions

Network operator “VV” has released a sensing service for road status sensing, and has deployed base stations especially at multiple crossroads to continuously sense the road status.

Due to the high buildings (e.g. Building A) near the crossroads, there are some areas with obstacles for 5G base stations. Some 5G system sensing entities are further deployed by the network operator ‘VV’ to help radio signal transmission and collect sensing measurement data.

Network operator “VV” has a collaboration with the ITS management department that the user who has registered the Network operator “VV”’s “road status sensing service” can receive real-time road status information, driving warning or assistant driving information from ITS management platform.

Bob has registered the road status sensing service from Network operator “VV”.

Network operator “VV” can also deliver the real-time road information and the real time location/ trajectory of vehicles to a map service provider. The map service provider can provide “assisted driving service” based on this information.

Tom has a vehicle with the “assisted driving service” provided by the map service provider.

Tom drives the vehicle from home to the company in the morning of a working day.

### 5.11.3	Service Flows

Figure 5.11.3-1. sensing at crossroads with/without obstacle

- The 5G base station continuously collects sensing measurement data of the road status and the sensing result is continuously reported to the trusted 3rd party (e.g. the map service provider or ITS management platform) by 5G network according to the preconfigured refresh rate (e.g. in the midnight, it uses slow refresh rate with 0.2Hz, and in the working day morning, it uses fast refresh rate with 10Hz). The refresh rate can be adjusted according to the trusted 3rd party demand and network operator’s policy.

- In the working day morning, Bob has started his road status sensing service when he begins driving his vehicle to his office.

- Tom drives his vehicle from home to his office and started assisted driving service. The map service provider sends the road sensing request to the 3GPP core network.

- In the crossroad, there are some higher buildings. It is difficult for Tom and Bob to timely detect other vehicles and VRUs in the area. As example in figure 5.x.3.1, Bob is driving his vehicle and crossing the crossroad toward the southeast of the crossroad. Linda is driving her motorcycle on a side road toward the main road which is also the southeast of the crossroad. The line of sight between Bob and Linda is blocked by the high building A which is at a corner of the intersection.

- Linda’s motorcycle activity is continuously sensed by the base station under the help of other 5G system sensing entities.

- The motorcycle sensing result which includes the motorcycles moving speed, moving direction, position etc. is periodically reported to the the map service provider and ITS management platform.

- The other vehicles in the crossroad have been sensed and related sensing result are also reported to the map service provider and ITS management platform.

- The map service provider fuses the sensing result with the map and then sends to the Tom’s vehicle.

- According to the continuously received motorcycle sensing results, the ITS management platform can analyze and identify that there will be a potential collision risk between Bob and Linda. The collision warning then is sent to Bob.

### 5.11.4	Post Conditions

Tom’s vehicle receives the real-time map information which warns Tom that there is another cross-direction motorcycle driving towards his vehicle. Tom stops his vehicle before the crossroad to avoid a potential collision. With the assistance of RAN sensing, Tom arrives in the company safely and easily. Tom starts the daily work in the office.

Bob receives the warning and drives safely through the crossroads.

Linda can also ride safely to the crossroad. The potential risk of collision is avoided.

### 5.11.5	Existing features partly or fully covering the use case functionality

None.

### 5.11.6	Potential New Requirements needed to support the use case

[PR 5.11.6-1] The 5G system shall be able to support a mechanism to provide available sensing service in a sensing service area location.

[PR 5.11.6-2] The 5G base station shall be able to collect sensing measurement data from requested sensing service area location according to the operator’s policy.

NOTE 1:	The operator policy means to configure the sensing service area location, real time sensing measurement data collection or periodic collection etc.

[PR 5.11.6-3] The 5G system shall be able to report the sensing result to the trusted 3rd party with refresh rate which is requested by the trusted 3rd party e.g. a map service provider, and controllable by the operator, according to a business agreement.

NOTE 2:	The sensing result can be the target object’s size, shape, position, moving direction, moving speed, etc.

[PR 5.11.6-4] The 5G system shall support means for a trusted 3rd party application, e.g. a map service provider to configure sensing per location.

[PR. 5.11.6-5] The 5G system shall be able to support the sensing service with given KPIs in Table 5.11.6-1.

Table 5.11.6-1 KPI Table of Sensing

| Scenario | Sensing Distance |  |  | Sensing Angle/direction |  |  | Moving Speed of target object |  |  | Interval between two consecutive sensing fixes | Latency (ms) | Service Availability (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Accuracy(m) | Resolution(cm) | Distance Range (m) | Accuracy | Resolution | Angle range | Accuracy | Resolution(m/s) | Speed range(km/h) |  |  |  |
| sensing at crossroads with/without obstacle | [2] (note 1) | [36.6] (note 2) | [500] (note 3) | FFS | FFS | FFS | FFS | [0.52] (note 2) | [<=70] (note3) | FFS | [<100] (note3) | [99.9] (note3) |
| NOTE 1: The value is sourced from [26]. NOTE 2: The value is sourced from [27] NOTE 3: The values are sourced from [28] |  |  |  |  |  |  |  |  |  |  |  |  |

Editor’s Note: Other KPIs are FFS.

Editor’s Note: The KPI term and table format will be updated.

## 5.12	Network assisted sensing to avoid UAV collision

### 5.12.1	Description

With the help of current 5G networks, the commercialization of low-altitude UAVs has entered a new stage. UAV can perform surveillance, early warning for many scenarios, and other tasks in low altitude airspace below commercial flights such as delivery. In the logistics industry, UAV delivery is developed very quickly and is estimated to become a nearly 10-billion-euro market. UAV delivery can be widely used in food distribution, retail commodity delivery, postal delivery, provision of medical aids, precision agriculture delivery, industrial delivery, etc.

While the UAV is applied in so many industries, how to avoid collision and effectively manage the UAV traffic are key challenges. In general, the UAV can provide its moving information and surrounding dynamic environment sensed by its own sensors to UTM (Uncrewed Aerial System Traffic Management), then the UTM controls the flight trajectory of the UAV accordingly. But the sensing range of a single UAV is limited and during a UAV flying, the UAV surrounding environment status will not be detected in time which will cause the UAV deviation or collision.

Using the wide coverage of 5G network, a UE on boarding UAV can be a subscriber of the 5G network and connect with UTM via the 5G network.

As shown in figure 5.12.1-1, through the communication connection between the 5G base station and the UE on boarding UAV, the UE can provide its positioning information and UE ID to 5G network. The 5G network and UTM can corelate the UE positioning information, UE ID with UAV ID.  Based on it, on one hand, the 5G RAN nodes can work together to send sensing signal toward specific direction, angle, area to track the flight of the UAV.  On the other hand, the UE can collect the reflection signals from its environments and send the sensing measurement data associated with the UE ID to 5G network via the communication connection.  Some sensing information of the UAV flying environment, e.g. higher building, obstacles and other UAVs nearby, which will impact its safe flying can be collected by UE onboarding a UAV and then reported to 5G core network to be exposed to the UTM. Furthermore, continuous sensing service can be provided during UAV flight.

The UTM is using different inputs like classic radar, via systems currently used in general aviation like FLARM or ADS-B. In this sense, UTM already combines different sources of location information and could further use 5G sensing as additional source for the specific UAV to avoid it deviating from course and collision. When multiple UAVs appear in the same area, the base station also can sense them at the same time.

Figure 5.12.1-1 Network assisted collision avoidance for the UAVs

The following service flow gives an example of UAV delivery in retail goods delivery.

### 5.12.2	Pre-Conditions

Network Operator ‘MM’ provides a new 5G service named ‘5G Sensing Service’.

The UAV City Express ‘SS’ uses a specific UTM to assist its retail goods UAV delivery.

This UTM uses ‘5G Sensing Service’ provided by 5G network Operator ‘MM’ as additional source of information and navigate the UAVs.

Tom has ordered online daily necessities from a supermarket.  Tom is living in downtown.

Jerry has also ordered online some food from a supermarket. Jerry is living in countryside.

The supermarket prepares the goods in packages and asks City Express ‘SS’ to deliver them to Tom and Jerry.

City Express ‘SS’ dispatches UAV A for Tom, and UAV B for Jerry.

UE A is on board UAV A and UE B is on board UAV B. Both UE A and UE B are subscribed to the 5G network of Operator ‘MM’.

Through the communication connections between the 5G base station and the UE A/ UE B, the UE provides its positioning information and UE ID. The 5G network and UTM corelate the UE positioning information, UE ID with associated UAV ID.

### 5.12.3	Service Flows

The UAV A and UAV B are flying to their destinations under the guidance of UTM with the assistance of the ‘5G Sensing Service’ provided by network Operator ‘MM’.

Considering that UAV A will fly to downtown, the UTM asks network Operator ‘MM’’s ‘5G Sensing Service’ to provide sensing service for UAV A, and the required sensing result includes the flying environment along its trajectory, e.g. altitude of the buildings, obstacles and other UAVs nearby.

Considering UAV B will fly to the countryside, the UTM asks network Operator ‘MM’’s ‘5G Sensing Service’ to provide sensing service for UAV B, and the required sensing result includes the flying environment along its trajectory e.g. obstacles, and other UAVs nearby.

The UTM requests the report period about UAV A and UAV B.

Each base station continuously sends sensing signaling along the UAV A’s trajectory, and the UE A on board of the UAV A can send the sensing measurement data which it collects for its surrounding environment back to the base station using the 5G communication connection. Then, the 5G network can obtain a comprehensive UAV A’s flying environment sensing result e.g. building position, altitude, other nearby moving objects e.g. other UAV’s relative position, altitude, degree of moving angle, moving speed etc. to UTM.

Same sensing operation is also for UAV B.

The 5G network reports the sensing result periodically according to UTM’s request.

The UTM adjusts and guides the UAV flying trajectories considering the received sensing result and input from other sources (e.g. FLARM, ADS-B).

Considering UAV A is flying toward downtown, both the flying environment (e.g. many buildings) and wireless environment are complex compared with UAV B and its environment in countryside, the 5G network need to configure different sensing operation for UAV A and UAV B to guarantee required sensing service quality, for example to operate sensing with shorter period, sensing KPI, and report sensing result with higher refresh rate for UAV A.

### 5.12.4	Post-Conditions

The UAV A successfully delivers package to Tom and UAV B successfully delivers package to Jerry and return safely.

### 5.12.5	Existing features partly or fully covering the use case functionality

None.

### 5.12.6	Potential New Requirements needed to support the use case

[PR 5.12.6 -1] The 5G system shall be able to provide a sensing service to track one specific target object and the environment around the target object with the assistant information provided by the UE on board the specific target object or authorized 3rd party.

NOTE 1: 	The assistant information for example can be UE position, UE ID.

[PR 5.12.6 -2] The base stations shall be able to sense multiple specific target objects and their environments at the same time.

[PR 5.12.6 -3] The 5G system shall be able to provide a mechanism controllable by the operator, according to a business agreement, for a trusted 3rd party to request the sensing service related with a certain target object or multiple target objects of a certain location area.

[PR 5.12.6 -4] Based on operator policy, the 5G system shall be able to provide a mechanism for a trusted 3rd party to request per location area different sensing services configuration (e.g. sensing KPI, report refresh rate etc.).

[PR 5.12.6 -5] The 5G system shall be able to report sensing result of the environment around a specific target object to a trusted 3rd party.

NOTE 2: 	The sensing result of the environment for example can be its position, the size of obstacles around, and other moving objects nearby.

[PR 5.12.6 -6] The 5G system shall be able to provide sensing service with follow KPIs.

Table 5.12.6-1	Performance requirements of sensing results for network assisted sensing to avoid UAV collision

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Network assisted sensing to avoid UAV collision | Outdoor | [99] | 1 | N/A | 1 (NOTE 2) | N/A | <1 (NOTE 2) | 1 | 500 | 0.5 | N/A | N/A |
| NOTE 1: The terms in Table 5.12.6-1 are found in Section 3.1. NOTE 2: The KPI values are sourced from [25] and [40] |  |  |  |  |  |  |  |  |  |  |  |  |

Editor's Note: Other potential new requirements are FFS.

## 5.13.	Use case on sensing for UAV intrusion detection

### 5.13.1	Description

UAV industry is developing quickly around the world with the widely usages in various scenarios such as aerial photography, police force, urban management, agriculture, geology, meteorology, electric power, emergency rescue and disaster relief, etc. Especially for the smart city in future, a large number of UAVs will be used to improve the quality of our daily life including industrial inspection, public security patrol, cargo transportation, live broadcast and so on. However, this also brings big challenges on UAV supervision due to the following reasons:

1) Low-altitude UAVs have characteristics as large number, small size, wide flying zone, widely used to execute complex and diverse tasks, which makes UAV supervision very difficult if only using the traditional radar system.

2) Non-cooperative UAVs could intrude some no-fly zone (e.g. airport, military base) intentionally or unintentionally which would lead to serious consequences, e.g. exposing private information using the camera, blocking other UAV traffic on the flying route.

5G radio signals can be used to provide wireless access for communication, meanwhile the 5G radio signals can also be used to generate sensing data for object detection e.g. sense presence or proximity of UAVs illegal flying in a specific area. 5G System could provide sensing service by processing sensing data and output sensing information (e.g. relative position, altitude, distance, velocity, direction).  In this case, 5G System could be used for sensing the UAV intrusion in the scenarios of UAV illegal flying in restricted area include light rail, airports, government facilities, research institutes, high-speed railway stations, temporary performance venue and other permanent or temporary restricted areas.

Furthermore, considering that the UAV entering the restricted area is illegal and the UAV itself even could be illegal, this kind of sensing operation doesn’t require the cooperation of the UAV. That means the UAV may be unaware of the sensing operation.  When multiple UAVs appear in the same restricted area, the 5G system can sense presence or proximity of multiple UAVs illegal flying at the same time.

Figure 5.13.1-1 UAV collision risks at light rail

### 5.13.2	Pre-conditions

Network operator ‘MM’ provides 5G sensing service for the park and the light rail area with its 5G network covering the park and the light rail track. ‘MM’ can make use of wireless base stations to sense the airspace within their coverage area and report the sensing information to the USS/UTM as defined in TS 23.256 clause 3.1.

The Light rail operator ‘XX’ uses a UTM to management potential UAV illegal intrusion along the light rail tracks. ‘XX’ has provided its restricted area information to the UTM.

There is a need to hold a ceremony with high security requirement in the park temporarily, turning the park in a restricted area where UAVs are not allowed to enter. The administrator has a subscription for UAV prevention service from the USS/UTM.

The UTM uses ‘5G Sensing Service’ provided by 5G network Operator ‘MM’ to detect potential UAV illegal intrusion for above scenarios.

The UTM requests that once a UAV is detected that its distance from the border of the restricted area is less than 10m, the 5G system should report the event to the UTM.

The Network operator ‘MM’ can configure energy consumption sensing mode with different sensing period, e.g. operate sensing one time per 50 seconds, per 10 seconds, per second etc. And in emergency condition, the 5G system can provide continuously sensing service according to the UTM’s request.

The light rail works from 5:30 am to 23:00 pm every day.

### 5.13.3	Service Flows

Figure 5.13.3-1: Sensing for UAV intrusion detection

The 5G system periodically senses the restricted area whether there are UAVs flying into the restricted area border for both the Park and the light rail area.

There are three UAVs (A, B, C) flying around the restricted areas.

When UAV A flying near the Park is detected and closely tracked with required accuracy in the sensing area, the 5G system reports the sensing results to the UTM in real time and begins continuously sensing.

When UAV B and UAV C flying near the light rail are detected, and closely tracked with required accuracy in the sensing area, the 5G system reports the sensing results to the UTM in real time and continuously senses.

To reduce energy consumption, the 5G system will notify the UTM that the 5G system cannot detect any UAVs illegal flying after a time period which is requested by the UTM. After that, the 5G system stops continuously sensing and begins periodically sensing operation according to the Network Operator’s policy.

The USS/UTM could trigger to send warning messages/notices to UAV controller based on analytical results based on the sensing information from the mobile network. Alternatively, the USS/UTM will trigger UAV countermeasures to prevent the UAV from flying in the no-fly area.

When the ceremony has been finished, the 5G system would stop sensing operation based on the request from UTM. And when the light rail stops operation between 23:00 pm to 5:00am next morning, the 5G system stops sensing operation to save energy.

### 5.13.4	Post-conditions

The mobile network can provide sensing service for UAV intrusion detection with high quality and continuity, to improve the accuracy and efficiency of public safety supervision and management.

USS/UTM interacts with the mobile network for sensing service and perform UAV intrusion detection based on the sensing information exposed by network.

### 5.13.5	Existing features partly or fully covering the use case functionality

None.

### 5.13.6	Potential New Requirements needed to support the use case

[P.R 5.13.6-001] The 5G system shall be able to provide a sensing service by using base stations to collect sensing measurements.

[P.R 5.13.6-002] The base station shall be able to sense a target object by obtaining sensing measurement without active involvement of the target object.

[P.R 5.13.6-003] The 5G system shall provide mechanisms for an operator to transport sensing data from base station towards the core network.

[P.R 5.13.6-004] Based on operator’s policy and subject to regulatory requirements, the 5G system shall be able to provide a mechanism for a trusted 3rd party to request the sensing service and based on the request, the base station shall be able to operate sensing periodically or continuously in certain location area for a certain amount of time.

[P.R 5.13.6-005] Based on operator’s policy and subject to regulatory requirements, the 5G system shall be able to periodically expose sensing results to a trusted 3rd party application.

[P.R 5.13.6-006] The 5G system shall provide a mechanism controllable by the operator, according to a business agreement, to report sensing result to a trusted 3rd party about a target object and multiple target objects when specific conditions are met.

NOTE: These conditions could be the target object distance from the restricted area border less than 10m or entering restricted area.

[P.R 5.13.6-007] The 5G system shall be able to support the activation and deactivation of the sensing service according to operator’s policy.

[P.R 5.13.6-008] The 5G system shall be able to provide a mechanism for network operator to configure and adjust sensing operation (e.g. authorization, sensing area, sensing operation period and sensing operation time window etc.) based on request from a trusted 3rd party.

[P.R 5.13.6-009] The 5G system shall be able to provide sensing with following KPIs.

Table 5.13.6-1	Performance requirements of sensing results for UAV intrusion detection

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| UAV intrusion detection | Outdoor | 95 | ≤10 | ≤10 | N/A | N/A | 10 | [5] | [≤5000] | [≤0.1Hz] | [≤5] | [≤5] |
| NOTE: The terms in Table 5.13.6-1 are found in Section 3.1. |  |  |  |  |  |  |  |  |  |  |  |  |

## 5.14.	Use case on sensing for tourist spot traffic management

### 5.14.1	Description

In order to ensure the sustainable development of tourist spots, the traffic flow management of tourist attractions should fully consider the space-carrying capacity, facility-carrying capacity, ecological-carrying capacity and other factors that may induce disasters within the area.

The scenic area controls the traffic flow through real-time monitoring, diversion of traffic and early warning and reporting. The flow control of tourist spots includes two aspects: passenger-flow management and vehicle-flow management.

Traffic data collection is an important part of traffic management. Base stations in tourist area can provide 5G communication service and also can sense the passenger and the vehicle in its coverage at gates or per unit area that are set with a finer granularity. For tourist spots with a large area, it will be convenient to use base station to have the traffic sensing data sources when it's difficult to deploy equipment like camera and other sensors.

### 5.14.2	Pre-conditions

Network Operator A provides 5G services for a famous tourist spot.

The management department of the tourist spot has subscribed the sensing service provided by 5G network Operator A. And the base stations in the tourist area can be used to sense the traffic flow and the crowd density (for both including the vehicles and passengers) constantly.

Jim is the worker of the tourist spot and responsible for traffic management.

### 5.14.3	Service Flows

Figure 5.14.3-1: Sensing for tourist spot traffic management

1.	When the scenic area begins to open. Jim will operate the scenic area traffic monitoring system to start real-time traffic control.

2.	The traffic management system of the scenic spot will send a service request to the operator network to start sensing the people and vehicles in the scenic spot.

3.	The base station at the entrance and exit of the scenic spot can sense the people and vehicles those will enter or leave the place, and the base stations in the scenic spot can sense the people and vehicles for certain area (e.g. walkway, parking area).

4.	Operator A reports the traffic sensing information from the base stations in the scenic spot to the traffic monitoring system. Based on the sense information, the traffic management system could analyse the traffic status and decides whether the traffic in  the area is congested.

5.	If the congestion exceeds the threshold, the management system would notice Jim about the detail, and Jim would trigger to limit traffic to avoid traffic overload in the scenic spot.

### 5.14.4	Post-conditions

With 5GS support to the traffic management system, the vehicles and tourists are controlled within a reasonable range, and the spot can operate normally during business hours.

### 5.14.5	Existing features partly or fully covering the use case functionality

None.

### 5.14.6	Potential New Requirements needed to support the use case

[P.R 5.14.6-001] The 5G system shall be able to provide means to use base station(s) to perform sensing in certain area.

[P.R 5.14.6-002] Subject to regulatory requirements and operator policy, the 5G system shall be able to expose sensing results to a trusted 3rd party application.

[P.R 5.14.6-003] Subject to regulatory requirements and operator policy, the 5G system shall be able to support the activation and deactivation of the sensing service based on location.

[P.R 5.14.6-004] The 5G system shall be able to provide sensing service with KPIs given in Table 5.14.6-1.

Table 5.14.6-1	Performance requirements of sensing results for tourist spot traffic management

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Tourist spot traffic management | Outdoor | 95 | [≤2] | N/A | N/A | N/A | [1] | [1] | [≤5000] | [≤0.1] | [≤5] | [≤5] |
| NOTE: The terms in Table 5.14.6-1 are found in Section 3.1. |  |  |  |  |  |  |  |  |  |  |  |  |

## 5.15	Use case on contactless sleep monitoring service

### 5.15.1	Description

Compared with wearable devices, contactless sensing technologies have more advantages in health status detection. With more room has reserved for un-intrusion, contactless sensing-based health monitoring gives people, especially who view object contact as uncomfortable, more options. 3GPP system are designed for catering people’s communication purpose, whose wireless signals are very rich and can be accessible ubiquitously. With additional processing, 3GPP system will breed new opportunities with contactless sensing technologies applied, such as smart health, smart home, smart city and even smart space.

Sleep Monitoring application describes the case that a human’s sleep situation is monitored without any wearable device [31]. Instead of utilizing capacitors as propagation medium, Sleep Monitoring application effectively reuses the current ubiquitously accessible medium, that is wireless signals to realize the sensing purpose. People’s presence, movement and even respiration will affect the wireless signal propagation, which on the receiving side will be presented as the fluctuation of waveform’s intensity, phase shift and etc.

Figure 5.15.1-1 describes how the wireless signals that propagated via the established direct network connection (i.e. between the radio access network and 5G UE) will be affected and distorted by the target sensing object. Generally, when people is sleeping, regular chest rise and fall will cause additional vibration of the target object when detecting the doppler [37], this is defined as the micro doppler effect in radar [32]. By observing the micro doppler effect, people’s respiration rate per minute can be counted.

Figure 5.15.1-1: People’s respiration affected 3GPP wireless signal propagation in an indoor environment

NOTE 1: The transmitter as shown in Figure 5.15.1-1 is an indoor small base station as described in TS22.261 [33].

NOTE 2: The transmitter as shown in Figure 5.15.1-1 can also be a CPE that is used for this service.

This sleep monitoring application can help to diagnose early symptoms of some diseases, e.g. milder symptoms of sleep apnea before it develops worse [xa]. Through monitoring people’s breathing, i.e. respiration rate, and the breathing stoppage duration, the application server can give instructions to the user on whether or not the user is experiencing sleep apnea, and the user in return can adjust lifestyles such as losing weight or quitting smoking to avoid worse cases.

-	A person's respiratory rate is the number of breaths you take per minute. The normal respiration rate for an adult at rest is 12 to 20 breaths per minute. A respiration rate under 12 or over 25 breaths per minute while resting is considered abnormal [42].

-	The breathing stoppage duration is the amount of time that a sleep apnea patient stops breathing, which can be from 10 seconds to two minutes or more [41]. We take breathing stoppage duration = 10 seconds for example as the trigger of the event reporting to the application server. When the user triggers this sensing service, the sensing system will monitor this special event and report it to the application server.

### 5.15.2	Pre-conditions

The device installing this sleep monitoring application is 5G UE.

There is a service agreement between MNO and sleep monitoring operator. The MNO can also be the sleep monitoring application provider.

### 5.15.3	Service Flows

1. The application user Bob triggers the sleep monitoring application on the 5G UE. When the application server receives the request, the application server contacts the 5G system to trigger the sensing service to monitor Bob’s respiration rate.

2. 5G system discovers a base station (or CPE) to start the sleep monitoring sensing service.

3. The base station (or CPE) coordinates with Bob’s phone (5G UE) to perform the sensing measurement. The base station and the 5G UE can be transmitter and receiver or vice versa. The receiver measures the 5G wireless signals (e.g., number of detected transmission paths, micro doppler shift, etc.) and collects them as the sensing measurement data.

4. Sensing measurement data is processed to derive the sensing results (e.g. respiration rate) locally or is provided to the 5G network: 5G network processes the sensing measurement data to derive the sensing results and exposes the sensing results to the sleep monitoring application server.

5. The 5G UE receives the sleep monitoring feedback from the application server and shows it to the application user Bob. Bob can have sleep apnea and needs the application to further monitor his breathing stoppage duration. An event with “breathing stoppages duration = 10 seconds” is triggered by Bob and received by the application server, which then contacts the 5G system to trigger this event.

6. 5G system adjusts the sensing measurement process and executes Steps 3-5. When the event report criteria are satisfied, i.e. Bob is detected to have a 10-second breathing stoppages duration, the application server will receive the notification sent by 5G system.

### 5.15.4	Post-conditions

The user experiences the sleep monitoring application enabled by the 5G network. Bob changes his lifestyle, he does more exercise, and tries to lose weight to avoid the sleep apnea problem.

### 5.15.5	Existing feature partly or fully covering use case functionality

None

### 5.15.6	Potential New Requirements needed to support the use case

[PR.5.15.6-1] The 5G system shall support mechanisms to discover and configure a UE and a base station to perform sensing measurements in a certain sensing service location area.

[PR.5.15.6-2] The 5G system shall support mechanisms to derive and expose sensing results to a trusted 3rd party.

[PR.5.15.6-3] The 5G system shall be able to provide 5G wireless sensing service with the following KPIs:

Table 5.15.6-1	Performance requirements of sensing results for contactless sleep monitoring

| Scenario | Sensing service area | Confidence level [%] | Motion rate accuracy | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Sleep monitoring | Outdoor (bedroom) | 95 | 2 times/min [0.033Hz] (NOTE 2) | N/A | N/A | N/A | N/A | N/A | N/A | 60s | 60 | 5 (NOTE3) | 5 (NOTE3) |
| NOTE 1: The terms in Table 5.15.6-1 are found in Section 3.1. NOTE 2: Respiration rate = 18 times/min as reference, any detected value in [16,20] satisfies accuracy requirements NOTE 3: Detect event = “breathing stoppages duration >= 10 seconds” as reference |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 5.16	Use case on Protection of Sensing Information

### 5.16.1	Description

This use case re-uses the scenario where a UE performs sensing to detect intruders in the home, as per use case 5.1 (intruder detection in smart home). The additional aspect introduced in this use case is that there is an unauthorised user that is attempting to collect sensing information from Mary's home.

### 5.16.2	Pre-conditions

Refer to use case 5.1 where 5G CPEs (i.e. UEs) are set up to detect intruders when Mary's home is vacant as her family is on holiday.

### 5.16.3	Service Flows

An unauthorised user is in the vicinity of Mary's home.

In Mary's home, the 5G CPE transmits 5G signals, and the reflected signals are used by the unauthorised user's device to collect sensing information.

As the 5G signals from the CPEs in Mary's home are protected, the unauthorised user's device fails to derive any sensing information.

### 5.16.4	Post-conditions

The privacy of sensing information in Mary's home is preserved.

The unauthorised user cannot use the 5G signals to detect that the family is not at home.

### 5.16.5	Existing features partly or fully covering the use case functionality

None

### 5.16.6	Potential New Requirements needed to support the use case

[PR 5.16.6-1] The 5G system shall provide a mechanism to protect identifiable information that can be derived from the sensing measurements data from eavesdropping.

## 5.17	Use case on health monitoring at home

### 5.17.1 	Description

Tom is an elderly person living in his house. Since he has become weaker, he has subscribed to a wireless sensing service of his MNO so that his health state (including e.g. lack of movement, detection of falls, breathing rate)  can be monitored 24/7 when he is at his home. Wireless sensing is a promising technology for health monitoring [34] [35] [36] [37] that does not require a person to wear a health monitoring device on his/her body (which people may forget, requires recharging, and can be uncomfortable to wear over long periods of time).

A single base station is not capable of covering Tom’s home with good coverage. Thus, multiple base stations capable of acting as wireless transmitters and/or receivers cooperate to ensure excellent coverage. Furthermore, the received reflected radar signal is sometimes weak, and thus, the MNO offers the possibility of using a phone with wireless sensing receiving capabilities. The usage of the phone also allows more accurate measurements of certain vital signs (e.g. breathing rate) since the phone is close to Tom. The usage of the phone also allows the MNO to offload the workload from the base station to the phone. Also other UEs in vicinity of Tom could take part in the sensing.

Fig. 5.x.1.1 shows a schematic illustration of how such system could look like, whereby the blue arrow indicates transmitted wireless sensing signals from Base Station A, and the green dashed arrows indicate reflected wireless sensing signals received by Base Station B and Tom’s phone.

Figure 5.17.1-1: Example of a distributed sensing system (incl. two base stations, a UE and a Sensing function).

### 5.17.2	Pre-conditions

1.	Tom has subscribed to the sensing service offered by an MNO.

2.	The MNO has deployed two RAN entities (e.g. base station A and base station B) that are capable of wireless communication and sensing. The base stations can act as wireless sensing transmitters and/or wireless sensing receivers.  These two base stations are sufficiently close to Tom's house to provide good coverage in and around Tom's house in the frequency bands used for wireless sensing. Tom’s subscription includes a phone with wireless sensing capabilities for more accurate sensing.

3.	Tom has a mobile phone that is capable of detecting wireless sensing signals. Tom can use it to directly and/or more accurately sense his health state.

4.	The sensing measurement data from the base stations and UEs is collected and processed by a sensing function that can be deployed in the 5GS or provided by an external application or a combination thereof. The exact separation of functionalities between those entities is not explored further in this use case. The sensing function is assumed to be capable of extracting health state information, e.g. lack of movement, detection of falls, breathing rate from these sensing measurements, determine the sensing requirements (e.g. accuracy), and determine the criteria/thresholds (e.g. lack of movement) on when to create an alert.

### 5.17.3	Service Flows

1.	Based on Tom’s sensing subscription, information about a user (in this case Tom) is obtained including information about where he lives and sensing requirements that are needed (e.g. sensing of movements which can be used to detect falls or sufficient activity of Tom).

2.	Base station A starts transmitting the wireless sensing signal.

3.	Tom is currently located in the living room. If Tom is at this location, base station A can hardly receive the reflection of its transmitted sensing signal. However, base station B can receive a strong reflection of that sensing signal. Base station A and B coordinate with each other so that Base Station B is capable of processing the received reflected wireless sensing signal, generating sensing measurement and/or sensing results that are sent to a sensing function for further processing. In this manner, movements of Tom in and around the house can be monitored, and it can be detected if Tom falls.

4.	Tom feels a bit weak today and decides to measure his health state in more detail. Tom was told that he needs to carry his phone to enable this. Tom picks up his phone and uses it as a wireless sensing receiver capable of picking up and processing the reflected wireless sensing signal transmitted by Base station A. This requires the phone to coordinate wireless sensing with Base station A, which includes for example exchanging of capabilities (since the sensing capabilities can differ per phone) and coordinating of timing/frequencies of sensing signals. Since Tom’s phone is very close to Tom, Tom can use his phone for more accurate sensing of certain vital signs, such as breathing rate and heart rate. The phone sends measurements to a sensing function for further processing. When Tom goes to sleep, he puts his phone next to him to monitor his vital signs also during the night.

5.	When Tom’s health state is determined to be in danger, e.g., when Tom falls or stops moving, the family or emergency services gets alerted of such event.

### 5.17.4	Post-conditions

The sensing service/application receives accurate sensing measurements about Tom and can generate alerts if an adverse event happens to Tom.

### 5.17.5	Existing features partly or fully covering the use case functionality

None

### 5.17.6	Potential New Requirements needed to support the use case

[PR 5.17.6-1] The 5G system shall be able to coordinate wireless sensing among a set of RAN entities and UEs.

[PR 5.17.6-2] The 5G system shall support a mechanism for the 5G network to retrieve the wireless sensing capabilities from UEs and RAN entities, and for the UEs and RAN entities to exchange capabilities amongst each other.

[PR 5.17.6-3] The 5G system shall support a mechanism for two or more authorized UEs and/or RAN entities to take part in the wireless sensing of a target, whereby the authorization may be provided based on location.

[PR 5.17.6-4] The 5G system shall support a mechanism to provide wireless sensing capable UEs and RAN entities with information of which network entity or destination server to send the sensing measurement data to.

## 5.18	Use case on service continuity of unobtrusive health monitoring.

### 5.18.1 	Description

An elderly home has installed a new 5G system capable of providing communication and sensing capabilities through the facilities as illustrated in Figure 5.18.1-1. The deployed 5G system includes multiple sensing devices, e.g., base stations, providing connectivity and sensing capabilities. These sensing devices can perform wireless sensing of a target, in this case, health monitoring (e.g. fall/activity detection [34][35][36] or wireless sensing of vital signs such as heart rate [38] or breathing rate [37] of one or more persons). Since elderly people move through the facilities, it is important to provide health monitoring independently of the base station used for sensing. The staff of the elderly home really likes this new 5G wireless sensing feature because it is unobtrusive and offers various advantages over the old system that they use with body worn sensors. For example, they don’t need to recharge or replace the batteries of body worn sensors anymore and remind people or help people to wear them after they took them off (for example to take a shower). The elderly people themselves also like it more, since the body worn sensors often made them feel uncomfortable, especially during sleep or during hot days. Installing cameras was not seen as a good alternative because of the privacy concerns.

In the provided use case, base stations cooperate with each other to ensure service continuity for sensing of a ‘target’ user. In this particular scenario, a user, Robert, is considered who moves through the facilities. Robert's health is quite frail and requires continuous monitoring of his health state without interruption. Robert is currently sensed by means of (indoor) base station A located near his room and is moving out of the sensing area of base station A and approaching the sensing area of base station B covering the recreation/eating area and part of the hallway. Base station A and base station B cooperate in such a way that it is ensured that base station B has started wireless sensing of Robert before base station A stops its wireless sensing of Robert. When Robert is in range of both base station A and B, both base stations can cooperate to perform simultaneous wireless sensing. Similarly, when Robert decides to go for a walk to the garden that is covered by Base Station C, the sensing of Robert is seamlessly continued by Base Station C. The sensing measurement data is collected and processed by the 5G system (e.g. to detect certain movement patterns) and then exposed to a sensing application that is automatically monitoring health anomalies. If a health anomaly is detected (e.g. Robert falls down), an alarm is triggered indicating the health condition as well as the location of the monitored user.

Figure 5.18.1-1: Example of service continuity between Base stations A, B and C.

### 5.18.2	Pre-conditions

1.	MNO operates a 5GS providing wireless sensing capabilities through a set of base stations installed in the elderly home and its garden, as illustrated in Figure 5.x.1-1.

2. 	Robert has subscribed to the wireless sensing service offered by the 5GS in cooperation with an external application provider. Robert provided some identification information, e.g. which room he resides in, the identity of his mobile phone and/or some physical characteristics (e.g. length). The application provider has no knowledge of the RAN infrastructure operated by the MNO.

### 5.18.3	Service Flows

1.	Robert is currently located in his room in the elderly home. The closest nearby base station, i.e. base station A infers, based on the identification information provided by Robert, that Robert is in his room. Base station A starts wireless sensing of Robert, whereby it sends the sensing measurements to the 5GC for further processing, after which the sensing results are sent to a sensing application to detect health anomalies

2. 	Robert starts moving toward the garden.

3.	When leaving his room and entering the hallway, the wireless sensing signal conditions of base station B become better than those of base station A.

4.	The 5G system coordinates the responsibility of sensing Robert from base station A to base station B. During this time, both base station A and B might sense Robert.

5. 	Base station B is used for sensing Robert

6. 	Base station A can stop sensing Robert.

7. 	When leaving the elderly home and entering the garden, Base Station C continues the sensing of Robert.

### 5.18.4	Post-conditions

Robert’s vital signs are monitored without interruption independently of his location.

### 5.18.5	Existing features partly or fully covering the use case functionality

None

### 5.18.6	Potential New Requirements needed to support the use case

[PR 5.18.6.1] The 5G system shall support service continuity for wireless sensing of a target between different sensing devices.

Editor’s Note: It is FFS if [PR 5.18.6.1] can be solved at application layer.

[PR 5.18.6.2] The 5G system shall support simultaneous wireless sensing of a target by means of multiple sensing devices.

## 5.19	Use case on Sensor Groups

### 5.19.1	Description

Sensing has been considered in this technical report in terms of interaction between a UE and a base station. This information however is only partial, as it extends along a limited UE-base station axis. This information however can be considered a component of a scene that, when gathered with other available sensor data, can be synthesized into more comprehensive information.

Where the sensor is video, LiDAR, sonar, etc., (that is, it operates in some other way than 3GPP defined radio access technology,) it is still valuable to gather simultaneous sensor data and combine it. Only this way can sensor data capturing a scene (such as the front and back and sides of an object of interest, etc.) be obtained.

In the Localized Mobile Metaverse Services use case 5.1 in [11], includes the following text "His mobile device begins to collect information about his surroundings. The collected information can include information that is obtained by interacting with nearby devices (e.g. sensors and other mobile devices)."

This use case explores the implications of this function. Specifically, how can a UE identify sensors that are present that can provide information sought by the UE?

In this particular use case, on a construction site, a crane is lifting a large object near a tower. Construction worker safety, efficient pursuit of tasks and other situational awareness to prevent disasters are important tasks. Instead of merely relying on the crane operator, this use case allows the 5G system to model and track the tower, crane and payload as it is in motion.

Figure 5.19.1-1: Group Sensing, to provide sensing services

In Figure 5.19.1-1, the UE is on a construction site. It seeks to identify sensors available at that site to provide sensing data potentially relevant to obtaining information for the user. These sensors - in the UE's proximity and available to provide sensor data (that is, this service is authorized,) comprise a sensor group.

The use case assumes that sensors that can form a sensor group are either UEs or communicate by means of a UE (as a kind of 'split terminal equipment (TE)' UE.)

It is essential to capture a 'synchronous' group of sensors and their movements in 3 dimensions in order to optimally combine the sensor data for the purpose of localization computations. This is particularly important when the viewer is near a large object or any non-static object must be modeled on all sides. This is an active area of research, for example in 6DoF Tracking and sensing. [44] The techniques described in this paper concern how to determine the 3D position and orientation of drones with synchronization, but it is clear that this is a fundamental requirement of a group of sensors providing input on a single physical object in (absolute or relative) motion.

Editor's Note: The relation between a sensor group and other group features defined in 3GPP (e.g. MCX groups) is FFS.

Editor's Note: The relation between synchronized sensing data and multistatic measurement is for further study.

In this use case, a user's UE identifies a sensor group, through interaction with an AS. Part of identification of the sensors in the group is obtaining sufficient information that the user can become authorized to obtain sensor data, and the relevant service access information is obtained. The goal of the use case is to enable the acquisition of sensor data in a UE's proximity. The communication of the sensing data itself is out of scope of this use case.

Discovery of the kind described here can potentially be accomplished by different radio technologies, e.g. NR ProSe, UWB, IEEE 802.11, etc.

### 5.19.2	Pre-conditions

Benoît inspects various construction sites. He has a UE equipped with a set of surveillance and appraisal applications.

On construction sites he visits, there are sensors deployed. Some are UEs, e.g. using NR-based sensing. Other sensors include video cameras, LiDAR equipment and passive infrared sensors. These are not, generally, installed directly in the terminal equipment, but rather use the terminal equipment to communicate, as shown in figure 5.19.2-1.

Figure 5.19.2-1: Sensors available to become a sensing group

In the scenario above, (a) is a UE that serves various sensors that are themselves not UEs. The means by which these sensors communicate with the UE is out of scope of this use case. They could be e.g. connected by means of a physical cable. (b) is a UE that is capable of 3GPP defined sensing. (c) is a UE that can operate the UE camera for sensing purposes.

In this example (a) and (b) are authorized and ready to send mobile originated sensing data to an AS.

The sensing measurement data (non-RF sensing) and sensing result (RF sensing) acquired by the AS can be combined in software in a manner that is out of scope of the 5G standard. In this use case, the combination is performed by the AS.

Benoît's UE, (c), is authorized to access the media (the sensing result output of the AS produced by taking account of the sensing measurement data from (a) and (b), etc.) provided by the AS.

NOTE:	The architecture shown above is only an example - to indicate that the media communication is out of scope (in the application domain.) Another architecture, e.g. where direct communication is used, or something else entirely, could be considered - and is out of scope of this use case.

Benoît, using UE (c), monitors the construction site for safety and efficiency.

### 5.19.3	Service Flows

Benoît's UE (c) uses functionality provided by the 5G system to seek to determine the existence of UEs (a) and (b), referring to Figure 5.19.2-1.

UE (c) has knowledge of the AS that accumulates sensor information that could be of interest.

UE (c) requests of the AS that accumulates sensor information for a sensor group: what sensors are in the proximity?

UE (c) is able to become authorized to receive information concerning the sensor group.

The AS requests the 5G system obtain a sensing group.

The 5G system will identify the set of UEs that provide the AS with sensor information that are in the proximity of UE(c). The 5G system will strive to synchronously locate 4 or more devices, to be localized within 10cm of accuracy, with accuracy of measurment within 5 ms of synchronization.

NOTE: 	It is impractical for the AS to continuously track the location of each of these UEs because their location can change and this information is needed only on demand. The group needs to be captured together, at the same time, since the sensor data of the group needs to be interpreted by UE (c) together.

The AS provides UE (c) with sufficient information to identify the sensing group of UEs that are ready to provide sensing measurement data (for non-RF sensors) and sensing results (for RF sensors,) as well as how to get sensing data from the sensing group from the AS.

UE (c) requests to obtain sensing measurement results from the sensing group. The sensors must be within proximity and their locations are known with great accuracy (within 10cm in 3D), with accuracy of measurement within 5 ms of synchronization.

UE (c) is authorized to obtain sensing measurement results from the sensing group.

UE (a) provides sensing measurement data and UE (b) or the network provides sensing results. These are received by the AS and combined. This sensing result is provided to UE (c) by the AS.

UE (b) is mobile and its position varies. UE (c) must identify its position with sufficient accuracy to interpret the sensing result meaningfully. Since UE (b) is mobile, its position and movements must be tracked to provide accuracy up to 10 cm, with accuracy of measurment within 5 ms of synchronization.

UE (b) leaves the proximity of UE (c). UE (c) identifies that UE (b) has left the sensing group.

Later, UE (b) returns to proximity of UE (c). UE (c) identifies that UE (b) has joined the sensing group, including its position within 10 cm, with accuracy of measurment within 5 ms of synchronization.

The crane operations are monitored by sensors (a) and (b), providing different perspectives by means of sensing measurement data from non-RF sensors and sensing results from RF sensors to the AS.

### 5.19.4	Post-conditions

Benoît, making use of UE (c), is able to ascertain with very high accuracy the location and movement of the entire group of UEs that can form a sensor group. The ability of UE (c) to identify the position and membership of the group continues over time, so that the current membership of the group is known, and that membership can change.

The AS is able to combine the sensing measurement data acquired by non-RF sensors and sensing results acquired by RF sensors from different perspectives and produce a useful 3D representation of the site to the site supervisor, who receives the sensing result by means of media delivered from the AS to the Benoît's UE (c).

### 5.19.5	Existing feature partly or fully covering use case functionality

There are requirements specified in 22.261, 6.37.2 to support ranging services that are relevant to this use case. These were developed in the FS_Ranging study. [43]

-	The 5G system shall be able to support for a UE to discover other UEs supporting ranging.

-	The 5G system shall be able to start ranging and stop ranging according to the application layer’s demand.

-	The 5G system shall be able to provide mechanisms for a MNO, or authorized 3rd party, to provision and manage ranging operation and configurations.

-	The 5G system shall be able to support ranging enabled UEs to determine the ranging capabilities (e.g. capabilities to perform distance and/or angle measurement) of other ranging enabled UEs.

-	The 5G system shall be able to allow a ranging enable UE to determine if another ranging enabled UE is stationary or mobile, before and/or during ranging.

-	The 5G system shall allow ranging service between 2 UEs triggered by and exposed to the application server.

Differences between this use case and the above ranging requirements, and ranging in general include:

-	Sensing measurement data is not acquired 'between UEs' but by means of different sensing technologies.

-	It is not sufficient to discover other UEs that support sensing: these must be in the discovering UE's proximity and the discovered UE's precise location must be ascertained.

-	In this use case, a 'sensing group' is formed, where in Ranging, all range information was acquired through interactions directly (or indirectly if relayed) between UEs.

### 5.19.6	Potential New Requirements needed to support the use case

[P.R.5.19.6-1]	The 5G system shall allow an authorized UE to discover the precise location of other UEs in proximity that are authorized to provide sensing measurement data from non-RF sensors and sensing results from RF sensors, within a maximum latency.

Table 5.19.6-1: KPIs for sensing sensors

| Use case | Position Accuracy of the sensing devices in the sensing group | Synchronization interval for UE localization and sensing | Range | Number of UEs to support sensing | Relative speed of UEs and object measured |
| --- | --- | --- | --- | --- | --- |
| Identify sensing group | 10 cm | <= 5ms | 10 - 100m | 6-8 | 3.6 kmph (walking speed) |

NOTE: 	The KPIs above suffice to determine the location of a set of sensors, so that these sensors can provide coordinated sensing results (from RF-sensors) and sensing measurement data (from non-RF sensors) from which can successfully be associated with objects in motion. Otherwise sensing data acquired from different perspectives could be effectively impossible to correlate.

Editor's Note: the KPIs in this table and in the rest of this use case have to be aligned.

[P.R.5.19.6-2]	The 5G system shall allow an authorized AS to discover a sensing group of UEs in the proximity of the UE that is requesting the service from the AS.

NOTE: 	This requirement assumes that a UE requests an AS to discover a set of UEs that have sensing functions that can provide sensing results from RF sensors and sensing measurement data from non-RF sensors to that AS.

## 5.20	Use case of Sensing for Parking Space Determination

### 5.20.1	Description

Sensing technology can improve the user experience in parking garage via enabling the vehicle and parking garage to get more information, e.g. information whether a parking space is available or not. The indoor/underground parking garage can install multiple Sensing receivers and Sensing transmitters throughout the concrete structure for detecting the availability of the parking space. The outdoor parking garage can also exploit multiple Sensing receivers and Sensing transmitters for detecting the availability of the parking space.

Another related use case is automated parking e.g. AVP (Automated Valet Parking) and AFP (Automatic Factory Parking) [45] where cars are provided with drive-path information to do automated parking in a given parking lot facility. Connectivity is an important component in automatic parking, and the 3GPP sensing technology can serve as the way to determine available parking spaces and the best route for a car to reach it.

The coverage could be either a public network or a private network specifically for the parking garage. For the Sensing receiver(s) and Sensing transmitter(s) indoor, see figure 5.20.1-1, one deployment scenario is that Sensing receiver(s) and Sensing transmitter(s) can be ceiling-mounted and located in such a way as to provide sensor coverage for the parking bays within the structure, where the Sensing transmitter and Sensing receiver can be co-located. Another deployment method is that some Sensing receiver(s) and Sensing transmitter(s) can be ceiling-mounted and some can be mounted on the floor or wall, where the Sensing transmitter and Sensing receiver can be separately located. For the Sensing receiver(s) and Sensing transmitter(s) outdoor, see figure 5.20.1-2, Sensing receiver(s) and Sensing transmitter(s) can be deployed at a relative high place to guarantee the coverage of the parking garage.

Figure 5.20.1-1: Parking space determination (indoor deployment)

Figure 5.20.1-2: Parking space determination (outdoor deployment)

Multiple methods can be used via using the sensing signals emitted from the Sensing transmitter to detect the target object/area and the sensing signals bounced/reflected. For example, the Sensing receiver can measure the reflected signal power of the target area [8]. Since the cars are usually made of metal materials and the ground is, on the contrary, covered by cement or plastic cement, these objects can vary significantly on the reflected signal power, thus the Sensing receiver can distinguish whether a parking space is taken by simply measuring the reflected signal power difference. Another example method is that the Sensing receiver(s) and Sensing transmitter(s) can identify a parking space availability by monitoring the target movements. Comparing with the stationary objects such as ground and poles, the Sensing receiver(s) and Sensing transmitter(s) can easily distinguish a car when a car is parking or leaving, thus by measuring the distance/angle/velocity, the Sensing receiver(s) and Sensing transmitter(s) can record that a parking space is occupied when a car is parking, and that a parking space is free when a car is leaving. Or the Sensing receiver(s) and Sensing transmitter(s) can also detect a parking space availability by generating a 3-D point cloud [9], then both the stationary and moving target can be easily detected in a 3-D point cloud especially with some backend data processing skills such as machine learning/deep learning.

### 5.20.2	Pre-conditions

This use case is about a public multi-storey parking garage who has installed Sensing receiver(s) and Sensing transmitter(s) throughout the concrete structure to detect the positions of people, objects, and vehicles within the garage. The concrete structure can make coverage difficult and, especially in underground levels, coverage provided by external transmitters can be very poor. This parking garage can provide the information to the entering vehicles about the availability of the parking space.

Consider an example scenario shown in Fig. 5.20.2-1. A typical parking space is with length 5 m and width 2.5 m. So, from the horizontal dimension, the resolution requires to distinguish different parking spaces. The results can be aggregated by the parking garage operator and the parking state can be updated in seconds even when the coverage of the parking garage is poor, which helps avoid a wasting of time for the entering vehicles.

Figure 5.20.2-1: Example of sensing scenario

### 5.20.3	Service Flows

- James wants to park his car at the public parking garage in floor #B2. His vehicle, on entering the public parking garage, queries the parking garage service for availability of a parking bay in floor #B2.

- The parking garage operator can activate the sensing devices in floor #B2 for sensing. Sensing receiver(s) and Sensing transmitter(s) sense the parking spaces without interfering each other or with bearable interference. The sensing results can be aggregated by the parking garage operator.

- With the aggregated sensing results, the parking garage operator can send sensing results back with an addition to the dynamic map corresponding to the floor #B2 which shows the current status of parking bays in the structure.

- James can see the available parking bays on his in-car display and choose a suitable one.

### 5.20.4	Post-conditions

Thanks to sensing, James has found a parking space on the correct floor without issue, making his life easier during daily travel.

### 5.20.5	Existing features partly or fully covering the use case functionality

None.

### 5.20.6	Potential New Requirements needed to support the use case

Editor’s note: A requirement on the operation of Sensing out of coverage when the Sensing receiver(s) and Sensing transmitter(s) are UEs is FFS

[PR 5.20.6-1] The 5G system shall be able to provide sensing services in licensed and unlicensed spectrum.

[PR 5.20.6-2] The 5G system shall be able to authorize Sensing receiver(s) and Sensing transmitter(s) to participate in a sensing service.

[PR 5.20.6-3] Based on operator’s policy, the 5G system shall enable a trusted 3rd party to request the activation of the sensing service with specific KPI requirement, as well as deactivation of the same service.

[PR 5.20.6-4] The 5G system shall be able to support charging for the sensing services (e.g. considering service type, sensing accuracy, target area, duration).

[PR 5.20.6-5] The 5G system shall be able to provide a sensing service considering the interference to the Sensing service caused by the sensing operations between multiple Sensing transmitter(s) and Sensing receiver(s).

[PR 5.20.6-6] The 5G system shall be able to provide sensing with following KPIs.

Table 5.20.6-1	Performance requirements of sensing results for parking space determination

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Parking space determination | Indoor/ outdoor | 95 | 0.5 | 0.5 | 0.1 | N/A | 2.5m perpendicular to the parking space 5m parallel to the parking space | N/A | 1000 | 1 | 1 | 5 |
| NOTE: The terms in Table 5.20.6-1 are found in Section 3.1. |  |  |  |  |  |  |  |  |  |  |  |  |

## 5.21.	Use case of Seamless XR streaming

### 5.21.1	Description

Extended Reality (XR) is an important 5G use case. Split-rendering architectures, where the heavy XR video rendering computation is done at the application server based on control information received from the UE, poses strict Quality-of-Service (QoS) requirements in terms of round-trip latency and throughput for delivering the video and control info.

It is therefore crucial to always maintain a high-quality wireless link for XR. Thus, it is critical to predict and adapt fast to wireless channel changes. This is especially true in Millimeter wave bands in which the channel and propagation characteristics are very sensitive to user and environment changes such as blockages, user motion or rotation.

To adapt fast to the wireless channel changes, an understanding of the wireless channel dynamics is required. The channel dynamics depend on understanding the surrounding environment such as the transmitter and receiver locations, geometry of the buildings, moving scatterers, location and material of blockers, etc.

Interestingly, most of the XR streaming devices (e.g., 5G phones, AR/VR headsets) and 3rd party entities that support 5G are also provided with sensors, such as RF sensors, Inertial Measurement Units (IMU) sensors, RGB cameras, position sensors, and others.

In light of the availability of sensors and the need of environment understanding, it is therefore natural to utilize the overall sensing information to acquire an understanding of the surrounding environment.

To this end, a “Sensing Assistance Service” can be envisioned that enables the collection of sensing information from multitude of sensors, process and provide that information to 3rd party applications.

- The input to this service, “Sensing Assistance information” could be sensing measurement data and sensing results from multiple sensors, e.g., RF sensing results, XR user position, camera images, depth maps, hand tracking, motion type, etc. Such input could be produced by a 5GS entity (sensing measurement data and sensing results) or by a 3rd party (e.g., surveillance camera). It is essential to note that the collection of this sensing assistance information should be done with appropriate user consent and adherence to regional and national regulations.

- The output of this service is some understanding of the environment and/or impact to communication performance of a service consumer, e.g., RF environment mapping, etc.

- The consumer of this service can be a 3rd party application.

It is important to note that while this service is provided by 5GS/edge, business model for the monetization of this service would need to consider factors such as the entities, transfer of the non-RF sensing data and the value of sensing assistance information produced by these entities as well as the value to the consumer of the service.

### 5.21.2	Pre-conditions

Jose is playing a game inside a gaming arena using a VR headset that is connected to a RAN entity.  The VR headset, RAN entity and 3rd party surveillance system are configured to provide “Sensing Assistance Information” to the “Sensing Assistance Service” which could be at the edge.

The VR headset is equipped with NR RF sensors, IMU sensors and cameras, and it can provide Sensing Assistance Information e.g., NR RF sensing results, headset pose and location, velocity, images of the environment and processed images (such as motion pattern and maps). Also, the VR headset can provide communication reference signal measurements or reports to a network entity, which to process and provide sensing measurements results to the Sensing Assistance Service.

RAN entity has NR RF capabilities and can provide NR RF sensing measurement data to a network entity, which to process and provide sensing results to the sensing assistance service.

The gaming arena also has cameras deployed by a trusted 3rd party surveillance camera company and can provide images of the environment and processed images (such as motion pattern and maps).

### 5.21.3	Service Flows

1.	Jose is playing a game using a VR headset in an arena with some obstacles and other gamers in the environment. Jose moves through the arena and approaches a communication blocker which could potentially impact the performance of the wireless communication between VR headset and the RAN entity.

2.	Jose’s VR headset, RAN entity and the 3rd party surveillance system provide sensing measurements results to the Sensing Assistance Service. The Sensing Assistance Service fuses together the sensing measurements results from RF and non-RF sources to produce a comprehensive RF map of the environment surrounding the headset (information such as the location of RAN entities, reflectors, static blockers, etc) and an indication of wireless link blockage event, e.g., people walking by blocking the 5G link.

Editor’s Note: Further clarification of RF Map is FFS.

- 5GS/edge uses the RF map to predict that Jose’s communication link is about to be blocked if he comes close to the blocker and such prediction is sent to the application layer of the game. The application layer adjusts the content of rendered video frames accordingly (e.g., lowers the frame rate, adds a virtual obstacle in the rendered video to prevent Jose from coming close to the blocker.)

Editor’s Note: Further clarification of the relation between this use case and “Use case on Transparent Sensing” in Clause 5.4 of the TR 22.837 is FFS.

### 5.21.4	Post-conditions

Jose enjoys seamless XR gaming application without video frame drops, i.e., no video glitches. This is because Sensing Assistance Information was leveraged to assist both the communication service as well as the application.

### 5.21.5	Existing features partly or fully covering the use case functionality

None

### 5.21.6	Potential New Requirements needed to support the use case

[PR.5.21.6 - 001] Subject to user consent and national or regional regulation, based on operator policy, the 5G system shall be able to support secure means for RAN entities and authorized UEs to provide NR RF sensing measurement data and/or sensing results to a 5G network entity for processing.

Editor’s Note: The sensing measurement data and sensing result terminologies can be revisited based on future updates.

## 5.22	Use case of UAVs/vehicles/pedestrians detection near Smart Grid equipment

### 5.22.1	Description

In the future, there will be more and more autonomous driving devices, such as drones and self-driving cars. These devices have a strong ability to affect the surrounding environment, which may have an impact on the operating equipment in Smart Grid.

For example, vehicles, such as UAVs and engineering vehicles, may affect the operation safety of multiple links such as power generation, power transmission, and power transformation.

At present, multiple scenarios of power transmission and transformation in the Smart Grid industry have potential combination with integrated sensing and communication technology. Among them, there are related accidents caused by hooking or damaging transmission lines by vehicles in the power transmission process. Thus, the transmission stations need to identify and warn vehicles. In the process of power transformation, there are security risks such as candid photography and attack by drones, getting electric shock when approaching, etc. In a word, there are requirements for perimeter intrusion detection and UAV detection in substations.

### 5.22.2	Pre-conditions

There are existing 5G base stations deployed near the transmission stations and substations, which can provide constant remote sensing of the location of intruders in the coverage area including UAVs, engineering vehicles and pedestrians. Network operator A can use these 5G base stations to provide 5G sensing service for the Smart Grid operator X, including sensing the motion trail of the UAVs, vehicles and pedestrians in their working area.

The Smart Grid Operator X uses the 5G sensing service provided by 5G network Operator A to detect potential intrusion/approaching of UAVs, vehicles and pedestrians near the transmission stations and substations.

The Smart Grid operator sets the border of restricted area for the transmission stations/lines and substations in which no UAVs, vehicles or pedestrians can be access, and define a warning distance value. Once a UAV, traffic vehicle, or pedestrian is detected that its distance from the border is less than the warning distance value, the 5G system will report the event to the Smart Grid operator to send the alerting message.

The 5G base stations can sense the location of the UAV/traffic vehicle/pedestrian constantly and send these data to the 5G core network. Then the sensing node and computing node can analyse and predict the path of the UAV or pedestrian according to a large amount of data and give early warning of potential security risks.

### 5.22.3	Service Flows

1.	The Smart Grid Operator X requests sensing service from network operator A to collect sensing data in the defined area (i.e., the park covering transmission stations and substations). The network operator A configures the base stations located in the defined area to perform sensing.

2.	The 5G base station constantly collects sensing measurement data of the location of UAVs/vehicles/pedestrians in the defined area and send the sensing data to the 5G core network with a defined frequency to obtain the sensing result (i.e., the distance between the UAV/traffic vehicle/pedestrian and the border or motion trail).

3.	The 5G system will send notification to UAVs/vehicles/pedestrians with UE that they are near a restricted area. 5G system will also report the sensing results to the Smart Grid operator. The Smart Grid operator determines to send the alerting message to the intruding/approaching UAVs/vehicles/pedestrians based on the sensing results. In addition, the staffs working in the park respond to the emergency and prepare to intercept the intruding/approaching if the UAVs/vehicles/pedestrians are not away.

### 5.22.4	Post-conditions

The UAVs/vehicles/pedestrians are away from the defined area. Potential security risks are avoided. Thanks to the wide-area and constant sensing capability of the 5G base station, and the precise data processing and prediction by the 5G core network, the safety supervision of the Smart Grid is improved.

### 5.22.5	Existing features partly or fully covering the use case functionality

In TS22.261, there are existing requirements on information exposure:

In clause 6.10:

The 5G system shall be able to:

-	provide a third-party with secure access to APIs (e.g. triggered by an application that is visible to the 5G system), by authenticating and authorizing both the third-party and the UE using the third-party's service.

-	provide a UE with secure access to APIs (e.g. triggered by an application that is not visible to the 5G system), by authenticating and authorizing the UE.

-	allow the UE to provide/revoke consent for information (e.g., location, presence) to be shared with the third-party.

-	preserve the confidentiality of the UE's external identity (e.g. MSISDN) against the third-party.

-	provide a third-party with information to identify networks and APIs on those networks.

### 5.22.6	Potential New Requirements needed to support the use case

[PR. 5.22.6 - 001] Subject to operator policy, the 5G system shall enable the network to expose a suitable API to a authorized third party to provide the information regarding sensing results.

[PR. 5.22.6 - 002] The 5G system shall be able to support the following KPIs:

Table 5.22.6-1	Performance requirements of sensing results for UAVs/vehicles/pedestrians’ detection near Smart Grid equipment

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Sensing for the use case in Smart Grid (NOTE 2) | Outdoor | 95 | ≤0.7 | N/A | UAV: ≤25; Pedestrian: ≤1.5; Vehicle: ≤15 | N/A | N/A | N/A | ≤5s | ≥10Hz | [≤5] | [≤5] |
| NOTE 1: The terms in Table 5.22.6-1 are found in Section 3.1. NOTE 2: The typical size (Length x Width x Height) of UAV is 1.6m x 1.5m x 0.7m, the typical size of pedestrian is 0.5m x 0.5m x 1.75m, and the typical size of engineering vehicle is 7.5m x 2.5m x 3.5 m. The size of the park of Smart Grid depends on the real environment. NOTE 3: The safe distance between pedestrian/vehicle and transmission station/line is 0.7m/0.95m [46]. |  |  |  |  |  |  |  |  |  |  |  |  |

## 5.23	AMR collision avoidance in smart factories

### 5.23.1	Description

Autonomous mobile robots (AMR) are currently being introduced in many logistics operations, e.g. manufacturing, warehousing, cross-docks, terminals, and hospitals.  Compared to an automated guided vehicle (AGV) system in which a central unit takes control of scheduling, routing, and dispatching decisions for all AGVs, AMRs are robots built with intelligence to autonomously move and perform tasks. AGVs is expected to further be evolved into intelligent AMR to meet the demand of intelligent factory.

Compared to AGVs which move on transport paths guided by rails, magnetic markers etc. AMRs can travel automatically without derivatives or guides. AMRs don’t rely on predetermined paths, they can easily adjust routes as user demands change, so AMRs have wider mobile range and more flexibility. AMRs can not only stop on time to avoid humans and other obstacles, but also adjust its route for its destination. However, during the AMR working process, the sensing range of a single AMR is limited and the AMR surrounding environment status may be not detected in time. For example, People or other machines that suddenly appear from behind the large factory equipment can affect the driving safety of the AMR. So, it is very challenge for AMR to get accurate and continuous sensing information along its route.

5G Base stations can be deployed in a factory not only to provide communication capabilities for equipments in the factory but also sense the surrounding environment e.g. obstacles or people in the trajectory of AMRs.  Base stations transmit the sensing signals and receive the reflected signals to get sensing information, then reports the real-time sensing measurement data to the core network. The core network can process and analyze the sensing measurement data for outputting the sensing result. Such sensing result can be exposed to a trusted 3rd party e.g. automation platform of the factory to enables AMRs to know more information about the surrounding environment to improve efficiency and driving safety.

In addition, when there are obstacles (e.g., the large factory equipments) to block the transmission of radio signals or AMR trajectory is across indoor and outdoor, multiple base stations with sensing capability can work together to improve the sensing accuracy and sensing service continuity.

### 5.23.2	Pre-Conditions

5G Network operator ‘MM’ provides 5G sensing service in the factory of Company A. Its 5G system has been deployed covering the factory to provide continuous sensing service indoor and outdoor.

Company A has placed two AMRs (AMR 1 and AMR2) in its factory for moving goods from workshop A to workshop B. At the same time, there are people moving around in both workshops, and other goods or tools may be temporarily placed on the route of the AMRs. The people walking in the workshop and the goods may block the AMR route, jeopardizing production safety. In addition, the two AMRs may collide considering their flexible routes.

The AMRs of Company A uses ‘5G Sensing Service’ provided by 5G network Operator ‘MM’ during they are working. In order to ensure data security, the related sensing data is not permitted to be delivered outside Company A.

### 5.23.3	Service Flows

Figure 5.23.3-1 shows the route change of AMR1 in the process of carrying goods.

Figure 5.23.3-1: Sensing People or obstacles detection in smart factory

- The AMR#1 is delivering the car parts from workshop A to Sam who is near the assembly line in workshop B.

- The sensing measurement data collected by Base station #1, the 5G system processes the sensing measurement data to obtain sensing results and detects the proximity of obstacles along the trajectory of AMR#1. 5G system provides the sensing result to the AMR#1, and AMR#1 then re-route and bypass the obstacles based on the sensing result.

- AMR#1 is leaving the Workshop A and across from indoor to outdoor.

- The sensing measurement data is collected by Base station #2, the 5G system processes the sensing measurement data to obtain sensing results and detects the proximity of Daming who is walking across the trajectory of AMR#1. 5G system provides the sensing result to the AMR#1, then AMR#1 stops to wait for Daming to leave.

- AMR#1 enters the Workshop B.

- The sensing measurement data is collected by Base station #3, the 5G system processes the sensing measurement data to obtain sensing results and detects the AMR#2 near AMR#1. 5G system provides the sensing result to AMR#1, then AMR#1 re-routes and bypasses the obstacles based on the sensing result.

- When AMR#1 enters the coverage of Base Station #4, using the sensing measurement data from Base station #4, the 5G system processes the data to obtain sensing results and detects the proximity of John who is behind a large machine. 5G system provides the sensing result to AMR#1, then AMR#1 stops and waits for John to leave.

AMR#1 successfully delivers the car parts to Sam in workshop B.

### 5.23.4	Post-Conditions

Based on the communication and sensing services provided by the 5G network, the AMRs in the factory operate normally and reduce safety incidents.

### 5.23.5	Existing features partly or fully covering the use case functionality

None.

### 5.23.6	Potential New Requirements needed to support the use case

[PR 5.23.6 -1] The 5G system shall be able to provide the continuity of sensing service for a specific target object, across indoor and outdoor.

[PR 5.23.6 -2] The 5G system shall be able to provide a secure mechanism to ensure sensing result data privacy within the sensing service area.

[PR 5.23.6-3] The 5G system shall be able to support the following sensing related KPIs:

Table 5.23.6-1	Performance requirements of sensing results for AMR collision avoidance in smart factories

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| AMR collision avoidance in smart factories | 100m | 99 | ≤1 | N/A | 1 | N/A | 1 | 10 | ˂500 | ≥20Hz | N/A | 5 |
| NOTE 1: The terms in Table 5.23.6-1 are found in Section 3.1. NOTE 2: The KPI values are sourced from [47] |  |  |  |  |  |  |  |  |  |  |  |  |

## 5.24	Use case on roaming for sensing service of sports monitoring

### 5.24.1	Description

Sports monitoring application describes the case of a human being monitored when doing exercise via utilizing wireless signals instead of cameras, or wearable devices. With enhanced privacy preservation, wireless signals that propagated in the 5G system (e.g. between 5G UE and 5G UE, and between the radio access network and device) can be further reused and processed to retrieve the target sensing object’s characteristics [48]. In a sports monitoring situation, the target object is human and the target object’s characteristic is human body gesture. By comparing the detected body gesture with the correct body gesture when people are doing exercises like sit-ups, and push-ups, this sport monitoring application will give feedback, for example, it can count the number of the exercise e.g. sit-ups, and calculate calories.

### 5.24.2	Pre-conditions

The sports monitoring application provider has a service agreement with mobile operator A in country X and mobile operator B in country Y.

Mobile operator A in country X and Mobile operator B in country Y has roaming agreements.

Bob installs a sports monitoring application on his mobile phone and subscribes to the sensing service with mobile operator A.

### 5.24.3	Service Flows

1. Bob is a sports fan and does exercise every day. He travels to country Y and gets accommodation in hotel M, where mobile operator B’s sensing service is available. Bob triggers the application and selects his favorite sport, sit-up.

2. The sensing request is received by Mobile operator B and Mobile operator B then authenticates whether the sensing request can be satisfied, e.g. by verifying the sensing system availability (infrastructure, sensing modalities), the roaming agreement with Bob’s home mobile operator A in country X, the identity of Bob and etc.

3. If the authentication succeeds, mobile operator B authorizes and provides such 5G sensing service to Bob with required performance targets and requirements and Bob pays for the sensing service to mobile operator B.

- Executes sensing measurement process (e.g. micro doppler shift) with the sensing entities (e.g. RAN entities and the roaming UE, or CPE and the roaming UE) to obtain sensing measurement data;

- Processes the sensing measurement data to derive sensing result and expose it to the application server.

4. If the authentication fails, mobile operator B does not provide such 5G sensing service to Bob and potentially, Bob or the sports monitoring application server will receive the reason of why mobile operator B cannot provide such sensing service.

### 5.24.4	Post-conditions

Bob can enjoy the sports monitoring application even when he travels in another country.

### 5.24.5	Existing features partly or fully covering the use case functionality

Roaming-related authentication and charging may refer to existing requirements as defined in Clause 9 in TS22.261:

The following set of requirements complement the requirements listed in 3GPP TS 22.115. The requirements apply for both home and roaming cases.

The 5G core network shall support collection of charging information for alternative authentication mechanisms

The 5G system shall be able to generate charging information regarding the used radio resources e.g. used frequency bands.

### 5.24.6	Potential New Requirements needed to support the use case

[PR 5.24.6-1] Based on operator policy, the 5G System shall be able to provide the 5G wireless sensing services in case of roaming.

[PR.5.24.6-2] The 5G system shall be able to provide 5G wireless sensing service with the following KPIs:

Table 5.24.6-1	Performance requirements of sensing results for sports monitoring

| Scenario | Sensing service area | Confidence level [%] | Motion rate accuracy | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Sports monitoring | Indoor (living room) | 95 | 3 times/min [0.05Hz] (NOTE 2) 4 times/min [0.07Hz] (NOTE 3) | N/A | N/A | N/A | N/A | N/A | N/A | 60s | 1min | N/A | N/A |
| NOTE 1: The terms in Table 5.24.6-1 are found in Section 3.1. NOTE 2: Sit-up rate = 30 times/min as reference NOTE 3: Push-up rate = 40 times/min as reference |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 5.25	Use Case on immersive experience based on sensing

### 5.25.1	Description

Sensing based on the 5G signals is a technology using the difference between wireless signals and its reflective signals including the Doppler frequency shift, time of flight (ToF), amplitude variation and so on, to sense the surroundings. The information collected during sensing cannot be directly understood by human beings, providing good privacy protection. Along with 5G stepping into the home, more interesting functions can be introduced based on the sensing using 5G signals.

It will be fantastic to have an immersive audio and light experience when watching movies and listening to music in the home. The speakers can provide this kind of audio experience if they can know the position of each other and also the user.  Usually, to have immersive audio experience, several speakers are needed. Ranging technology can help the speakers to determine position relative to each other and this gives a chance for speakers to provide a fantastic experience together to a listener by adjusting the audio field at a place when the listener stays at that special place.

Different from ranging service where only the UE’s position can be identified, sensing can identify the relative position of the reflector even the reflector is not a UE. If the speakers can obtain the sensing results, this will give a chance to the speakers to follow the listener and provide an immersive audio experience, even when the listener is moving around. The speakers can follow the position of the human and adjust the audio field anytime anywhere. Similar to the audio case, if the smart light can obtain the sensing results, the light can also track the user anytime anywhere to provide an immersive experience.

For the immersive experience scenario, the audio field and light adjustment should be based on the user’s position. For example, the smart screen, lights and speakers can provide immersive sound and light experience for the user who sits at the sofa area (around 2~3m2 area) and at the same time lower the sound volume and turn down the light at other places of the home avoiding the interference on others. The sensing node (e.g., smart screen, i.e., a UE) can perform sensing operations to track the user’s movement using RF signals [29] and provide the information to the speakers and lights for adjustment. Then both the lights and speakers can provide a cosy zone around the user.

In home, there is usually a mixed deployment of smart screen, lights, and speakers. We take smart screen as the sensing node as an example. Assume that the smart screen, the smart lights, and the speakers are placed in the drawing-room, and the smart screen can sense the object in the room as shown in Fig. 5.25.1-1. The smart screen (i.e., UE) can receive the reflected sensing signals transmitted by UEs in the room, or by the gNB to perform sensing operation on the user in the room.

Figure 5.25.1-1: Example deployment of the smart screen, speakers and lights

The smart screen can track the user’s position via sensing operation. Each user is assumed to occupy an area 0.5m * 0.5m and move with a speed lower than 2m/s in horizontal dimension. Then the audio field and light can be adjusted based on the user position (e.g. an area 1.5m * 1.5m) to avoid the experience deterioration even when the user is walking around.

To identify multiple users in the room, in the horizontal dimension, the resolution requires to distinguish objects as large as 0.5m. With higher accuracy on distance, each user in the room can be identify with a more accurate location such as 0.2m, the light and audio can target the user better [30]. If the results are accurate but out of date due to the movement of the user, this will also degrade the experience as the cosy zone is lagging the movement of the user. Consider the audio field and lights are adjusted in an area with 1.5m * 1.5m, to avoid the experience deterioration, the service latency should smaller than 0.5m/2m/s, i.e., 250ms. To track the user’s movement, the results need to be refreshed in 250ms, i.e., 4 times per second (4 Hz).

Figure 5.25.1-2: Example of accuracy required for the use case

### 5.25.2	Pre-conditions

This use case is about Tom’s home theatre plan. Tom bought a set of speakers, and placed them in his home to create an immersive audio experience. Tom also bought a set of smart lights to create an immersive light experience.  Together with a smart screen in home, these smart devices compose the home theatre. According to the instruction of the speakers, and also based on the availability of audio and power wiring Tom distributed the speakers in his living room. The smart lights are installed on the ceiling of the room. After detecting the position of each distributed speakers, the home theatre system can adjust the audio field in Tom’s home. After detecting the position of smart lights, the home theatre system can control the light variation in home. This detection mechanism of the speakers and smart lights is outside the scope of this document.

There exists a sensing device in the home, which can sense Tom’s position without requiring Tom to take a UE with him. For example, the sensing node is the smart screen belonging to the home theatre system at Tom’s home. Based on the sensing results from the sensing node, the home theatre system can adjust the audio field and light based on the sensing results.

The home theatre system can be deployed by user where the smart screen, lights and speakers communicate with each other via direct device communication using unlicensed band. As an alternative, the home theatre system can be deployed with the help of operator using licensed band when the units of the home theatre system are in coverage. In this case, operator can control the performing of sensing operation based on the location of the deployment of the home theatre.

### 5.25.3	Service Flows

Tom activates his home theatre and the speaker begin to play music based on the audio on demand. The sensing device in the room (i.e., Smart screen) performs sensing operation and the sensing results begin helping adjust the audio field.

Fig. 5.25.3-1 Immersive experience with tracking light and sound

Tom immerses himself in music and begins to dance. The sensing device (e.g., smart screen) tracks Tom’s position via the processing of the receiving sensing signals, the Sensing result is calculated, and the Sensing result is sent to the control unit of the home theatre system.

Based on the sensing results, the home theatre system can adjust the audio field and lights according to Tom’s movement.

Thanks for the sensing service provided by the sensing node (e.g. smart screen), no matter where Tom stands, he always experiences the best surround sound and tracking light.

### 5.25.4	Post-conditions

Thanks to the sensing service provided in the intelligent home, Tom can have an immersive experience via his home theatre.

### 5.25.5	Existing features partly or fully covering the use case functionality

None.

### 5.25.6	Potential New Requirements needed to support the use case

[PR 5.25.6-1] The 5G system shall be able to configure and authorize sensing for a Sensing device or a group of Sensing devices when using licensed spectrum based on the Sensing device’s location.

[PR 5.25.6-2] The 5G system shall be able to enable a Sensing device to perform sensing with licensed band under operator’s control based on the Sensing device’s location.

[PR 5.25.6-3] The 5G system shall be able to enable UEs without 5G coverage to use unlicensed spectrum to perform sensing.

[PR 5.25.6-4] Subject to user consent and national or regional regulation, based on operator policy, the 5G system shall be able to allow a Sensing device to provide sensing results to a trusted third party.

[PR 5.25.6-5] The 5G system shall be able to provide sensing with following KPIs.

Table 5.25.6-1	Performance requirements of sensing results for immersive experience

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Immersiveexperience | Indoor | 95 | 0.5 | 0.5 | 0.1 | N/A | 0.5 | N/A | 250 (granularity of field is 1.5m x 1.5m) | 0.25 | 5 | 5 |
| NOTE: The terms in Table 5.25.6-1 are found in Section 3.1. |  |  |  |  |  |  |  |  |  |  |  |  |

## 5.26	Use case on accurate sensing for automotive maneuvering and navigation service

### 5.26.1	Description

It is forecasted that there will be approximately 8 million autonomous or semi-autonomous vehicles on the road by 2025 [49]. NR wireless sensing will assist with automotive maneuvering and navigation, especially in scenarios where single car-mounted sensors collecting information is not enough for making safe and reliable decisions, e.g. to avoid a collision, pedestrians, etc.

This scenario reuses the use case as defined in section 5.8, where NR wireless sensing is utilized to assist automotive maneuvering, i.e. sensing results play an important role in making the maneuvering decisions. However, the sensing environment when RAN entities and Ues execute the sensing measurement process may subject to high interference (e.g. interference caused by adjacent RAN entities, radars, fake base stations) and cause the sensing information as collected to be wrong.

The sensing information provided to the Automated Driving System (ADS) server need to be fully trustworthy: reliability, integrity, high confidence level and protection against tampering are key aspects. Users (and third parties) should not be able to fraud the ADS Server by tampering the sensing information in order to influence the maneuvering decision.

### 5.26.2 	Pre-conditions

Refer to 5.8, where Bob’s vehicle is detected to be blocked by other vehicle and cannot do the decision for autonomous driving with sensors collected data (e.g. from lidar, radar, camera, etc). Bob recognizes the needs of 5G system assistance and requests 5G System for coordination of the sensing service.

### 5.26.3		Service Flows

1. Bob drives in down town, and is approaching a cross road with regulatory signs, where the sensors on Bob’s vehicle are blocked by other vehicles. Bob’s vehicle cannot see the surroundings and Bob sends the sensing request to 5G system.

2. The 5G system gives instructions about how to proceed the sensing to Bob’s vehicle and Bob’s vehicle selects Joe’s vehicle to assist the sensing service. Joe’s vehicle transfers the sensing information to Bob’s ADS server.

3. Bob’s ADS server utilizes the sensing assistance information to do the decision, Bob decelerates before the traffic light.

4. Bob continues the journey and drives in a tunnel (10km length), the sensors on Bob’s vehicle are detected to be blocked by a big truck. Bob sends the sensing request to 5G system.

5. The 5G system selects RAN entities (e.g. road side units) to assist the sensing service. The 5G system transfer the sensing information collected by RAN entities to Bob’s ADS server.

6. Bob’s ADS server utilizes the sensing information to do the decision, and Bob accelerates to overtake the big truck.

7. Bob continues the journey and drives in a desert area and some sensors on Bob’s vehicle are detected to be broken (e.g. because of heat). Bob’s vehicle sends the sensing request to 5G system.

8. The 5G system selects RAN entities (sparsely deployed in desert area) to assist the sensing service. With the feedback from RAN entities, the 5G system transfer the sensing information together with the indication of confidence level as 60% to the ADS server.

9. Bob’s ADS server utilizes the indication and sensing information, and decides to return to Level 0 driving for safety.

### 5.26.4	Post-conditions

Bob’s vehicle is able to drive with high reliability by utilizing accurate 5G sensing service.

### 5.26.5		Existing features partly or fully covering the use case functionality

None

### 5.26.6		Potential New Requirements needed to support the use case

[PR 5.26.6-1] The 5G system shall be able to determine the confidence level of the sensing results.

# 6	Considerations

Editor’s Note: This clause can capture privacy, charging, public safety considerations.

Editor’s note: This chapter is FFS

# 6.1	Considerations on confidentiality and integrity,

### 6.1.1	General

When introducing sensing technology, new aspects on confidentiality, integrity, and availability need to be considered, to ensure that these aspects are considered already when proposing service requirements.

For instance, with sensing technology by-standers can be affected in a completely new way, previously only Ues have been able to be tracked but now sensing capabilities may enable tracing and potentially identification of anything in the environment, including humans that do not carry a UE, or any objects. This has implications for privacy. Obviously humans should have a right to privacy.

Of course, factors such as resolution, updating frequency, and type of application influence the security implications.

Requirements to minimize the risk of unwanted usage and awareness of the usage needs to be considered in stage 1. These are captured in the next chapter.

### 6.1.2	Potential New Requirements

A set of general new requirements can be identified:

[P.R.6.1.2-1] The 5G system shall limit the sensing information to users authorized to receive that information.

[P.R.6.1.2-2] The 5G system shall support encryption and integrity protection of the sensing  result, to protect the data inside the 5G system and when used.

[P.R.6.1.2-3] The 5G system shall support appropriate level of sensing for both situations where consent can be obtained from the sensing targets, and where it cannot.

## 6.2 Considerations on Privacy

Since Sensing is almost for the whole environment under the RF signal coverage, the privacy of sensing operation is an important consideration factor for real deployment.

For the private area, the private permission is required for sensing operation from such as the homeowner for in-home sensing or the building management for the in-building sensing.

For the public area, such as the public road, park and airport, it is required to obtain the permission of the public area management.

For the specific object (e.g. rain), the content of the sensing result report is limited according to the authority of the sensing requester, for example, the climate and rainfall detection can be reported to the Meteorological Bureau.

For the sensed object that supports 3GPP UE capability, the 5GS should notify the UE about the sensing event and request consent of the user before sensing the object.

These privacy policies need to be configured on the 5GS and can be flexible modified by the related parties under the operator control.

## 6.3 Considerations on Mission Critical and other priority services

The sensing operation in 5GS shall support commercial services (e.g. driving assistance), Mission Critical services (e.g. public safety, Utilities, Railways) and other priority services (e.g. MPS) with requirements for priority treatment.

The 5G system should allow flexible means to provide relative priority treatment among the sensing services (e.g. driving assistance, Mission Critical, MPS) and among the users of these services subject to regional/national regulatory rules and operator policy.

# 7	Consolidated potential requirements and KPIs

## 7.1	Consolidated potential requirements

## 7.2	Consolidated potential KPIs of sensing results

Table 7.2-1	Performance requirements of sensing results for integrated sensing and communication scenarios

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| NOTE: The terms in Table 7.2-1 are found in Section 3.1. |  |  |  |  |  |  |  |  |  |  |  |  |

# 8	Conclusion and recommendations

Annex <Z> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 5.2022 | SA1#98e | S1-221249 | - | - | - | Initial Skeleton | 0.0.0 |
| 5.2022 | SA1#98e | S1-221250 S1-221251 S1-221252 | - | - | - | Output of approved pCRs from SA1 #98e | 0.1.0 |
| 9.2022 | SA1#99e | S1-222300 S1-222301 S1-222302 S1-222303 S1-222304 S1-222305 S1-222306 S1-222307 S1-222308 S1-222309 S1-222310 S1-222311 S1-222312 S1-222313 S1-222314 S1-222315 S1-222316 S1-222317 S1-222318 S1-222319 S1-222320 S1-222321 S1-222322 | - | - | - | Output of approved pCRs from SA1 #99e | 0.2.0 |
| 11.2022 | SA1#100 | S1-223333 S1-223484 S1-223485 S1-223061 S1-223716 S1-223577 S1-223494 S1-223495 S1-223496 S1-223578 S1-223498 S1-223579 S1-223580 S1-223701 S1-223690 S1-223730 S1-223731 S1-223590 S1-223592 S1-223488 S1-223604 S1-223606 S1-223607 | - | - | - | Output of approved pCRs from SA1#100 | 0.3.0 |