3GPP TSG-SA WG1 Meeting #101 	S1-230016

Athens, Greece, 20 Feb – 24 Feb 2023	(revision of S1-23xxxx)

Source:	Charter Communications

pCR Title:	Add Public Warning System Requirement

Draft Spec:	3GPP TR 22.851 V1.0.0

Agenda item:	7.4

Document for:	Approval

Contact:	Richard Wu <richard.wu@charter.com>

Abstract: Introduce the potential PWS requirement for the non-N2 shared network in TR 22.851 V1.0.0.

1. Introduction

PWS support is listed in the TR 22.851 v1.0.0, chapter 1 Scope.

2. Use Cases

A use case has been introduced on emergency service illustrating the needs to support regulatory services. We want to add PWS requirement.

3. Conclusions

It is proposed to add the introduced regulatory services into the TR 22.851.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.851 v1.0.0.

* * * First Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

HTA	High Traffic Areas

LTA	Low Traffic Areas

MOCN	Multi-Operator Core Network

NG-RAN	Next Generation Radio Access Network

PWS	Public Warning System

SST	Slice/Service Type

* * New Use Case * * * *

## 5 Use cases

## 5.X 	Use case on Emergency call

### 5.X.1	Description

The network is allowed to route the emergency call to the emergency response centre in the area where the operator provides services, as specified by TS 22.101[2], in accordance with national regulations and where the subscriber is located.

In order to route emergency calls to the correct emergency response centre, operators may need to use the UE location information. Operators may use different location information based on the requirement of national regulation. Sometimes operators have to change the network configuration to provide correct routing or services related to location information in an area.

This use case provides the possibility that the subscriber of the participating operator may initiate an emergency call in the shared network, which leads to the problem to the service provider of routing the emergency call according to the location information of the Hosting operator.

### 5.X.2	Pre-conditions

Assumptions:

1. OP-1 is the Hosting RAN operator of NG-RAN access network. There is a connection between the OP-1’s CN and OP-2’s CN, while OP-2 as a participating operator indirectly sharing the network.

2. OP-2, as the participating operator, is an emergency call service provider. The network of OP-2 is connected to the emergency response centre located in Area 1.

3. OP-3 is the Hosting RAN operator of NG-RAN and E-UTRAN. And OP-3 and OP-2 have a sharing agreement with MOCN.

### 5.X.3	Service Flows

OP-1 IMS

network

OP-1 network

with E-UTRAN

and Shared NG-RAN

OP-2 IMS

network

OP-2 network

with E-UTRAN

and NG-RAN

connection between the OP-1’s CN and OP-2’s CN

UE of

OP-2

OP-3 shared E-UTRAN

and shared NG-RAN

connection between the OP-3’s RAN and OP-2’s CN

Area 1

Area 2

Area 3

Figure 5.X.3-1: Emergency call routing with network sharing scenarios

1. UE is registered to OP-2. UE accesses the shared network of OP-1, and registered with the OP-2’s IMS. The nearest emergency response centre to UE's current geographic location is in Area 1, shown as Fig. 5.X.3-1.

2. UE initiates an emergency call in the shared network, it is most reasonable for the core network of OP-2 to route the emergency call to PSAP within area 1.

3. OP-2’s network routes the emergency call to PSAP in Area 1 according to the location information transmitted by the shared network.

### 5.X.4	Post-conditions

The emergency call can be correctly routed to the appropriate PSAP to serve the UE according to its location information.

### 5.X.5	Existing feature partly or fully covering use case functionality

Chapter 10 of TS 22.101[2] defines the emergency call service. According to National regulations, the requirement of UE’s location of emergency call service is different in deployment scenarios, examples are:

National regulations may require wireless networks to provide the emergency caller's location. This requirement typically overrides the caller's right to privacy with respect to their location being revealed, but remains in effect only as long as the authorities need to determine the caller's location. The interpretation of the duration of this privacy override may also be different, subject to national regulation. For example, some countries require location to be available from the wireless network only while the call is up, while others may allow PSAP's to unilaterally decide how long the location must be made available.

Therefore, the requirement for providing location availability is to allow the network to support providing a mobile caller's location to the authorities for as long as required by the national regulation in force for that network.

Note:	See TS 22.071 [x1] for location service requirements on emergency calls.

### 5.X.6	Potential New Requirements needed to support the use case

[PR 5.X.6-001] Subject to national or regional regulatory and operator policies, the 5G system shall support emergency call from a UE accessing a shared network via Indirect Network Sharing.

[PR 5.X.6-002] Subject to national or regional regulatory and operator policies, the 5G system shall provide to the participating operator the location information of an emergency caller accessing a shared network via Indirect Network Sharing.

[PR 5.X.6-003] Subject to regulatory requirements, the 5G system shall be able to support PWS when connectivity is provided via Indirect Network Sharing.