3GPP TSG SA WG 1 Meeting #101 	S1-230018

Athens, Greece, 20-24 February 2023	(revision of S1-23xxxx)

Source:	ZTE Wistron Telecom AB,  one2many, Charter Communications Inc., China Unicom

pCR Title:	Use Case on Support of PWS in 5G Shared Access Network with Indirect Interconnection

Draft Spec:	3GPP TR 22.851 V1.0.0

Agenda item:	7.4

Document for:	Approval

Contact:	Yvette Koza

Yvette.koza@zte.com.cn

Abstract: Define new service requirements to support PWS

1. Introduction

According to the objective of the SID the impact on PWS in indirect RAN sharing needs be analysed.

2. Reason for Change

Requirements for Support of PWS is described

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to approve the following changes to 3GPP TR 22.851 V1.0.0.

* * * First Change * * * *

## 5.x	Use case on Support of PWS in Shared NG-RAN in Indirect Interconnection with the Participating Operators

### 5.x.1	Description

Operators 1 and 2, both licensed to operate a 4G network and a 5G network share the 5G part of the radio access network in some areas of the country.

The Public Safety Agency sends the PWS message in a certain area covered by NG-RAN1 and RAN2 of Operator 1 and Operator 2 respectively and by the shared NG-RAN area of Operator 1 and Operator 2.

### 5.x.2	Pre-conditions

- Operator 1 owns NG-RAN 1 which is not shared.

- Operator 2 owns RAN 2 which is not shared.

- NG-RAN is a shared NG-RAN between Operator 1 and Operator 2.

- Operator 1 is the hosting operator.

- Operator 2 is a participating operator. Operator 1 and Operator 2 share NG-RAN in an indirect sharing method.

- Operator 1 has a regulatory obligation to broadcast PWS message to all UEs operating on NG-RAN 1 and NG-RAN.

- Operator 2 has a regulatory obligation to broadcast PWS message to all UEs operating on NG-RAN and RAN 2.

- UEs operating in NG-RAN 1 or RAN 2 may handover to NG-RAN during the period the PWS message is periodically rebroadcasting.

- UEs subscribed to Operator 1 operating in NG-RAN may handover to NG-RAN 1 during the period the PWS message is periodically rebroadcasting (and vice versa).

- UEs subscribed to Operator 2 operating in NG-RAN may handover to RAN 2 during the period the PWS message is periodically rebroadcasting (and vice versa).

Legend:
PWM: Public Warning Message
CBCS: Cell Broadcast System
PWS: Public Warning System

Figure 5.x.2-1 PWS Scenario in Indirect Network Sharing

### 5.x.3	Service flows

1) The Public Safety Agency (PSA) sends the PWS message in a certain shared NG-RAN radio coverage area designated by the PSA where Operator 1 and Operator 2 share the RAN access.

- Operator 1 and Operator 2 both  broadcast the PWS message in the area covered by their respective not shared RAN they have regulatory obligations (NG-RAN 1 and RAN 2).

- The UEs served by NG-RAN 1 and RAN 2  receive the warning message from their corresponding operating radio access.

- The UEs served by shared NG-RAN of the hosting Operator 1 receive the public warning message created for the specific region which is covered by NG-RAN

NOTE: Today this is not the case. The UEs served by this area will present 2 PWS messages, one from each operator.

### 5.x.4	Post-conditions

For UEs subscribed to unshared accesses NG-RAN 1 and RAN 2, the Public Warning message can only be received by UEs subscribed to these operators owning the access.

UEs served by shared NG-RAN receive the public warning message issued by the PSA in the radio coverage area of the shared NG-RAN designated for the warning message.

The UEs only display one copy of the PWS message according to and regardless of the RAN it is operating in or may handover to.

The UEs shall neither miss to present new content, nor present duplicate content of public warning messages after entering or moving out of a shared network.

### 5.x.5	Existing requirements to PWS Support in Shared Networks

TS 22.101

28.2.5 PWS support of Shared E-UTRAN/NG-RAN

A Participating Operator potentially has regulatory obligations to initiate the broadcast of PWS messages regardless of E-UTRAN/NG-RAN sharing.

The following requirements apply:

The shared E-UTRAN/NG-RAN shall be able to broadcast PWS messages originated from the core networks of all Participating Operators.

NOTE 1: 	Rel-11 design requires a shared PWS core. However, some regulatory obligations require a solution in which no common PWS core network entity is involved.

28.3.5 PWS support of Shared GERAN or UTRAN

A Participating Operator potentially has regulatory obligations to initiate the broadcast of PWS messages regardless of GERAN or UTRAN sharing.

The following requirements apply:

The shared GERAN or UTRAN shall be able to broadcast PWS.

NOTE 2: 		The Hosting RAN Operator is responsible for the delivery of PWS messages to the UEs.

Identifying, changing, and adding appropriate functionality in the network will definitely lead to a better shared-network operation.

### 5.x.6	Requirements

[PR 5.x.6-001] Based on regional or national regulations and on mutual agreement between the participating operators the 5G system shall support PWS for a shared NG-RAN in indirect interconnection with the participating operators.

[PR 5.x.6-002] All the UEs served by the shared NG-RAN in the designated coverage of the shared area and based on regional or national regulations and on mutual agreement between the participating operators, shall receive PWS messages broadcasted by the hosting network.

[PR 5.x.6-003] Based on regional or national regulations and on mutual agreement between the participating operators the 5G system shall ensure the UEs served by the shared NG-RAN to neither miss to present new content, nor present duplicate content of public warning messages after entering or moving out of a shared network.

* * * End of Changes * * * *