3GPP TSG-SA WG1 Meeting #101	S1-230086

# Athens, Greece, 20th - 24th February 2023									(revision of S1-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0674 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to PALS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | PALS |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In SA#94-e the CR pack SP-211493 on PALS was approved. However, when the approved CRs were implemented in TS 22.261 V18.5.0, the CR0614r1 on Clarification of Service Providers for PALS was not completely implemented. That means it was forgotten to implement one change in clause 6.41.2.2 (replacing “3rd party” by “service provider of localized services”). - The term “Service provider of localized services” also includes 3rd party service provider, so a general clarification should be added in clause 6.41.2.2. - In clause 6.41.2.3 the wording of the note can be improved. - In clause 6.41.2.7 the line spacing of the second requirement should be fixed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 6.41.2.2 the missing change from CR0614r1 has been implemented. - In clause 6.41.2.2 the general note below has been added. NOTE: The term “Service provider of localized services” includes also 3rd party service provider. - In clause 6.41.2.3 the wording of the note has been improved. - In clause 6.41.2.7 the line spacing of the second requirement has been fixed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies in the description of PALS remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.41.2.2, 6.41.2.3, 6.41.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

#### 6.41.2.2	Configuration of Localized Services in Hosting Network

NOTE:	The term “Service provider of localized services” includes also 3rd party service provider.

The 5G system shall support suitable mechanisms to allow automatically establishing localized service agreements for a specific occasion (time and location) and building temporary relationship among hosting network operator and other service providers including network operators or 3rd party application providers.

The 5G system shall support means for the service provider to request the hosting network via standard mechanisms to provide access to 3rd party services at a specific period of time and location. This period of time shall be flexible, so that a change in service provision can be decided at any time (e.g., to cancel or prolong local services in the locality of service delivery) based on localized services agreements.

Based on localized services agreements, the 5G system shall provide suitable means to allow the service provider to request and provision various localized service requirements, including QoS, expected/maximum number of users, event information for discovery, network slicing, required IP connectivity etc, and routing policies for the application of the localized services via the hosting network.

The 5G system shall support means for a hosting network to create policies and configure resources for the requested time and location for the 3rd party services based on the received request.

The 5G system shall support means for a hosting network to notify the service provider of the accepted service parameters and routing policies.

Subject to regulatory requirements and localized service agreements, the 5G system shall allow a home network operator to automatically negotiate policies with the hosting network for allowing the home network’s subscribers to connect at a specific occasion, e.g., time and location, for their home network services.

Subject to the automatic localized services agreements between the hosting network operator and home network operator, for UE with only home network subscription and with authorization to access hosting networks the 5G system shall support:

-	access to the hosting network and use home network services or selected localized services via the hosting network,

-	seamless service continuity for home network services or selected localized services when moving between two hosting networks or a host network and the home network.

The 5G system shall support a mechanism to enable configuration of a network that provides access to localized services such that the services can be limited in terms of their spatial extent (in terms of a particular topology, for example a single cell), as specified by a service provider of localized services.

The 5G system shall support a mechanism to enable configuration of a network that provides access to localized services such that the services can be limited in terms of the resources or capacity available, to correspond to requirements that apply only to the locality of service delivery, as specified by a service provider of localized services.

The 5G system shall support means for a hosting network to provide a 3rd party service provider with information for automatic discovery of the hosting network by the UEs to allow access to specific 3rd party services.

The 5G system shall support secure mechanisms to allow a home network to coordinate with a hosting network for a subscriber to temporarily access the hosting network (e.g., based on temporary credentials) at a given time (start time and duration) and location.

#### 6.41.2.3	User Manual Selection of Localized Services via Hosting Network

The hosting network shall allow a UE to manually select temporary localized services which are provided via local breakout at the hosting network.

NOTE:	Localized services which are provided via local breakout at the hosting network can be based on interworking scenarios for hosting network owned/collaborative services as indicated in Annex H.

#### <Text omitted>

#### 6.41.2.7	Returning to Home Network

The 5G system shall provide mechanisms to mitigate user plane and control plane overload caused by a high number of UEs returning from a temporary local access of a hosting network to their home network in a very short period of time.

The 5G system shall provide mechanisms to minimize the impact on the UEs communication e.g., to prevent user plane and control plane outages when returning to a home network together with other high number of UEs in a very short period of time, after terminating their temporary local access to a hosting network.

End of changes