3GPP TSG SA WG 1 Meeting #101 	S1-230089

Athens, Greece, 20 – 24 February 2023	(revision of S1-23xxxx)

Source:	Xiaomi, Qualcomm, OPPO

pCR Title:	New use case: Vehicle Sensing for ADAS

Draft Spec:	3GPP TR 22.837 V0.3.0

Agenda item:	7.1

Document for:	Approval

Contact:	Chunhui Zhu<zhuchunhui@xiaomi.com>

Abstract: It is proposed to add a new use case: Vehicle Sensing for ADAS to 3GPP TR 22.837 V0.3.0.

1. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 V0.3.0.

* * * First Change * * * *

2	References

…

[x0]	S. Saponaraet. al, "Radar-on-Chip/in-Package in Autonomous Driving Vehicles and Intelligent Transport Systems: Opportunities and Challenges," inIEEE Sig. Proc. Mag., Sept. 2019.

[x1]	J. Hasch, E. Topak, R. Schnabel, T. Zwick, R. Weigel and C. Waldschmidt, "Millimeter-Wave Technology for Automotive Radar Sensors in the 77 GHz Frequency Band," inIEEE Transactions on Microwave Theory and Techniques, vol. 60, no. 3, pp. 845-860, March 2012.

[x2]	S. M. Patole, M. Torlak, D. Wang and M. Ali, "Automotive radars: A review of signal processing techniques," inIEEE Signal Processing Magazine, vol. 34, no. 2, pp. 22-35, March 2017.

[x3]	R. Bosch, “LRR3 3rd Generation Long-Range Radar Sensor,” Robert Bosch GmbH, Germany, 2009.

[x4]	Continental, A.G., ARS 408-21 Premium Long RangeRadar Sensor 77 GHz.ARS, pp.408-21.

[x5]	F. Engels et. al, "Automotive Radar Signal Processing: Research Directions and Practical Challenges," in IEEE JSTSP, June2021.

[x6]	I. Greshamet al., "Ultra-wideband radar sensors for short-range vehicular applications," inIEEE Transactions on Microwave Theory and Techniques, vol. 52, no. 9, pp. 2105-2122, Sept. 2004.

[x7]	AinsteinAutomotive Safety Radar T-79 short-range radar: https://ainstein.ai/vehicle-radar/short-range-wideband-high-resolution-automotive-radar-sensor/

* * * Second Change * * * *

5.A	Use case on Vehicle Sensing for ADAS

5.A.1	Description

Advanced Driving Assistance System(ADAS) uses various sensors (Wireless Sensing millimeter wave radar, lidar, monocular / binocular camera and satellite navigation) installed on the vehicle to sense the surrounding environment at any time during the driving process, collect data, identify, detect and track static and dynamic objects, and carry out systematic calculation and analysis in combination with navigation map data, so as to make the driver aware of the possible dangers in advance, and effectively increase the comfort and safety of driving.

Figure 5.A.1-1 ADAS overview

There is an opportunity for 3GPP New Radio (NR) based sensing technologies to be added into ADAS. 5G based wireless sensing service could improve the ADAS reliability and quality.

The ADAS has the map information, the real time location/ trajectory of the car and can assist the car driving, e.g. stop the car for avoiding collision. The car (as 3GPP UE) are equipped with 3GPP NR based sensing technology. When the UE initially access the 5G network, the UE is authorized by the 5G network to participate in sensing under the operator’s control. The sensing measurement data is from the NR based sensor, and the sensing result is sent to the ADAS system of the car. Collaborating with other non-3GPP sensing devices/technologies, NR based sensing result as input to ADAS could improve the comfort and safety of driving.

The vehicle as a 3GPP UE based sensing is important for automotive use cases, it can operate under network control & complements network based sensing. Network based sensing alone cannot fully address the automotive use case needs, e.g.:

- There may be a blockage from the network (base station) to the sensed target.

- ADAS concerns more on relative positioning other than absolute position. Network based sensing introduces additional errors (due to compounding errors of two separate positions).

- Automobiles applications may determine sensing priority/performance requirements locally (not visible to gNB).

UE based sensing resources allocations can be under the control of the network (e.g. base station). For UEs inside the coverage of enhanced network for sensing, resources can be directly controlled. For UEs outside of the coverage, UE Policy based control (e.g. similar to V2X) can be used for control. Sensing results of the automobiles can be shared via 3GPP connections (Uu or PC5).

It is expected that the 3GPP NR sensing service for ADAS should meet the requirement and level of commercial ADAS radar sensing performance. Based on requirements for existing automotive sensors, the RF based sensing requirements for automotive applications are as the following table.

Table 5.A.1-1 RF based sensing requirements for automotive applications of existing automotive sensors

| Parameter​ | Typical Automotive Radar KPIs |  |
| --- | --- | --- |
|  | Long Range Radar​[x0-x5] | Short Range Radar[x0-x2, x5-x7] |
| Maximum range​ | 250-300m​ (for RCS of 10dBsm with >90%detection probability)​ | 30-100m |
| Range resolution​ | 10-75cm | 5-20cm​ |
| Range accuracy | ±10 to ±40cm | ±2 to 10cm |
| FOV azimuth​ | ±9-15deg​ | ±60-85deg​ |
| Azimuth resolution​ | 1-3deg​ | 3-9deg​ |
| Azimuth accuracy | ±0.1-0.3deg | ±0.3-5deg |
| Update rate​ | 5 to 20 fps | 20 to 50 fps​ |
| Max one-way velocity​ | ±50m/s to ±70m/s​ | ±30m/s |
| Velocity resolution | 0.1 –0.6 m/s​ | 0.1 - 0.6 m/s |
| Velocity accuracy | ±0.03m/s to ±0.12 m/s | ±0.03m/s to ±0.12 m/s |

Although all vehicles on the road are expected to be equipped with NR radio, but not necessarily utilizing them for 100 percent of time. Enabling NR radio based sensing for ADAS can be a better utilization of the capability and radio resources.

5.A.2	Pre-conditions

The 3GPP UE in the car has 3GPP subscription and is authorized by the operator to perform sensing.

5.A.3	Service Flows

1. Tom buys a new car with the latest ADAS equipped.

2. Tom wants to drive the car from home to the company in the morning of a working day. Tom drives from home to the road. The 3GPP NR based sensor in Tom’s car transmits the 3GPP NR signal to the other car(s) in the same road, and receives the reflected signal to detect the distance and speed of the other car(s) to feed to the ADAS in Tom’s car.

3. While the car is driving on the highway, suddenly a car is stopped before the Tom’s car due to an accident, fortunately it is timely detected by the NR based sensor.

4. The NR based sensor sends the collision warning to the ADAS, the ADAS stops the car immediately.

5. Finally, Tom’s car avoids a collision accident and leaves the highway safely.

5.A.4	Post-conditions

With the safely driving experience provided by ADAS, Tom arrives in the company safely and easily. Tom starts the daily work in the office.

5.A.5	Existing features partly or fully covering the use case functionality

There are features of Sidelink positioning for the car moving along the LOS road (e.g., using Sidelink positioning for car ranging on the same road), which requires the participant cars are 3GPP UEs.

5.A.6	Potential New Requirements needed to support the use case

[PR 5.A.6 -1] The 5G system shall be able to support a vehicle as a 3GPP UE to perform sensing under operator’s control.

[PR 5.A.6-2] The 5G system shall be able to manage 3GPP Radio resource usage for 3GPP RF based sensing.

[PR 5.A.6-3] The 5G system shall be able to charge the sensing service for the use of the 3GPP sensing radio and network resources.

[PR 5.A.6-4] Both Online and Offline charging shall be supported for the in-coverage sensing service.

[PR 5.A.6-5] The Offline charging shall be supported for the out-of-coverage sensing service.

[PR 5.A.6-6] The UE based usage reporting for charging can be utilized for the sidelink sensing and sensing result based charging.

[PR 5.10.6-7] The 5G system shall support sensing for ADAS services with KPIs as given in Table 5.A.6-1.

NOTE:	The KPIs for Vehicle Sensing for ADAS should meet the requirement and level of commercial ADAS radar sensing performance.

Table 5.A.6-1 KPIs for Vehicle Sensing for ADAS

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| ADAS [long range Radar] | Targets within: FoV: ±9-15deg Max Range: 250-300m Max velocity:±50m/s to ±70m/s | TBD | ~[0.1]-[1.3] NOTE 1 | TBD | [±0.03] -[±0.12] m/s NOTE 3 | N/A | [0.1] -[0.4] NOTE 4 | [0.1] –[0.6] NOTE 3 | [50] | [0.05] -[0.2] | [1-10]% | [<1]% |
| ADAS [Short range Radar] | Targets within: FoV:±60-85deg Max Range: 30-100m Max Velocity:±30m/s | TBD | ~[0.02]-[2.6] NOTE 2 | TBD | [±0.03] -[±0.12] m/s NOTE 3 | N/A | [0.02] –[0.1] NOTE 4 | [0.1] –[0.6] NOTE 3 | [20] | [0.02] –[0.05] | [1-10]% | [<1]% |
| NOTE 1: Assuming typical max range of 250m, range accuracy of 10cm, azimuth accuracy of ±0.3deg.Positioning accuracy as (Min-Max) within field of view. NOTE 2: Assuming typical max range of 30m, range accuracy of 2cm, azimuth accuracy of ±5deg.Positioning accuracy as (Min-Max) within field of view. NOTE 3: Velocity accuracy and resolution is typically reported for the radial velocity (not absolute H/V velocity) NOTE 4: Range resolution typically reported as 3D ranging distance accuracy. |  |  |  |  |  |  |  |  |  |  |  |  |