3GPP TSG SA WG 1 Meeting #101		 S1-230109

Athens, Greece, 20 - 24 February 2023

Title:	[Draft] Reply LS on KPIs for AI/ML model transfer in 5GS

Response to:	LS S2-2301578 from SA2

Release:	Rel-18

Work Item:	AIML-MT, AIMLsys

Source:	Qualcomm [SA1]

To:	SA2

Cc:	-

Contact person:	Francesco Pica

Fpica<at>qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR in S1-22xxxx

# 1	Overall description

SA1 thanks SA2 for their LS S2-2301578, and questions. SA1 answers and clarifications are provided below:

Q1: Could SA1 check whether the 2 ms Max allowed DL end-to-end latency in Table 7.10-1 of TS 22.261 for Split AI/ML image recognition is really necessary KPI value for 5GC? If not, SA2 kindly asks SA1 to revise this KPI.

[SA1 answer]: Assuming SA2 is referring to the Max allowed UL end-to-end latency in Table 7.10-1 of TS 22.261 (see below), SA1 agreed to revise the KPI value, as per enclosed CR.

| # Uplink KPI |  |  |  |  | # Downlink KPI |  |  |  | # Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Max allowed UL end-to-end latency | # Experienced data rate | # Payload size | # Communication service availability | # Reliability | # Max allowed DL end-to-end latency | # Experienced data rate | # Payload size | # Reliability |  |
| 2 ms | 1.08 Gbit/s | # 0.27 MByte | # 99.999 % | # 99.9 % |  |  |  | # 99.999 % | # Split AI/ML image recognition |
| … |  |  |  |  |  |  |  |  |  |

Q2: Could SA1 clarify if it is really necessary to update the model in 1 sec?

[SA1 answer]: 1sec is confirmed to be a suitable requirement.

Q3: SA2 would like to ask if any suggested PDB value from SA1? For example, Does PDB = 10ms for AI/ML inference between UE and Application Function and PDB = 100ms for AI/ML model downloading/Federated learning can fulfil the SA1 requirement or not?

[SA1 answer]: the indicated PDB values are reasonable.

Q4: Any other feedback SA1 would like to provide?

[SA1 answer]: no further feedback.

# 2	Actions

To SA2

ACTION: SA1 kindly asks SA2 to take the above into consideration.

# 3	Dates of next TSG SA WG 1 meetings

SA1#102	22-26 May 2023			Berlin (Europe)

SA1#103	21-25 Aug 2023			Gothenburg (Europe)