3GPP TSG SA WG 1 Meeting #101 	S1-230136

Athens, Greece, 20 - 24 February 2023	(revision of S1-23xxxx)

Source:	Novamint

pCR Title:	Text Proposal for the Overview section

Draft Spec:	3GPP TR 22.865 version 0.2

Agenda item:	7.7

Document for:	Approval

Contact:	Thierry Bérisot (tberisot@novamint.com)

Abstract: This document provides a text proposal to update the overview section of TR 22.865 version 0.2

1. Introduction

The overview of TR 22.865 is currently not included in version 0.2

2. Reason for Change

Adding an overview

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.865 v0.2.0.

* * * First Change * * * *

# 4	Overview

The present document captures a set of use cases and potential service requirements related to the 5G system with satellite access taking into account new capabilities such as:

- Store and forward satellite operation with discontinuous feeder link for delay-tolerant communication services: Store and Forward (S&F) operation for a satellite allows to provide autonomously service to UEs without the satellite being always connected to a gateway (i.e. S&F allows service link access to continue to be operational even at times when the feeder link is not connected). This is particularly relevant for delay-tolerant IoT services via NGSO space segment. In this context, each satellite of a constellation acts similarly to a SMSC and store the message until there is a feeder link available.

- GNSS independent operation: This would allow to provide satellite access to UE without GNSS receiver or with no access to GNSS service.

- Communication between UEs under the same satellite’s coverage: In some scenarios, UEs may need to communicate through a satellite access without going to the ground network in order to avoid long delays and limited data rate as well as reducing the consumption of backhaul resources (including resources of ISL and feed link).

In addition, the TR includes several use cases around positioning aspects with satellite access in order to update existing satellite positioning KPIs.

Editor’s Note: the overview maybe updated based on new inputs / use cases agreed during this or next SA1 meeting.