3GPP TSG SA WG 1 Meeting #101 	S1-230137

Athens, Greece, 20 - 24 February 2023

Source:	Novamint, TNO

pCR Title:	Update of 5.2

Draft Spec:	3GPP TR 22.865 version 0.2

Agenda item:	7.7

Document for:	Approval

Contact:	Thierry Bérisot (tberisot@novamint.com)

Abstract: This document provides a text proposal to TR 22.865 to update the use case “store and forward – MT” including the potential requirements

1. Introduction

The use case of “store and forward – MT” and the associated potential requirements have been agreed and captured in TR 22.865, where an editorial note is included in the PR-002

2. Reason for Change

How security would apply is not the responsibility of SA1 so the Editorial Note can be removed

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.865 v0.2.0.

* * * First Change * * * *

## 5.2	Use case on store and forward - MT

### 5.2.1	Description

This use case illustrates the realization of a S&F service between a UE with satellite access and an Application Server for a delay-tolerant/non-real-time IoT NTN service in the case of a Mobile Terminated message.

A description of store and forward operation is provided in Annex A.

Company TrackingInc is offering a service of remote monitoring of fields and is deploying and tracking many battery-powered IoT type UEs across the globe. All the IoT remote monitoring UEs deployed are including a 5G communication with satellite access. Some of them are deployed in remote area where there is no coverage by MNO and only satellite is possible.

For the satellite access, TrackingInc is using the service of IoTSAT for the 5G IoT connectivity by satellite and IoTSAT is using a LEO constellation which supports S&F operation mode.

All IoT remote monitoring UEs are regularly sending information related to the area they are monitoring to the application server of TrackingInc and some times receive new parameters from the application server. In most of the cases, the messages exchanged are delay-tolerant/non-real-time IoT.

### 5.2.2	Pre-conditions

In the present use case, the IoT remote monitoring UE is in a remote area with no ground stations available for feeder link connectivity and the IoT remote monitoring UE is aware that IoTSAT constellation operates in S&F mode.

### 5.2.3	Service Flows

The TrackingInc application server needs to send new parameters to the IoT remote monitoring UE through a dedicated message.

The TrackingInc application server message is sent by conventional means (e.g. IP routing, tunnels) to the network entry-point (e.g. a SCEF, PDN-GW, SMSC).

At this point:

- Limitations on the amount of data to be transferred to the IoT remote monitoring UE could be enforced.

- Delivery priority could be established.

- Acknowledgement of the received data by the network could be issued.

- End-to-end acknowledgement policy can be established.

The network stores the message until it can be delivered/relayed to a satellite expected to fly over and provide coverage to the destination IoT remote monitoring UE.

When the satellite is connected via the feeder link to the ground network, the message is uploaded into the satellite.

When flying over the area that the IoT remote monitoring UE is located, the satellite with the stored message triggers paging over the service link for the UE to connect to the network.

The stored message is delivered/downloaded from the satellite to the IoT remote monitoring UE. Acknowledgment may be requested/issued. Mechanisms to ensure integrity of the delivered information may be in place.

### 5.2.4	Post-conditions

The message generated by the TrackingInc application server has been delivered successfully to the IoT remote monitoring UE without relying on a continuous end-to-end network connectivity path between them.

### 5.2.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2], clause 6.3.2.3 on satellite access includes the following requirements:

The 5G system shall be able to provide services using satellite access.

The 5G system with satellite access shall be able to support low power MIoT type of communications.

However, it is not sufficient in regards of S&F operation especially for the delivery of delay-tolerant/non-real-time IoT NTN services with NGSO satellites.

### 5.2.6	Potential New Requirements needed to support the use case

[PR 5.2.6-001] The 5G system with satellite access shall be able to inform an application server whether store and forward operation is applied for communication with a UE.

[PR 5.2.6-002] The 5G system with satellite access shall be able to provide integrity protection and confidentiality for communications between an authorized UE and the network when store and forward operation is applied.