3GPP TSG SA WG 1 Meeting #101 	S1-230149

Athens, Greece, 20 - 24 February 2023	(revision of S1-23xxxx)

Source:	CATT

pCR Title:	Pseudo-CR on updates of clause 5.7

Draft Spec:	3GPP TR 22.851 v0.3.0

Agenda item:	7.4

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This document provides a proposal on updating use case of long-distance mobility in and across shared networks in clause 5.7 of TR22.851.

1. Introduction

The use case of “long-distance mobility in and across shared networks” and associated potential requirements have been agreed and captured in TR22.851 v0.3 with Editor’s Notes, which intend to be further clarified in this contribution.

2. Reason for Change

As discussed in SA1 #100, the following Editor’s Note was added for future study on the potential requirements:

Editor’s Note: Requirement #001, #002 are FFS.

Regarding more study on the mechanism of mobility management and access control in MOCN configuration of network sharing, PR#001 is updated to address the enhancement on the rule management in different coverage area of Shared RAN. Then the service flow and the analysis of existing features are updated accordingly.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree on the following changes to 3GPP TR22.851 v0.3.0

* * * First Change * * * *

5.7	Use case of long-distance mobility in and across shared networks

5.7.1	Description

Despite the aircraft and rail have been developed to serve more and more regions, road transport remains the most important way to deliver goods. As the logistics industry is booming, the total length of long-distance road transport has increased every year in the last ten years. How to provide communication service during long-distance transport is valuable to be discussed to serve the transportation and logistics customers. Network sharing offers a way for the network operator to share the heavy deployment of mobile networks, as well as improve the service availability and user experience along the transport routes.

Compared with MOCN configuration, network sharing with the indirect connection between shared RAN and the core network of Participating Operators allows flexible access options for the sharing parties to consider. For example, one network operator needs to provide communication service along a dedicated transport route across dense urban, rural, and desert, but can’t find one Hosting RAN Operator with full coverage of the whole route. It’s possible to cooperate with a terrestrial network operator and a 3GPP satellite network operator separately to ensure the full coverage. Also, with the agreement, it’s possible to leverage the temporary coverage enhancement plan of Hosting RAN Operators for certain regions, such as deploying onboard base stations in dense urban, to provide better user experience to their own users.

Figure 5.7.1-1 Desert Highway across shared networks [9]

5.7.2	Pre-conditions

It is assumed that network operators OP1, OP2, and OP3 have deployed 5G networks respectively in a country. Their radio access network can provide coverage to different regions of the country, and together cover the entire country with overlapping in certain regions.

OP1 and OP3 are agreed with OP2 to share radio access network in the planned geographic area via the indirect connection between their shared radio access network and OP2’s core network.

- OP1 is a Hosting RAN Operator, who has deployed different types of 5G access nodes (e.g. Macro, Micro, onboarding, and IAB).It only shares Macro base stations (5G BS#A, BS#B, BS#C) with Participating Operator OP2 but allows to access to dedicated type of access nodes(e.g. onboarding) temporarily regarding the agreement and the policy.

- OP3 is another Hosting RAN Operator, sharing 3GPP Satellite access nodes with OP2.

- OP2 has non-shared 5G access network deployed in some regions of the country, as 4G&5G BS.

- UE on the vehicle supports all 5G RATs and has the subscription with OP2‘s network.

Figure 5.7.2-1: Long-distance mobility in and across shared networks

5.7.3	Service Flows

1. UE on the vehicle connects to OP2’s network via OP1 shared access nodes and is authorized to initiate a communication service (e.g., data transfer, IMS voice call) during the movement.

2. In geographic Area A, UE connects to OP2’s network only through the access nodes like 5G BS #A based on the agreement and the policies of OP1 and OP2.to initiates a communication service (e.g., data transfer, IMS voice call) during the movement.

3. A regional earthquake happens and results in unstable performance of 5G BS #B in Area B. OP1’s onboarding base stations are shared to OP2 as a supplementary or an alternative access node based on OP1 and OP2 agreement.

4. When UE reaches Location Loc #A, where it can detect the radio signal of both 5G BS #B and onboarding base station, UE will be steered to access a proper OP1 access node (e.g. onboarding base station) to continue the service regarding the subscription and the policies of OP1 and OP2.

5. When UE reaches Location Loc #B, where it can detect the radio signal of OP1 5G BS#C and OP3 Satellite access node, UE will be steered to access a proper Hosting RAN operator’s access node (e.g., OP3 Satellite) to continue the service regarding the subscription and the policies of OP1, OP3 and OP2.

6. When UE reaches Location Loc #C, where it can detect the radio signal of OP3 Satellite access node and OP2 4G&5G BS, UE will be steered to select OP2 5G cell to continue the service regarding the policies of OP3 and OP2.

5.7.4	Post-conditions

UE can get services with the displayed serving network as OP2’s network when move in OP1’s shared RAN, across OP1’s and OP3’s shared RANs, and across OP3’s shared RAN and OP2’s non-shared RAN.

OP2 ensures service continuity with proper mobility and access control to the shared access nodes of the same or different Host RAN Operators.

5.7.5	Existing features partly or fully covering use case functionality

The normative stage1 requirements of network sharing have been introduced to TS22.101 [2] and TS 22.261 [3] in previous releases. TS 22.101[2] specifies the service principle about Network Sharing that

It shall be possible to support different mobility management rules, service capabilities and access rights as a function of the home PLMN of the subscribers. [clause 4.9]

The 3GPP System shall support a Shared RAN (E-UTRAN or NG-RAN) to cover a specific coverage area (e.g. from a complete network to a single cell, both outdoor and indoor). [clause 28.2.1]

The network sharing has been extended to different types of access networks, as stated TS 22.261 [3].

A 5G satellite access network shall support NG-RAN sharing.

TS 22.261[3] has also illustrated general requirements about access, the mobility management and service continuity between different access networks as below.

Based on operator policy, the 5G system shall support steering a UE to select certain 3GPP access network(s). [clause 6.3.2]

The 5G system shall support inter- and/or intra- access technology mobility procedures within 5GS with minimum impact to the user experience (e.g., QoS, QoE). [clause 6.2.2]

The 5G system shall support service continuity between 5G terrestrial access network and 5G satellite access networks owned by the same operator or owned by different operators having an agreement. [clause 6.2.3]

TS 22.261[3] has defined requirements about multi-network connectivity and service delivery across operators as below.

To provide a better user experience for their subscribers with UEs capable of simultaneous network access, network operators could contemplate a variety of sharing business models and partnership with other network and service providers to enable its subscribers to access all services via multiple networks simultaneously, and with minimum interruption when moving.

For a user with a single operator subscription, the use of multiple serving networks operated by different operators shall be under the control of the home operator.

In MOCN configuration, that multiple shared access networks have direct connections to the Participating Operator’s core network, the access control and mobility management are performed by Participating Operator’s core network directly.

However, in Indirect Network Sharing, besides Participating Operator’s core network, Host RAN Operator’s core network may be involved in the access control and mobility management. Also, UE may connect to Participating Operator’s core network via the shared access nodes of different Host RAN Operators.

TS 22.261[3] has defined requirements about multi-network connectivity and service delivery across operators as below.

To provide a better user experience for their subscribers with UEs capable of simultaneous network access, network operators could contemplate a variety of sharing business models and partnership with other network and service providers to enable its subscribers to access all services via multiple networks simultaneously, and with minimum interruption when moving.

For a user with a single operator subscription, the use of multiple serving networks operated by different operators shall be under the control of the home operator.

5.7.6	Potential New Requirements needed to support the use case

[PR 5.7.6-001] In case of Indirect Network Sharing, the 5G system shall support different mobility management rules and access rights for Participating Operator’s subscribers in different coverage areas of a Shared RAN based on the Hosting and Participating Operator’s agreement and policies.

[PR 5.7.6-002] In case of multiple Shared RANs of different Hosting Operators using Indirect Network Sharing, the 5G system shall support steering UE to select certain Shared RAN based on the Hosting and Participating Operators’ policies.

* * * End of Changes * * * *