3GPP TSG SA WG 1 Meeting #101		 S1-230154

Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.153 | CR | 0058 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MPS requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MPS-WLAN, TEI-18 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In recent stage-2 Rel-18 work (MPS WLAN WI of SA2), there have been discussions and concerns related to mechanisms to prioritise access to MPS over NSWO (passing the MPS priority using SWa interface), referring to SA1 MPS requirements in 22.153 sec 10.1.3. In fact, there is an issue with current requirements in sec 10.1.3, as they are not consistent with the intent of the functionalities targeted in the entire sec.10.1, appearing to be out of scope. In particular: - Sec 10.1 talks about “requirements specific to WLAN access to EPC/5GC“, whilst - Sec 10.1.3 refers to NSWO, which, based on stage-2 description (in TS 23.501), is for services that are not traversing 5GC: " A UE supporting Non-seamless WLAN offload may, while connected to WLAN access, route specific data flows via the WLAN access without traversing the 5GC. These UE data flows are identified using URSP configuration for Non-Seamless Offload, or UE Local Configurations as defined in TS 23.503 [45]. For these data flows, the UE uses the local IP address allocated by the WLAN access network and no IP address preservation is provided between WLAN and NG-RAN." |  |  |  |  |  |  |  |  |
| Summary of change: |  | Propose to remove the requirements in sec. 10.1.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements are incorrect and/or inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

================ Changes ====================

# 10	Non-3GPP access

## 10.1	MPS when access to the EPC/5GC is WLAN

MPS can be supported for MMTEL voice/video calls and data sessions when the access to the EPC/5GC is WLAN and the 3GPP system is used for the MPS authorization.  In this case, MPS priority treatment can be provided by:

-	both the WLAN access and the EPC/5GC (i.e., when the WLAN supports MPS), or

-	only the EPC/5GC (i.e., when the WLAN does not support MPS).

The following subclauses provide requirements specific to WLAN access to EPC/5GC.

### 10.1.1	MPS for MMTEL voice and video

#### 10.1.1.1	General

For a UE with WLAN access to the EPC/5GC, the system shall support the requirements in clauses 9.1 and 9.2 for MMTEL voice and voice conference calls and MMTEL video and video conference calls by an authorized MPS Service User using

-	a UE with a subscription for MPS, and

-	a UE that does not have an MPS subscription.

The system shall support the requirements in clauses 9.1 and 9.2 for MPS for MMTEL voice and voice conference calls and MPS for MMTEL video and video conference calls terminating to a UE with WLAN access to the EPC/5GC regardless of the terminating UE subscription for MPS.

#### 10.1.1.2	Authentication and authorization

For a UE with a 3GPP subscription for MPS and with WLAN access to the EPC/5GC, the system shall support MPS for MMTEL voice/video authorization based on the UE subscription information.

For a UE that does not have a 3GPP subscription for MPS and with WLAN access to the EPC/5GC, the system shall support MPS for MMTEL voice/video authorization based on Service User credentials not associated with the UE (e.g., a calling card number, PIN or security token).

#### 10.1.1.3	WLAN Interworking

For a UE with WLAN access to the EPC/5GC, the 3GPP system shall support delivery of the following to the WLAN:

-	indication of MPS for MMTEL voice/video authorization,

-	priority marking of transport parameters of MPS for MMTEL voice/video signalling and media, and

-	QoS characteristics of MPS for MMTEL voice/video signalling and media.

### 10.1.2	MPS for DTS

#### 10.1.2.1	General

For a UE or IoT device with WLAN access to the EPC/5GC, the system shall support the requirements in clause 9.3 on MPS for DTS sessions for:

-	an authorized Service User using a UE with a subscription for MPS,

-	an authorized Service User using a UE that does not have an MPS subscription,

-	an authorized IoT device with a subscription for MPS, and

-	an authorized enterprise network activation of MPS for DTS for remote MPS subscribed UEs/IoT devices associated with the enterprise network.

##### 10.1.2.2	Authentication and authorization

For a UE or IoT device with a 3GPP subscription for MPS and with WLAN access to the EPC/5GC, the system shall support MPS for DTS authorization based on the UE subscription information.

For a UE that does not have a 3GPP subscription for MPS and with WLAN access to the EPC/5GC, the system shall support MPS for DTS authorization based on Service User credentials not associated with the UE (e.g., a calling card number, PIN or security token).

For an authorized enterprise network that is activating MPS for DTS for a remote UE or IoT device with a 3GPP subscription for MPS, the system shall support MPS for DTS authorization based on the UE or IoT device subscription information.

#### 10.1.2.3	WLAN interworking

For a UE or IoT with WLAN access to the EPC/5GC, the 3GPP system shall support delivery of the following to the WLAN:

-	indication of MPS for DTS authorization,

-	priority marking of transport parameters of MPS for DTS signalling and media, and

-	QoS characteristics of MPS for DTS signalling and media.

### 10.1.3	Void