3GPP TSG SA WG 1 Meeting #101 	S1-230157

Athens, Greece, 20 - 24 February 2023	(revision of S1-223454)

Title:	Use Case on UAV simultaneous traffic over two networks

Agenda item:	7.8

Source:	Qualcomm, Futurewei, InterDigital, Lockheed Martin

Document for:	Approval

Contact:	Francesco Pica
fpica<at>qti.qualcomm.com

Abstract: This paper proposes a new use case to be captured in TR 22.843 v.0.2.0. This is a revised version of a use case submitted, and postponed, in the last SA1 meeting (S1-223454).

======================= [First Change] ========================

## 5.x	Use case on UAV simultaneous traffic over two networks

### 5.x.1	Description

Related to discussions within the drones’ aviation industry, on how to improve reliability of UAV traffic, e.g., for command and control (C2) or other critical data, one scenario of interest includes the ability for drones provided with dual SIM/subscription to connect simultaneously via two networks for transferring different and specific type of traffic, for instance use the most reliable NW for C2 or other critical data.

As an example, let us assume the case of a UAV company (“SafeSky”) that offers monitoring and surveillance services for specific venues (e.g., stadiums) or public events. Their drones are 5G-capable and configurable with dual subscription, one for nationwide PLMN connectivity (with MNO-A), one for ad-hoc UAV coverage using networks deployed in specific public safety venues/locations (can be a PLMN or NPN, managed by MNO-A or other MNO).

In such scenario, when there is available coverage from both NWs, SafeSky can configure their UAVs to transfer C2 traffic with a local UAV-Controller (UAV-C) via the ad-hoc PLMN1/NPN (most suitable for low volume/data rate, but very stringent real-time latency and reliability), while less critical user data is exchanged with the UAV Supplier Service (USS) and UTM via the wide-area PLMN2 coverage. This is illustrated, at high level, in Fig.1, where PLMN1/NPN is the local NW and PLMN2 is the wide area network.

Fig.5.x.1 Example of separating UAV traffic over two PLMNs.

In other examples, different traffic differentiation configurations are possible (in a similar venue scenario), e.g., UAVs may transfer real-time critical payload traffic (e.g., streaming a 4K video of a sport event) with a local endpoint via the ad-hoc PLMN1/NPN, while C2 and UAV monitoring data via the wide-area PLMN2 network.

### 5.x.2	Pre-conditions

UAVs have dual subscription (dual SIM, or SIM and NPN credentials) to two different PLMNs (or a PLMN and an NPN).

UAVs are capable of dual radio operation (e.g., NR+NR).

Specific conditions and policies, configurable by the UAV owner/provider, regulate which NW the UAVs should use, for which traffic.

It is assumed, in this use case, that UAVs connects to the same USS/UTM and UAV-C during the whole flight operation.

### 5.x.3	Service Flows

1. Based on configured policies, UAVs are registered to PLMN1/NPN and PLMN2, using their respective home subscription.

2. When monitoring surveillance service is started, UAVs connect with both networks.

3. UAVs traffic is active on both NWs, C2 communication with the UAV-C is routed over PLMN1 and other traffic monitoring data with USS/UTM is routed over PLMN2. USS/UTM is aware of the UAV traffic routing over two subscribed PLMNs and can interact with each PLMN for traffic management (PLMN1 for C2 traffic and PLMN2 for other traffic).

### 5.x.4	Post-conditions

The surveillance service runs smoothly, with no glitch in UAVs’ operation and manoeuvring.

### 5.x.5	Existing features partly or fully covering the use case functionality

There are no service requirements in 22.125 about UAS multi-NW/PLMN support. Other requirements (e.g., from TS 22.261 or 22.101) do not cover the specific target UAV scenarios and functionalities. Some examples of existing requirements are captured below:

[from 22.261 sec. 6.18: Multi-network connectivity and service delivery across operators]

The 5G system shall enable users to obtain services from more than one network simultaneously on an on-demand basis.

For a user with a single operator subscription, the use of multiple serving networks operated by different operators shall be under the control of the home operator.

When a service is offered by multiple operators, the 5G system shall be able to maintain service continuity with minimum service interruption when the serving network is changed to a different serving network operated by a different operator.

NOTE 1: A business agreement is required between the network operators.

In the event of the same service being offered by multiple operators, unless directed by the home operator's network, the UE shall be prioritized to receive subscribed services from the home operator's network.

NOTE 2: If the service is unavailable (e.g., due to lack of network coverage) from the home operator's network, the UE may be able to receive the service from another operator's network.

NOTE 3: QoS provided by the partner operator's network for the same service will be based on the agreement between the two operators and could be different than that provided by the home operator's network.

[from 22.101 sec. 13.4]

The 3GPP system shall support ME with multiple USIMs (on the same UICC or on different UICCs) that are registered at the same time.

The 3GPP system shall treat each registration from the USIMs of a MUSIM UE independently. Each registered USIM in a MUSIM UE shall be associated with a dedicated IMEI/PEI.

[Others]
In addition, there are existing UAV requirements (from TS 22.125) on 3GPP system to provide monitoring and notification regarding C2 communication changes, which are not specific to inter-PLMN connectivity, for example:

The 3GPP system shall support C2 communication with required QoS for pre-defined C2 communication models (e.g. using direct ProSe Communication between UAV and the UAV controller, UTM-navigated C2 communication based on flight plan between UTM and the UAV).

The 3GPP system shall support C2 communication with required QoS when switching between the C2 communication models.

The 3GPP system shall support a mechanism for the UTM to request monitoring of the C2 communication with required QoS for pre-defined C2 communication models (e.g. using direct ProSe Communication between UAV and the UAV controller, UTM-navigated C2 communication between UTM and the UAV).

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-001] The 5G system shall be able to support means to configure UAVs, equipped with dual PLMN subscription, to use simultaneous connectivity with both subscribed PLMNs for transferring different user data over the two connection paths, e.g., for reliable C2 traffic over one PLMN and other data over a different PLMN.

[PR 5.x.6-002] The 5G system shall be able to support means to expose UAV configuration policies related to traffic routing over two subscribed PLMNs (simultaneously) to an authorized 3rd-party (e.g., UTM) and/or to the UAV application layer (e.g., managed by the UAV provider/owner).

NOTE: the above requirements can be extended to scenarios where one network is a PLMN and one is an NPN.

====================== [End of Changes] =======================