3GPP TSG SA WG 1 Meeting #101 	S1-230177

Athens, Greece, 20 - 24 February 2023

Source:	Huawei

pCR Title:	Update of KPI table for railway intrusion detection

Draft Spec:	3GPP TR 22.837 v0.3.0

Agenda item:	7.1

Document for:	Approval

Contact:	< Fangyuan Zhu (zhufangyuan@huawei.com)>

Abstract: This document proposes to address the Editor’s Note of the KPI table for railway intrusion detection.

1. Introduction

It is proposed to update the Clause 5.7: Use case on sensing for railway intrusion detection.

2. Reason for Change

There is one Editor's Note for the KPI table for railway intrusion detection.

Editor's Note: the KPI values for Missed Deteciton and False Alarm are FFS.

There are 6 different intruder detection use cases currently captured in TR 22.837 v0.3.0.

| Scenario | Missed detection [%] | False alarm [%] |
| --- | --- | --- |
| Intruder detection in smart home | < 5 | < 2 |
| Pedestrian/animal intrusion detection on a highway | ≤5 | ≤5 |
| UAV intrusion detection | [≤5] | [≤5] |
| Sensing for the use case in Smart Grid | [≤5] | [≤5] |

In the use case of railway intrusion detection, the value of missed detection is updated as ≤2% and the value of false alarm is updated as ≤2%.

3. Conclusions

It is proposed to update the KPI value for this UC.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 V0.3.0.

* * * First Change * * * *

5.7.6	Potential New Requirements needed to support the use case

[PR. 5.7.6 - 001] Subject to operator policy, the 5G system shall enable the core network to collect and aggregate sensing measurement data from base stations.

[PR. 5.7.6 - 002] Subject to operator policy, the 5G system shall enable the core network to expose a suitable API to provide the information regarding sensing results to authorized third parties.

[PR. 5.7.6 - 002] The 5G system shall be able to support the following KPIs:

Table 5.7.6-1	Performance requirements of sensing results for railway intrusion detection

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Intrusion detection on a railway | Outdoor (Along railway) | 95 | ≤1.5 | N/A | N/A | N/A | N/A | N/A | ˂1500 | ≤ 0.1 | ≤2 | ≤2 |
| NOTE: The terms in Table 5.7.6-1 are found in Section 3.1. |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *