3GPP TSG SA WG 1 Meeting #101 	S1-230178

Athens, Greece, 20 - 24 February 2023

Source:	Huawei

pCR Title:	Update of Clause 5.13

Draft Spec:	3GPP TR 22.837 v0.3.0

Agenda item:	7.2

Document for:	Approval

Contact:	< Fangyuan Zhu (zhufangyuan@huawei.com)>

Abstract: This document proposes to update Clause 5.13.

1. Introduction

It is proposed to update the Clause 5.13: Use case on sensing for UAV intrusion detection.

2. Reason for Change

The logistics industry is growing quickly in introducing the UAV to deliver the cargo in our daily lives. But the large-scale UAVs cannot fly safely and efficiently at low altitudes due to the complex geographic environments. To promote the development of commercial UAV industry, the local governments and airspace regulatory authorities start to plan the low-altitude public flight routes as a safe and efficient infrastructure for UAVs.

Compared to the wide no-fly zone (e.g. airport, military base), the flight route protection area of UAV has a limited range and space. Therefore, the sensing accuracy for intrusion detection in case the UAV flies into the flight route protection area is much strict.

3. Conclusions

It is proposed to add the flight route protection area intrusion detection into Clause 5.13.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 V0.2.0.

* * * First Change * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	W. Favoreel, "Pedestrian sensing for increased traffic safety and efficiency at signalized intersections," 2011 8th IEEE International Conference on Advanced Video and Signal Based Surveillance (AVSS), 2011, pp. 539-542, doi: 10.1109/AVSS.2011.6027406.

[3]	Advances in Wildlife Crossing Technologies: https://highways.dot.gov/public-roads/septoct-2009/advances-wildlife-crossing-technologies.

[4]	Protection Detection: Making Roads Safe for Drivers and Wildlife: https://onlinepubs.trb.org/onlinepubs/webinars/201118.pdf.

[5]	F. Liu et al., "Integrated Sensing and Communications: Towards Dual-functional Wireless Networks for 6G and Beyond," in IEEE Journal on Selected Areas in Communications, doi: 10.1109/JSAC.2022.3156632.

[6]	T. S. Rappaport, G. R. MacCartney, M. K. Samimi and S. Sun, "Wideband Millimeter-Wave Propagation Measurements and Channel Models for Future Wireless Communication System Design," in IEEE Transactions on Communications, vol. 63, no. 9, pp. 3029-3056, Sept. 2015, doi: 10.1109/TCOMM.2015.2434384.

[7]	C. Han, Y. Bi, S. Duan and G. Lu, "Rain Rate Retrieval Test From 25-GHz, 28-GHz, and 38-GHz Millimeter-Wave Link Measurement in Beijing," in IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing, vol. 12, no. 8, pp. 2835-2847, Aug. 2019, doi: 10.1109/JSTARS.2019.2918507.

[8]	IEEE 802.11-18/0611r16: “Wireless LANs, WiFi Sensing Uses Cases”

[9]	TEM STANDARDS TEM STANDARDS AND RECOMMENDED PRACTICE: https://unece.org/fileadmin/DAM/trans/main/tem/temdocs/TEM-Std-Ed3.pdf

[11]	3GPP TR 22.856, "Localized Mobile Metaverse Services".

[13]	“Velodyne™ LiDAR VPL-16 User Manual,” 63-9243 Rev. E, Velodyne™ LiDAR, https://velodynelidar.com/wp-content/uploads/2019/12/63-9243-Rev-E-VLP-16-User-Manual.pdf

[14] 		Liu, A., Huang, Z., Li, M., Wan, Y., Li, W., Han, T.X., Liu, C., Du, R., Tan, D.K.P., Lu, J. and Shen, Y., 2022. A survey on fundamental limits of integrated sensing and communication. IEEE Communications Surveys & Tutorials, 24(2), pp.994-1034.

[15] 		https://medium.com/desn325-emergentdesign/s-l-a-m-and-optical-tracking-for-xr-cfabb7dd536f

[16] 		Dwivedi, S., Shreevastav, R., Munier, F., Nygren, J., Siomina, I., Lyazidi, Y., Shrestha, D., Lindmark, G., Ernström, P., Stare, E. and Razavi, S.M., 2021. Positioning in 5G networks. IEEE Communications Magazine, 59(11), pp.38-44.

[17]	T. Murakami et al, “Wildlife Detection System Using Wireless LAN Signal,” in NTT Technical Review vol.17, No.6, pp. 45-48, June 20019, https://www.ntt-review.jp/archive/ntttechnical.php?contents=ntr201906fa13.pdf&mode=show_pdf

[18]	Eradication of elephant mortality and injury due to railway accidents through automatic tracking and alert system in IEEE Conference Publication, IEEE Xplore

[19]	Impact of wild animals (deer and bears) on train operations 210616_KO_Animal2.pdf (jrhokkaido.co.jp) (in Japanese) [z]	Rail Industry Safety Induction Handbook: https://railsafe.org.au/__data/assets/pdf_file/0009/32022/Rail-Industry-Safety-Induction-RISI-Handbook-V5.1.pdf

[21]	Society of Automotive Engineers (SAE), “Taxonomy and definition for terms related to Driving automation systems for on-Road Motor Vehicles”, https://www.sae.org/standards/content/j3016_202104/

[22]	Census of Fatal Occupational Injuries Summary, 2020, https://www.bls.gov/news.release/cfoi.nr0.htm

[23]	Javed MA, Muram FU, Hansson H, Punnekkat S, Thane H. Towards dynamic safety assurance for Industry 4.0. Journal of Systems Architecture. 2021 Mar 1; 114:101914.

[24]	American National Standards Institute/Industrial Truck Safety Development Foundation, Safety standard for driverless, automatic guided industrial vehicles and automated functions of manned industrial vehicles, December 2019, 2019, [Online] http://www.itsdf.org.

[25]	Moore, Erik George, "Radar Detection, Tracking and Identification for UAV Sense and Avoid Applications" (2019). Electronic Theses and Dissertations. 1544.

[26] 	I. Sobron, I. Landa, I. Eizmendi and M. Velez, "Adaptive TOA Estimation with Imperfect LOS and NLOS Knowledge in UWB Positioning Systems," 2020 IEEE SENSORS, 2020, pp. 1-4. available at: https://sci-hub.wf/10.1109/sensors47125.2020.9278589

[27] 	L. Wang, Z. Zhang, X. Di and J. Tian, "A Roadside Camera-Radar Sensing Fusion System for Intelligent Transportation," 2020 17th European Radar Conference (EuRAD), 2021, pp. 282-285.

[28]	5GAA_White_Paper_C-V2X Use Cases Volume II: Examples and Service Level Requirements

[29] 	https://www.ieee802.org/11/Reports/tgbf_update.htm

[30]	https://mentor.ieee.org/802.11/dcn/20/11-20-1712-02-00bf-wifi-sensing-use-cases.xlsx

[31]	X. Liu, J. Cao, S. Tang and J. Wen, "Wi-Sleep: Contactless Sleep Monitoring via WiFi Signals," 2014 IEEE Real-Time Systems Symposium, 2014, pp. 346-355, doi: 10.1109/RTSS.2014.30.

[32]	Chen V C. The micro-Doppler effect in radar. Artech house, 2019.

[33]	3GPP TS 22.261: “Service requirements for the 5G system”.

[34]	A. Chebrolu, "FallWatch: A Novel Approach for Through-Wall Fall Detection in Real-Time for the Elderly Using Artificial Intelligence", 2021 Third International Conference on Transdisciplinary AI (TransAI), 2021, pp. 57-63, doi: 10.1109/TransAI51903.2021.00018, https://ieeexplore.ieee.org/document/9565618

[35]	B. A. Alsaify et al., “A CSI-Based Multi-Environment Human Activity Recognition Framework” Applied Sciences 12, no. 2: 930, 2022. https://doi.org/10.3390/app12020930

[36]	U. Saeed U et al., "Discrete Human Activity Recognition and Fall Detection by Combining FMCW RADAR Data of Heterogeneous Environments for Independent Assistive Living", Electronics 10(18):2237, 2021. https://doi.org/10.3390/electronics10182237

[37]	C. Dou, H. Huan, "Full Respiration Rate Monitoring Exploiting Doppler Information with Commodity Wi-Fi Devices". Sensors 21, 3505, 2021. https://doi.org/10.3390/s21103505

[38]	J. Pu, H. Zhang, "RF-Heartbeat: Robust and Contactless Heartbeat Monitoring Based on FMCW Radar", 2021. TechRxiv Preprint. https://doi.org/10.36227/techrxiv.15021645.v2

[39] 	H. V. Habi and H. Messer, “Recurrent Neural Network for Rain Estimation Using Commercial Microwave Links,” in IEEE Transactions on Geoscience and Remote Sensing, vol. 59, no. 5, pp. 3672-3681, May 2021, doi: 10.1109/TGRS.2020.3010305.[40]                    Roberto Opromolla, etc., “Perspectives and Sensing Concepts for Small UAS Sense and Avoid”, 2018 IEEE/AIAA 37th Digital Avionics Systems Conference (DASC)

[41]	https://www.rwjbh.org/trinitas-regional-medical-center/treatment-care/sleep-disorders/sleep-apnea/

[42]	https://my.clevelandclinic.org/health/articles/10881-vital-signs

[43]	3GPP TR 22.855, "Study on Ranging-based Services".

[44]	Guoxuan Chi, et. al., "Wi-Drone: Wi-Fi-based 6-DoF Tracking for Indoor Drone Flight Control", MobiSys 22, Association for Computing Machinery, 2022.

[45]	Report on Automated Valet Parking: technology assessment and use case implementation description – 5G Automotive Association (5gaa.org). https://5gaa.org/news/report-on-automated-valet-parking-technology-assessment-and-use-case-implementation-description/

[46]	Nie Y B , Zhang L . Main amendments to Working Safety Regulation of State Grid Company(Dynamical Part for Hydrodynamic Power Plant)[J]. East China Electric Power, 2008.

[47] 	Giuseppe Fragapane, René de Koster, Fabio Sgarbossa, Jan Ola Strandhagen, Planning and control of autonomous mobile robots for intralogistics: Literature review and research agenda, European Journal of Operational Research, Volume 294, Issue 2,2021, Pages 405-426

[48]	Li S, Li X, Lv Q, et al. WiFit: Ubiquitous bodyweight exercise monitoring with commodity wi-fi devices, 2018 IEEE SmartWorld, Ubiquitous Intelligence & Computing, Advanced & Trusted Computing, Scalable Computing & Communications, Cloud & Big Data Computing, Internet of People and Smart City Innovation, IEEE, 2018: 530-537.

[49]	https://www.synopsys.com/automotive/autonomous-driving-levels.html

[x]	Civil Aviation Administration of China, “Route design specification of the light-small unmanned aircraft system for urban logistics”, 2022. http://www.caac.gov.cn/XXGK/XXGK/BZGF/HYBZ/202208/P020220811600885528858.pdf

* * * Second Change * * *

5.13.	Use case on sensing for UAV intrusion detection

5.13.1	Description

UAV industry is developing quickly around the world with the widely usages in various scenarios such as aerial photography, police force, urban management, agriculture, geology, meteorology, electric power, emergency rescue and disaster relief, etc. Especially for the smart city in future, a large number of UAVs will be used to improve the quality of our daily life including industrial inspection, public security patrol, cargo transportation, live broadcast and so on. However, this also brings big challenges on UAV supervision due to the following reasons:

1) Low-altitude UAVs have characteristics as large number, small size, wide flying zone, widely used to execute complex and diverse tasks, which makes UAV supervision very difficult if only using the traditional radar system.

2) Non-cooperative UAVs could intrude some no-fly zone (e.g. airport, military base) intentionally or unintentionally which would lead to serious consequences, e.g. exposing private information using the camera, blocking other UAV traffic on the flying route.

5G radio signals can be used to provide wireless access for communication, meanwhile the 5G radio signals can also be used to generate sensing data for object detection e.g. sense presence or proximity of UAVs illegal flying in a specific area. 5G System could provide sensing service by processing sensing data and output sensing information (e.g. relative position, altitude, distance, velocity, direction).  In this case, 5G System could be used for sensing the UAV intrusion in the scenarios of UAV illegal flying in restricted area include light rail, airports, government facilities, research institutes, high-speed railway stations, temporary performance venue and other permanent or temporary restricted areas.

Furthermore, considering that the UAV entering the restricted area is illegal and the UAV itself even could be illegal, this kind of sensing operation doesn’t require the cooperation of the UAV. That means the UAV may be unaware of the sensing operation.  When multiple UAVs appear in the same restricted area, the 5G system can sense presence or proximity of multiple UAVs illegal flying at the same time.

Figure 5.13.1-1 UAV collision risks at light rail

Compared to the wide no-fly zone (e.g. airport, military base), the flight route protection area of UAV has a limited range and space. If the illegal and uncollaborated UAVs fly into the flight route protection area intentionally or unintentionally, it would cause severe accidents. Normally the cross section of the route protection area is rectangular in size, and the cross section size is determined based on the horizontal yaw tolerance X and vertical yaw tolerance Y of UAV in the air. In GNSS mode, the UAV's flight route accuracy can be controlled within 10 m in the horizontal and 5 m in the vertical [x]. The horizontal width of the cross section of the protection area is set to 4X (i.e. 40m). The vertical width of the cross section of the protection area is set to 4Y (i.e. 20m) [x]. Therefore, the typical size of the cross section of the horizontal protection area is 40m x 20m. When another UAV flying at the outermost side of protection area heads towards the protection area, it means the flight routes intrusion happens.

Figure 5.13.1-2 UAV flight routes protection area

5.13.2	Pre-conditions

The UAVs owned by the logistics operator ‘YY’ will take off and fly. The logistics operator ‘YY’ requests UTM to manage potential illegal intrusion into the UAV flight routes protection area. The logistics operator ‘YY’ has provided the information of flight routes protection area to the UTM.

Network operator ‘MM’ provides 5G sensing service for the park, flight routes protection area of the logistics UAVand the light rail area with its 5G network covering the park, flight routes protection area of the logistics UAV and the light rail track. ‘MM’ can make use of wireless base stations to sense the airspace within their coverage area and report the sensing information to the USS/UTM as defined in TS 23.256 clause 3.1.

The Light rail operator ‘XX’ uses a UTM to management potential UAV illegal intrusion along the light rail tracks. ‘XX’ has provided its restricted area information to the UTM.

There is a need to hold a ceremony with high security requirement in the park temporarily, turning the park in a restricted area where UAVs are not allowed to enter. The administrator has a subscription for UAV prevention service from the USS/UTM.

The UTM uses ‘5G Sensing Service’ provided by 5G network Operator ‘MM’ to detect potential UAV illegal intrusion for above scenarios.

The UTM requests that once a UAV is detected that its distance from the border of the restricted area is less than 10m, the 5G system should report the event to the UTM.

The Network operator ‘MM’ can configure energy consumption sensing mode with different sensing period, e.g. operate sensing one time per 50 seconds, per 10 seconds, per second etc. And in emergency condition, the 5G system can provide continuously sensing service according to the UTM’s request.

The light rail works from 5:30 am to 23:00 pm every day.

5.13.3	Service Flows

Figure 5.13.3-1: Sensing for UAV intrusion detection

The 5G system periodically senses the restricted area whether there are UAVs flying into the restricted area border for both the Park, flight routes protection area of the logistics UAV and the light rail area.

There are four UAVs (A, B, C, D) flying around the restricted areas.

When UAV A flying near the Park is detected and closely tracked with required accuracy in the sensing area, the 5G system reports the sensing results to the UTM in real time and begins continuously sensing.

When UAV B and UAV C flying near the light rail are detected, and closely tracked with required accuracy in the sensing area, the 5G system reports the sensing results to the UTM in real time and continuously senses.

When UAV D flying into the flight routes protection area of the logistics UAV is detected, and closely tracked with required accuracy in the sensing area, the 5G system reports the sensing results to the UTM in real time and continuously senses.

To reduce energy consumption, the 5G system will notify the UTM that the 5G system cannot detect any UAVs illegal flying after a time period which is requested by the UTM. After that, the 5G system stops continuously sensing and begins periodically sensing operation according to the Network Operator’s policy.

The USS/UTM could trigger to send warning messages/notices to UAV controller based on analytical results based on the sensing information from the mobile network. Alternatively, the USS/UTM will trigger UAV countermeasures to prevent the UAV from flying in the no-fly area or flight routes protection area of the logistics UAV.

When the ceremony has been finished or the logistics UAV lands, the 5G system would stop sensing operation based on the request from UTM. And when the light rail stops operation between 23:00 pm to 5:00am next morning, the 5G system stops sensing operation to save energy.

5.13.4	Post-conditions

The mobile network can provide sensing service for UAV intrusion detection with high quality and continuity, to improve the accuracy and efficiency of public safety supervision and management.

USS/UTM interacts with the mobile network for sensing service and perform UAV intrusion detection based on the sensing information exposed by network.

5.13.5	Existing features partly or fully covering the use case functionality

None.

5.13.6	Potential New Requirements needed to support the use case

[P.R 5.13.6-001] The 5G system shall be able to provide a sensing service by using base stations to collect sensing measurements.

[P.R 5.13.6-002] The base station shall be able to sense a target object by obtaining sensing measurement without active involvement of the target object.

[P.R 5.13.6-003] The 5G system shall provide mechanisms for an operator to transport sensing data from base station towards the core network.

[P.R 5.13.6-004] Based on operator’s policy and subject to regulatory requirements, the 5G system shall be able to provide a mechanism for a trusted 3rd party to request the sensing service and based on the request, the base station shall be able to operate sensing periodically or continuously in certain location area for a certain amount of time.

[P.R 5.13.6-005] Based on operator’s policy and subject to regulatory requirements, the 5G system shall be able to periodically expose sensing results to a trusted 3rd party application.

[P.R 5.13.6-006] The 5G system shall provide a mechanism controllable by the operator, according to a business agreement, to report sensing result to a trusted 3rd party about a target object and multiple target objects when specific conditions are met.

NOTE: These conditions could be the target object distance from the restricted area border less than 10m or entering restricted area.

[P.R 5.13.6-007] The 5G system shall be able to support the activation and deactivation of the sensing service according to operator’s policy.

[P.R 5.13.6-008] The 5G system shall be able to provide a mechanism for network operator to configure and adjust sensing operation (e.g. authorization, sensing area, sensing operation period and sensing operation time window etc.) based on request from a trusted 3rd party.

[P.R 5.13.6-009] The 5G system shall be able to provide sensing with following KPIs.

Table 5.13.6-1	Performance requirements of sensing results for UAV intrusion detection

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| UAV intrusion detection | Outdoor | 95 | ≤10 NOTE 2 | No-fly zone intrusion :≤10; Flight routes protection area intrusion : ≤5 NOTE 2 | N/A | N/A | 10 | [5] | [≤5000] | [≤0.1Hz] | [≤5] | [≤5] |
| NOTE 1: The terms in Table 5.13.6-1 are found in Section 3.1. NOTE 2: In case of the flight routes protection area intrusion detection, the typical size of the cross section of the horizontal protection area is 40m x 20m [x]. |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *