3GPP TSG-SA WG1 Meeting #101	S1-230211

Athens, Greece, 20 February – 24 February 2023	(revision of S1-22xxxx)

Title:	Annex for Communication Modes of Low Power Consumption

Agenda Item:	7.2

Source:	CMCC

Contact:	Pengtai Qin (qinpengtai@chinamobile.com)

Yuan Li (liyuanyjy@chinamobile.com)

Yulu Zhang（zhangyulu@chinamobile.com）

______________________________________________________________

Abstract: In the current version of TR22.840, availability scenarios, ambient power and energy storage methods, harvesting source selection considerations have been discussed firmly in Annex. However, due to the stringent requirements for complexity and power consumption, the communication modes of Ambient IoT devices might be quite different with previously defined 3GPP IoT devices (e.g., NB-IoT/eMTC devices), thus, this contribution proposes an introduction of communication modes of low power consumption, aiming to give a reference for Ambient IoT communication.

1. Introduction

Generally, there are two mainly communication modes of wireless communication equipment, which are active communication modes and passive communication modes. At present, 3GPP IoT devices such as NB IoT devices, use active communication modes to communicate with a network, which means the devices can transmit data over self-generated carrier waves. However, taking the advantage of no internal RF signal is need to generate, passive communication mode e. g., backscatter communication is a promising energy efficient communication technology for the IoT devices which has been studied in many papers [x1-4] and maybe it is a candidate technique of Ambient IoT system for reducing power consumption. Because of this reason, a discussion of low power consumption communication modes is given.

2. Reason for Change

An introduction of low power consumption communication modes would help to propose the following technical specifications of Ambient IoT.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840-1.0.0.

* * * Start of First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	International Energy Agency (IEA), "World Energy Outlook 2020". Available online: https://www.iea.org/reports/world-energy-outlook-2020

[3] 	Motlagh, N.H.; Mohammadrezaei, M.; Hunt, J.; Zakeri, B. "Internet of Things (IoT) and the Energy Sector." Energies 2020, 13, 494.

[4] 	Pereira F., Correia R., Carvalho N.B. "Passive Sensors for Long Duration Internet of Things Networks." Sensors 2017;17:2268. doi: 10.3390/s17102268.

[5] 	https://www.gs1.org/standards/rfid.

[6]	3GPP TS 22.368: "Service requirements for Machine-Type Communications (MTC)".

[7]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.011: “Service accessibility”.

[10]	5G for smart manufacturing: https://www.gsma.com/iot/wp-content/uploads/2020/04/2020-04_GSMA_SmartManufacturing_Insights_On_How_5G_IoT_Can_Transform_Industry.pdf

[11] 	 https://forcetechnology.com/en/articles/batteryless-electronics-energy-harvesting

[12] 	R. Ding and B. Xing, "Comparative Research on the Way of Energy Harvesting of the Wireless Sensor Network Nodes," 2013 6th International Conference on Intelligent Networks and Intelligent Systems (ICINIS), 2013

[13] 	P. D. Hung et al., "A Self-Powered Wireless Gas Sensor Node Based on Photovoltaic Energy Harvesting," 2021 Symposium on VLSI Circuits, 2021.

[14] 	GS1: "EPC Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard (Release 2.1) "

[15]                      EPC Tag Data standards version 1.1 Rev.1.24

[16] 	Noghabaei S M, Radin R L, Savaria Y, et al. A high-efficiency ultra-low-power CMOS rectifier for RF energy harvesting applications[C]//2018 IEEE International Symposium on Circuits and Systems (ISCAS). IEEE, 2018: 1-4

[17] 	Wu Z, Zhao Y, Sun Y, et al. A Self-Bias Rectifier with 27.6% PCE at-30dBm for RF Energy Harvesting[C]//2021 IEEE International Symposium on Circuits and Systems (ISCAS). IEEE, 2021: 1-5.

[18] 	Valenta C R, Durgin G D. Harvesting wireless power: Survey of energy-harvester conversion efficiency in far-field, wireless power transfer systems[J]. IEEE Mi crowave Magazine, 2014, 15(4): 108-120.

[18] 	Kim S, Vyas R, Bito J, et al. Ambient RF energy-harvesting technologies for self-sustainable standalone wireless sensor platforms[J]. Proceedings of the IEEE, 2014, 102(11): 1649-1666.

[20] 	Green M, Dunlop E, Hohl‐Ebinger J, et al. Solar cell efficiency tables (version 57)[J]. Progress in photovoltaics: research and applications, 2021, 29(1): 3-15.

[21] 	Mishu M K, Rokonuzzaman M, Pasupuleti J, et al. An adaptive TE-PV hybrid energy harvesting system for self-powered iot sensor applications[J]. Sensors, 2021, 21(8): 2604.

[22]	Prauzek M, Konecny J, Borova M, et al. Energy harvesting sources, storage devices and system topologies for environmental wireless sensor networks: A review[J]. Sensors, 2018, 18(8): 2446.

[23] 	Kim S, Vyas R, Bito J, et al. Ambient RF energy-harvesting technologies for self-sustainable standalone wireless sensor platforms[J]. Proceedings of the IEEE, 2014, 102(11): 1649-1666.

[24] 	H. S. Kim, J. -H. Kim, and J. Kim, ‘‘A review of piezoelectric energy harvesting based on vibration,’’ Int. J. Precision Eng. Manuf., vol. 12, no. 6, pp. 1129–1141, Dec. 2011.

[25] 	Hoang T, Ferin G, Bantignies C, et al. Aging assessment of piezoelectric energy harvester using electrical loads[C]//Journal of Physics: Conference Series. IOP Publishing, 2019, 1407(1): 012078.

[26] 	https://www.usda.gov/foodwaste/faqs

[27] 	https://www.fda.gov/food/new-era-smarter-food-safety#:~:text=Welcome%20to%20the%20New%20Era,reducing%20the%20number%20of%20illnesses

[28] 	Alam AU, Rathi P, Beshai H, Sarabha GK, Deen MJ. Fruit Quality Monitoring with Smart Packaging. Sensors (Basel). 2021 Feb 22;21(4):1509. doi: 10.3390/s21041509. PMID: 33671571; PMCID: PMC7926787.

[29] 	Taoukis, Petros & Nychas, George-John. (2000). Use of time-temperature integrators and predictive modeling for shelf life control of chilled fish under dynamic storage conditions. International journal of food microbiology. 53. 21-31. 10.1016/S0168-1605(99)00142-7.

[30] 	Mishu, M.K., Rokonuzzaman, M., Pasupuleti, J., Shakeri, M., Rahman, K.S., Hamid, F.A., Tiong, S.K. and Amin, N., 2020. Prospective efficient ambient energy harvesting sources for iot-equipped sensor applications. Electronics, 9(9), p.1345.

[31] 	Bagchi, S., Abdelzaher, T.F., Govindan, R., Shenoy, P., Atrey, A., Ghosh, P. and Xu, R., 2020. New frontiers in IoT: Networking, systems, reliability, and security challenges. IEEE Internet of Things Journal, 7(12), pp.11330-11346.

[32] 	O’Neill, V. and Soh, B., 2022. Improving Fault Tolerance and Reliability of Heterogeneous Multi-Agent IoT Systems Using Intelligence Transfer. Electronics, 11(17), p.2724.

[33] 	Katanbaf, M., Jain, V. and Smith, J.R., 2020. Relacks: Reliable backscatter communication in indoor environments. Proceedings of the ACM on Interactive, Mobile, Wearable and Ubiquitous Technologies, 4(2), pp.1-24.

[34] 	3GPP TR 23.501 release-17 - https://www.3gpp.org/ftp/Specs/archive/23_series/23.501/23501-h50.zip

[35] 	3GPP TR 38.300 release-17 - https://www.3gpp.org/ftp/Specs/archive/38_series/38.300/38300-h10.zip

[36] 	https://www.mokosmart.com/use-iot-fire-detector-sensor/

[37]  	https://www.5gamericas.org/wpcontent/uploads/2019/07/5G_Americas_URLLLC_White_Paper_ Final__updateJW.pdf

[38] 	TS 22.261 https://www.3gpp.org/ftp/Specs/archive/22_series/22.261/22261-j00.zip

[39] 	 	    TR 22.866: "enhanced Relays for Energy Efficiency and Extensive Coverage".

[40] 		     		Global RFID Sensor Market - Global Industry Analysis, Size, Share, Growth, Trends

and Forecast 2021-2031, Transparency Market Research, June 2021

[41]		     		Sensors: Technologies and Global Markets (

https://www.bccresearch.com/market-

research/instrumentation-and-sensors/sensors-technologies-markets-report.html),BCC

Research

[42]		     		Berckmans, D. Precision livestock farming technologies for welfare management in

intensive livestock systems. Rev. Sci. Tech. Off. Int. Epiz. 2014, 33, 189–196.

[43]	Precision Agriculture for Crop and Livestock Farming - Brief Review, Animals 2021

[44]                	Journal of Dairy Science - Comparison of 2 systems of pasture allocation on milking intervals and total daily milk yield of dairy cows in a pasture-based automatic milking system, American Dairy Science Association, 2013

[45]	Grazing and automation, Proceedings 4th Meeting European Grassland Federation WG “Grazing”, 2015

[46] 	Grazing Methods, UK Agriculture Food and Environment, https://grazer.ca.uky.edu/content/grazing-methods-which-one-you#:~:text=Strip%20Grazing%20%E2%80%93%20This%20technique%20involves,a%20new%20allocation%20of%20forage.

[47]                	Nora O’Donovan, https://www.independent.ie/business/farming/dairy/dairy-    advice/putting-a-size-on-paddocks-30447793.html

[48]	A. van den Pol-van Dasselaar, P. W. Blokland, et al, Beweidbare oppervlakte en weidegang op melkveebedrijven in Nederland (Pasture area and grazing on dairy farms in the Netherlands), Livestock Research Wageningen, 2015

[49]	Laura Paine, A Summary of Dairy Grazing Practices in Wisconsin

[50]	R. J. Rendell, Designing Paddocks on an Irrigated Dairy Farm, Victoria, Australia

[51]	The EU pig meat sector, European Parliament

[52]	http://www.bestgenetics.com.cn/EN/PigFarm1

[53]		    			Lisette. E. van der Zande, et al, Individual detection and tracking of group housed pigs

in their home pen using computer vision, Front. Anim. Sci. 2:669312. doi:

10.3389/fanim.2021.669312, April 2021

[54]	Carol Souza de Silva, Wageningen University, the impact of Pen Size and Stocking Density on behaviour and welfare of growing pigs

[55]	Xue Li, Xia Xiong, et al, Effects of stocking density on growth performance, blood parameters and immunity of growing pigs, December, 2020

[56] 	https://www.concreteconstruction.net/projects/infrastructure/so-many-manholes-so-little-time_o#:~:text=U.S.%20EPA%20estimates%20the%20number,from%20100%20to%20500%20feet.

[57]	http://news.cjn.cn/24hour/wh24/201101/t1276749.htm

[58] 	https://www.dibblecorp.com/insights/manhole-infrastructure-assessments/

[59] 	https://www.francebleu.fr/infos/faits-divers-justice/un-homme-retrouve-mort-dans-une-bouche-d-egout-a-saint-nazaire-1666456765

[60]	https://guancha.gmw.cn/2022-05/16/content_35739227.htm

[61]	Intelligent manhole cover national standard, GB/T 41401, 2022, the Ministry of Housing and Urban-Rural Development of the People's Republic of China

[62]	MEMS gauge pressure sensor NSPGD1 Novosense, liquid level measurement, https://www.novosns.com/Public/Uploads/uploadfile/files/20220726/-897.pdf

[63]	Tilt sensor, Clinometer LE-60, http://www.tuoluoyi.com/upfile/201610/2016100853701905.pdf

[64]	https://www.analog.com/media/en/technical-documentation/data-sheets/adxl375.pdf

[65] 	Sung-Pil CHANG, Jinkyo CHOO, Values of Bridge in the Formation of Cities

[66]	https://www.thepaper.cn/newsDetail_forward_4643273

[67] 	https://news.sina.cn/gn/2021-12-19/detail-ikyamrmy9954352.d.html

[68] 	https://www.nytimes.com/2020/12/22/world/europe/genoa-bridge-collapse.html

[69]	https://www.theguardian.com/world/2022/oct/31/india-bridge-collapse-death-toll-rises-rescue-efforts-continue

[70]	Role of the Local Government in Monitoring and Maintenance Bridges, IAPA Proceedings Conference, 2019

[71]	https://elastisense.com/structural-health-monitoring-bridges

[72]	GB/T 39559.2-2020 National Specification for operational monitoring of urban rail transit facilities – Part 2: Bridge

[73]	Tilt sensor, Clinometer LE-60, http://www.tuoluoyi.com/upfile/201610/2016100853701905.pdf

[74]	https://www.analog.com/media/en/technical-documentation/data-sheets/adxl375.pdf

[75]	Big data analysis for bridges health monitoring, Tongji University, 2017

[76]	Beijing Dongsha bridge safe operation monitoring, http://www.ccea.zju.edu.cn/_upload/article/files/64/e6/a824c7d04932af200750fbd9e690/fb0a5e9c-a1e7-47b8-bdb4-82b584d1c52f.pdf

[77]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

[x1] Yao C, Liu Y, Wei X, et al, Backscatter technologies and the future of internet of things: Challenges and opportunities, Intelligent and Converged Networks, vol. 1, no. 2, pp. 170-180, 2020.

[x2] Liu W, Huang K, Zhou X, et al. Next Generation Backscatter Communication: Systems, Techniques and Applications, 10.48550/arXiv.1701.07588, 2017.

[x3] Tang X, Xie G. and Cui Y, Self-Sustainable Long-Range Backscattering Communication Using RF Energy Harvesting, IEEE Internet of Things Journal, 2021, 8(17), 13737-13749.

[x4] Mostafa A. E and Wong V. W. S, Transmit or Backscatter: Communication Mode Selection for Narrowband IoT Systems, IEEE Transactions on Vehicular Technology, 2022, 71(5), 5477-5491.

[x5] 3GPP Release 13: Cellular system support for ultra-low complexity and low throughput Internet of Things (CIoT)

[x6] Lauridsen M, Krigslund R, Rohr M, et al. An Empirical NB-IoT Power Consumption Model for Battery Lifetime Estimation, 2018:1-5.

[x7] Jorke P, Falkenberg R, Wietfeld C. Power Consumption Analysis of NB-IoT and eMTC in Challenging Smart City Environments, 2018 IEEE Globecom Workshops (GC Wkshps). IEEE, 2019.

[x8] Bouguera T, Diouris J F, Chaillout J J , et al. Energy consumption modeling for communicating sensors using LoRa technology, 2018 IEEE Conference on Antenna Measurements & Applications (CAMA). IEEE, 2018.

[x9] Cunha D.d.O., Costa L.H.M.K., Duarte O.C.M.B. Analyzing the Energy Consumption of IEEE 802.11 Ad Hoc Networks, Belding-Royer, E.M., Al Agha, K., Pujolle, G. (eds) Mobile and Wireless Communication Networks, 2004.

[x10] Kazeem O O, Akintade O O, Kehinde L O. Comparative Study of Communication Interfaces for Sensors and Actuators in the Cloud of Internet of Things, The Internet of Things. Scientific & Academic Publishing, 2017.

[x11] Man A, Zhang E S, Chan H T, et al. Design and Implementation of a Low-power Baseband-system for RFID Tag, IEEE International Symposium on Circuits & Systems. IEEE, 2007.

[x12] Ashry A, Sharaf K. Ultra low power UHF RFID tag in 0.13 μm CMOS, Conference on Microelectronics. IEEE, 2007.

[x13] Yeager D, Zhang F, Zarrasvand A, et al. A 9.2A gen 2 compatible UHF RFID sensing tag with 12dBm Sensitivity and 1.25Vrms input-referred noise floor, Solid-State Circuits Conference Digest of Technical Papers (ISSCC), 2010 IEEE International. IEEE, 2010.

[x14] Karthaus U and Fischer M, Fully integrated passive UHF RFID transponder IC with 16.7-μW minimum RF input power, IEEE J. SolidState Circuits, 2003, 38(10), 1602–1608.

[x15] He Z, Zhu X, Xian L, et al. Design and implementation of a 2.62 uW low-power baseband processor for Passive UHF RFID Tags, International Conference on Advanced Communication Technology. IEEE, 2010.

[x16] Rodriguez-Rodriguez J A, Masuch J, Delgado-Restituto M. An EPC class-1 generation-2 baseband processor for passive UHF RFID tag, Research in Microelectronics and Electronics, 2009.

[x17] Marjonen J, Pesonen N, Vermesan O, et al. An 8-bit 10 kS/s 0.18 μm CMOS SAR ADC for RFID applications with sensing capabilities, Norchip. IEEE, 2007:1-4.

[x18] Talla V, Hessar M, Kellogg B, et al. LoRa Backscatter: Enabling The Vision of Ubiquitous Connectivity, arXiv, 2017.

[x19] Kellogg B, VamsiTalla,  Smith J R , et al. PASSIVE WI-FI: Bringing Low Power to Wi-Fi Transmissions, Getmobile Mobile Computing & Communications, 2017, 20(3):38-41.

[x20] Rosenthal J. D, Mandal A, Sathe V, et al, An All-Digital 1 Mbps, 57 pJ/bit Bluetooth Low Energy (BLE) Backscatter ASIC in 65 nm CMOS, 2022 IEEE International Conference on RFID (RFID), Las Vegas, NV, USA, 2022, 109-113

* * * Start of Second Change * * * *

Annex X (Informative): Communication Modes of Low Power Consumption

There are two types of communication modes for wireless low power consumption communication devices, which are active communication mode and passive communication mode.

X.1 Active communication mode

In the active communication mode, the device uses internal power source to transmit data to the transceiver/receiver. Since an active communication device need to generate RF signals, it has many active RF components such as frequency synthesizer, RF modulator, and PA (Power Amplifier), and thus, cause high power consumption.

Many 3GPP defined IoT techniques like NB-IoT, eMTC, etc., support PSM (Power Saving Mode) and eDRX (extended Discontinuous Reception) to reduce power consumption and to extend battery life, but the average power consumption for active communication is still higher than 100 mW. Some examples of power consumption of different active communication devices are listed in Table x-1-1.

Table x-1-1 Power Consumption of Different Active Communication Techniques/Devices

| Ref. | Technique/Device | Center Freq. | Data Rate | Power Consumption for Comm. | Note |
| --- | --- | --- | --- | --- | --- |
| [x5] | NB-IoT (Release 13 standard) | 800MHz /900MHz | DL: <26kbps UL: <62kbps | 545 mW for transmit 90 mW for receive 3 mW for idle | Typical assumption (Table 7.2.4.5-1 in TR 45.820 V13.1.0) |
| [x6] | NB-IoT (a commercially available device) | 800MHz | N.A. | 716 mW for transmit 213 mW for receive 21 mW for idle | UL Max. TBS: 1000 bits DL Max. TBS :680 bits |
| [x7] | eMTC | 1.08MHz | 1Mbps | 709.5 mW for transmit 62.5 mW for receive 3.63 mW for idle | Typical IoT smart city use cases |
| [x8] | LoRa (only transmit 32bits sensor data) | 868MHZ | 0.08kbps-1.33kbps | 105 mW for transmit |  |
| [x9] | Wi-Fi (802.11b) | 2.4GHz | 1-11Mbps | 1350 mW for transmit 900 mW for receive 740 mW for idle |  |
| [x10] | Bluetooth (802.15.1) | 2.4GHz | 1Mbps | 128.7 mW for transmit 178.2 mW for receive 39.6 μW for idle |  |

X.2 Passive communication mode

In the passive communication mode, the device is powered by the electromagnetic energy transmitted from the transceiver and therefore, need no internal power source.

Generally, there are two techniques that can achieve passive communication, which are inductive coupling and backscatter. However, inductive coupling relies on the magnetic field of the transceiver, which means that this coupling only occurs in the near-field, thus, the operating range of this technique is usually shorter than 1m. Inductive coupling technology can barely satisfy the potential requirements and KPIs (such as the communication range) have been proposed in Ambient IoT study. Because of the reason above, in this text, we will focus on the other passive communication technology – Backscatter.

A backscatter device modulates and reflects received RF signals to transmit data instead of generating RF signals by itself. The operating range of backscatter is typically from 5m-10m and could be further extend if advance techniques are employed. As a result, backscatter has found many useful applications in practice such as RFID (Radio Frequency Identification) to achieve ultra-low power communication.

Since a backscatter device only transmit data using the incident signal when inquired by backscatter transceivers. It has no active RF components and as a result can be made to have miniature hardware with extremely low power consumption, i.e., not larger than tens of μW. Some examples of power consumption of different passive communication devices are listed in Table x-2-1.

Table x-2-1 Power Consumption of Different Passive Communication Devices

| Ref. | Device Type | Center Freq. | UL Data Rate | Power Consumption for Comm. |
| --- | --- | --- | --- | --- |
| [X11] | RIFD Tag (CMOS 0.18 μm process) | UHF | 40-640kbps | 1 μW |
| [x12] | RIFD Tag (CMOS 0.13 μm process) | 900MHz |  | 1 μW |
| [x13] | RFID Tag (CMOS 0.13 μm process with sensor interface) | 900MHz |  | 1.44 μW |
| [x14] | RFID Tag (CMOS 0.5 μm, Schottky diodes) | 900MHz |  | 2.3 μW |
| [x15] | RIFD Tag (CMOS 0.18 μm process) | 915MHz |  | 2.62 μW |
| [x16] | RFID Tag (CMOS 0.35 μm process) | UHF |  | 2.8 μW |
| [x17] | RFID Tag (CMOS 0.18 μm process with pressure sensor) | UHF |  | 25 μW |
| [x18] | LoRa Backscatter Device | 900MHz | 18bps-37.5kbps | 9.25 μW |
| [x19] | Passive Wi-Fi (802.11b) | 2.4GHz | 1-11Mbps | 14.5−59.2 μW |
| [x20] | Bluetooth Backscatter Tag (CMOS 65 nm process) | 2.4GHz | 1Mbps | 57 μW |

* * * End of Changes * * * *