3GPP TSG-SA WG1 Meeting #101	S1-230212

Athens, Greece, 20 February – 24 February 2023	(revision of S1-22xxxx)

Source:	CMCC

pCR Title:	Consolidated KPI for Ambient IoT

Draft Spec:	3GPP TR 22.840

Agenda item:	7.2

Document for:	Approval

Contact:	     Pengtai Qin (qinpengtai@chinamobile.com)

Yuan Li (liyuanyjy@chinamobile.com)

Yulu Zhang（zhangyulu@chinamobile.com）

Abstract: This contribution proposes the consolidation on KPI for Ambient IoT.

Discussion

According to different information the Ambient IoT devices can collect and transmit, the 28 agreed use cases and 2 agreed traffic scenarios in TR 22.840 v0.3.0. can be put into the following 2 categories for KPI consolidation as a way forward.

- Use case category 1: Basic information transmission

- Use case category 2: Additional information transmission

Explanation and Motivation of use cases categorizations:

- In use case Category 1, an Ambient IoT device only needs to transmit messages which are related to the Ambinet IoT device itself (e.g., the device identification) to the 5G network. By transmitting these messages, many services such as inventory, positioning/ranging, and activation can be provided.

- In use case Category 2, except transmitting basic information in category 1, an Ambient IoT may need to transmit additional information which is collected from external environment (e.g., sensing data generated by the embedded sensor) to the 5G network. By transmitting these messages, sensor service can be provided.

- There are different requirements for these two categories, especially in max. allowed E2E latency and message size. Since different KPI requirements may correspond to different solutions proposed by other 3GPP work groups, thus, to try to clarify different KPIs and to avoid overlaps, we suggest that the KPIs be divided into the two categories as Table 2.

Table 1: Use cases/traffic scenarios index

| Index | Use Case/Traffic Scenario Title |
| --- | --- |
| 1 | Use case on Ambient IoT on automated warehousing |
| 2 | Use case on medical instruments inventory management and positioning |
| 3 | Use Case on Ambient IoT devices in substations in smart grids |
| 4 | Use case on supporting Ambient IoT in Non-Public Network for logistics |
| 5 | Use case on intralogistics in automobile manufacturing |
| 6 | Use case on Ambient IoT sensors in smart homes |
| 7 | Use Case on Ambient IoT for airport terminal / shipping port |
| 8 | Use case on Finding Remote Lost Item |
| 9 | Use case on LCS for Ambient IoT |
| 10 | Use case on Ranging for Ambient IoT |
| 11 | Use case on online modification of medical instruments status |
| 12 | Use case on Ambient IoT service for personal belongings finding |
| 13 | Use case on Ambient IoT for Base Station Machine Room Environmental Supervision |
| 14 | Use case on indoor positioning in shopping centre using Ambient IoT |
| 15 | Use Case on Ambient IoT enablement for smart laundry |
| 16 | Use case on Ambient IoT service for automated supply distribution |
| 17 | Use case on Device Activation and Deactivation |
| 18 | Use case on Fresh Food Supply Chain |
| 19 | Use case on Forest Fire Monitoring using Ambient IoT devices |
| 20 | Use case on Smart Agriculture |
| 21 | Use Case on Ambient IoT for Museum Guid |
| 22 | Use case of smart grazing dairy farming enabled by Ambient IoT |
| 23 | Use case of smart pig farm enabled by Ambient IoT |
| 24 | Use case of smart manhole cover safety monitoring using Ambient IoT |
| 25 | Use case of smart bridge health monitoring using Ambient IoT |
| 26 | Elderly Health Care |
| 27 | Use case on end-to-end logistics |
| 28 | Use case on pressure powered switch |
| 29 | Traffic scenario on flower auction |
| 30 | Traffic Scenario on cow stable |

Table 2: Ambient IoT use case classification

| Use cases Categories | Use cases agreed in TR 22.840 v0.3.0 |
| --- | --- |
| Basic information transmission | 1; 2; 4; 5; 7; 8; 9; 10; 11; 12; 14; 16; 17; 21; 26; 27; 28; 29 |
| Additional information transmission | 3; 6; 13; 15; 18; 19; 20; 22; 23; 24; 25; 30 |

****************Start of Change *************

7.2	Consolidated potential KPIs

Table 7.2.1 KPI for basic information transmission using Ambient IoT

| Indoor/ Outdoor coverage | Index | Scenarios | Max allowed E2E latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | No. 1 | Automated warehousing | 1s |  |  | 100/ 128bits/s | 96/128 bits |  | Indoor- 30m indoors |  | 5~10 km/h |  |  |  | 2~3 m |
|  | No. 5 | Automatic Intralogistics in automobile manufacturing | 100ms |  |  | 1 kbit/s | 96 bits |  | Indoor- 30 meters | 600 000 m2 | 1,5 Million /km2 |  | [3] m |  | [3] m |
|  | No. 9 | Absolute positioning |  |  |  |  |  |  |  |  | Indoor - up to [5 km/h] Outdoor - up to [20 km/h] |  | [10 s] | [95 %] | [10 m] Horizontal Accuracy [3 m] Vertical Accuracy |
|  | No. 10 | Finding Items in a home |  |  |  |  |  | 20 Ambient IoT devices/ (100m2) | Indoor- 10m IC/PC/OOC |  | Static/ Moving | 500ms |  | 95 % | 100 cm 10 degrees |
|  | No. 14 | Parking area/ Shopping area | 0.5 s | 99.9% |  | 1 kbit/s | 96 bits | 2500/ 10000m2 | Indoor- 10 m |  |  |  | 0.5 s | 90% | 3 m |
|  | No. 21 | Museum guide (indoor) | [2] s | 99.9% |  | [ 1 kbit/s] UL | [96] bits | [10,000 /km²] | Indoor- [30] m | [20,000] m² | [3] km/h |  |  | 90% | [3] m |
|  | No. 29 | Container logistics in a flower auction | [<10 s] | [99,99%] |  | [5] kbit/s | [96] bits | [< 1,3] Million/km2 | Indoor- [35] m Indoors | [1 700 000] m2 |  |  |  |  |  |
| Outdoor | No. 7 | Airport Terminal/ Shipping Port | 1 sec | 99% |  |  | 256 bits (UL) | FFS | FFS | 1-10km2 |  |  |  |  |  |
| Indoor/ Outdoor | No. 2 & No. 11 | Medical instrument inventory management and positioning | hundreds ms level | 99% |  | 2kbps | 176bits | 1000 /km2 | Indoor- 50m In Outdoor- 200m |  | Static or walking speed <6km/h | hundreds ms level |  |  |  |
|  | No. 8 | Remote lost item finding | 5s | 99% |  |  | 256 bits | 750 devices /100m2 | Indoors- 10m Outdoor- 100m |  |  |  |  |  |  |
|  | No. 12 | Personal belongings finding | 1s | 99.9% |  | 1kbit/s | 1kbit | Indoor- 5 per 100 m2 Outdoor-10 per 100 m2 | Indoor- 10 m Outdoor- 100m | Indoor- 200 m2 Outdoor-Up to the whole PLMN | Static | 1 per hour |  |  | Indoor- [1-3] m, 90% availability Outdoor- FFS |
|  | No. 16 | Ambient IoT devices for automated supply distribution | 10 s |  |  | 100 bit/s | Typically, <100 bytes | 1,5 Million/km2 |  | 600 000 m2 |  |  |  |  | [3] m (Indoor, 90% confidence level and in horizontal) |
| Undeclared | No. 4 | Non-Public Network for logistics |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | No. 17 | Device Activation and Deactivation |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | No. 26 | Elderly Health Care |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | No. 27 | End-to-end logistics |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | No. 28 | Pressure powered switch |  |  |  |  |  |  |  |  |  |  |  |  |  |

Editor’s Notes: Five use cases (No. 4, No. 17, No. 26, No. 27 and No. 28) are classified as undelared in Table 7.2.1 because no potential key performance requirement is given in these documents at present.

Table 7.2.2 KPI for additional information transmission using Ambient IoT

| Indoor/ Outdoor coverage | Index | Scenarios | Max allowed E2E latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | No. 6 | Ambient IoT devices in smart home | 20 s | 99.9 % |  |  | 8~96bits |  | Indoor- [10-30]m |  | Stationary |  |  |  |  |
|  | No. 13 | BSMR environmental supervision | 1s |  | 99.9% | 1kbit/s |  |  | Indoors- 30m |  |  |  |  |  |  |
|  | No. 23 | Smart livestock farming (Pig barns) | 10 s |  |  | 500 bit/s | Typically, [< 100 bytes] | 850 000 devices / km2 | Indoor- 250 m | 6000 m2 |  | 15 minutes to half an hour |  |  |  |
|  | No. 30 | Cows in dairy stable | 1 s | [99,9%] |  | [< 0.5] kbit/s | [500] bits | [< 1] /km2 | Indoors- [35] m | [1 800] m2 |  |  |  |  |  |
| Outdoor | No. 3 | Remote monitoring of transmission and distribution networks in smart grids | 1 s |  |  | [< 1kbit/s] | Typically [< 100 bytes] | [< 10,000 /km2] | Outdoor-typically [50-200] meters | several km2 up to 100 000 km2] |  |  |  |  |  |
|  | No. 19 | Forest Fire Monitor | 10sec |  | 99.9% with fault tolerance. |  |  | FFS | [150-FFS] meters | FFS |  | FFS |  |  |  |
|  | No. 24 | Smart manhole cover remote monitoring | 10 s - 30 s |  |  | 1 kbit/s | Typically, [< 100 bytes] | 1000 devices / km2 | FFS | City wide including rural areas |  | 15 min |  |  |  |
|  | No. 25 | Smart bridge health monitoring | 10 s |  |  | 1 kbit/s | Typically, [< 100 bytes] | 1000 devices / km2 | FFS | Along the bridge |  | 15 min |  |  |  |
| Indoor/ Outdoor | No. 15 | Ambient IoT devices for smart laundry | 10 s |  |  | 100bit/s | Typically < 100 bytes | 20 / 100m2 |  | several m2 up to 1000 m2 |  |  |  |  |  |
|  | No. 18 | Ambient IoT devices for food supply chain | 1 minute |  |  | 0.12 bit/s | Typically, < 100 bits | 1.5 million devices/ km2 |  | 30,000m2 |  |  |  |  |  |
|  | No. 20 | Smart Agriculture | 1s | 99.9% |  | 1 kbit/s | 1000 bits | 1 per m2 | 30-100m | 500-70000 m2 per greenhouse | Static | 1 hour |  |  |  |

Editor’s Note: Whether a use case/traffic scenario belongs to an indoor, outdoor or indoor/outdoor category is open to be modified because some KPIs (e.g., communication range) in some use case/traffic scenarios are still FFS.

*******************End of Change*****************