3GPP TSG SA WG 1 Meeting #101 	S1-230222

Athens, Greece, 20 - 24 February 2023

Source:	Huawei

pCR Title:	Update of sensing definitions

Draft Spec:	3GPP TR 22.837 v0.3.0

Agenda item:	7.1

Document for:	Approval

Contact:	< Fangyuan Zhu (zhufangyuan@huawei.com)>

Abstract: This document proposes to update sensing definitions.

1. Introduction

It is proposed to update sensing definitions.

2. Reason for Change

In order to better understand the difference between the sensing measurement data and sensing result, it is propose to  differentiate them based on data accessibility as the following:

- The 3GPP sensing measurement data is to be processed inside 5G system.

- The sensing result is intended to be used outside 5G system for sensing service.

In the SID proposal, agreement is to focus on 3GPP based sensing. The non-3GPP sensing “could be” (see SP-220717) considered. Fusion (however one may interpret) is not in the objectives, hence there is no need to define praseology for “sensing fusion” specifcially. This proposal intends to servce the goal to specify clear requiements for 5GS, however, it does not prevent “fusion” either in 5GS or outside of 5GS.

The updated definitions are the following:

3GPP sensing measurement data: data obtained from 5G system about radio/wireless signals impacted (e.g. reflected, refracted, diffracted) by an object or environment of interest for sensing purposes by using 5G Wireless sensing and to be processed inside 5G system.

Non-3GPP sensing measurement data: data obtained from non-3GPP sensors (e.g. video, LiDAR, sonar) directed at an object or environment of interest for sensing purposes.

sensing result: the information derived from 3GPP sensing measurement data and/or Non-3GPP sensing measurement data, which is intended to be used outside 5G system for sensing service.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 V0.3.0.

* * * First Change * * * *

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G Wireless sensing: 5GS feature providing capabilities to get information about characteristics of the environment and/or objects within the environment (e.g. shape, size, speed, location, distances or relative motion between objects, etc) using NR RF signals and, in some cases, previously defined information available in EPC and/or E-UTRA.

Motion rate accuracy describes the closeness of the measured magnitude of the target object’s vibration frequency caused by part(s) of the target object to the true magnitude of the target object’s vibration frequency.

Editor’s Note: terminology of motion rate accuracy needs to be refined.

sensing group: a set of UEs that support sensing operations all of whose location is known and whose sensing measurement data can be collected synchronously.

3GPP sensing measurement data: data obtained from 5G system about radio/wireless signals impacted (e.g. reflected, refracted, diffracted) by an object or environment of interest for sensing purposes by using 5G Wireless sensing and to be processed inside 5G system.

Non-3GPP sensing measurement data: data obtained from non-3GPP sensors (e.g. video, LiDAR, sonar) directed at an object or environment of interest for sensing purposes.

sensing measurement process: process of collecting 3GPP sensing measurement data.

Sensing receiver: A Sensing receiver is an entity that receives the sensing signal which the sensing service will use in its operation. A sensing receiver is an NR RAN node or a UE. A Sensing receiver can be located in the same or different entity as the Sensing transmitter.

sensing result: the information derived from 3GPP sensing measurement data and/or Non-3GPP sensing measurement data, which is intended to be used outside 5G system for sensing service.

NOTE: Examples of sensing result are characteristics of an object or environment, etc.

Sensing service area location:  an area location whether with or without obstacle, the 5G system can provide sensing service with certain quality.

Sensing transmitter: A Sensing transmitter is the entity that sends out the sensing signal which the sensing service will use in its operation. A Sensing transmitter is an NR RAN node or a UE. A Sensing transmitter can be located in the same or different entity as the Sensing receiver.

transparent sensing: sensing measurements are communicated such that they can be discerned and interpreted by the 5G system, e.g. the data is communicated using a standard protocol to an interface defined by the 5G system.

The following KPIs apply to the definition of the use cases on sensing qualitative requirements:

-	Sensing service area: a service area where sensing services would solely rely on infrastructures and sensing technologies that can be assumed to be present anywhere where 5G is present. This includes both indoor and outdoor environments.

The following KPIs apply to the definition of the use cases on sensing quantitative requirements:

-	Accuracy of positioning estimate describes the closeness of the measured sensing result (i.e. position) of the target object to its true position value. It can be further derived into a horizontal sensing accuracy – referring to the sensing result error in a 2D reference or horizontal plane, and into a vertical sensing accuracy – referring to the sensing result error on the vertical axis or altitude.

-	Accuracy of velocity estimate describes the closeness of the measured sensing result (i.e. velocity) of the target object’s velocity to its true velocity.

-	Confidence level describes the percentage of all the possible measured sensing results that can be expected to include the true sensing result considering the accuracy.

-	Sensing Resolution describes the difference of the measured magnitude of two target objects, which is the magnitude needed to distinguish the two target objects.

-	Range resolution denotes the minimum difference in distance between target objects to have a measurably different range.

-	Velocity resolution denotes the minimum difference in velocity between target objects to have a measurably different velocity.

-	Missed detection describes the probability of missing to acquire a sensing result when the 5G system attempts to acquire a sensing result. It applies only to binary sensing results.

-	False alarm describes the probability of detecting a false sensing result that does not represent the characteristics of a target object or environment when the 5G system attempts to acquire a sensing result. It applies only to binary sensing results.

-	Max sensing service latency: time elapsed between the event triggering the determination of the sensing result and the availability of the sensing result at the sensing system interface.

-	Refreshing rate: rate at which the sensing result is generated by the sensing system. It is the inverse of the time elapsed between two successive sensing results reporting to the application server.

* * * End of Change * * * *