3GPP TSG SA WG 1 Meeting #101 	S1-230251

Athens, Greece, 20 – 24 February 2023	(revision of S1-23xxxx)

Source:	KPN

pCR Title:	Pseudo-CR on update of use case 5.10 in TR 22.837

Draft Spec:	3GPP TR 23.837 v0.3.0

Agenda item:	7.1

Document for:	Approval

Contact:	Kallol Das, kallol.das@tno.nl

Abstract:

1. Introduction

TR 22.837 contains the use case on UAV flight trajectory tracing.

2. Reason for Change

The use case on UAV flight trajectory tracing should contain requirements that also consider NR network conditions in the desired sensing area, related to the support of  sensing services.

3. Conclusions

The use case is made more complete.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 v0.3.0

* * * First Change * * * *

## 5.10	Use case on UAV flight trajectory tracing

### 5.10.1	Description

With the development of UAV technologies and the increase of demands on rapid logistics, aerial photographing, environmental monitoring and public security, a variety of commercial UAV services gradually become reality.

Normally the commercial UAVs fly based on predetermined flight routes, following regulated positions, heights, speeds, and directions. E.g., a package-delivery UAV flies from the package sender to the package recipient; a task-execution (such as environmental monitoring) UAV flies from the UAV airport to the target area.

On-route flying is important for these commercial UAVs. Their flight routes are optimized and permitted by UAV service operators, UAV management department, or USS (Uncrewed Aerial System Service Supplier)/UTM (Uncrewed Aerial System Traffic Management). Usually, they have the shortest flight distance, avoid no-fly zone, and keep safe distance from obstacles (e.g., building, trees, hills) and other commercial UAVs.

Although a UAV is equipped with sensors to keep itself along the flight route, the external UAV flight trajectory tracing function is still necessary because these sensors sometimes are restricted. E.g., the camera is impacted by light situation; the UAV-borne radar is impacted by rainfall or snowfall, etc. If these events occur, UAV cannot correctly decide its own position, height or speed, and thus cannot follow the traced route.

Although there exist dedicated UAV surveillance equipment and radar, their large-scale deployment has great challenges due to lack of available sites and high installation and maintenance cost.

In comparison, using the 5G system can provide a cost-effective way to trace these UAVs, e.g., 5G network infrastructures with ubiquitous coverage can better trace the flight trajectory of each UAV.

Specifically, 5G RAN entities can rely on radio sensing to obtain the information on UAV position and motion (e.g., distance, angle) and send sensing measurement data to a sensing processing entity located in the 5G system.

As shown in Figure 5.10.1-1, the UEs that are connected to the 5G RAN entities can be configured to assist in the sensing operations, which can increase the sensing coverage, provide more positioning reference points, and improve sensing result accuracy and robustness. This improvement is a result of  higher density of UEs compared to the base stations, which increases the probability that some UEs are located in positions that have shorter distance away from UAV than 5G RAN entities (e.g., UAV located in the middle of two 5G RAN entities while UE locates under UAV), or some UEs are located in the reflection directions that have larger radar cross section (RCS) than 5G RAN entities considering the UAV RCS variation in different reflection directions.

The 5Gsensing processing entity can collect the sensing data from one or multiple network infrastructures.

The 5G network operator can provide the UAV flight trajectory tracing service to a trusted 3rd party application (e.g., UAV service operator, UAV management department, USS/UTM) as requested.

Figure 5.10.1-1:  UAV flight trajectory tracing by 5G system

### 5.10.2	Pre-conditions

A UAV operator/UTM provides package delivery service in an area which is covered by 5G network. The UAV operator/UTM subscribes to the UAV flight trajectory tracing service from the 5G network operator.

The UAV operator/UTM provides the 5G network operator the characteristics of the UAV to be sensed, time and space (covering the regulated UAV flight routes and possible off-route locations) of the UAV flight trajectory tracing service.

### 5.10.3	Service Flows

When the appointed time starts, 5G network operator activates the UAV flight trajectory tracing function at the appointed space until the appointed time ends.

The UAV operator controls UAV#1 to take off from package delivery source and fly toward package delivery destination along a regulated flight route.

By radio sensing, a set of 5G base stations and UEs detect UAV#1, and then estimate the position and motion related metrics (e.g., distance, angle) as well as the target object is in coverage, resulting in sensing measurement data. The 5G base stations and UEs then send the sensing measurement data to the 5Gsensing processing entity.

In certain cases, during the flying course, based on sensing and location information, if it is detected that UAV#1 has left the coverage of an old base station and entered the coverage of a new base station, the old base station could stop radio sensing and operate in a power saving mode. The new base station starts and keeps on sensing UAV#1 until it is out of coverage. Note that the determination that the UAV#1 has left the coverage of a base station or not could be determined based on the UAV positions and velocities estimated at the 5G sensing processing entity. Therefore, the network could then decide to activate and deactivate sensing in certain base stations based on this information. Additionally, the selection and the activation/deactivation of sensing transmitters and/or sensing receivers shall take the current network conditions (e.g., network-load, energy consumption) into account. In other cases, the network could configure a start and stop of sensing operations for a base station based on a specified time period.

The 5G sensing processing entity collects the UAV sensing measurement data from one or multiple base stations and UEs, and estimates the positions and velocities, and sends in real time the sensing results (e.g., UAV positions, velocities) to the UAV operator and/or UTM.

Based on the received sensing results, the UAV operator and/or UTM traces the flight trajectory of UAV#1. Once the UAV operator and/or UTM detects an off-route event, it further steers UAV#1.

### 5.10.4	Post-conditions

UAV#1 delivers package to the destination along the traced flight route or its off-route behavior is sensed.

### 5.10.5	Existing features partly or fully covering the use case functionality

None.

### 5.10.6	Potential New Requirements needed to support the use case

[P.R 5.10.6-1] Based on operator policy, request from UTM and sensing configuration (e.g. sensing area), the 5G system shall be able to support RAN entities and UEs in sensing the characteristics of an airborne object of interest (e.g., UAV), including generating sensing measurement data related to the object’s location and motion metrics (see examples in Table 5.10.6-1).

[P.R 5.10.6-2] The 5G system shall be able to support means to authorize RAN entities and UEs in certain location area generating and reporting sensing measurement data (e.g., related to a UAV position, velocity) to a 5G sensing processing entity.

NOTE: 	The requirement above assumes that the sensing measurement data is post-processed in 5G sensing processing entity which is located within the 5G system.

[P.R 5.10.6-3] The 5G system shall be able to support means to process the sensing measurement data and expose in real time the sensing results (e.g., related to a UAV position, velocity) from a 5G sensing processing entity to a trusted 3rd party application.

[P.R 5.10.6-4] The 5G system shall support energy efficient sensing operations.

[P.R.5.10.6-5] The 5G system shall provide means to authorize sensing transmitters and sensing receivers for sensing in the desired sensing area based on network conditions (e.g., network-load, energy consumption).

[P.R.5.10.6-6] The 5G system shall provide means to assign the sensing transmitter role and/or the sensing receiver role for sensing in the desired sensing area based on network conditions (e.g., network-load, energy consumption).

[P.R 5.10.6-6] The 5G system shall support sensing services with KPIs as given in Table 5.10.6-1.

Table 5.10.6-1	Performance requirements of sensing results for UAV flight trajectory tracing

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution (horizontal/vertical) [mxm] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| UAV flight trajectory tracing | Outdoor | N/A | N/A | N/A | N/A | N/A | 1m x 1m ~10m x 10m (NOTE 2) | 1m/s x 1m/s ~ 10m/s x 10m/s (NOTE 3) | 5~50 (NOTE 4) | 0.3~5Hz (NOTE 5) | 5 | 5 |
| NOTE 1: The terms in Table 5.10.6-1 are found in Section 3.1. NOTE 2: To detect the UAV existence (e.g., for invation detection), the sensing resolution of distance is 10m [25]. To track the UAV flying (e.g., for collision detection and warning), the sensing resolution of distance is 1m [25] NOTE 3: To detect the UAV existence, the sensing resolution of velocity is 10m/s [25]. To track the UAV flying, the sensing resolution of velocity is 1m/s [25] NOTE 4: To detect the UAV existence, the sensing latency is approximate 50ms [25]. To track the UAV flying, the sensing latency is approximate 5ms [25] NOTE 5: To detect the UAV existence, the sensing refreshing rate is 0.3~5Hz. To track the UAV flying, the sensing latency is 0.3~2Hz. Echodyne MESA-DAATM has approximate 1Hz scan rate [40]. |  |  |  |  |  |  |  |  |  |  |  |  |

* * * Next Change * * * *

* * * Next Change * * * *