3GPP TSG SA WG 1 Meeting #101 	S1-230269

Athens, Greece, 20 - 24 February 2023	(revision of S1-23xxxx)

Source:	Rakuten Mobile Inc

pCR Title:	Use Case on Air Pollution Monitoring using Sensing

Draft Spec:	3GPP TR 22.837 V0.3.0

Agenda item:	7.1

Document for:	Approval

Contact:	Manmeet Bhangu <manmeet.bhangu@rakuten.com>

Abstract: This document proposes a use case of NR based sensing for air pollution monitoring

1. Introduction

Introduce a air pollution monitoring use case into the sensing study

2. Reason for Change

Adding a use case with potential functional requirements and KPIs for air pollution monitoring to the study

3. Conclusions

none

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 v0.3.0.

* * * First Change * * * *

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	W. Favoreel, "Pedestrian sensing for increased traffic safety and efficiency at signalized intersections," 2011 8th IEEE International Conference on Advanced Video and Signal Based Surveillance (AVSS), 2011, pp. 539-542, doi: 10.1109/AVSS.2011.6027406.

[3]	Advances in Wildlife Crossing Technologies: https://highways.dot.gov/public-roads/septoct-2009/advances-wildlife-crossing-technologies.

[4]	Protection Detection: Making Roads Safe for Drivers and Wildlife: https://onlinepubs.trb.org/onlinepubs/webinars/201118.pdf.

[5]	F. Liu et al., "Integrated Sensing and Communications: Towards Dual-functional Wireless Networks for 6G and Beyond," in IEEE Journal on Selected Areas in Communications, doi: 10.1109/JSAC.2022.3156632.

[6]	T. S. Rappaport, G. R. MacCartney, M. K. Samimi and S. Sun, "Wideband Millimeter-Wave Propagation Measurements and Channel Models for Future Wireless Communication System Design," in IEEE Transactions on Communications, vol. 63, no. 9, pp. 3029-3056, Sept. 2015, doi: 10.1109/TCOMM.2015.2434384.

[7]	C. Han, Y. Bi, S. Duan and G. Lu, "Rain Rate Retrieval Test From 25-GHz, 28-GHz, and 38-GHz Millimeter-Wave Link Measurement in Beijing," in IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing, vol. 12, no. 8, pp. 2835-2847, Aug. 2019, doi: 10.1109/JSTARS.2019.2918507.

[8]	IEEE 802.11-18/0611r16: “Wireless LANs, WiFi Sensing Uses Cases”

[9]	TEM STANDARDS TEM STANDARDS AND RECOMMENDED PRACTICE: https://unece.org/fileadmin/DAM/trans/main/tem/temdocs/TEM-Std-Ed3.pdf

[11]	3GPP TR 22.856, "Localized Mobile Metaverse Services".

[13]	“Velodyne™ LiDAR VPL-16 User Manual,” 63-9243 Rev. E, Velodyne™ LiDAR, https://velodynelidar.com/wp-content/uploads/2019/12/63-9243-Rev-E-VLP-16-User-Manual.pdf

[14] 		Liu, A., Huang, Z., Li, M., Wan, Y., Li, W., Han, T.X., Liu, C., Du, R., Tan, D.K.P., Lu, J. and Shen, Y., 2022. A survey on fundamental limits of integrated sensing and communication. IEEE Communications Surveys & Tutorials, 24(2), pp.994-1034.

[15] 		https://medium.com/desn325-emergentdesign/s-l-a-m-and-optical-tracking-for-xr-cfabb7dd536f

[16] 		Dwivedi, S., Shreevastav, R., Munier, F., Nygren, J., Siomina, I., Lyazidi, Y., Shrestha, D., Lindmark, G., Ernström, P., Stare, E. and Razavi, S.M., 2021. Positioning in 5G networks. IEEE Communications Magazine, 59(11), pp.38-44.

[17]	T. Murakami et al, “Wildlife Detection System Using Wireless LAN Signal,” in NTT Technical Review vol.17, No.6, pp. 45-48, June 20019, https://www.ntt-review.jp/archive/ntttechnical.php?contents=ntr201906fa13.pdf&mode=show_pdf

[18]	Eradication of elephant mortality and injury due to railway accidents through automatic tracking and alert system in IEEE Conference Publication, IEEE Xplore

[19]	Impact of wild animals (deer and bears) on train operations 210616_KO_Animal2.pdf (jrhokkaido.co.jp) (in Japanese) [z]	Rail Industry Safety Induction Handbook: https://railsafe.org.au/__data/assets/pdf_file/0009/32022/Rail-Industry-Safety-Induction-RISI-Handbook-V5.1.pdf

[21]	Society of Automotive Engineers (SAE), “Taxonomy and definition for terms related to Driving automation systems for on-Road Motor Vehicles”, https://www.sae.org/standards/content/j3016_202104/

[22]	Census of Fatal Occupational Injuries Summary, 2020, https://www.bls.gov/news.release/cfoi.nr0.htm

[23]	Javed MA, Muram FU, Hansson H, Punnekkat S, Thane H. Towards dynamic safety assurance for Industry 4.0. Journal of Systems Architecture. 2021 Mar 1; 114:101914.

[24]	American National Standards Institute/Industrial Truck Safety Development Foundation, Safety standard for driverless, automatic guided industrial vehicles and automated functions of manned industrial vehicles, December 2019, 2019, [Online] http://www.itsdf.org.

[25]	Moore, Erik George, "Radar Detection, Tracking and Identification for UAV Sense and Avoid Applications" (2019). Electronic Theses and Dissertations. 1544.

[26] 	I. Sobron, I. Landa, I. Eizmendi and M. Velez, "Adaptive TOA Estimation with Imperfect LOS and NLOS Knowledge in UWB Positioning Systems," 2020 IEEE SENSORS, 2020, pp. 1-4. available at: https://sci-hub.wf/10.1109/sensors47125.2020.9278589

[27] 	L. Wang, Z. Zhang, X. Di and J. Tian, "A Roadside Camera-Radar Sensing Fusion System for Intelligent Transportation," 2020 17th European Radar Conference (EuRAD), 2021, pp. 282-285.

[28]	5GAA_White_Paper_C-V2X Use Cases Volume II: Examples and Service Level Requirements

[29] 	https://www.ieee802.org/11/Reports/tgbf_update.htm

[30]	https://mentor.ieee.org/802.11/dcn/20/11-20-1712-02-00bf-wifi-sensing-use-cases.xlsx

[31]	X. Liu, J. Cao, S. Tang and J. Wen, "Wi-Sleep: Contactless Sleep Monitoring via WiFi Signals," 2014 IEEE Real-Time Systems Symposium, 2014, pp. 346-355, doi: 10.1109/RTSS.2014.30.

[32]	Chen V C. The micro-Doppler effect in radar. Artech house, 2019.

[33]	3GPP TS 22.261: “Service requirements for the 5G system”.

[34]	A. Chebrolu, "FallWatch: A Novel Approach for Through-Wall Fall Detection in Real-Time for the Elderly Using Artificial Intelligence", 2021 Third International Conference on Transdisciplinary AI (TransAI), 2021, pp. 57-63, doi: 10.1109/TransAI51903.2021.00018, https://ieeexplore.ieee.org/document/9565618

[35]	B. A. Alsaify et al., “A CSI-Based Multi-Environment Human Activity Recognition Framework” Applied Sciences 12, no. 2: 930, 2022. https://doi.org/10.3390/app12020930

[36]	U. Saeed U et al., "Discrete Human Activity Recognition and Fall Detection by Combining FMCW RADAR Data of Heterogeneous Environments for Independent Assistive Living", Electronics 10(18):2237, 2021. https://doi.org/10.3390/electronics10182237

[37]	C. Dou, H. Huan, "Full Respiration Rate Monitoring Exploiting Doppler Information with Commodity Wi-Fi Devices". Sensors 21, 3505, 2021. https://doi.org/10.3390/s21103505

[38]	J. Pu, H. Zhang, "RF-Heartbeat: Robust and Contactless Heartbeat Monitoring Based on FMCW Radar", 2021. TechRxiv Preprint. https://doi.org/10.36227/techrxiv.15021645.v2

[39] 	H. V. Habi and H. Messer, “Recurrent Neural Network for Rain Estimation Using Commercial Microwave Links,” in IEEE Transactions on Geoscience and Remote Sensing, vol. 59, no. 5, pp. 3672-3681, May 2021, doi: 10.1109/TGRS.2020.3010305.[40]                    Roberto Opromolla, etc., “Perspectives and Sensing Concepts for Small UAS Sense and Avoid”, 2018 IEEE/AIAA 37th Digital Avionics Systems Conference (DASC)

[41]	https://www.rwjbh.org/trinitas-regional-medical-center/treatment-care/sleep-disorders/sleep-apnea/

[42]	https://my.clevelandclinic.org/health/articles/10881-vital-signs

[43]	3GPP TR 22.855, "Study on Ranging-based Services".

[44]	Guoxuan Chi, et. al., "Wi-Drone: Wi-Fi-based 6-DoF Tracking for Indoor Drone Flight Control", MobiSys 22, Association for Computing Machinery, 2022.

[45]	Report on Automated Valet Parking: technology assessment and use case implementation description – 5G Automotive Association (5gaa.org). https://5gaa.org/news/report-on-automated-valet-parking-technology-assessment-and-use-case-implementation-description/

[46]	Nie Y B , Zhang L . Main amendments to Working Safety Regulation of State Grid Company(Dynamical Part for Hydrodynamic Power Plant)[J]. East China Electric Power, 2008.

[47] 	Giuseppe Fragapane, René de Koster, Fabio Sgarbossa, Jan Ola Strandhagen, Planning and control of autonomous mobile robots for intralogistics: Literature review and research agenda, European Journal of Operational Research, Volume 294, Issue 2,2021, Pages 405-426

[48]	Li S, Li X, Lv Q, et al. WiFit: Ubiquitous bodyweight exercise monitoring with commodity wi-fi devices, 2018 IEEE SmartWorld, Ubiquitous Intelligence & Computing, Advanced & Trusted Computing, Scalable Computing & Communications, Cloud & Big Data Computing, Internet of People and Smart City Innovation, IEEE, 2018: 530-537.

[49]	https://www.synopsys.com/automotive/autonomous-driving-levels.html

[50]	N. H. Motlagh et al., "Toward Massive Scale Air Quality Monitoring," in IEEE Communications Magazine, vol. 58, no. 2, pp. 54-59, February 2020, doi: 10.1109/MCOM.001.1900515.

[51]	S. Durjan, V. Bassoo and R. Ahmad, "Effects of Major Air Pollutants on Millimeter Wave Spectrum," 2020 3rd International Conference on Emerging Trends in Electrical, Electronic and Communications Engineering (ELECOM), Balaclava, Mauritius, 2020, pp. 185-190, doi: 10.1109/ELECOM49001.2020.9297032.

[52]	T. S. Rappaport, J. N. Murdock and F. Gutierrez, "State of the Art in 60-GHz Integrated Circuits and Systems for Wireless Communications," in Proceedings of the IEEE, vol. 99, no. 8, pp. 1390-1436, Aug. 2011, doi: 10.1109/JPROC.2011.2143650.

* * * Next Change * * * *

## 5.x	Use case on pollution monitoring using Integrated Sensing and Communication

### 5.x.1	Description

Air pollution monitoring is critical for protecting public health and the environment. The air we breathe has a direct impact on our health, and exposure to certain pollutants can cause a range of health problems, including respiratory and cardiovascular disease, cancer, and other illnesses.

Current air quality monitoring is based on either professional monitoring systems are using large number of low-cost air quality monitoring sensors into public transportation or other parts of urban infrastructure. The professional monitoring system are highly expensive and suffers from low spatial resolution while using millions of low-cost sensors results in poor accuracy unless sensors are periodically re-calibrated [50].

In a 5G network, base stations are deployed by operators with radio cell planning that allows them to cover a wide area. By using these base stations to monitor air pollution, it is possible to track levels of pollutants across cities and along the highways.

The mmwave frequencies used for New Radio (NR) can be affected by the absorption of molecular substances in the air, which can reduce the strength of the signal. The major air pollutants in atmosphere have known to degrade the performance of mmWave links, and in particular the particulate matter (PM) causes the highest attenuation to mmWave signals [51][52].

The impairment of mmWave due to atmospheric attenuation cause by the presence of particulate matter (PM) can be quantified and modelling the base station signal measurements, we are able to know the pollution level.

### 5.x.2	Pre-conditions

City A is a mid-size city in a developing world with high level of air pollution. The quality of air pollution is worse along the highways during the morning and evening commute hours. City needs to monitor the pollution level for statistical as well as practical purposes. High levels of air, water, and soil pollution can cause respiratory problems, neurological damage, and a range of other health issues. Additionally, pollution can also harm the environment by damaging ecosystems and wildlife. By monitoring pollution levels, a city can take steps to reduce pollution and prevent harmful health and environmental impacts.

### 5.x.3	Service Flows

- CityA has a agreement with OperatorA for measuring the air pollution along the highways during the peak traffic hours.

- OperatorA has intensive coverage of the parts of city the govt is interested to monitor the pollution and has deployed mmwave frequencies in its network.

- The base station (or CPE) coordinates with customer’s phone (5G UE) to perform the sensing measurement. The base station and the 5G UE can be transmitter and receiver or vice versa. The receiver measures the 5G wireless signal’s attenuation of mmWave signals as they travel from the transmitter to the receiver and collects them as the sensing measurement data.

- The receiver sends the NR based sensing measurement data every minute (time duration is configurable) and the 5G system processes the sensing measurement data to obtain sensing results and exposes the NR based sensing results to operator’s data center or authorised 3rd party application server via the core network.

- Based on the sensing results above, the application server obtains the pollution information associated with location information. Operator A or 3rd party processes the data to generate air pollution maps and other relevant metrics, which are then made available to City A.

- Operator A provides regular reports to City A on the air pollution levels, enabling the city to track progress and evaluate the effectiveness of its pollution reduction measures.

### 5.x.4	Post-conditions

City A will get data for the levels of pollution along a certain part of the city. City A can use this data to respond promptly and take action to reduce the pollution, such as imposing stricter regulations or enforcing penalties on polluters.

### 5.x.5	Existing feature partly or fully covering use case functionality

[PR. 5.3.6 – 001] The 5G system shall support collection of the NR based sensing measurement data from the base station.

[PR. 5.3.6 – 002] Based on operator’s policy, the 5G system shall support mechanisms to process the sensing measurement data to derive the sensing results.

[PR. 5.3.6 – 003] Based on operator’s policy, the 5G system shall provide mechanisms to expose NR based sensing results with assisted information, e.g. location, to a trusted 3rd party application via the core network.

### 5.x.6	Potential New Requirements needed to support the use case

[PR. 5.x.6 – 001] Based on operator’s policy, the 5G system shall provide mechanism to authorised 3rd party to configure the rate at which sensing is performed.

[PR. 5.x.6 – 002] The 5G system shall support sensing services with KPIs as given Table 5.3.6-1.

Table 5.3.6-1	Performance requirements of sensing results for rainfall monitoring

| Scenario | Sensing service area | Confidence level [%] | Maximum Range[m] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing Services latency[ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Pollution monitoring | outdoor | 95 | 200 | N/A | N/A | N/A | N/A | N/A | N/A | 60 sec | 1 min, application configurable | 5 | 5 |
| NOTE 1: The terms in Table 5.3.6-1 are found in Section 3.1. NOTE 2: Numbers are derived from [5],[50] |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Changes * * * *