3GPP TSG SA WG 1 Meeting #101 	S1-230274

Athens, Greece, 20 - 24 February 2023	(revision of S1-23xxxx)

Source:	Deutsche Telekom

pCR Title:	pCR on updating use case 5.2 Supporting UAV flight preparation

Draft Spec:	3GPP TR 22.843

Agenda item:	7.8

Document for:	Approval

Contact:	Vasil Aleksiev, vasil.aleksiev@magenta.at

Abstract: This is a proposal to clarify further the 5GS part in the process of UAV flight preparation

1. Introduction

There is existing use case on supporting UAV flight preparation where the role of 5G system is not entirely clarified.

2. Reason for Change

As the 5GS can have varios statistics and predictive knowledge about the QoS under the network coverage, it can assist in providing specific flight path. This could save some ping pong messages between UAV controller, UTM and 5GS in trying to check different posibilities in preparing a flight. QoS information can be also sensitive information to be widely provided to 3rd parties. UTM has the control and can evaluate and finally decide.

In cases where the flight path cannot be changed (e.g. gas pipes inspection), the 5GS can reconfigure network resources providing the needed QoS in specific geographic area/waypoints.

4. Proposal

It is proposed to agree the following changes to 3GPP TS / TR <TS/TR number and version>.

* * * First Change * * * *

## 5.2	Use case on supporting UAV pre-flight preparation

### 5.2.1		Description

During the operation of UAVs using the 3GPP network access, the network needs to ensure that the UAVs can access the network smoothly and timely. And UAV flight mission need to be ensured for the real-time performance and stability of flight control link to guarantee the continuity of the flight.

5G network can provide wide-area, high-quality and secure connectivity that can enable cost-efficient drone operations beyond visual line-of-sight (BVLOS) range. In some industrial UAV scenarios, the UAV would be provided pre-scheduled flight plan for autonomous flying. When UAV uses 3GPP network as the access type, the UTM/drone operator may need to acknowledge the network status for flight path scheduling with the help of 3GPP network. 5G system can assist the UTM and drone operator with providing flight path based on the supported QoS in specific area or reconfigure network resources to achieve the needed QoS.

### 5.2.2		Pre-conditions

In the logistics company, UAV A and UAV B are both individual terminals for express delivery and subscribe the communication service from 5G network.

Operator A provides 5G network coverage and communication services for remote commands and control (C2) communication between UAV and UAV control platform.

The UTM has good cooperation with Operator A to have UAV-related information which is fetched from 5G network.

### 5.2.3		Service Flows

Figure 5.2.3-1: Supporting UAV pre-flight preparation

1.	The logistics company plans to use UAV A to provide video inspection of pipelines from X to Y and use UAV B to deliver express package from X to Z.

2.	UAV/UAV-C A and UAV/UAV-C B register in 5G network and can communicate with UTM though 5G network.

3.	The UAV-C A provides planned flight path information from X to Y to 5G system (e.g. waypoints and required QoS from X to Y). The UAV-C B provides planned flight path or start point X, end point Z and needed QoS.

4.	5G system checks for required and needed QoS information in predictions/statistics (e.g. end-to-end latency, QoE) along required and possible flight paths.

5.	Based on the available analytics information, the 5G system can provide confirmation that QoS is available for UAV A flight path or alternatively can reconfigure network resources to be able to provide the needed QoS to UAV A among the flight path. As UAV B can take alternative routes, 5G system can provide optimized flight path considering where the QoS is available.

6.	UTM/UAS determines the final flight paths based on the information from Operator A and sends to UAV/UAV-C A and UAV/UAV-C B separately.

7.	UAV A and UAV B receive the flight path information from UTM/UAS via 5G system and start flight mission.

### 5.2.4		Post-conditions

UAV A and UAV B can finish their delivery tasks with the support of 5G system.

### 5.2.5		Existing features partly or fully covering the use case functionality

None.

### 5.2.6		Potential New Requirements needed to support the use case

[P.R 5.2.6-001] Based on operator’s policy, the 5G system shall be able to provide a method to monitor and predict network condition (e.g. bitrate, latency, reliability) along the UAV continuous geographic path provided from UTM or UAV controller.

[P.R 5.2.6-002] The 5G system shall be able to provide UTM and UAV controller with the information about geographic areas/waypoints where UAV service requirements could be met based on predicted QoS and network conditions (e.g. bitrate, latency, reliability).

[P.R 5.2.6/003] The 5G system shall be able to support UTM and UAV controller with providing flight path based on the needed QoS.

[P.R 5.2.6/004] Based on operators’ policy, the 5G system shall be able to reconfigure network resources providing the needed QoS among a flight path.

* * * End of Changes * * * *