3GPP TSG SA WG 1 Meeting #101 	S1-230305

Athens, Greece, 20 - 24 February 2023	(revision of S1-23xxxx)

Source:	KPN

pCR Title:	Pseudo-CR on store and forward messaging for Ambient IoT

Draft Spec:	3GPP TR 22.840v1.0.0

Agenda item:	7.2

Document for:	Approval

Contact:	Toon .norp@ tno. nl

Abstract: <provide a short description of the content>

1. Introduction

This contribution provides an update (with additional requirement) for the use case 5.27 use case on end-to-end logistics.

2. Reason for Change

In the use case of end-to-end logistics it is quite possible that a delivery driver has to deliver a package in an area that does not have good mobile network coverage. A handheld relay device can relay communication from the Ambient IoT device on the package. However, it is possible that also the handheld relay device does not have coverage. In that case a message from the Ambient IoT device can be sent using ‘store and forward’, where the message is first sent from the Ambient IoT device to the handheld device. The handheld device subsequently forwards the message to the network when it is back in mobile network coverage.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840v1.0.0.

* * * First Change * * * *

## 5.27	Use case on end-to-end logistics

### 5.27.1	Description

In a logistics scenario, Ambient IoT devices can be used to track specific goods (e.g. TVs) from the factory where they are made until delivery at the final end-customer. E.g. an Ambient IoT device could be attached to the packaging of a TV, which can then be used for tracking in the factory, in one or more warehouses on the way from factory to the customer and finally to the track the delivery at the end customer.

In this end-to-end logistics trajectory, an Ambient IoT device will encounter multiple different networks (NPNs, PLMNs) in different regions and countries. Each of these networks have to follow different frequency regulations and have different frequency licenses. The Ambient IoT device needs to be able to deal with these differences.

Limitations in range for Ambient IoT can imply that some additional infrastructure needs to be deployed. However, it is not assumed that a network needs to be deployed specifically for Ambient IoT support. It should be possible to integrate Ambient IoT communication in networks that are also used for other 5G communication.

The assumption in this use case is that the Ambient IoT devices need to be triggered to send their identification. When the Ambient IoT devices are not triggered they remain in a suspended state and cannot send or receive information.

The owner of a network (e.g. in a factory or warehouse) can decide which part of the network (e.g. which base stations) to use to trigger Ambient IoT devices. This way, e.g. Ambient IoT devices in only part of the factory are triggered. It should also be possible to trigger only a selected group of Ambient IoT devices (e.g. through broadcasting a group ID), or to trigger the Ambient IoT devices for specific actions (e.g. receive information instead of broadcasting an ID).

Figure 5.27.1-1: Tracking a cardboard box a TV is transported in by attaching an Ambient IoT Device to the box

Where the network itself does not have the support to trigger the Ambient IoT device (e.g. in the PLMN where the TV is delivered to the door of the customer), a handheld device can be used. This handheld device can, when needed, relay the communication from the Ambient IoT device towards the network. The handheld device can collect several messages from one or more Ambient IoT devices before forwarding these messages to the network. This ‘store-and-forward’ messaging also allows messaging from an Ambient IoT device when there is no end-to-end connectivity from Ambient IoT device to the network.

### 5.27.2	Pre-Conditions

Factory A has deploys a standalone NPN.

TV_company has an agreement with the NPN in Factory A and with PLMN_X to use a third party interface to trigger Ambient IoT devices.

The Ambient IoT device has credentials with PLMN_X that can also be used for the standalone NPN in factory A. Furthermore, PLMN_X has roaming agreements with operators in other countries.

Warehouse B deploys a NPN implemented as a slice on PLMN_Y.

### 5.27.3	Service Flows

- TV_company uses Factory A to manufacture its TVs. After manufacturing, the TVs are packaged in a cardboard box and the box is then equipped with an Ambient IoT device (in the form of a sticker) for tracking/tracing of the TV.

- Within the Factory A, the TVs are stored for a while in internal storage. Tracking of the TVs in storage is possible through Ambient IoT. The TV_company instructs the NPN that is deployed in the Factory A to trigger the TVs in the internal storage only. TVs that are still in the production area do not need to be triggered. The Ambient IoT devices are triggered to send their ID. This way the TV_company can create an inventory of all TVs that are still in store at the factory. The Ambient IoT devices can also be triggered to receive information (e.g. instructions).

- A shipment of TV is sent to a warehouse in a Country_X. The warehouse uses a NPN implemented as a slice on PLMN_Y. Because of roaming agreements between PLMN_Y and PLMN_X, the Ambient IoT devices can access the NPN network in the warehouse. Note for the NPN in Country_X different regulations apply than for the NPN in Factory A.

- TV_company wants to create an inventory of the TVs that are stored in the warehouse. However, the warehouse also stores products from other companies. Therefore the TV_company instructs the NPN to trigger only the TVs from TV_company through the use of a group ID that identifies these TVs. Ambient IoT devices on other products will not be triggered to respond.

- Finally the TV is delivered to the end-customer. The delivery driver uses a handheld device to trigger the Ambient IoT device on the box to send its ID to the network. If the network is within range, the Ambient IoT device can communicate directly to the network. Otherwise the handheld device can relay the communication. Even if the handheld device cannot get connectivity to the network either, the Ambient IoT device can still send a message to the handheld device. The handheld device can forward that message to the network when it gets back within coverage.

### 5.27.4	Post-Conditions

TV_Company has full traceability of its products from the factory in one country, to the delivery at the end-customer in another country.

### 5.27.5	Existing features partly or fully covering the use case functionality

3GPP TS22.101 Clause 29.2 contains the following requirements:

Support of 3rd party requested broadcast

-	The 3GPP Core Network shall enable a 3rd party service provider to request sending a broadcast message in a specified geographic area (as specified in TS 22.368 [52]) expecting to reach a group of devices that are served by the 3rd party service provider.

A 3rd party requested broadcast can be used to trigger a group of IoT devices in a specific geographic area. But this requirement does not include the feature of waking up Ambient IoT devices.

### 5.27.6	Potential New Requirements needed to support the use case

[PR.5.27-001] The 5G system shall be able to support an Ambient IoT device to function in different countries in accordance with local regulations.

[PR.5.27-002] The 5G system shall support functionality to trigger Ambient IoT devices to communicate.

Editor’s Note: terminology of triggering to be checked with existing MTC triggering terminology. Definitions still to be provided in the definition section

[PR.5.27-003] The 5G network shall be able to trigger Ambient IoT devices. The 5G system shall enable an authorized 3rd party to instruct the 5G network in which area, which group of Ambient IoT devices needs to be triggered and which action the Ambient IoT devices need to perform when triggered (e.g. send ID, receive further information, send measurement value).

[PR.5.27-004] The 5G system shall support relaying communication from an Ambient IoT device to the network.

[PR.5.27-005] The 5G system shall support secure and authenticated messaging from an Ambient IoT device to the network, even when there is no end-to-end connection between the Ambient IoT device and the network

NOTE:	An example of such a scenario is where the Ambient IoT device can communicate to a relay device, but where there is no concurrent communication from the relay to the network and subsequently when the relay device can communicate to the network, it can no longer communicate with the Ambient IoT device.