3GPP TSG SA WG 1 Meeting #101 	S1-230316

Athens, Greece, 20 - 24 February 2023	(revision of S1-23xxxx)

Source:	Philips International B.V.

pCR Title:	Addressing Editor’s Note in Use Case 5.18

Draft Spec:	3GPP TR 22.837

Agenda item:	7.1 FS_Sensing

Document for:	Approval

Contact:	Walter Dees (walter.dees@philips.com)

Abstract: This contribution proposes to address the Editor’s Note in use case 5.18

1. Introduction

During SA1#99e use case 5.18 related to service continuity of wireless sensing of a target was accepted with the following Editor’s Note in [PR 5.18.6.1]:

“[PR 5.18.6.1] The 5G system shall support service continuity for wireless sensing of a target between different sensing devices.

Editor’s Note: It is FFS if [PR 5.18.6.1] can be solved at application layer.”

2. Reason for Change

We propose that the Editor’s note can be removed based on the following reasons:

- The application layer typically does not have knowledge about the details of the RAN architecture and deployment, i.e. it might not know which particular gNB(s) would be used for sensing a target area, and which gNB(s) would be better suited to sense the target area when the sensing target gets close to the edge of the target area or leaves the target area.

- Not all gNB(s) may be capable of wireless sensing and some gNBs may be too overloaded, e.g. with normal communication tasks. The 5G system itself is much better suited to determine which gNB(s) to involve to sense a target and configure/instruct those gNB(s) accordingly. This is of particular importance in mobility scenarios.

- If service continuity needs to be done by the application, the application may need to get sensing data from multiple/all base stations covering a certain area and possibly beyond depending on where the target can move towards. This would be highly inefficient communication wise (all this information needs to be sent to the AF) and computation wise (all this information needs to be processed at the AF), and could possibly lead to some privacy concerns as too much information may be provided to the AF. It is simpler and safer if the 5GS / RAN can keep track of the targets and only sense where and when required and let the RAN determine which base stations to involve.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837.

* * * First Change * * * *

## 5.18 Use case on service continuity of unobtrusive health monitoring.

### 5.18.1 	Description

An elderly home has installed a new 5G system capable of providing communication and sensing capabilities through the facilities as illustrated in Figure 5.18.1-1. The deployed 5G system includes multiple sensing devices, e.g., base stations, providing connectivity and sensing capabilities. These sensing devices can perform wireless sensing of a target, in this case, health monitoring (e.g. fall/activity detection [34][35][36] or wireless sensing of vital signs such as heart rate [38] or breathing rate [37] of one or more persons). Since elderly people move through the facilities, it is important to provide health monitoring independently of the base station used for sensing. The staff of the elderly home really likes this new 5G wireless sensing feature because it is unobtrusive and offers various advantages over the old system that they use with body worn sensors. For example, they don’t need to recharge or replace the batteries of body worn sensors anymore and remind people or help people to wear them after they took them off (for example to take a shower). The elderly people themselves also like it more, since the body worn sensors often made them feel uncomfortable, especially during sleep or during hot days. Installing cameras was not seen as a good alternative because of the privacy concerns.

In the provided use case, base stations cooperate with each other to ensure service continuity for sensing of a ‘target’ user. In this particular scenario, a user, Robert, is considered who moves through the facilities. Robert's health is quite frail and requires continuous monitoring of his health state without interruption. Robert is currently sensed by means of (indoor) base station A located near his room and is moving out of the sensing area of base station A and approaching the sensing area of base station B covering the recreation/eating area and part of the hallway. Base station A and base station B cooperate in such a way that it is ensured that base station B has started wireless sensing of Robert before base station A stops its wireless sensing of Robert. When Robert is in range of both base station A and B, both base stations can cooperate to perform simultaneous wireless sensing. Similarly, when Robert decides to go for a walk to the garden that is covered by Base Station C, the sensing of Robert is seamlessly continued by Base Station C. The sensing measurement data is collected and processed by the 5G system (e.g. to detect certain movement patterns) and then exposed to a sensing application that is automatically monitoring health anomalies. If a health anomaly is detected (e.g. Robert falls down), an alarm is triggered indicating the health condition as well as the location of the monitored user.

Figure 5.18.1-1: Example of service continuity between Base stations A, B and C.

### 5.18.2	Pre-conditions

1.	MNO operates a 5GS providing wireless sensing capabilities through a set of base stations installed in the elderly home and its garden, as illustrated in Figure 5.x.1-1.

2. 	Robert has subscribed to the wireless sensing service offered by the 5GS in cooperation with an external application provider. Robert provided some identification information, e.g. which room he resides in, the identity of his mobile phone and/or some physical characteristics (e.g. length). The application provider has no knowledge of the RAN infrastructure operated by the MNO.

### 5.18.3	Service Flows

1.	Robert is currently located in his room in the elderly home. The closest nearby base station, i.e. base station A infers, based on the identification information provided by Robert, that Robert is in his room. Base station A starts wireless sensing of Robert, whereby it sends the sensing measurements to the 5GC for further processing, after which the sensing results are sent to a sensing application to detect health anomalies

2. 	Robert starts moving toward the garden.

3.	When leaving his room and entering the hallway, the wireless sensing signal conditions of base station B become better than those of base station A.

4.	The 5G system coordinates the responsibility of sensing Robert from base station A to base station B. During this time, both base station A and B might sense Robert.

5. 	Base station B is used for sensing Robert

6. 	Base station A can stop sensing Robert.

7. 	When leaving the elderly home and entering the garden, Base Station C continues the sensing of Robert.

### 5.18.4	Post-conditions

### Robert’s vital signs are monitored without interruption independently of his location.

### 5.18.5	Existing features partly or fully covering the use case functionality

None

### 5.18.6	Potential New Requirements needed to support the use case

[PR 5.18.6.1] The 5G system shall support service continuity for wireless sensing of a target between different sensing devices.

[PR 5.18.6.2] The 5G system shall support simultaneous wireless sensing of a target by means of multiple sensing devices.