3GPP TSG SA WG 1 Meeting #101 	S1-230338

Athens, Greece, 20 – 24 February 2023	(revision of S1-23xxxx)

Source:	Intel

pCR Title:	Use case on Compute Offload for Metaverse MMO Gaming

Draft Spec:	3GPP TR 22.856

Agenda item:	7.3

Document for:	Approval

Contact:	Suresh Srinivasan <suresh.srinivasan@intel.com>

Abstract: This pCR proposes a new use case to TR 22.856.

1. Introduction

This CR introduces a use case on Compute Offload for Metaverse MMO Gaming.

2. Reason for Change

This p-CR proposes a new use case on Compute Offload for Metaverse MMO Gaming

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.856 v1.0.0

* * * 1st Change * * * *

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.228: "Service requirements for the Internet Protocol (IP) Multimedia core network Subsystem (IMS)".

[3]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services".

[4]	3GPP TS 22.101: "Service principles".

[5]	3GPP TS 22.261: "Service requirements for the 5G system".

[6] 	Talaba D, Horvath I, Lee K H, "Special issue of Computer-Aided Design on virtual and augmented reality technologies in product design," Computer-Aided Design 42:361 - 363.

[7] 	Wang X, Tsai J J-H, "Collaborative Design in Virtual Environments," In: ISCA 48:1-2.

[8] 	Benford S, Greenhalgh C, Rodden T, "Collaborative virtual environments." Communications of the ACM 44: 79–85.

[9]	M. Eid, J. Cha, and A. El Saddik, "Admux: An adaptive multiplexer for haptic-audio-visual data communication", IEEE Tran. Instrument. and Measurement, vol. 60, pp. 21–31, Jan 2011.

[10]	K. Iwata, Y. Ishibashi, N. Fukushima, and S. Sugawara, "QoE assessment in haptic media, sound, and video transmission: Effect of playout buffering control", Comput. Entertain., vol. 8, pp. 12:1–12:14, Dec 2010.

[11]	N. Suzuki and S. Katsura, "Evaluation of QoS in haptic communication based on bilateral control", in IEEE Int. Conf. on Mechatronics (ICM), Feb 2013, pp. 886–891.

[12]	E. Isomura, S. Tasaka, and T. Nunome, "A multidimensional QoE monitoring system for audiovisual and haptic interactive IP communications", in IEEE Consumer Communications and Networking Conference (CCNC), Jan 2013, pp. 196–202.

[13]	A. Hamam and A. El Saddik, "Toward a mathematical model for quality of experience evaluation of haptic applications", IEEE Tran. Instrument. and Measurement, vol. 62, pp. 3315–3322, Dec 2013.

[14]	O. Holland et al., "The IEEE 1918.1 "Tactile Internet" Standards Working Group and its Standards," Proceedings of the IEEE, vol. 107, no. 2, Feb. 2019.

[15]	Lee, Donghwan et. al., “Large-scale Localization Datasets in Crowded Indoor Spaces,” NAVER LABS, NAVER LABS Europe, CVPR 2021.

[16]	3GPP TR 22.837, " Study on Integrated Sensing and Communication."

[17]	Alriksson, F., Kang, D.H., Phillips, C., Pradas, J.L. and Zaidi, A., 2021. XR and 5G: Extended reality at scale with time-critical communication. Ericsson Technology Review.

[18]	3GPP TR 26.928, "Extended Reality (XR) in 5G."

[19] 	Ge, Y., Wen, F., Kim, H., Zhu, M., Jiang, F., Kim, S., Svensson, L. and Wymeersch, H., 2020. 5G SLAM using the clustering and assignment approach with diffuse multipath. Sensors, 20(16), p.4656.

[20] 	Kim, H., Granström, K., Gao, L., Battistelli, G., Kim, S. and Wymeersch, H., 2020. 5G mmWave cooperative positioning and mapping using multi-model PHD filter and map fusion. IEEE Transactions on Wireless Communications, 19(6), pp.3782-3795.

[21] 	Liu, A., Huang, Z., Li, M., Wan, Y., Li, W., Han, T.X., Liu, C., Du, R., Tan, D.K.P., Lu, J. and Shen, Y., 2022. A survey on fundamental limits of integrated sensing and communication. IEEE Communications Surveys & Tutorials, 24(2), pp.994-1034.

[22] 	Dwivedi, S., Shreevastav, R., Munier, F., Nygren, J., Siomina, I., Lyazidi, Y., Shrestha, D., Lindmark, G., Ernström, P., Stare, E. and Razavi, S.M., 2021. Positioning in 5G networks. IEEE Communications Magazine, 59(11), pp.38-44.

[23]	Suhr, C. "S.L.A.M. and Optical Tracking for XR", https://medium.com/desn325-emergentdesign/s-l-a-m-and-optical-tracking-for-xr-cfabb7dd536f <accessed 1.9.22>

[24]	Kim, H., Granstrom, K., Svensson, L., Kim, S. and Wymeersch, H., 2022. PMBM-based SLAM Filters in 5G mmWave Vehicular Networks. IEEE Transactions on Vehicular Technology.

[25]	5GAA " C-V2X Use Cases Volume II: Examples and Service Level Requirements", 5G Automobile Assocaition White Paper,  https://5gaa.org/wp-content/uploads/2020/10/5GAA_White-Paper_C-V2X-Use-Cases-Volume-II.pdf <accessed 02.09.22>

[26]	A. Ebrahimzadeh, M. Maier and R. H. Glitho, "Trace-Driven Haptic Traffic Characterization for Tactile Internet Performance Evaluation," 2021 International Conference on Engineering and Emerging Technologies (ICEET), 2021, pp. 1-6.

[27]	Lee, L.-H., Braud, T., Zhou, P., Wang, L., Xu, D., Lin,  Z., Kumar, A., Bermejo, C., and Hui, P. All One Needs to Know about Metaverse: A Complete Survey on Technological Singularity, Virtual Ecosystem, and Research Agenda, 2021.

[28]	Halbhuber, David & Henze, Niels & Schwind, Valentin. (2021). Increasing Player Performance and Game Experience in High Latency Systems. Proceedings of the ACM on Human-Computer Interaction. 5. 1-20. 10.1145/3474710.

[29] 	Holland, O., Steinbach, E., Prasad, R.V., Liu, Q., Dawy, Z., Aijaz, A., Pappas, N., Chandra, K., Rao, V.S., Oteafy, S. and Eid, M., 2019. The IEEE 1918.1 “tactile internet” standards working group and its standards. Proceedings of the IEEE, 107(2), pp.256-279.

[30] 	Skalidis, I., Muller, O. and Fournier, S., 2022. CardioVerse: The Cardiovascular Medicine in the Era of Metaverse. Trends in Cardiovascular Medicine.

[31] 	Koo, H., 2021. Training in lung cancer surgery through the metaverse, including extended reality, in the smart operating room of Seoul National University Bundang Hospital, Korea. Journal of educational evaluation for health professions, 18.

[32] 	Mozumder, M.A.I., Sheeraz, M.M., Athar, A., Aich, S. and Kim, H.C., 2022, February. Overview: technology roadmap of the future trend of metaverse based on IoT, blockchain, AI technique, and medical domain metaverse activity. In 2022 24th International Conference on Advanced Communication Technology (ICACT) (pp. 256-261). IEEE.

[33] 	Ning, H., Wang, H., Lin, Y., Wang, W., Dhelim, S., Farha, F., Ding, J. and Daneshmand, M., 2021. A Survey on Metaverse: the State-of-the-art, Technologies, Applications, and Challenges. arXiv preprint arXiv:2111.09673.

[34] 	https://www.xrtoday.com/virtual-reality/ukrainian-doctors-perform-first-vr-surgery/

[35] 	https://www.cryptotimes.io/surgeon-performed-surgery-remotely-through-metaverse/

[36]	 Senk, S., Ulbricht, M., Tsokalo, I., Rischke, J., Li, S.C., Speidel, S., Nguyen, G.T., Seeling, P. and Fitzek, F.H., 2022. Healing Hands: The Tactile Internet in Future Tele-Healthcare. Sensors, 22(4), p.1404.

[37] 	Chen, D. and Zhang, R., 2022. Exploring Research Trends of Emerging Technologies in Health Metaverse: A Bibliometric Analysis. Available at SSRN 3998068.

[38] 	https://www.yankodesign.com/2022/05/15/the-metaverse-has-the-power-to-improve-healthcare-and-it-has-already-begun/

[39] 	3GPP TR 23.501 release-17 - https://www.3gpp.org/ftp/Specs/archive/23_series/23.501/23501-h50.zip

[40] 	3GPP TR 38.300 release-17 - https://www.3gpp.org/ftp/Specs/archive/38_series/38.300/38300-h10.zip

[41] 	3GPP TR 22.826 release-17 - https://www.3gpp.org/ftp/Specs/archive/22_series/22.826/22826-h20.zip

[42] 	Tataria, H., Shafi, M., Molisch, A.F., Dohler, M., Sjöland, H. and Tufvesson, F., 2021. 6G wireless systems: Vision, requirements, challenges, insights, and opportunities. Proceedings of the IEEE, 109(7), pp.1166-1199.

[43]	3GPP TS 23.682, "Architecture enhancements to facilitate communications with packet data networks and applications".

[44]	3GPP TS 23.273, "5G System (5GS) Location Services (LCS); Stage 2".

[45]	"MPEG-4 Face and Body Animation", https://visagetechnologies.com/mpeg-4-face-and-body-animation/ (accessed 2.11.22).

[46]	3GPP TS 22.105, "Services and service capabilities".[47]	Virtual humans: https://en.wikipedia.org/wiki/Virtual_humans

[48]	Kwang Soon Kim, et al., "Ultrareliable and Low-Latency Communication Techniques for Tactile Internet Services", PROCEEDINGS OF THE IEEE, Vol. 107, No. 2, February 2019

[49]	I.-J. Hirsh and C. E. J. Sherrick, “Perceived order in different sense modalities,” Journal of Experimental Psychology, vol. 62, no. 5, pp. 423–432, 1961. [50]                       VESA Compression Codecs - vesa.org/vesa-display-compression-codecs

[51]	3GPP TR 23.700-80: "Study on 5G System Support for AI/ML-based Services".

[52]	EU data protection rules | European Commission (europa.eu) (https://ec.europa.eu/info/law/law-topic/data-protection/eu-data-protection-rules_en)

[53] 	California Consumer Privacy Act (CCPA) | State of California - Department of Justice - Office of the Attorney General (https://oag.ca.gov/privacy/ccpa)

[54]	Xu, M., Ng, W.C., Lim, W.Y.B., Kang, J., Xiong, Z., Niyato, D., Yang, Q., Shen, X.S. and Miao,       C., 2022. A full dive into realizing the edge-enabled metaverse: Visions, enabling technologies, and challenges. IEEE Communications Surveys & Tutorials.

[54]	Xu, M., Ng, W.C., Lim, W.Y.B., Kang, J., Xiong, Z., Niyato, D., Yang, Q., Shen, X.S. and  Miao, C., 2022. A full dive into realizing the edge-enabled metaverse: Visions, enabling technologies, and challenges. IEEE Communications Surveys & Tutorials.

[55]	Xu, X., Chen, Y., Zhang, X., Liu, Q., Liu, X. and Qi, L., 2021. A blockchain‐based    computation offloading method for edge computing in 5G networks. Software: Practice and Experience, 51(10), pp.2015-2032.

[56]	Dinh, T.Q., Tang, J., La, Q.D. and Quek, T.Q., 2017. Offloading in mobile edge computing: Task allocation and computational frequency scaling. IEEE Transactions on Communications, 65(8), pp.3571-3584.

[57]	Ning, Z., Dong, P., Wang, X., Hu, X., Liu, J., Guo, L., Hu, B., Kwok, R.Y. and Leung, V.C.,          2020. Partial computation offloading and adaptive task scheduling for 5G-enabled vehicular networks. IEEE Transactions on Mobile Computing, 21(4), pp.1319-1333.

[58] 	3GPP TR 26.118 release-19, https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=3325

[59] 	3GPP TR 22.261 release-18 and release-15,  https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=3107

[60] 	     https://www.sportskeeda.com/esports/massively-multiplayer-games

* * * Next Change * * * *

# 5	Use cases

## 5.A	Use case on Compute Offload for Metaverse MMO Gaming

### 5.A.1	Description

Massive Multiplayer Online (MMO) games require graphical (GPU) intense computation since it host more than hundred players in a single session. Whereas MMO for metaverse are still in rudimentary stages due to mobile metaverse enablers – using devices such as HMD and VR glasses. These are usually designed to be compact and wearable and have limitations in computation power, memory storage and energy.  Metaverse enablers require additional powerful computing resource to render both immersive virtual worlds and interactions with hundreds of other players.

5G systems with Edge computing is a promising technology for the mobile metaverse users, since metaverse applications such as MMO games require low latency to be as low as possible to deliver real-time, advanced, and computation-intense capabilities. Edge computing serves GPU computation capability in the local network reducing latency and burden on backbone networks.

Metaverse MMO games must determine the movement and positioning of an avatar to be consistent and fit in with foreground and background rendering. Most delay sensitive computation such as positioning of an avatar and foreground rendering are offloaded in smaller GPU computable tiles to the edge computing. The 5G system with distributed edge computing not only aids additional computing but also accelerates computing. Remote direct memory access to share the results among distributed computing along with URLCC will reduce the computing offload communication delay [55-58]

Figure 5.A.1-1: Metaverse MMO gaming requiring Computing offload to 5G system [60]

Limitation with Edge/Cloud/Split Rendering. In Network based or split based rendering, most of the computation or partial computation are done in the cloud/Edge server and the resulting video are rendered to the mobile metaverse enablers over the 5G network. Tracking and sensor information from the metaverse enablers are sent to the network server to duly update viewport rendering. Such network-based rendering urges for high data rate and low latency [58], and with multiple users using applications such as MMO games would almost occupy bandwidth to serve the required high data rate and low latency. Due to these shortcomings, MMO gaming for metaverse has been far from reality. With the possibility to offload computation, the video for viewport tiles is further split into smaller group of tile videos and are offloaded to a distributed network-based compute resources to compute in parallel. Eventually, the results from the network-based compute are transferred to the metaverse enablers using Remote Direct Memory Access (RDMA) and the metaverse enablers will render video locally. With this architecture, 5G's URLLC and eMMB would serve such offload and make MMO games for metaverse a reality [58].

### 5.A.2	Pre-conditions

1) Bob is an intense player of mobile metaverse MMO game services and has a 5G capable Head Mount Device (HMD).

2) Bob’s HMD device is designed to be compact and wearable, and it is suitable only for low power and low computation applications.

2) Bob has a 5G subscription with the operator Y – who supports compute offload as service.

3) Operator-Y has subscribers various plans for the compute offload as service based on the time and intensity of the compute offload.

4) Bob is registered to a popular Massive Multiplayer Online Game X.

### 5.A.3	Service Flows

1) On a weekend, Bob wants to play his favourite MMO game using his HMD device.

2) MMO games generally require a graphics intensive device but unfortunately the bob’s HMD device is unable or slow in updating MMO games.

3) Since Bob’s HMD device is registered to the compute offload services with the operator. The HMD device discovers all the available BS and it’s compute offload services, and connects – using dual-connectivity or carrier aggregation- to the required number of BS. The HMD creates a link - using the additional or enhanced control signals- with all the discovered compute offload services. (Note: Link could be a dedicated logical channel with RRC state or an IP link)

4) Operator-Y provides a distribute offload compute service. This service has high-speed processors, which are sufficient to execute the computation that Bob’s HMD device offloads. The HMD device can work in parallel to the offloaded computation. This will reduce the effective computation time as experienced by Bob.

5) The results of the distributed computation are periodically directly transferred to the application (MMO gaming) of the HMD device due to which MMO’s foreground rendering is lot faster.

6) When it is done, it concludes its use of offloading services. The service resources for bob’s resource are no longer required.

### 5.A.4	Post-conditions

1) Due to computing offload, Bob was able to play MMO Games using his favourite HMD device, even though this device has limited computational capacity.

2) Without compute offload, Either Bob wouldn’t have used HMD for MMO Games or Bob would have to depend on Cloud/edge rendering, which is bandwidth intense and intermittent.

### 5.A.5	Existing feature partly or fully covering use case functionality

1) System requirements for high data rate and low latency for AR/VR in clause 7.6 of TS-22.261 [59] specify the KPI table for Cloud/Edge/Split rendering. These AR/VR videos are rendered from cloud/edge using conventional 5G data communication, which consumes significant amount of 5G bandwidth.

2) System design for cloud/Edge/split rendering is specified in TS 26.118 clause 4.3 [58]. In this design, viewport streaming, distributed rendering, split rendering, are proposed where the computation of XR/VR rendering are done either in the server or both server and the metaverse enablers respectively. Further, the system design addresses display, tracking and Pose generation, media coding/encoding and formats, viewport rendering and its associated 5G communication. This architecture is application oriented and motivates a high data rate, which is a limitation with the existing shared 5G spectrum allocation. Though these high data rates are achievable with mmwave spectrum but not immediately realizable.

3) The KPI values for this use case will be within the KPI values specified for Ultra Reliable Low Latency communication (URLLC) and enhance Mobile Broadband communication (eMMB) in TS-22.261 [59], since we split and distribute our computing into tiny tiles rather than rendering from the network.

### 5.A.6	Potential New Requirements needed to support the use case

[PR 5.A.6-001] The 5G system shall support UE to discover compute offload services in the 5G system

[PR 5.A.6-002] The 5G system shall support to discover compute offload services availability to the UE upon request.

[PR 5.A.6-003] The 5G system shall support UE to discover multiple BS supporting compute offload services.

[PR 5.A.6-004] The 5G system shall support in agreement with the UE to decide single/multiple BS to offload compute offload services.

[PR 5.A.6-005] The 5G system shall support additional or enhancement of control plane required to support compute offload services.

[PR 5.A.6-006] The 5G system shall support to setup communication link between compute offload service in the 5G system and the UE with minimal communication overhead.

Note: Link could be a dedicated logical channel with RRC state or an IP link.

[PR 5.A.6-007] The 5G system shall support transfer of distributed compute offload results to the UE application directly1.

1 Transferring results from compute offload using Remote direct memory transfer (RDMA) to the UE's application will reduce the header overhead.

* * * End of Change * * * *