3GPP TSG-SA WG1 Meeting #101 	S1-230344

Athens, Greece, 20 - 24 February 2023	(revision of S1-230314)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.104 | CR | <094> | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | An additional usecase for Industrial edge cloud regarding digital twin usage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Verizon, Orange, Vodafone, Nokia, Huawei, Deutsche Telekom, NTT Docomo, Lenovo, China Unicom, Ericsson, Interdigital, Siemens(?) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGINDUS |  |  |  |  |  | Date: |  |  | 2023-01-30 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding clarification on Mobile robots section regarding the usage of edge computing plus adding an example edge cloud usecase in production manufacturing– it is a follow-up for 5G-ACIA-LS-2022-004 (S1-223255) to 3GPP |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added text in additional information section of Mobile Robots (A 2.2.3) to clarify that having an edge cloud system enables lower latency for Mobile robots control for critical operations - Added a new use case (digital twin in a production quality inspection) and possible corrective action. This use case deploys telemetry data/video processing locally for immediate inspection and corrective action if needed.A new subsection is added ( A.2.3.x) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The edge usecase benefits for Mobile robots and the production quality maintenance are perceived to be an important applications in Industrial environment. Without this update, an important usecase of edge computing and its requirements will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A 2.2.3 and the new Appendix subsection (A.2.3.x) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******       Start of  changes  ***********

### A.2.2.3	Mobile robots

Mobile robots and mobile platforms, such as automated guided vehicles, have numerous applications in industrial and intra-logistics environments and will play an increasingly important role in the Factory of the Future. A mobile robot essentially is a programmable machine able to execute multiple operations, following programmed paths to fulfil a large variety of tasks. This means, a mobile robot can perform activities like assistance in work steps, collaboration with other robots, e.g. for car assembly, and transport of goods, materials and other objects. Mobile robot systems are characterised by a maximum flexibility in mobility relative to the environment, with a certain level of autonomy and perception ability, i.e., they can sense and react with their environment.

Autonomous guided vehicles (AGVs) are a sub-group of mobile robots. AGVs are driverless and used for moving materials efficiently within a facility. A detailed overview of the state of the art of autonomous-guided-vehicle systems is provided elsewhere in the literature [16]. All mobile robots incorporate all functionalities needed for an AGV.

Today, the AGV’s control intelligence is hosted inside the AGV. In the future, centralised fleet control will be hosted in the edge cloud, which will require reliable wireless communication between the control entity and all AGVs. Also, the current paradigm of pre-describing a route for the AGV will be replaced with target-based navigation. This paradigm change will make AGV routes more flexible.

Mobile robots are monitored and controlled from a guidance control system. Radio-controlled guidance control is necessary to get up-to-date process information, to avoid collisions between mobile robots, to assign driving jobs to the mobile robots, and to manage the traffic of mobile robots. The mobile robots are track-guided by the infrastructure with markers or wires in the floor or guided by own surround sensors, like cameras and laser scanners.

Mobile robot systems can be divided in operation in indoor, outdoor and both indoor and outdoor areas. These environmental conditions have an impact on the requirements of the communication system, e.g., the handover process, to guarantee the required cycle times.

Where this document does not explicitly refer to AGVs, the term mobile robots applies to AGVs as well as to mobile robots.

Table A.2.2.3-1: Service performance requirements for mobile robots

| Use case # | Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value [%] | Communication service reliability: mean time between failures | End-to-end latency: maximum | Service bitrate: user experienced data rate | Message size [byte] | Transfer interval: lower bound | Transfer interval: target value (note) | Transfer interval: upper bound | Survival time | UE speed | # of UEs | Service area |
| 1 | > 99.999 9 | ~ 10 years | < target transfer interval value | – | 40 to 250 | – < 25 % of target transfer interval value | 1 ms to 50 ms | + < 25 % of target transfer interval value | target transfer interval value | ≤ 50 km/h | ≤ 2,000 | ≤ 1 km2 |
| 2 | > 99.999 9 | ~ 1 year | < target transfer interval value | – | 15 k to 250 k | – < 25 % of target transfer interval value | 10 ms to 100 ms | + < 25 % of target transfer interval value | target transfer interval value | ≤ 50 km/h | ≤ 2,000 | ≤ 1 km2 |
| 3 | > 99.999 9 | ~ 1 year | < target transfer interval value | – | 40 to 250 | – < 25 % of target transfer interval value | 40 ms to 500 ms | + < 25 % of target transfer interval value | target transfer interval value | ≤ 50 km/h | ≤ 2,000 | ≤ 1 km2 |
| 4 | > 99.999 9 | ~ 1 week | 10 ms | > 10 Mbit/s | – | – |  | – | – | ≤ 50 km/h | ≤ 2,000 | ≤ 1 km2 |
| NOTE: The transfer interval is not so strictly periodic in these use cases. The transfer interval deviates around its target value within bounds. The mean of the transfer interval is close to the target value. |  |  |  |  |  |  |  |  |  |  |  |  |

Use case one

Periodic communication for the support of precise cooperative robotic motion control (transfer interval: 1 ms), machine control (transfer interval: 1 ms to 10 ms), co-operative driving (10 ms to 50 ms).

Use case two

Periodic communication for video-operated remote control.

Use case three

Periodic communication for standard mobile robot operation and traffic management.

Use case four

Real-time streaming data transmission (video data) from a mobile robot to the guidance control system.

Additional information

AGVs have the following needs.

–	The direct-device control is time-critical since the communication involves safety-relevant functions such as emergency stop and the avoidance of obstacles.

–	For the implementation of swarm intelligence, position and availability information are needed. A possible route change due to a blocked route affects the routes of all other AGVs that will follow. The communication is less time-critical than for safety-relevant functions.

–	Camera-based navigation requires high data rates. Examples for camera-based navigation are the positioning of boxes/containers, detection of persons and obstacles, as well as creation and administration of a map for flexible navigation. Note that sensor-based navigation requires lower data rates than camera-based navigation.

- AGV route control and emergency safety related time critical latency and response can be achieved with an edge cloud where the edge infrastructure is located close to the AGVs

Mobile robots have additional needs.

–	The mobile robot provides an additional service during transport (for instance quality inspection, scanning of surroundings, asset identification, carrying of work pieces). In order to reduce the uplink data rate, pre-compression of data is possible directly on the device. The communication is not or at least less time-critical than the motion control of the mobile robot.

–	The mobile robot needs to interact with the periphery (for instance intelligent storage racks, stationary robots, and moving machines). This communication is time-critical. Interaction with the periphery can be relevant at the start point, end point, and also at several intermediate stations (for instance the collection of parts from intelligent storage racks). The exact position and orientation can be determined by a centering station and the AGV sensors. Additional scanning by the robot with a video camera may be necessary.

- In order to be able to process time-sensitive data from local sensors and devices (AGVs, robots, cobots, etc.) in an edge cloud, the edge infrastructure needs to be on-premise or close to the factory.

### A.2.3.x	Inspection in production systems

An Edge Computing use case example: Digital twin based production quality maintenance:

A digital twin is a virtual representation of a product or production systems. Digital twins are used to simulate, predict and optimize products and production systems. In future, digital twin production control system based on augmented reality based will be used in the factories.  In this usecase,  a digital twin’s  digital and virtual model of a function combined with other physical data to simulate real-time aspects of how a system operates.  A digital twin production control system can  be automated using machine data and the AI/ML trained data after applying the AI/ML algorithm  and further processing. The processed output can be translated to a control command  back to the device by a control function running on the edge cloud.

In another case, using telemetry data as input, the digital twin model’s output may be fed to an AR server for sending low latency AR streams toward the manual operator in the factory production area. At the same time, it can further be utilized as input by an AI/ML model.  A process control function can compare the machine data (example:position, rotation level, speed, sensor data, high-speed photography etc.) and perhaps a high-resolution video from the manufacturing line and  if necessary, it can send commands for corrective measure. In this example, the process control functions reside at the edge infrastructure and the inspection related corrective input is sent back to the production system control function.

Corrective actions/commands for misalignments from the processed output can be sent in two ways:

- Manual process with AR server: In this case the service performance requirements should follow the table A.2.4.2-1.

- Automatic process with AI/ML: This usecase and service performance requirements are described below in table A.2.3.x

The general high-level service requirements of the edge computing usecase for the digital twin based production inspection:

- High bandwidth wireless data for both uplink and downlink

  - exact number depends on video encoding, frame rate and video-resolution requirements

- Timing accuracy and low latency ( <=20ms)

- High availability of the communication network

- Security requirements: Data encryption, connection authentication, user authorization

- QoS methods to ensure quality of service performance over different UE to Application connection sessions (video streaming, sensor data, control data)

- UE Mobility and session continuity (optional)

Table A.2.3.x: Service performance requirements for automated inspection

| Use case # | Characteristic parameter |  |  | Influence quantity |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value [%] | Communication service reliability: mean time between failures | End-to-end latency: maximum | Message size [byte] | Transfer interval: lower bound | Transfer interval: target value | Transfer interval: upper bound | Survival time | UE speed | # of UEs | Service area (note) |
| 1 | 99.999 | ≥ 1 year | < target transfer interval value | 20 – large packets | -20 % of target transfer interval value | <=20 ms | +20% of target transfer interval value | Variable depending upon vertical industry | typically stationary | < 5 typically | typically ≤100 m x 100 m x 50 m |
| NOTE: Length x width x height. |  |  |  |  |  |  |  |  |  |  |  |

Use case one

The periodic telemetry data and video images are used from the digital twin in the production system for analysis and then the processed outcome is sent back to the system for any adjustment of  the machine components.

The following diagram explains the above digital twin usecase steps to manage the production in a factory (both manual and automated operations)

- Low-latency AR overlays and incorporation of AI/ML techniques to identify manufacturing issues and improve product quality as well as to enable offline adjustments for optimization, adaptations, and preventive operations on the machines

******       End of  changes  ***********