3GPP TSG SA WG 1 Meeting #101 	S1-230383230353

Athens, Greece, 20 - 24 February 2023	(revision of S1-230104,353)

Source:	Tencent

pCR Title:	Pseudo-CR on New Use Cases on Machine-type communication

Draft Spec:	3GPP TR 22.916-020

Agenda item:	7.x

Document for:	Approval

Contact:	Gilles Teniou (teniou@tencent.com)

1. Introduction

The Study on Network of Service Robots with Ambient Intelligence (FS_SOBOT) has the following objectives as described in the Planning of Stage-1 Rel-19 topics.

SA1#101 (Feb 2023): 70%

Remarks: add necessary terminology, add new use cases relating to (representative) application fields /scenarios with focus on collaborative operations of multiple robots (e.g., unstructured job site env., media communication for robotic applications, aerial/underwater and safety-critical env.)

TR sent for information

This contribution proposes to progress the use cases aspects with a focus on media communication for robotic applications.

2. Reason for Change

New use cases related to media formats for machine-type communication in the FS_SOBOT study.

3. Conclusions

None.

4. Proposal

If agreeable, it is proposed to add the following changes to the draft TR 22.916.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 22.104, "Service requirements for cyber-physical control applications in vertical domains; Stage 1".

[3] 	3GPP TS 22.261, "Service requirements for the 5G system; Stage 1".

[4] 	3GPP TS 22.263, "Service requirements for video, imaging and audio for professional applications (VIAPA); Stage 1".

[5] 	Next G Alliance Report: 6G Applications and Use Cases, May 2022; https://www.nextgalliance.org/wp-content/uploads/dlm_uploads/2022/07/NGA-Perspective-Brochure-V6.pdf

[6] 	K.-D. Lee, "A Smart Network of Service Robots: Technical Challenges and Design Considerations, " IEEE Communications Magazine, pp. 28-34, August 2021.

[7] 	K.-D. Lee and C. Gray-Preston, "Everyday Living Assisted by 6G Applications and Solutions," IEEE Wireless Communications Magazine, October 2022.

[8]	W. Zhuang, Y. Shen, L. Li, C. Gao and D. Dai. “Develop an Adaptive Real-Time Indoor Intrusion Detection System Based on Empirical Analysis of OFDM Subcarriers.” Sensors (Basel, Switzerland). 2021 Mar;21(7):2287

[XX]	MPEG Use cases and requirements for Video Coding for Machines, https://www.mpeg.org/wp-content/uploads/mpeg_meetings/138_OnLine/w21545.zip

[YY]	3GPP TR 22.886, "Study on enhancement of 3GPP support for 5G V2X services”.

* * * Second Change (All new text) * * * *

## 5.X	Media-related use cases

### 5.X.1	General description

#### 5.X.1.1	Video surveillance

The use cases focus on media aspects of service robots composed of visual sensors such as cameras and equiped with capabilities to analyze video signals (such as feature extraction, object tracking and detection…) and are able to process this information (i.e., take consequent action). Those use cases are directly related to the standardization effort in MPEG for defining a media compression format called Video Coding for Machines (VCM). Associated MPEG use cases and requirements are available here [XX].

This use case addresses the scenarios in which automatic object detection and tracking is achieved with video cameras. One illustration is the monitoring of indoor environments in which features of the captured video need to be extracted, such as intrusion detection, fire detection, trusted individuals recognition…

The basic service configuration is depicted in the figure AA below.

Figure AA: Use case on video surveillance

In this scenario, video cameras capture a video stream together with related features extracted from pre-processing such as object detection and tracking and are sent to the back-end server for analysis and processing.

The uplink bandwidth is expected to be used in an optimized way in a configuration in which there is no need for human understandability of the signal. The image quality requirements are thus limited to the machine interpretation.

#### 5.X.1.2	Intelligent transportation

In a smart traffic system, cars may need to communicate features between each other and other sensors in order to perform different tasks. Sensors in the infrastructure may communicate features towards different vehicles, which then use these features to do object detection, lane tracking, etc. Final processing of these features is done on the individual vehicles.

An example is illustrated Figure BB. The front car with multiple cameras sees the surrounding environment and detect and recognize objects such as cars, pedestrians, or street furniture or even recognize events such as traffic jams or accidents potentially using (deep) neural networks. The processed data (feature maps) may be consumed internally for desired tasks and/or the extracted features may be compressed and transmitted to other surrounding cars/infrastructure (e.g., side road cell/grid) for further analysis. Sending a standardized compact bitstream is essential for interoperability between various vendors, IoV (Internet of Vehicle), and IoT (Internet of Things) applications.

Figure BB: Use case on V2X communications

The bandwidth is expected to be used in an optimized way in a configuration in which there is no need for human understandability of the signal. The image quality requirements are thus limited to the machine interpretation.

### 5.X.2	Related existing service requirements

There is no service requirement known that relates to the bitrate efficiency of robotics video signal, however the Release 16 study on enhancement of 3GPP support for 5G V2X services in TR 22.886 [YY] describes the KPIs for video data sharing particularly for machine-centric video data analysis. The following parameters are used as related KPIs:

-	Latency: less than 10ms.

-	Data rate 100 – 700 Mbps.

-	Reliability 99.99%.

### 5.X.3	Challenges and potential gaps

The following applicable aspects are identified and recommended for further study and can be further considered with other ongoing or recently completed Studies if applicable.

Functionality aspects:

[CPG-5.X-001] 5G system is expected to convey media signals for machine type communications that support feature extraction and descriptor signalling.

NOTE : The type of feature and descriptor is out of scope of this document.

Efficiency aspects:

[CPG-5.X-002] 5G system is expected to meet the service requirements to enable communication between robots using media formats offering at least a better compression efficiency than the 3GPP codecs already specified for human consumption.

* * *End of Change * * * *