3GPP TSG SA WG 1 Meeting #101	S1-230385

Athens, Greece, 20 - 24 February 2023	(revision of S1-230066)

Source:	Nokia

pCR Title:	22.851 pCR: Clarifications on UE steering

Draft Spec:	3GPP TR 22.851-030

Agenda item:	7.4 (FS_NetShare: Study on Network Sharing Aspect)

Document for:	Approval

Contact:	Laurent-Walter Goix (laurent-walter.goix@nokia.com)

Abstract: This document provides clarifications on UE steering for FS_NetShare, which is currently FFS.

1. Introduction

In SA1 #100, a use case was added to TR 22.851 that introduced the notion of “steering” UEs. However, the related requirements were put FFS.

230385 : changes as discussed :

- updated “steering guidance information” > “steering indication”

- reworded to highlight the action of the Participating operator

- updated “may” to “could” (informative wording)

2. Reason for Change

Steering UE towards certain 3GPP access networks is already specified in TS22.261. However, it has to be clarified that within another operator, such steering only represents a guidance information that may or may not be applied.

3. Conclusions

This contribution proposes a rewording to address PR#2 FFS.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.851-030.

First Change

### 5.7.6	Potential New Requirements needed to support the use case

[PR 5.7.6-001] Subject to the agreement between Hosting Operators and a Participating Operator, the 5G system shall support a mechanism to authorize UE with a subscription to the Participating Operator to obtain services in a Shared RAN of Hosting Operators using Indirect Network Sharing.

Editor’s Note: Requirement #001 is FFS.

[PR 5.7.6-002] In case of Indirect Network Sharing, based on the Hosting and Participating Operators’ policies, the 5G system shall enable the Participating Operator to provide steering indication to the Hosting Operator.

Note: Such indication for a given UE of the Participating Operator could be applied by the Hosting Operator to select amongst its available shared access network(s).

:

End of Changes