3GPP TSG-SA WG1 Meeting #101 	S1-230133

Athens, Greece, 20 – 24 Feb., 2023

Source:	OPPO

pCR Title:	Consolidation on Functional Requirement of AIML-Ph2

Draft Spec:	3GPP TR 22.840

Agenda item:	7.6

Document for:	Approval

Contact:	Yang Xu (xuyang@oppo.com)

Abstract: it is proposed to make CPR mapping table for consolidation of AIML-Ph2.

Discussion

Based on all functional requirements captured in the latest version of TR 22.876v020, it made a mapping table for consolidation.

| Potential Requirement | Clean-up Steps Taken | Consolidated Potential Requirement |
| --- | --- | --- |
| [P.R.5.1-001] The 5G system shall have the means to modify the QoS of communication via direct device connectionpath. NOTE 1: The split point selection is dynamic. In consequence, the amount of intermediate data will vary. To maintain the end to end latency, the QoS is adjusted. | Adopt it to CPR-1 with further clarifying the modification of QoS is to ensure an E2E latency is not affected when a relay UE is involved for task offloading | [CPR-1] The 5G system shall have the means to modify the QoS of communication via direct device connection path ensuring an E2E latency is not affected when a relay UE is involved for task offloading |
| [P.R.6.1-003] The 5G system shall support a mechanism for an authorized third-party to negotiate a suitable QoS of direct device connections for a group of UEs to exchange data with each other | Adopt it to CPR-2 | [CPR-2] The 5G system shall support a mechanism for an authorized third-party to negotiate a suitable QoS of direct device connections for a group of UEs to exchange data with each other |
| [P.R. 7.2-001] 5GS shall be able to support an aggregated QoS for a group UEs served by a relay UE. | Merge them into one PCR for aggregated QoS. Adopt it to CPR-3 | [CPR-3] 5GS shall be able to support an aggregated QoS for a group UEs served by a relay UE by provision an aggregated QoS to a relay UE. |
| [P.R. 7.2-002] 5GS shall be able to provision an aggregated QoS to a relay UE for a group-based service. |  |  |
| [P.R.6.1-002] Subject to user consent and operator policies, the 5G system shall be able to provide means for an authorized third-party to authorize a group of UEs to exchanging data with each other via direct device connection. | Authorizaition for UEs in a group to perform communication via SL Aopt it to CPR-4 | [CPR-4] Subject to user consent and operator policies, the 5G system shall be able to provide means for an authorized third-party to authorize a group of UEs to exchanging data with each other via direct device connection. |
| [P.R.6.1-001] Subject to user consent, operator policies and regional or national regulatory requirements, the 5G system shall be able to support means to monitor a direct device connection and expose corresponding monitoring information (e.g. experienced data rate, latency) to an authorized 3rd party. NOTE: The monitoring information in [P.R.6.1-001] doesn’t include any user position-related data. | Monitoring and exposure of SL resource such as latency and data rate Aopt it to CPR-5 | [CPR-5] Subject to user consent and operator policies, the 5G system shall be able to provide means for an authorized third-party to authorize a group of UEs to exchanging data with each other via direct device connection. |
| [P.R.6.1-004] Subject to user consent, operator policies and regional or national regulatory requirements, the 5G system shall support means to monitor and expose to a 3rd party, characteristics of traffic intended to be relayed by a UE participating in the communication. | Traffic characteristics for relay service. Adopt it to CPR-6 | [CPR-6] Subject to user consent, operator policies and regional or national regulatory requirements, the 5G system shall support means to monitor and expose to a 3rd party, characteristics of traffic intended to be relayed by a UE participating in the communication. |
| [P.R.5.1-002] The 5G system shall be able to collect charging information for proximity-based work task offloading. | Charging requirement for offloading task. Adopt it to CPR-7 | [CPR-7] The 5G system shall be able to collect charging information for proximity-based work task offloading. |

****************Start of Change *************

8	Consolidated potential requirements and KPIs

8.1	Consolidated potential requirements

| Potential Requirement | Clean-up Steps Taken | Consolidated Potential Requirement |
| --- | --- | --- |
| [P.R.5.1-001] The 5G system shall have the means to modify the QoS of communication via direct device connectionpath. NOTE 1: The split point selection is dynamic. In consequence, the amount of intermediate data will vary. To maintain the end to end latency, the QoS is adjusted. | Adopt it to CPR-1 with further clarifying the modification of QoS is to ensure an E2E latency is not affected when a relay UE is involved for task offloading | [CPR-1] The 5G system shall have the means to modify the QoS of communication via direct device connection path ensuring an E2E latency is not affected when a relay UE is involved for task offloading |
| [P.R.6.1-003] The 5G system shall support a mechanism for an authorized third-party to negotiate a suitable QoS of direct device connections for a group of UEs to exchange data with each other | Adopt it to CPR-2 | [CPR-2] The 5G system shall support a mechanism for an authorized third-party to negotiate a suitable QoS of direct device connections for a group of UEs to exchange data with each other |
| [P.R. 7.2-001] 5GS shall be able to support an aggregated QoS for a group UEs served by a relay UE. | Merge them into one PCR for aggregated QoS. Adopt it to CPR-3 | [CPR-3] 5GS shall be able to support an aggregated QoS for a group UEs served by a relay UE by provision an aggregated QoS to a relay UE. |
| [P.R. 7.2-002] 5GS shall be able to provision an aggregated QoS to a relay UE for a group-based service. |  |  |
| [P.R.6.1-002] Subject to user consent and operator policies, the 5G system shall be able to provide means for an authorized third-party to authorize a group of UEs to exchanging data with each other via direct device connection. | Authorizaition for UEs in a group to perform communication via SL Aopt it to CPR-4 | [CPR-4] Subject to user consent and operator policies, the 5G system shall be able to provide means for an authorized third-party to authorize a group of UEs to exchanging data with each other via direct device connection. |
| [P.R.6.1-001] Subject to user consent, operator policies and regional or national regulatory requirements, the 5G system shall be able to support means to monitor a direct device connection and expose corresponding monitoring information (e.g. experienced data rate, latency) to an authorized 3rd party. NOTE: The monitoring information in [P.R.6.1-001] doesn’t include any user position-related data. | Monitoring and exposure of SL resource such as latency and data rate Aopt it to CPR-5 | [CPR-5] Subject to user consent and operator policies, the 5G system shall be able to provide means for an authorized third-party to authorize a group of UEs to exchanging data with each other via direct device connection. |
| [P.R.6.1-004] Subject to user consent, operator policies and regional or national regulatory requirements, the 5G system shall support means to monitor and expose to a 3rd party, characteristics of traffic intended to be relayed by a UE participating in the communication. | Traffic characteristics for relay service. Adopt it to CPR-6 | [CPR-6] Subject to user consent, operator policies and regional or national regulatory requirements, the 5G system shall support means to monitor and expose to a 3rd party, characteristics of traffic intended to be relayed by a UE participating in the communication. |
| [P.R.5.1-002] The 5G system shall be able to collect charging information for proximity-based work task offloading. | Charging requirement for offloading task. Adopt it to CPR-7 | [CPR-7] The 5G system shall be able to collect charging information for proximity-based work task offloading. |

*******************End of Change*****************