3GPP TSG-SA WG1 Meeting #101 	   S1-230396

Athens, Greece, 20 - 24 February, 2023

Title:	Update on AI Model Transfer Management through Direct Device Connection

Agenda Item:	7.7

Source:	China Telecom, OPPO

Contact:	wangheng26@chinatelecom.cn, xuyang@oppo.com

Abstract: This document adds the KPI requirements for use case 6.1 and makes related editorial works in FS_AIML_Ph2 TR 22.876 version 0.2.0.

**************** First Change ******************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]		3GPP TR 22.874, Study on traffic characteristics and performance requirements for AI/ML model transfer in 5GS (Release 18)

[3]					3GPP TS 22.104, Service requirements for cyber-physical control applications in vertical domains

[4]		Huaijiang Zhu, Manali Sharma, Kai Pfeiffer, Marco Mezzavilla, Jia Shen, Sundeep Rangan, and Ludovic Righetti, “Enabling Remote Whole-body Control with 5G Edge Computing”, to appear, in Proc. 2020 IEEE/RSJ International Conference on Intelligent Robots and Systems. Available at: https://arxiv.org/pdf/2008.08243.pdf

[5]	B. Kehoe, S. Patil, P. Abbeel, and K. Goldberg, “A survey of research on cloud robotics and automation,” IEEE Transactions on automation science and engineering, vol. 12, no. 2, pp. 398–409, 2015.

[6]	M. Chen, K. Huang, W. Saad, M. Bennis, A. V. Feljan, and H. V. Poor, “Distributed Learning in Wireless Networks: Recent Progress and Future Challenges”IEEE JOURNAL ON SELECTED AREAS IN COMMUNICATIONS, VOL. 39, NO. 12, DECEMBER 2021

[7]	M. Chen, H. V. Poor, W. Saad, and S. Cui, “Wireless communications for collaborative federated learning,” IEEE Commun. Mag., vol. 58, no. 12, pp. 48–54, Dec. 2020

[8]		Jianmin Chen, Xinghao Pan, Rajat Monga, Samy Bengio, Rafal Jozefowicz, “Revisiting Distributed Synchronous SGD,” arXiv preprint arXiv: 1604.00981, 2016

[9] 		Shuxin Zheng, Qi Meng, Taifang Wang, Wei Chen, Nenghai Yu, Zhi-Ming Ma, Tie-Yan Liu, “Asynchronous Stochastic Gradient Descent with Delay Compression” arXiv: 1609.08326, 2020

[10]	3GPP TR 21.905: "Service requirements for the 5G system".

[x1]			https://github.com/open-mmlab/OpenPCDet

…

**************** Second Change ******************

6.1	AI Model Transfer Management through Direct Device Connection

6.1.1	Description

Based on the earlier study in phase one 3GPP TR 22.874 [2], operators can provide services to help manage and distribute the AI/ML models especially in the edge server so that the UE can acquire a proper model immediately. However, when a lot of UEs requesting for the same model at the same time or the UE is blocked by barriers with poor connection with the base station, the model transfer process will become longer than expected.

To overcome this difficulty, as shown in Fig.1, a volunteer UE which is well connected to the base station can help relaying AI/ML models or receive and store AI/ML models first. Then, the other UEs can download AI/ML models from the volunteer UE through direct device connection. In this way, all UE can have a stable and reliable model transfer process while the radio resource of the base station can be saved. Besides, the volunteer UE can transfer the stored models to other volunteer UEs under operator’s control.

The selection of volunteer UE can be realized by local network policies and strategies. And it also can be exposed as a capability to the 3rd party company when the company wants to choose one or a few certain UEs to be volunteer UEs in an activity. For example, a travel company may assign the tour guides’ Augmented Reality (AR) headsets as volunteer UEs in a carnival through the operator’s network exposure. The travel company may sign a higher quality plan for tourist guides’ devices to provide better user experience for following tourists. Meanwhile, operator can benefit from the alternative open service based on AI/ML model management capabilities and may avoid low Quality of Service due to crowding direct connections to base stations during the carnival.

Fig.1 AI/ML Model management through direct device connection

6.1.2	Pre-conditions

The operator’s MEC near the Jurassic Park stores a variety of AI/ML models according to the the park company’s requirements. And it is capable to transfer the stored model to the device such as AR headset.

The operator rolls out a new high quality plan which can allow the user customizes own Service Level Agreement (SLA) for specific network address access and data (e.g. AI/ML Models) download. As a trade-off, the user’s device will help transfer the same data through direct device connection to nearby devices sharing common aspiration.

The AR headset can transfer the stored AI/ML model to the other AR headsets. However, the AR headset cannot store all models for different scenarios due to limited storage. Indeed, a model needs to be downloaded when or a few seconds before the UE first appears in the certain area.

Alice and Bob are tour guide hired by Jurassic Park and their real-time positions can be acquired when they are in the park based on signed agreements.

All of AR headsets should in the coverage of the base stations serving for the Jurassic Park.

6.1.3	Service Flows

1.	Jurassic Park provides panorama AR tour guide services in a commercial area and a tropical rainforest area. AR headsets need to download Model A and B (both are VGG-16, 552MByte) respectively.

2.	To provide high quality user experience, Jurassic Park company indicates to the operator that AR headsets require to download model A in area A and model B in area B.

3.	The Jurassic Park company signs a high quality plan for tour guide Alice and Bob’s AR Headsets for providing better service to the tour group using direct device connection.

4.	When Bob and his tour group enter area A, their headsets request for the Model A. The operator network finds they requested the same model and Bob is a signed volunteer UE, then triggers to establish a QoS acceleration for Bob’s model downloading timely within 1 second. Meanwhile, Jurassic Park requests the operator network to inform Bob to help transfer the model to all other UE near Bob. Also, the operator network informs all other UE near Bob that Bob can provide the model as well. The UE which is a little far from Bob (e.g. out of Bob’s coverage) will still download the model through the base station directly.

5.	Alice and her group are 10 meters far from Bob and also move towards to area A. Jurrassic Park predicts their desire model based on their movement and finds Bob has already downloaded it based on the model transferring records. Jurassic Park requests operator network to inform Alice that she can request model from Bob. Meanwhile, the operator network indicates all other UE near Alice to download the model from Alice.

6.	For Alice and Bob, they can see the status of all direct device connections to themselves through network exposures providing by operators (e.g. monitored bandwidth and latency of each direct device connection)

7.	When Alice and Bob notice that their groups have a poor QoS of model transfer through direct device connection, they can send a request to the park company for promoting the performance of their direct device connections and the park company will send a similar message to the operator through network exposure to active a temporary acceleration of these direct device connections (e.g. expand the bandwidth of each direct device connection).

6.1.4	Post-conditions

1.	The tourists can enjoy the continuous AR services with smooth model switchover when their location and responding models change.

2.	Tour group’s AR headsets provides user experience of the panorama AR tour guide services that can help retrain and improve AI/ML models in operator’s MEC by Jurassic Park company (e.g. Federated/Distributed Learning).

3.	the operator network performs analytics, based on network statistics and quality of experience reported by Jurassic Park company, to improve and optimized the model transfer process (e.g. setting constraints for maximum direct device connection for one volunteer UE and choose a temporary volunteer UE for sharing model transfer task).

6.1.5	Existing features partly or fully covering the use case functionality

In 3GPP TS 22.261 [8] clause 6.27.2 "Requirements"

The 5G system shall be able to make the position-related data available to an application or to an application server existing within the PLMN, external to the PLMN, or in the User Equipment.

In 3GPP TS 22.261 [8] clause 6.9.2.4 "Relay UE Selection"

The 3GPP system shall support selection and reselection of relay UEs based on a combination of different criteria e.g.

-	the characteristics of the traffic that is intended to be relayed (e.g. expected message frequency and required QoS),

-	the subscriptions of relay UEs and remote UE,

-	the capabilities/capacity/coverage when using the relay UE,

-	the QoS that is achievable by selecting the relay UE,

-	the power consumption required by relay UE and remote UE,

-	the pre-paired relay UE,

-	the 3GPP or non-3GPP access the relay UE uses to connect to the network,

-	the 3GPP network the relay UE connects to (either directly or indirectly),

-	the overall optimization of the power consumption/performance of the 3GPP system, or

-	battery capabilities and battery lifetime of the relay UE and the remote UE.

NOTE:	Reselection may be triggered by any dynamic change in the selection criteria, e.g. by the battery of a relay UE getting depleted, a new relay capable UE getting in range, a remote UEs requesting additional resources or higher QoS, etc.

22.261 v18.6.0 6.40.2

Based on operator policy, the 5G system shall be able to provide an indication about a planned change of bitrate, latency, or reliability for a QoS flow to an authorized 3rd party so that the 3rd party AI/ML application is able to adjust the application layer behaviour if time allows. The indication shall provide the anticipated time and location of the change, as well as the target QoS parameters.

Subject to user consent, operator policy and regulatory constraints, the 5G system shall be able to support a mechanism to expose monitoring and status information of an AI-ML session to a 3rd party AI/ML application.

NOTE 2:	Such mechanism is needed for AI/ML application to determine an in-time transfer of AI/ML model.

Subject to user consent, operator policy and regulatory requirements, the 5G system shall be able to expose information (e.g. candidate UEs) to an authorized 3rd party to assist the 3rd party to determine member(s) of a group of UEs (e.g. UEs of a FL group).

6.1.6	Potential New Requirements needed to support the use case

6.1.6.1	Potential Functionality Requirements

[P.R.6.1-001] Subject to user consent, operator policies and regulatory requirements, the 5G system shall be able to support means to monitor a direct device connection and expose corresponding monitoring information (e.g. experienced data rate, latency) to an authorized 3rd party.

NOTE:  The monitoring information in [P.R.x.y-001] doesn’t include any user position-related data.

[P.R.6.1-002] Subject to user consent and operator policies, the 5G system shall be able to provide means for an authorized third-party to authorize a group of UEs to exchanging data with each other via direct device connection.

[P.R.6.1-003] The 5G system shall support a mechanism for an authorized third-party to negotiate a suitable QoS of  direct device connections for a group of UEs to exchange data with each other.

[P.R.6.1-004] Subject to user consent, operator policies and regulatory requirements, the 5G system shall support means to monitor, characteristics of traffic relayed by a UE participating in the communication and expose to 3rd party.

6.1.6.2	Potential KPI Requirements

[P.R.x.y-005] The 5G system shall support to use direct device communication to transmit the AI/ML model of image recognition and 3D object recognition with the following KPIs.

Table 6.1.6.2-1: KPIs for image recognition and 3D object recognition using direct device connection

| # Model Type | # Max allowed DL end-to-end latency | # Experienced data rate # in PC5 | # Model size | # Communication service availability |
| --- | --- | --- | --- | --- |
| AlexNet | 1s | 1.92 Gbit/s | 240 MByte | 99.9 % |
| ResNet-152 | 1s | 1.92 Gbit/s | 240 Mbyte | 99.9 % |
| ResNet-50 | 1s | 0.8 Gbit/s | 100 Mbyte | 99.9 % |
| GoogleNet | 1s | 0.218 Gbit/s | 27.2 Mbyte | 99.9 % |
| Inception-V3 | 1s | 0.736 Gbit/s | 92 Mbyte | 99.9 % |
| PV-RCNN | 1s | 0.4 Gbit/s | 50 Mbyte | 99.9 % |
| PointPillar | 1s | 0.14 Gbit/s | 18 Mbyte | 99.9 % |
| SECOND | 1s | 0.16 Gbit/s | 20 Mbyte | 99.9 % |
| For the size of image recognition model, it refers to table 6.1.1-1 in TR22.874 [2], for the size of 3D object recognition model, see [x1]. Reliability is assumed to be [99.9 – 99.999]% |  |  |  |  |

************* End of Change ***************