3GPP TSG SA WG 1 Meeting #101 	S1-230409

Athens, Greece, 20 - 24 February 2023	(revision of S1-220192, 3620)

Source:	China Mobile

Title:	Use case of cooperation between metaverse service and network

Draft Spec:	3GPP TR 22.856-030

Agenda Item:	7.3

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com)

Abstract: This paper proposes a deep cooperation between metaverse and network in order to support the better communication and user experience in metaverse.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.228: "Service requirements for the Internet Protocol (IP) Multimedia core network Subsystem (IMS)".

[3]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services".

[4]	3GPP TS 22.101: "Service principles".

[5]	3GPP TS 22.261: "Service requirements for the 5G system".

…

[xx]	Y. Sun, Z. Chen, M. Tao, and H. Liu, “Communications, caching, and computing for mobile virtual reality: Modelingand trade off,” IEEE Trans. Commun., vol. 67, no. 11, pp. 7573–7586, Nov. 2019.

[xy]	Y. Cai, J. Llorca, A. M. Tulino and A. F. Molisch, "Compute- and Data-Intensive Networks: The Key to the Metaverse," 2022 1st International Conference on 6G Networking (6GNet), 2022

* * * Next Change * * * *

5.R	Use case on cooperation between metaverse and network using interactive XR

5.R.1	Description

The mobile metaverse allows users to access an endless virtual world at anytime and anywhere through their terminals. The mobile metaverse are expected to behave as the real world, which means in addition to rendering a virtual environment like the physical world, the perceived spatial-temporal consistency is also the key point to achieve an immersive location agnostic service experience.

In mobile metaverse, spatial-temporal consistency for single user could mean, for example, dropping a virtual pen and seeing this pen fall subsequently. While for multiple players, this consistency could mean, for example, that one person cuts down a tree and other people see the tree falling down. This user experience requires the motion-to-photon latency in the range of 7 ms to 15ms [5] at least for a single user viewing the consequence of her own actions. Immersive VR requires the delivery of massive amount of data (in the order of Gigabyte) at ultra-low latency (less than 20 ms) [xx].

NOTE: 	For location agnostic service experience involving multiple users who are not in the same location, the requirements above do not apply, since the service can impose ordering and timing of representations of virtual events in an arbitrary manner.

It should be noted that the computation resources for rendering involved in the mobile metaverse is different from the cloud gaming and traditional VR. For example, running a typical massively multiplayer online game today requires multiple teraFLOPS of graphics horsepower, and the demand is expected to grow by two orders of magnitude to create fully immersive mobile metaverse experiences. [xy]

For the mobile metaverse world, distributed computation is an inevitable processing mode, so the selection of proper servers and data centers should consider the requirements of network delay, processing delay, storage and computation resource. The goal is to minimize the user's perception of delay.

Therefore, in order to obtain consistent experience in mobile metaverse service anytime and anywhere, deep collaboration between mobile metaverse and 5G network is needed. The potential collaboration aspects may include caching location, computation location, communication path, traffic scheduling and resource allocation in network. For example, when a service request emerges, the network control policy needs to coordinate the selection of (i) caching locations to provide digital objects, (ii) computation locations to execute service functions, and (iii) communication paths to route all associated data streams, jointly optimized with dynamic decisions on (iv) traffic scheduling and (v) resource allocation at all network locations. [xy]

5.R.2	Pre-conditions

There are about 12,000 players sign up for a popular game and appear simultaneously in a specific setting, such as Eve Online in 2021. Due to the limitations of the existing server processing, it is not possible to support such a large number of high concurrency, so the network and application server need to cooperate to support the distribution of visitors to other servers while ensuring low latency requirement by XR applications.

5.R.3	Service Flows

1. Bob is a player who attends a popular AR interactive game and gathers with others in a shared environment, they are aware of each other’s action so that they need high synchronization.

2. The service provider will provide deployment information of each server to the 5G network, and request the Bob’s physical location and transmission delay in 5G network.

3. According to the cooperation agreement with application, 5G network will expose information to service providers, including the physical location and network delay of specific terminals or a group of terminals. The network delay includes the delay inside 5G system (UE to PSA UPF) and the latency information between PSA UPF and some potential servers.

4. The new server is selected by the service provider according to the UE location, network delay, business requirements, computation resource and storage resource of application servers. The decision result will be sent back to 5G network. Then the 5G network can then formulate corresponding policies for the service flows.

5. The content information will be synchronized to the new server in real time. 5G network should support the ultra-low latency data transmission, potentially among multiple operators.

5.R.4	Post-conditions

Bob will have a good experience in this interactive AR game.

5.R.5	Existing features partly or fully covering the use case functionality

3GPP started the work of edge computing from R15 to R18. In R15, AF influence mechanism is introduced to inform the 5G network of the application deployment information to assist UPF selection. In R16, 5G system supports QoS monitoring mechanism for end-to-end delay monitoring for URLLC services. In R17, 5GS supports to solve the problem of edge DNS selection and service migration between different edge platforms. In R18, the work focuses on the edge computing platform access from other operator network, and the distribution of network policies for a group of local UEs.

5.R.6	Potential New Requirements needed to support the use case

[PR 5.R.6-1] Subject to operators’ policy, upon request from the mobile metaverse service, the 5G system shall be able to expose the network latency information to the mobile metaverse service.

Note: Network latency information refer to the latency 5G system can acquire directly (e.g. latency from UE to UPF) and latency information 5G system can acquire assisted by other mechanisms (e.g. between UPF and specific server, etc.).  This information might be used by the mobile metaverse service, which is out of scope of 3GPP, for example to select a suitable QoS, transmission path and server.