3GPP TSG SA WG 1 Meeting #101 	S1-230413

Athens, Greece, 20 – 24 February 2023	(revision of S1-230266)

Source:	Samsung

pCR Title:	Pseudo-CR on 5.X: New use case on IMS-based Avatar Call Support for Accessibility

Draft Spec:	3GPP TR 23.856 v.0.3.0

Agenda item:	7.3

Document for:	Approval

Contact:	Erik Guttman <erik.guttman@samsung.com>

Abstract: This use case considers the potential for use of IMS 3D Avatar Call to improve the possibilities for accessible communication and charging requirements.

1. Introduction

Accessibility is a very important goal: those with diverse communication capabilities use telecommunications systems. The ITU-T SG16 defines 'Total Conversation' as the extension of communication services to those for whom additional accessibility is required. "Total Conversation is an ITU-T defined concept that encompasses voice telephony, video telephony and text telephony. The idea is that it gives everyone the chance to communicate with one another regardless of whether they are hearing, hearing impaired or deaf." [1]

This use case considers the possibility to extend support beyond voice, video and text telephony to avatar communication to demonstrate exciting opportunities.

[1] https://www.itu.int/en/ITU-T/studygroups/com16/accessibility/Pages/telecom.aspx

2. Reason for Change

Accessibility is an important goal for telecommunications. There are existing 3GPP standards in this area that may benefit from additional support that avatar communication makes possible.

3. Discussion

Please see the use case below.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.856 v.0.3.0.

FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.228: "Service requirements for the Internet Protocol (IP) Multimedia core network Subsystem (IMS)".

[3]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services".

[4]	3GPP TS 22.101: "Service principles".

[5]	3GPP TS 22.261: "Service requirements for the 5G system".

[6] 	Talaba D, Horvath I, Lee K H, "Special issue of Computer-Aided Design on virtual and augmented reality technologies in product design," Computer-Aided Design 42:361 - 363.

[7] 	Wang X, Tsai J J-H, "Collaborative Design in Virtual Environments," In: ISCA 48:1-2.

[8] 	Benford S, Greenhalgh C, Rodden T, "Collaborative virtual environments." Communications of the ACM 44: 79–85.

[9]	M. Eid, J. Cha, and A. El Saddik, "Admux: An adaptive multiplexer for haptic-audio-visual data communication", IEEE Tran. Instrument. and Measurement, vol. 60, pp. 21–31, Jan 2011.

[10]	K. Iwata, Y. Ishibashi, N. Fukushima, and S. Sugawara, "QoE assessment in haptic media, sound, and video transmission: Effect of playout buffering control", Comput. Entertain., vol. 8, pp. 12:1–12:14, Dec 2010.

[11]	N. Suzuki and S. Katsura, "Evaluation of QoS in haptic communication based on bilateral control", in IEEE Int. Conf. on Mechatronics (ICM), Feb 2013, pp. 886–891.

[12]	E. Isomura, S. Tasaka, and T. Nunome, "A multidimensional QoE monitoring system for audiovisual and haptic interactive IP communications", in IEEE Consumer Communications and Networking Conference (CCNC), Jan 2013, pp. 196–202.

[13]	A. Hamam and A. El Saddik, "Toward a mathematical model for quality of experience evaluation of haptic applications", IEEE Tran. Instrument. and Measurement, vol. 62, pp. 3315–3322, Dec 2013.

[14]	O. Holland et al., "The IEEE 1918.1 "Tactile Internet" Standards Working Group and its Standards," Proceedings of the IEEE, vol. 107, no. 2, Feb. 2019.

[15]	Lee, Donghwan et. al., “Large-scale Localization Datasets in Crowded Indoor Spaces,” NAVER LABS, NAVER LABS Europe, CVPR 2021.

[16]	3GPP TR 22.837, " Study on Integrated Sensing and Communication."

[17]	Alriksson, F., Kang, D.H., Phillips, C., Pradas, J.L. and Zaidi, A., 2021. XR and 5G: Extended reality at scale with time-critical communication. Ericsson Technology Review.

[18]	3GPP TR 26.928, "Extended Reality (XR) in 5G."

[19] 	Ge, Y., Wen, F., Kim, H., Zhu, M., Jiang, F., Kim, S., Svensson, L. and Wymeersch, H., 2020. 5G SLAM using the clustering and assignment approach with diffuse multipath. Sensors, 20(16), p.4656.

[20] 	Kim, H., Granström, K., Gao, L., Battistelli, G., Kim, S. and Wymeersch, H., 2020. 5G mmWave cooperative positioning and mapping using multi-model PHD filter and map fusion. IEEE Transactions on Wireless Communications, 19(6), pp.3782-3795.

[21] 	Liu, A., Huang, Z., Li, M., Wan, Y., Li, W., Han, T.X., Liu, C., Du, R., Tan, D.K.P., Lu, J. and Shen, Y., 2022. A survey on fundamental limits of integrated sensing and communication. IEEE Communications Surveys & Tutorials, 24(2), pp.994-1034.

[22] 	Dwivedi, S., Shreevastav, R., Munier, F., Nygren, J., Siomina, I., Lyazidi, Y., Shrestha, D., Lindmark, G., Ernström, P., Stare, E. and Razavi, S.M., 2021. Positioning in 5G networks. IEEE Communications Magazine, 59(11), pp.38-44.

[23]	Suhr, C. "S.L.A.M. and Optical Tracking for XR", https://medium.com/desn325-emergentdesign/s-l-a-m-and-optical-tracking-for-xr-cfabb7dd536f <accessed 1.9.22>

[24]	Kim, H., Granstrom, K., Svensson, L., Kim, S. and Wymeersch, H., 2022. PMBM-based SLAM Filters in 5G mmWave Vehicular Networks. IEEE Transactions on Vehicular Technology.

[25]	5GAA " C-V2X Use Cases Volume II: Examples and Service Level Requirements", 5G Automobile Assocaition White Paper,  https://5gaa.org/wp-content/uploads/2020/10/5GAA_White-Paper_C-V2X-Use-Cases-Volume-II.pdf <accessed 02.09.22>

[26]	A. Ebrahimzadeh, M. Maier and R. H. Glitho, "Trace-Driven Haptic Traffic Characterization for Tactile Internet Performance Evaluation," 2021 International Conference on Engineering and Emerging Technologies (ICEET), 2021, pp. 1-6.

[27]	Lee, L.-H., Braud, T., Zhou, P., Wang, L., Xu, D., Lin,  Z., Kumar, A., Bermejo, C., and Hui, P. All One Needs to Know about Metaverse: A Complete Survey on Technological Singularity, Virtual Ecosystem, and Research Agenda, 2021.

[28]	Halbhuber, David & Henze, Niels & Schwind, Valentin. (2021). Increasing Player Performance and Game Experience in High Latency Systems. Proceedings of the ACM on Human-Computer Interaction. 5. 1-20. 10.1145/3474710.

[29] 	Holland, O., Steinbach, E., Prasad, R.V., Liu, Q., Dawy, Z., Aijaz, A., Pappas, N., Chandra, K., Rao, V.S., Oteafy, S. and Eid, M., 2019. The IEEE 1918.1 “tactile internet” standards working group and its standards. Proceedings of the IEEE, 107(2), pp.256-279.

[30] 	Skalidis, I., Muller, O. and Fournier, S., 2022. CardioVerse: The Cardiovascular Medicine in the Era of Metaverse. Trends in Cardiovascular Medicine.

[31] 	Koo, H., 2021. Training in lung cancer surgery through the metaverse, including extended reality, in the smart operating room of Seoul National University Bundang Hospital, Korea. Journal of educational evaluation for health professions, 18.

[32] 	Mozumder, M.A.I., Sheeraz, M.M., Athar, A., Aich, S. and Kim, H.C., 2022, February. Overview: technology roadmap of the future trend of metaverse based on IoT, blockchain, AI technique, and medical domain metaverse activity. In 2022 24th International Conference on Advanced Communication Technology (ICACT) (pp. 256-261). IEEE.

[33] 	Ning, H., Wang, H., Lin, Y., Wang, W., Dhelim, S., Farha, F., Ding, J. and Daneshmand, M., 2021. A Survey on Metaverse: the State-of-the-art, Technologies, Applications, and Challenges. arXiv preprint arXiv:2111.09673.

[34] 	https://www.xrtoday.com/virtual-reality/ukrainian-doctors-perform-first-vr-surgery/

[35] 	https://www.cryptotimes.io/surgeon-performed-surgery-remotely-through-metaverse/

[36]	 Senk, S., Ulbricht, M., Tsokalo, I., Rischke, J., Li, S.C., Speidel, S., Nguyen, G.T., Seeling, P. and Fitzek, F.H., 2022. Healing Hands: The Tactile Internet in Future Tele-Healthcare. Sensors, 22(4), p.1404.

[37] 	Chen, D. and Zhang, R., 2022. Exploring Research Trends of Emerging Technologies in Health Metaverse: A Bibliometric Analysis. Available at SSRN 3998068.

[38] 	https://www.yankodesign.com/2022/05/15/the-metaverse-has-the-power-to-improve-healthcare-and-it-has-already-begun/

[39] 	3GPP TR 23.501 release-17 - https://www.3gpp.org/ftp/Specs/archive/23_series/23.501/23501-h50.zip

[40] 	3GPP TR 38.300 release-17 - https://www.3gpp.org/ftp/Specs/archive/38_series/38.300/38300-h10.zip

[41] 	3GPP TR 22.826 release-17 - https://www.3gpp.org/ftp/Specs/archive/22_series/22.826/22826-h20.zip

[42] 	Tataria, H., Shafi, M., Molisch, A.F., Dohler, M., Sjöland, H. and Tufvesson, F., 2021. 6G wireless systems: Vision, requirements, challenges, insights, and opportunities. Proceedings of the IEEE, 109(7), pp.1166-1199.

[43]	3GPP TS 23.682, "Architecture enhancements to facilitate communications with packet data networks and applications".

[44]	3GPP TS 23.273, "5G System (5GS) Location Services (LCS); Stage 2".

[45]	"MPEG-4 Face and Body Animation", https://visagetechnologies.com/mpeg-4-face-and-body-animation/ (accessed 2.11.22).

[46]	3GPP TS 22.105, "Services and service capabilities".[47]	Virtual humans: https://en.wikipedia.org/wiki/Virtual_humans

[48]	Kwang Soon Kim, et al., "Ultrareliable and Low-Latency Communication Techniques for Tactile Internet Services", PROCEEDINGS OF THE IEEE, Vol. 107, No. 2, February 2019

[49]	I.-J. Hirsh and C. E. J. Sherrick, “Perceived order in different sense modalities,” Journal of Experimental Psychology, vol. 62, no. 5, pp. 423–432, 1961. [50]                       VESA Compression Codecs - vesa.org/vesa-display-compression-codecs

[51]	3GPP TR 23.700-80: "Study on 5G System Support for AI/ML-based Services".

[52]	EU data protection rules | European Commission (europa.eu) (https://ec.europa.eu/info/law/law-topic/data-protection/eu-data-protection-rules_en)

[53] 	California Consumer Privacy Act (CCPA) | State of California - Department of Justice - Office of the Attorney General (https://oag.ca.gov/privacy/ccpa)

[A]	3GPP TS 22.226: "Global Text Telephony"

[B]	ITU-T SG16 "ITU-T SG 16 Work on Accessibility - How ITU is Pioneering Telecom Accessibility for All", https://www.itu.int/en/ITU-T/studygroups/com16/accessibility/Pages/telecom.aspx, accessed 30.1.23.

[C]	https://www.ijert.org/sign-language-to-text-and-speech-translation-in-real-time-using-convolutional-neural-network

SECOND CHANGE

## 5.W	IMS-based 3D Avatar Call Support for Accessibility Use Case

### 5.W.1	Description

3GPP has long standardized functionality to support availability of communication for users with disabilities. Global Text Telephony [A] provides a character by character text conversation to enable Global Text for those who rely on it, even for emergency service access. With the advent of speech recognition, it is possible to encode audio calls into text and text can be converted to speech. This kind of conversion goes a long way to achieve ITU-T SG16's Total Conversation vision: "Total Conversation is an ITU-T defined concept that encompasses voice telephony, video telephony and text telephony. The idea is that it gives everyone the chance to communicate with one another regardless of whether they are hearing, hearing impaired or deaf." [B]

There are a number of additional valuable scenarios that could be enabled through the use of IMS 3D Avatar Call, as described in 5.11.

Figure 5.W.1-1: Accessibility Scenarios for IMS 3D Avatar Call

In scenario 1, above, a hearing impaired user communicates with another using signage. Each user's gestures as well as facial expression and movements are captured by sensors (e.g. these sensors could be part of the terminal equipment) and transformed into an avatar encoding before transmission to the conversational partner. The experience of both parties is natural, and the user experience should resemble that of a video call, albeit with 'idealized lighting and contrast' due to the animation.

In scenario 2, one person speaks while the other signs. The signage of the person on the right is captured as described in scenario 1, but in addition it is analyzed. Research results indicate the likelihood that it will soon be possible to reliably use AI-based programs to capture signage to generate text. [C] It is clear that text to speech is possible. Thus for the user on the left, they can see the person on the right signing and receive an audio rendering of the text they generate.

The speech of the user on the left can be converted to text by means of voice recognition. There is extensive research into text to signage [D] as well as some commercial products already available in this area. It is therefore possible for the user on the right to both see the user on the left speaking, as well as an avatar providing signage, or even an avatar rendering of the user on the left performing signage.

In scenario 3, one of the users may not be able to use IMS 3D Avatar call, e.g. they use terminal equipment without this support. In this case, the user on the left enters text and this is rendered as an avatar signing for the user on the right, if this is desired. The user on the right can express herself using signing, which is captured as text (as described for scenario 2) and sent as GTT text media to the user on the left.

One element is currently not possible with text conversion to other media, be it speech or generated avatar media of signage: the timing and emotions expressed in the communication. As part of scenario 3, we consider the possibility of capturing specific text conventions to indicate speech pauses or emotions.

An additional consideration is that the display equipment used to present the IMS 3D Avatar call may either be a UE itself or a separate monitor that the UE is able to use or is available through the display connected another UE, as by Inter-Device Connectivity (a feature of IMS.)

Finally, the possibility to support a communicating user that is 'software generated' is supported well by this use case. In this case, a variant of scenario 2 could be used where the user on the left is in fact an automated customer support centre representative. The computer generated speech is rendered as signage to the user on the right, and the signage of the user on the right is rendered as speech to the software-based customer service party.

### 5.W.2	Pre-conditions

Both communicating parties AeCha, Bharathi and Carlos have a mobile subscription with PLMNs Absolute Telecom (PLMN A) and Benefit Wireless (PLMN B) and Celestial Cellular (PLMN C).

Both Arndt and Berndt have UEs that support sensors capable of capturing their facial expressions and movements as well as gestures sufficiently for this use case. They also are able to set their terminal equipment down so they have free hands (either on a tripod or table, etc.) Carl has a UE that is only capable of voice calls.

### 5.W.3	Service Flows

Scenario 1: IMS 3D Avatar Call between two callers employing accessibility features and translation

AeCha calls Bharati. AeCha and Bharati use IMS 3D Avatar Call to communicate through sign language.

AeCha signs using Korean sign language. Bharathi signs using Hindi sign language. There are many forms of sign language in the world that are not mutually comprehensible internationally, and we assume that AeCha and Bharathi would not be able to understand each other's signage directly.

There are a set of services available in the communication channel between AeCha and Bharathi that enable the two of them can communicate.

Figure 5.W.3-1: IMS Avatar Call with Services for Signage to Text and Text Translation

AeCha's signing is captured by the sensors and encoded as Avatar Codec, capturing her use of Korean sign language. In the network, the signage is transcoded into Korean text. The Korean text can be translated into English text. This text can then be used to generate Indian Sign Language (shown in the transcoding function in PLMN B).

It is acknowledged that the translation services included in this use case are not exact, however the possibility to communicate directly using signage, and even with the avatar of the corresponding party could be quite valuable.

The avatars seen by AeCha and Bharathi are a representation of the other party, as sufficient information is exchanged by the 5G system to enable the transcoders that produce the avatar codec in PLMN A and PLMN B to do so.

Scenario 2: IMS 3D Avatar Call and Audio between two callers, with accessibility enhancements

Figure 5.W.3-2: IMS Avatar Call with Services for Signage to Text and Text to Voice

Carlos speaks. His speech is recognized (in a transcoder in PLMN C) and encoded as English Text. This text is transported as media. The text is transcoded (in PLMN B) to Indian sign language encoded in an avatar codec. Bharathi views an avatar signing, using Indian sign language to represent Carlos' speech. The avatar is not a representation of Carlos as there are no sensors capturing Carlos, unless there is a means to configure the transcoder in PLMN B with the avatar information corresponding to Carlos' appearance. This is out of scope of this use case.

Bharathi signs, and this is captured in an Avatar codec that identifies her gestures, facial expression and movements. This is converted to English text in a transcoder function in PLMN B. The English text is sent as media to PLMN C, where a transcoder converts the text to speech. This speech is transported as audio media to Carlos, who hears a synthesized voice expressing the communication that Bharathi signed.

Scenario 3: IMS 3D Avatar Call and GTT between two callers, with accessibility enhancements

Figure 5.W.3-3: IMS Avatar Call with Services for Signage to Text and Text to Voice

In this scenario, Carlos uses a GTT terminal to supply GTT media uplink. This media is converted in a transcoder to avatar codec representing signing in Indian sign language to Bharathi. The avatar is not a representation of Carlos as there are no sensors capturing Carlos, unless there is a means to configure the transcoder in PLMN B with the avatar information corresponding to Carlos' appearance. This is out of scope of this use case.

In order to provide the possibility of richer communication as a result of text expression, the text converter recognizes conventions that can indicate a pause or a specific emotion or emphasis (variation in volume or emphatic gestures) in the text itself. For example, "..." could indicate a pause, "*louder text*" could indicate emphasis, and a smiley could result in a smile. This functionality would require that the user know the associated conventions and be able to employ them by means of the MMI.

Bharathi signs, and this is captured in an Avatar codec that identifies her gestures, facial expression and movements. This is converted to GTT text in a transcoder function in PLMN B. The GTT media is delivered to Carlos, who reads text expressing the communication that Bharathi signed.

### 5.W.4	Post-conditions

In each of three scenarios one or both parties are able to sign and see signage in their native sign language in order to communicate with the other party. The possibility to interwork with legacy GTT terminals and legacy audio terminals is also supported.

### 5.W.5	Existing feature partially or fully covering use case functionality

The 5G system supports IMS which is able to handle diverse media, establish calls and support media codec transcoding services.

The 5G system supports GTT.

### 5.W.6	Potential New Requirements

[P.R.-5.W.6-1] The 5G system shall support the encoding of sensor data capturing the facial expression and movement and gestures of a person, in a standard form, such that this avatar encoding can be transcoded into text.

[P.R.-5.W.6-2] The 5G system shall support a set of transcoders to facilitate accessibility including but not limited to:

- text to speech

- GTT to speech

- speech to text

- speech to GTT

- avatar encoding of sign language to text

- avatar encoding of sign language to GTT

- text to avatar encoding sign language

- GTT to avatar encoding sign language

[P.R.-5.W.6-3] The 5G system shall support the generation of avatar encoding on the basis of text such that the characteristics of the avatar encoding (i.e. determining the avatar encoding's appearance) can be determined by the calling party.

[P.R.-5.W.6-4] The 5G system shall support transcoding of avatar encoding to and from other media (e.g. text, binary metadata, etc.)

[P.R-5.W.6-5] The 5G system shall support the avatar transcoding functionality to control the appearance of the avatar based on the preferences of its associated user.  Examples of the controlled appearance could be for the avatar to express behavior, movement, affect, emotions, etc.

[P.R.-5.W.6-6] The 5G system shall be able to collect charging information for transcoding services associated with IMS-based avatar call.

END OF CHANGES