3GPP TSG SA WG 1 Meeting #101 	S1-230427

Athens, Greece, 20 – 24 February 2023	(revision of S1-230091)

Source:	CMCC, Huawei, Orange

pCR Title:	Pseudo-CR on updates to clause 5.16

Draft Spec:	3GPP TR 22.856

Agenda item:	7.3

Document for:	Approval

Contact:	Yue Hu (huyueyjy@chinamobile.com); Alice Li (alice.li1@huawei.com)

Abstract: This document provides a TP to TR 22.856 to update clause 5.16 on the use case of virtual store in a metaverse marketplace.

Revision includes the changes to address the comments received.

1. Introduction

The use case of “virtual store in a metaverse marketplace” has been agreed and captured in TR 22.856, which includes the use case description and some potential requirements. An editor’s note is also captured to indicate the detail of avatar call need further clarification.

2. Reason for Change

To address the EN, the following updates are included in the pCR:

- provide information on the 3D avatar communications using IMS;

- new potential service requirements are proposed.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.856 v0.3.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.228: "Service requirements for the Internet Protocol (IP) Multimedia core network Subsystem (IMS)".

[3]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services".

[4]	3GPP TS 22.101: "Service principles".

[5]	3GPP TS 22.261: "Service requirements for the 5G system".

[6] 	Talaba D, Horvath I, Lee K H, "Special issue of Computer-Aided Design on virtual and augmented reality technologies in product design," Computer-Aided Design 42:361 - 363.

[7] 	Wang X, Tsai J J-H, "Collaborative Design in Virtual Environments," In: ISCA 48:1-2.

[8] 	Benford S, Greenhalgh C, Rodden T, "Collaborative virtual environments." Communications of the ACM 44: 79–85.

[9]	M. Eid, J. Cha, and A. El Saddik, "Admux: An adaptive multiplexer for haptic-audio-visual data communication", IEEE Tran. Instrument. and Measurement, vol. 60, pp. 21–31, Jan 2011.

[10]	K. Iwata, Y. Ishibashi, N. Fukushima, and S. Sugawara, "QoE assessment in haptic media, sound, and video transmission: Effect of playout buffering control", Comput. Entertain., vol. 8, pp. 12:1–12:14, Dec 2010.

[11]	N. Suzuki and S. Katsura, "Evaluation of QoS in haptic communication based on bilateral control", in IEEE Int. Conf. on Mechatronics (ICM), Feb 2013, pp. 886–891.

[12]	E. Isomura, S. Tasaka, and T. Nunome, "A multidimensional QoE monitoring system for audiovisual and haptic interactive IP communications", in IEEE Consumer Communications and Networking Conference (CCNC), Jan 2013, pp. 196–202.

[13]	A. Hamam and A. El Saddik, "Toward a mathematical model for quality of experience evaluation of haptic applications", IEEE Tran. Instrument. and Measurement, vol. 62, pp. 3315–3322, Dec 2013.

[14]	O. Holland et al., "The IEEE 1918.1 "Tactile Internet" Standards Working Group and its Standards," Proceedings of the IEEE, vol. 107, no. 2, Feb. 2019.

[15]	Lee, Donghwan et. al., “Large-scale Localization Datasets in Crowded Indoor Spaces,” NAVER LABS, NAVER LABS Europe, CVPR 2021.

[16]	3GPP TR 22.837, " Study on Integrated Sensing and Communication."

[17]	Alriksson, F., Kang, D.H., Phillips, C., Pradas, J.L. and Zaidi, A., 2021. XR and 5G: Extended reality at scale with time-critical communication. Ericsson Technology Review.

[18]	3GPP TR 26.928, "Extended Reality (XR) in 5G."

[19] 	Ge, Y., Wen, F., Kim, H., Zhu, M., Jiang, F., Kim, S., Svensson, L. and Wymeersch, H., 2020. 5G SLAM using the clustering and assignment approach with diffuse multipath. Sensors, 20(16), p.4656.

[20] 	Kim, H., Granström, K., Gao, L., Battistelli, G., Kim, S. and Wymeersch, H., 2020. 5G mmWave cooperative positioning and mapping using multi-model PHD filter and map fusion. IEEE Transactions on Wireless Communications, 19(6), pp.3782-3795.

[21] 	Liu, A., Huang, Z., Li, M., Wan, Y., Li, W., Han, T.X., Liu, C., Du, R., Tan, D.K.P., Lu, J. and Shen, Y., 2022. A survey on fundamental limits of integrated sensing and communication. IEEE Communications Surveys & Tutorials, 24(2), pp.994-1034.

[22] 	Dwivedi, S., Shreevastav, R., Munier, F., Nygren, J., Siomina, I., Lyazidi, Y., Shrestha, D., Lindmark, G., Ernström, P., Stare, E. and Razavi, S.M., 2021. Positioning in 5G networks. IEEE Communications Magazine, 59(11), pp.38-44.

[23]	Suhr, C. "S.L.A.M. and Optical Tracking for XR", https://medium.com/desn325-emergentdesign/s-l-a-m-and-optical-tracking-for-xr-cfabb7dd536f <accessed 1.9.22>

[24]	Kim, H., Granstrom, K., Svensson, L., Kim, S. and Wymeersch, H., 2022. PMBM-based SLAM Filters in 5G mmWave Vehicular Networks. IEEE Transactions on Vehicular Technology.

[25]	5GAA " C-V2X Use Cases Volume II: Examples and Service Level Requirements", 5G Automobile Assocaition White Paper,  https://5gaa.org/wp-content/uploads/2020/10/5GAA_White-Paper_C-V2X-Use-Cases-Volume-II.pdf <accessed 02.09.22>

[26]	A. Ebrahimzadeh, M. Maier and R. H. Glitho, "Trace-Driven Haptic Traffic Characterization for Tactile Internet Performance Evaluation," 2021 International Conference on Engineering and Emerging Technologies (ICEET), 2021, pp. 1-6.

[27]	Lee, L.-H., Braud, T., Zhou, P., Wang, L., Xu, D., Lin,  Z., Kumar, A., Bermejo, C., and Hui, P. All One Needs to Know about Metaverse: A Complete Survey on Technological Singularity, Virtual Ecosystem, and Research Agenda, 2021.

[28]	Halbhuber, David & Henze, Niels & Schwind, Valentin. (2021). Increasing Player Performance and Game Experience in High Latency Systems. Proceedings of the ACM on Human-Computer Interaction. 5. 1-20. 10.1145/3474710.

[29] 	Holland, O., Steinbach, E., Prasad, R.V., Liu, Q., Dawy, Z., Aijaz, A., Pappas, N., Chandra, K., Rao, V.S., Oteafy, S. and Eid, M., 2019. The IEEE 1918.1 “tactile internet” standards working group and its standards. Proceedings of the IEEE, 107(2), pp.256-279.

[30] 	Skalidis, I., Muller, O. and Fournier, S., 2022. CardioVerse: The Cardiovascular Medicine in the Era of Metaverse. Trends in Cardiovascular Medicine.

[31] 	Koo, H., 2021. Training in lung cancer surgery through the metaverse, including extended reality, in the smart operating room of Seoul National University Bundang Hospital, Korea. Journal of educational evaluation for health professions, 18.

[32] 	Mozumder, M.A.I., Sheeraz, M.M., Athar, A., Aich, S. and Kim, H.C., 2022, February. Overview: technology roadmap of the future trend of metaverse based on IoT, blockchain, AI technique, and medical domain metaverse activity. In 2022 24th International Conference on Advanced Communication Technology (ICACT) (pp. 256-261). IEEE.

[33] 	Ning, H., Wang, H., Lin, Y., Wang, W., Dhelim, S., Farha, F., Ding, J. and Daneshmand, M., 2021. A Survey on Metaverse: the State-of-the-art, Technologies, Applications, and Challenges. arXiv preprint arXiv:2111.09673.

[34] 	https://www.xrtoday.com/virtual-reality/ukrainian-doctors-perform-first-vr-surgery/

[35] 	https://www.cryptotimes.io/surgeon-performed-surgery-remotely-through-metaverse/

[36]	 Senk, S., Ulbricht, M., Tsokalo, I., Rischke, J., Li, S.C., Speidel, S., Nguyen, G.T., Seeling, P. and Fitzek, F.H., 2022. Healing Hands: The Tactile Internet in Future Tele-Healthcare. Sensors, 22(4), p.1404.

[37] 	Chen, D. and Zhang, R., 2022. Exploring Research Trends of Emerging Technologies in Health Metaverse: A Bibliometric Analysis. Available at SSRN 3998068.

[38] 	https://www.yankodesign.com/2022/05/15/the-metaverse-has-the-power-to-improve-healthcare-and-it-has-already-begun/

[39] 	3GPP TR 23.501 release-17 - https://www.3gpp.org/ftp/Specs/archive/23_series/23.501/23501-h50.zip

[40] 	3GPP TR 38.300 release-17 - https://www.3gpp.org/ftp/Specs/archive/38_series/38.300/38300-h10.zip

[41] 	3GPP TR 22.826 release-17 - https://www.3gpp.org/ftp/Specs/archive/22_series/22.826/22826-h20.zip

[42] 	Tataria, H., Shafi, M., Molisch, A.F., Dohler, M., Sjöland, H. and Tufvesson, F., 2021. 6G wireless systems: Vision, requirements, challenges, insights, and opportunities. Proceedings of the IEEE, 109(7), pp.1166-1199.

[43]	3GPP TS 23.682, "Architecture enhancements to facilitate communications with packet data networks and applications".

[44]	3GPP TS 23.273, "5G System (5GS) Location Services (LCS); Stage 2".

[45]	"MPEG-4 Face and Body Animation", https://visagetechnologies.com/mpeg-4-face-and-body-animation/ (accessed 2.11.22).

[46]	3GPP TS 22.105, "Services and service capabilities".[47]	Virtual humans: https://en.wikipedia.org/wiki/Virtual_humans

[48]	Kwang Soon Kim, et al., "Ultrareliable and Low-Latency Communication Techniques for Tactile Internet Services", PROCEEDINGS OF THE IEEE, Vol. 107, No. 2, February 2019

[49]	I.-J. Hirsh and C. E. J. Sherrick, “Perceived order in different sense modalities,” Journal of Experimental Psychology, vol. 62, no. 5, pp. 423–432, 1961. [50]                       VESA Compression Codecs - vesa.org/vesa-display-compression-codecs

[51]	3GPP TR 23.700-80: "Study on 5G System Support for AI/ML-based Services".

[52]	EU data protection rules | European Commission (europa.eu) (https://ec.europa.eu/info/law/law-topic/data-protection/eu-data-protection-rules_en)

[53] 	California Consumer Privacy Act (CCPA) | State of California - Department of Justice - Office of the Attorney General (https://oag.ca.gov/privacy/ccpa)

[54]	B. Egger, W. A. P. Smith, A. Tewari, S. Wuhrer, M. Zollhoefer, T. Beeler, F. Bernard, T. Bolkart, A. Kortylewski, S. Romdhani, C. Theobalt, V. Blanz, and T. Vetter. "3D Morphable Face Models—Past, Present, and Future". ACM Trans. Graph. 39, 5, Article 157 (Jun 2020)

[55]	M. Zollhöfer, J. Thies, P. Garrido, D. Bradley, T. Beeler, P. Pérez, M. Stamminger, M. Nießner, and C. Theobalt. "State of the Art on Monocular 3D Face Reconstruction, Tracking, and Applications". Computer Graphics Forum (2018).

* * * Second Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AI	Artificial Intelligent

FACS	Facial Action Coding System

AI	Artificial Intelligent

* * * Third Change * * * *

## 5.16	Use case of virtual store in a mobile metaverse marketplace

### 5.16.1	Description

5G technologies especially the XR communication technologies make it possible to run business online (with or without physical offices/stores) to offer various services. Examples include fashion store online, drop-shipping business, virtual real estate agency, virtual assistants, teaching an online course, and online fitness training. Running a business online is particularly attractive for start-ups and small even medium sized businesses. For end consumers, visiting a market is a real feast for the senses. It is a multisensory experience that combines tradition with human contact and involves a series of decisions about which products to choose for the shopping basket.

Mobile metaverse services are expected to help to transfer a world so rich in sensations to the virtual sphere, which offer rich XR enabled multimedia communication services together with security mechanisms for data protection, user identity/profile management as well as digital asset management and protection. IMS based avatar calls are among these new features/services, where AI avatar can be used to help facilitate social interactions. An AI avatar [54] [55] is a digital character powered by artificial intelligence, which lives in a virtual setting, like a game, social network or online world. More frequently avatars are designed as human-like bots that can be controlled by real human using AI technologies and can easily engage with real humans and maintain relationship - to varying degrees - with them.

### 5.16.2	Pre-conditions

Magnificent Muggles, a niche fashion company, has set up virtual stores in a metaverse marketplace (a mobile metaverse service) provided by GreenMobile. The corresponding 5G communication subscriptions provided by GreenMobile include the XR enabled multimedia communication services (including IMS based avatar calls), which enable the efficient near-real life interaction between the virtual shop assistant and the online shoppers, an immersive location agnostic service experience. As a fashion retailer, Magnificent Muggles expect to provide similar if not better experience to their online shoppers in their virtual stores. To enable this, they have their shop assistants equipped with XR devices to interact with online buyers. In their virtual stores, the end consumers can “see” the products as if they were buying them face-to-face and they can interact live with the people selling them.

As part of the service level agreement, GreenMobile provides storage and communication services to Magnificent Muggles:

- 	to store the digital representations (e.g. avatar) for the virtual shop assistance

- 	to assist the authentication of their employees to use the digital representations (e.g. avatar) in the IMS based XR communication when assisting online buyers

- 	to render the digital representations (e.g. avatar) based on the voice, facial expression or body motion of a human user

The service flows below illustrate how the virtual shop assistant and an online shopper interact with each other using services provided by 5G system.

### 5.16.3	Service Flows

0.1	Magnificent Muggles register with GreenMobile (the provider of 5G communication services and the mobile metaverse services) the digital representations (e.g. avatar) to be used by the virtual shop assistants. Subject to regulatory requirements, the digital representations are then certified to be legally used in a certain region. The digital representations are stored at GreenMobile’s edge sites.

0.2	Mrs. Dursley, an end consumer, registers and stores her digital representations with GreenMobile to be used in the metaverse marketplace. Subject to regulatory requirements, the digital representations are then certified to be legally used in a certain region.

1. 	Humphrey is the shop assistant in the Magnificent Muggles virtual store. Due to the ongoing pandemic he works from home. As part of the security requirements, he needs to be verified as an employee of Magnificent Muggles before having an XR communication with online buyers.

2.	Mrs. Dursley decides to pay a visit at the virtual store of Magnificent Muggles to check out the new clothing. Humphrey, the shop assistant, greets her and offers to set up an XR communication to show her around the new lines. Mrs. Dursley thinks a good idea and agrees. Having completed the authentication of the participants, the multimedia communication session is set up between Mrs. Dursley and Humphrey as well as the associated XR devices.

3.1	For this session Humphrey uses one of the Magnificent Muggles registered digital representations. An example of the functional flow of the avatar call from Humphrey to Mrs. Dursley is illustrated in figure 5.16.3-1. Note that in this option the required 3D avatar model is made available at the network.

1)  The body motion or facial expressions of Humphrey are captured at UE1, which is transmitted to the network.

2)  With the received information about the user’s motion or facial expressions, the network renders the avatar (the dynamic 3D object).

3)  The media data (converted from the 3D object) is then transmitted to the recipient, UE2 of Mrs. Dursley.

4)  The video image (with the rendered avatar) is displayed at the screen of Mrs. Dursley’s terminal.

Figure 5.16.3-1: An example of avatar call functional flow (image rendering at the network)

NOTE:     it is also possible for UE1 to send video stream to the network, with which the network can  render the avatar (the dynamic 3D object). This is particularly useful for UEs with limited capability.

3.2	Mrs. Dursley downloads, from the network, one of her registered digital representations to be used for the XR communication. An example of the functional flow from Mrs. Dursley to Humphrey is illustrated in figure 5.16.3-2. Note that in this option the required 3D avatar model needs to be made available at all the recipients (UE1 in this option).

1)  The body motion or facial expressions of Mrs. Dursley are captured at UE2, which is transmitted to the recipient via the network.

2)  With the received information about Mrs. Dursley’s motion or facial expressions, UE1 renders the avatar (the dynamic 3D object). The video image (with the rendered avatar) is displayed at the screen of Humphrey’s terminal.

Figure 5.16.3-2: An example of avatar call functional flow (image rendering at the receiving side)

NOTE:     this use case shows the example of rendering of the video-based avatar media to 3D avatar. It is also possible to render other media to 3D avatar.

### 5.16.4	Post-conditions

The 5G system with a combination of various technologies offers the users an immersive shopping experience equivalent to a face-to-face purchase in a crowded market.

### 5.16.5	Existing features partly or fully covering the use case functionality

The service requirements on IMS Multimedia Telephony Service and supplementary services are well documented in TS 22.173 since Rel-7, many of which have been implemented in stage-2 and stage-3 WGs. The requirements on 5G IMS Multimedia Telephony Service are captured in TS 22.261 [5] clause 6.39 as a result of Rel-18 work.

On the user identity related aspects, there are several features defined including:

-	Support of Multi-device and Multi-Identity in IMS MMTEL service is captured in TS 22.173 clause 4.6 [3]:

The support of multiple devices is inherent in IMS. In addition, a service provider may allow a user to use any public user identities for its outgoing and incoming calls. The added identities can but do not have to belong to the served user. Identities may be part of different subscriptions and different operators.

-	TS 22.101 [4] has specified in clause 26a a set of service requirements on User Identity:

Identifying distinguished user identities of the user (provided by some external party or by the operator) in the operator network enables an operator to provide an enhanced user experience and optimized performance as well as to offer services to devices that are not part of a 3GPP network. The user to be identified could be an individual human user, using a UE with a certain subscription, or an application running on or connecting via a UE, or a device (“thing”) behind a gateway UE.

Network settings can be adapted and services offered to users according to their needs, independent of the subscription that is used to establish the connection. By acting as an identity provider, the operator can take additional information from the network into account to provide a higher level of security for the authentication of a user.

The 3GPP System shall support to authenticate a User Identity to a service with a User Identifier.

-	Clause 8 of TS 22.261 [5] specifies the security related requirements covering aspects such as authentication and authorization, identity management, and data security and privacy.

The functional requirement and performance KPIs in support of XR applications are mainly captured in TS 22.261 [5]:

-	clause 7.6.1 AR/VR;

-	clause 6.43 Tactile and multi-modal communication service;

-	clause 7.11 KPIs for tactile and multi-modal communication service

In support of metaverse services, additional consideration need to be given on the following aspects:

- 	securely register and store the digital representations (e.g. avatar) for the users. The user could be an individual human user using a UE with a certain subscription, or an application running on or connecting via a UE, or a device behind a gateway UE. The user could also be a third party, which is typically an enterprise customer having service level agreement with the operator and interacting with the 3GPP network via an application server.

- 	assist the authorization of the use of third party’s digital assets (e.g. the digital representations (e.g. avatar)) in the XR communication. The third party is also involved in the procedure to certify the user identity (e.g. an employee of the company).

- 	when required render the digital representations (e.g. avatar) based on the voice, facial expression or gesture in the live communication video

### 5.16.6	Potential New Requirements needed to support the use case

[PR 5.16.6.2-1] Subject to user’s consent, the 5G system shall support mechanisms to securely register, store and update the digital assets for a user.

NOTE 1: 	The user could be a human user using a UE with a certain subscription, or an application running on or connecting via a UE, or a device behind a gateway UE. The user could also be a third party, which is typically an enterprise customer having service level agreement with the operator and interacting with the 3GPP network via an application server.

[PR 5.16.6.2-2] Subject to regulatory requirements and operator’s policy, the 5G system shall provide suitable and secure means to allow trusted third-party to authorize the use of the digital assets (that belong to the third party enterprise customer) by a user.

NOTE 2: 	In a typical example the user is an employee of the third party enterprise customer.

[PR 5.16.6.2-3] The 5G system shall be able to collect charging information per UE for managing (e.g. register, store and update) the digital assets for an end user (e.g. typically a human user with a certain subscription).

[PR 5.16.6.2-4] The 5G system shall be able to collect charging information per application for managing (e.g. register, store and update) the digital assets for the third party (e.g. typically an enterprise customer having service level agreement with the operator).

[PR 5.11.6.2-5] Subject to user’s consent, the 5G system shall support the real-time transmission of the body movement information (e.g. body motion or facial expressions) of a human user in order to ensure immersive voice/audio and visual experience.

NOTE 3: 	The body movement information (e.g. body motion or facial expressions) of a human user is used for rendering of the avatar of this user.

[PR 5.11.6.2-6] Subject to user’s consent and operator’s policy, the IMS shall support the capabilities of rendering the avatar based on the body movement information (e.g. body motion or facial expression) of a human user.