3GPP TSG-SA WG1 Meeting #101 	S1-230448

Athens, Greece, 20 - 24 February 2023	(revision of S1-230187)

Title:	Consolidation requirements on FS_EnergyServ

Agenda Item:	7.10

Source:	China Mobile, Samsung

Contact:	Xiaonan Shi, shixiaonan@chinamobile.com

Abstract: This pCR provides consolidation requirements on FS_EnergyServ.

6	Consolidated Potential Requirements

Editor's Note: The following consolidated potential requirements will be revised and updated as discussion going on to cover updates in existing use cases and potential requirements in new use cases.

6.1	Energy Utililization as a Service Criteria

This subclause contains the requirements related to Energy Utilization as a Performance Criteria and supporting energy credit limit for specific service.

Table 6.1-1 – Energy Utililization as a Service Criteria Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 1.1-1 | Subject to operator’s policy, the 5G system shall support subscription policies that define a maximum energy utilization rate for services without QoS criteria (also termed "best effort" services.) | PR 5.1.6-1 Subject to operator’s policy, the 5G system shall support subscription policies that define a maximum energy utilization rate for services without QoS criteria (also termed "best effort" services.) |  |
| CPR 1.1-2 | Subject to operator’s policy, the 5G system shall support enforcement of subscription policies that define a maximum energy utilization rate for services without associated QoS criteria (also termed "best effort" services.) | PR 5.1.6-2 Subject to operator’s policy, the 5G system shall support enforcement of subscription policies that define a maximum energy utilization rate for services without associated QoS criteria (also termed "best effort" services.) |  |
| CPR 1.1-3 | Subject to operator’s policy, the 5G system shall support subscription policies that define a maximum energy credit limit for services. | P.R 5.5.6-1 Subject to operator’s policy, the 5G system shall support subscription policies that define a maximum energy credit limit for services. |  |
| CPR 1.1-4 | Subject to operator’s policy, the 5G system shall support subscription policies that define an energy charging rate for services. | P.R 5.5.6-2 Subject to operator’s policy, the 5G system shall support subscription policies that define an energy charging rate for services. |  |
| CPR 1.1-5 | Subject to operator’s policy, the 5G system shall support a mechanism to perform energy utilization credit limit control for services. NOTE x: The result of the credit control is not specified by this requirement. Examples include gating, increased charging rates, etc. | P.R 5.5.6-4 Subject to operator’s policy, the 5G system shall support a mechanism to perform energy utilization credit limit control for services. The result of the credit control is not specified by this requirement. Examples include gating, increased charging rates, etc. | Change the last sentence into NOTE. |

6.2	Different energy efficiency modes of network functions

This subclause contains the requirements related to different energy efficiency modes of network functions and dynamic changes.

Table 6.2-1 – Different energy efficiency modes of network functions Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.2-1 | 5G system shall support different energy efficiency modes of network functions based on pre-configured policy with authorised 3rd party. | PR 5.2.6-1 5G system shall support different energy efficiency modes of network functions based on pre-configured policy with authorised 3rd party. |  |
| CPR 6.2-2 | 5G system shall support dynamic changes of energy efficiency modes of network functions based on pre-configured policy with authorised 3rd party. NOTE 1: Pre-configured policy may include the time of changing energy efficiency modes, which energy efficiency modes map to which level of network function load, etc. | PR 5.2.6-2 5G system shall support dynamic changes of energy efficiency modes of network functions based on pre-configured policy with authorised 3rd party. NOTE 1: Pre-configured policy may include the time of changing energy efficiency modes, which energy efficiency modes map to which level of network function load, etc. |  |
| CPR 6.2-3 | 5G system shall support different charging mechanisms considering different energy efficiency modes of network functions during the usage within one service. | PR 5.2.6-3 5G system shall support different charging mechanisms considering different energy efficiency modes of network functions during the usage within one service. |  |

6.3	Information exposure related to Energy Usage

This subclause contains the requirements related to information exposure related to energy usage.

Table 6.2-1 – Information exposure related to Energy Usage Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.3-1 | Subject to operator’s policy and consent by the vertical customer, the 5G system shall be able to acquire energy usage information of the network functions serving the customer, independently from NG-RAN deployment scenarios, and expose this information to the customer and authorized third parties. | PR.5.3.6-1 Subject to operator’s policy and consent by the vertical customer, the 5G system shall be able to acquire energy usage information of the network functions serving the customer, independently from NG-RAN deployment scenarios, and expose this information to the customer and authorized third parties. |  |
| CPR 6.3-2 | Subject to operator’s policy and consent by the customer of NPN, the 5G system shall be able to acquire energy usage information of the NPN, including the shared network function(s) which is (are) serving the NPN, and expose this information to the NPN customer and authorized third parties. | PR.5.4.6-1 Subject to operator’s policy and consent by the customer of NPN, the 5G system shall be able to acquire energy usage information of the NPN, including the shared network function(s) which is (are) serving the NPN, and expose this information to the NPN customer and authorized third parties. |  |
| CPR 6.3-3 | Subject to operator’s policy, the 5G system shall support a means to expose energy utilization to authorized third parties for services, such that the energy utilization information clearly identifies the 'approaching' enforcement of an energy credit limit. | P.R 5.5.6-3 Subject to operator’s policy, the 5G system shall support a means to expose energy utilization to authorized third parties for services, such that the energy utilization information clearly identifies the 'approaching' enforcement of an energy credit limit. |  |